# 3GPP TSG-RAN WG4 Meeting # 108bis			    	   				    	  		        	     R4-2315424

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interruptions to WAN due to Sidelink Carrier Aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4#108, it was agreed to introduce interruption requirements to WAN due to Sidelink Carrier Aggregation: Issue 1-1: Interruption location of the interruption to WAN at SL carrier addition/release - Agreements - For Mode 2 SL CA operation do not specify the exact interruptions location - FFS if constraints on interruptions during WAN paging or SIB reception shall be introduced Issue 1-2: Interruption length of the interruption to WAN at SL carrier addition/release - Agreements - Interruption length is NR Slot length (ms); Interruption length (slots). 0; 1; 2. 1; 0.5; 3. 2; 0.25; 5.. |
| Summary of change: |  | Introduce RRM requirements for Interruptions to WAN due to Sidelink Carrier Aggregation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for Interruptions to WAN due to Sidelink Carrier Aggregation will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=================================Start of Change #1====================================

### 12.7.X	Interruptions to WAN due to Sidelink Carrier Aggregation

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to V2X sidelink communication. It is applicable for UE is in sidelink resource allocation mode 2.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS38.331[2].

The UE is allowed an interruption of up to the duration shown in table 12.7.X-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the V2X sidelink carrier addition/release command. This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.X-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

Editor’s Note: FFS if constraints on interruptions during WAN paging or SIB reception shall be introduced.

=================================End of Change #1====================================