3GPP TSG-RAN WG4 Meeting #108-bis                              R4-2315470

Xiamen, China, 09-13 Oct, 2023

Source:	Samsung, TELUS, Bell Mobility

Title:	TP for TR 38.899 to include HPUE CA_n13-n77

Agenda item:			4.20.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TP for TR 38.899 to include HPUE CA_n13-n77. All PC2 fallbacks have been specified.

2. Reference

[1] RP-232256_ Revised WID: Rel-18 High power UE (power class 1,5 and 2) for a single FR1 NR TDD band in UL of NR inter-band CA/DC combinations with/without NR SUL (supplementary uplink) with y bands downlink (y=2,3,4,5,6) and x bands uplink (x=1,2)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n13-n77

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n77(2A) | CA_n77(2A)8 | n13 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.x.3.1	Power class 2 case a, b

PC3 DL_n13A-n77(2A)_UL_n77(2A) is specified while IMD4/6 are missing. We submit a draft CR R4-2315469 to correct the error. For PC2 case, IMD 4/6 of dual n77 UL falls into n13 DL, which is the same as PC3. The MSD exception is defined as below.

Table 5.x.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13-n77 | n13 | 781 | 5 | 25 | 750 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 RBSTART=25 | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 RBSTART=25 | 3885 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 14: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

<End of Text Proposal>