3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315476

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.7.4.2

Source:	Apple

Title:	On PDSCH demod requirements with multi-RX in FR2

Release:	Rel-18

Document for:	Information

# 1. Introduction

In RAN4#108 demodulation requirements for multi-RX in FR2 were discussed and way forward [1] was agreed.  In this contribution we present our views on PDSCH demod requirements for multi-RX reception on the DL in FR2.

# 2. Discussion

In RAN4#108 the following agreements were made and captured in [1]:

Issue 2-1-2: PTRS Port for sDCI schemes Agreement: - Further evaluate both cases: one PTRS port across TRPs and one PTRS port for each TRP - Companies are encouraged to evaluate the performance impact and decide suitable parameters for each case. - Test applicable rules can be considered based on UE capability. Issue 2-1-3: PDSCH rate matching in mDCI transmission Agreement: - Following the features introduced by RAN1 in Rel-16, PDSCH rate matching in mDCI transmission. - PDSCH shall rate matching around PTRS from same TRP only. And no LS needed. Issue 2-1-4: Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous) Agreement: - No LS needed. Issue 2-1-5: Time/frequency offset between TRPs Agreements: - sDCI SDM: (-0.0625us, 600Hz) and (0.25us, 0Hz) - mDCI non-overlapping: (-0.0625us, 600Hz) - mDCI fully-overlapping: (-0.0625us, 600Hz) and (0.25us, 0Hz) - Interested companies can provide simulation results with (0.25us, 600Hz) for mDCI fully-overlapping scenario. Issue 2-1-6: PDSCH Configurations for demodulation requirements Agreements: Evaluate PDSCH demodulation performance with the following configurations. - Multi-DCI with fully-overlapping PDSCH, 1 layer per TRP - Multi-DCI with fully-overlapping PDSCH, 2 layers per TRP - Multi-DCI with non-overlapping PDSCH, 2 layers per TRP - Single-DCI SDM with 1+1 - Single-DCI SDM with 2+2

The baseline simulation assumptions for evaluation of different configurations for PDSCH performance with multi-RX were agreed. Our simulation results from initial evaluation are provided in our companion paper [2].

Multi-DCI transmission scheme

For multi-DCI with separate processing:

- With separate processing, non-overlapping PDSCH performance is not impacted with different levels of crosstalk.

- For overlapping PDSCH, 1 layer per TRP is not severely degraded with crosstalk.

- For overlapping PDSCH with 2 layers per TRP, performance is severely degraded.

- Performance is better with smaller TO than with larger TO for overlapping PDSCH.

- Multi-DCI with non-overlapping PDSCH is not impacted by cross talk.

- Multi-DCI with 1 layer per TRP overlapping PDSCH is not severely impacted by cross talk with separate processing.

- Multi-DCI with 2 layers per TRP overlapping PDSCH is severely impacted by cross talk with separate processing.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

For multi-DCI with joint processing:

- For overlapping PDSCH joint processing is more robust to crosstalk.

- Performance is better with smaller TO than with larger TO for overlapping PDSCH.

- For multi-DCI with 1 or 2 layers per TRP overlapping PDSCH, joint processing is more robust to cross talk.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

Single-DCI SDM transmission scheme

For joint processing:

- For overlapping PDSCH with single DCI SDM, joint processing is robust to crosstalk.

- Performance is better with smaller TO than with larger TO

- For single-DCI SDM with 1 or 2 layers per TRP, joint processing is robust to cross talk.

- Single-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

Based on the above observations from our results, we see that for multi DCI scheme it is feasible to define requirements with separate processing for with 2 layers non overlapping, and 1 layer overlapping. For single DCI SDM scheme, separate processing doesn’t give good performance and would need joint processing for any layer combination. We recommend the following for PDSCH requirements for multi-RX:

- Multi DCI Fully overlapping

  - MCS/Layers: 1+1 – MCS 17

  - UE processing: Separate processing

  - FO/TO for TRP2: 600 Hz, -0.0625us

  - Cross talk power ρ  =  -15dB

- Single DCI SDM:

  - MCS/Layers : 2+2 – MCS 13

  - UE processing: Joint processing

  - FO/TO for TRP2: 0 Hz, 0.25us

  - Cross talk power ρ  =  -12dB

If requirements are defined with joint processing, new UE capability for joint processing should be introduced.

- Define PDSCH demodulation requirements with multi-RX in FR2 for multi-DCI with the following configuration:
- PDSCH transmission: Fully overlapping
- MCS/Layers: 1+1 – MCS 17
- UE processing: Separate processing
- FO/TO for TRP2: 600 Hz, -0.0625us
- Cross talk power ρ  =  -15dB

- Define PDSCH demodulation requirements with multi-RX in FR2 for single-DCI SDM with the following configuration:
- MCS/Layers: 2+2 – MCS 13
- UE processing: Joint processing
- FO/TO for TRP2: 0 Hz, 0.25us
- Cross talk power ρ  =  -12dB

- Introduce UE capability for joint processing if requirements with joint processing are introduced.

# 3. Conclusion

In this paper, we provide our views on open issues on PDSCH demodulation requirements. Our observations and proposals are captured below:

Simulation Results

- Multi-DCI with non-overlapping PDSCH is not impacted by cross talk.

- Multi-DCI with 1 layer per TRP overlapping PDSCH is not severely impacted by cross talk with separate processing.

- Multi-DCI with 2 layers per TRP overlapping PDSCH is severely impacted by cross talk with separate processing.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

- For multi-DCI with 1 or 2 layers per TRP overlapping PDSCH, joint processing is more robust to cross talk.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

- For single-DCI SDM with 1 or 2 layers per TRP, joint processing is robust to cross talk.

- Single-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

PDSCH requirements

- Define PDSCH demodulation requirements with multi-RX in FR2 for multi-DCI with the following configuration:
- PDSCH transmission: Fully overlapping
- MCS/Layers: 1+1 – MCS 17
- UE processing: Separate processing
- FO/TO for TRP2: 600 Hz, -0.0625us
- Cross talk power ρ  =  -15dB

- Define PDSCH demodulation requirements with multi-RX in FR2 for single-DCI SDM with the following configuration:
- MCS/Layers: 2+2 – MCS 13
- UE processing: Joint processing
- FO/TO for TRP2: 0 Hz, 0.25us
- Cross talk power ρ  =  -12dB

- Introduce UE capability for joint processing if requirements with joint processing are introduced.

# Reference

- R4-2313977, “WF for Rel-18 Multi-Rx performance requirements”, Qualcomm.

- R4-2315475, “Simulation results for PDSCH with multi-RX in FR2”, Apple.