3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315478

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.18.2.1

Source:	Apple

Title:	On NWA for advanced receiver to cancel intra-user interference for MU-MIMO

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108 PDSCH demodulation requirements for MU-MIMO with advanced receiver were discussed and way forward [1] was agreed.  In this contribution we present our views on network assistance information for advanced receiver considered for mitigating inter- user interference in MU-MIMO.  RAN4 has received reply LS [2] from RAN1 on introducing DCI based assistance information for R-ML receiver with additional questions to RAN4. We provide our response to RAN1’s questions.

# 2. Discussion

## RRC based Network Assistance Information for Advanced Receivers

The RRC based NWA for advanced receivers for MU-MIMO can be categorized as

- Signaling for validity of default assumptions

- Signaling to reduce UE complexity

On the signalling to invalidate default assumptions, we have the following agreements and options from [1]:

Issue 1-2-1-2: The PRB bundling size and frequency domain resource allocation for the co-UE within each PRG of the target UE

Updated RAN4 default assumption:

  - For the target and any co-scheduled UEs in different CDM groups and with the same DMRS sequence, the target UE assumes the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE when PRG=2 or 4.

On RRC signaling details:

  - Option 1: Define RRC bit to indicate assumption information when the related RRC bit is set to true or false

  - Option 2: Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not, up to RAN2 decision on the details

  - Option 3: Define RRC signalling to indicate the above assumption information is valid or not

Not introduce separate UE capabilities to proposed 1-bit RRC signalling

Issue 1-2-1-3: The DMRS power boosting for the co-scheduled UE

On RRC signaling details:

  - Option 1: Define RRC bit to indicate assumption information when the related RRC bit is set to true or false

  - Option 2: Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not, up to RAN2 decision on the details

  - Option 3: Define RRC signalling to indicate the above assumption information is valid or not

Not introduce separate UE capabilities to proposed 1-bit RRC signalling

Issue 1-2-1-4: Time domain resource allocation information of the co-scheduled UE

On RRC signaling details:

  - Option 1: Define RRC bit to indicate assumption information when the related RRC bit is set to true or false

  - Option 2: Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not, up to RAN2 decision on the details

  - Option 3: Define RRC signalling to indicate the above assumption information is valid or not

Not introduce separate UE capabilities to proposed 1-bit RRC signalling

The open issue for the signaling for validity of default assumptions, the main open issue is on the RRC signaling. Whether we introduce a 1 bit dedicated signaling to indicate if default assumption is valid or invalid, or only introduce the bit when default assumption is invalid. Our preference is to introduce a dedicated 1-bit RRC signaling to indicate if the default assumption is valid or invalid. The behavior at the UE when the bit is absent would be ambiguous. If the UE expected to detect presence, ports, etc. of the co-UE when there is no bit would lead to additional complexity at UE side when MU-MIMO is not configured. Hence a dedicated bit to indicate if default assumptions are valid or invalid is preferred.

- There would be ambiguity in UE behavior if the RRC signaling is only present when default assumptions are invalid.

- Introduce dedicated RRC signaling to indicate if the default assumption is true or false for the following parameters:
- PRB bundling size and frequency domain resource allocation
- DMRS power boosting for the co-scheduled UE
- Time domain resource allocation information of the co-scheduled UE

On signaling to reduce UE complexity, the following additional signaling were discussed in [1]:

Issue 1-2-1-1: The DMRS port information for the co-scheduled UE

Candidate options on additional RRC based assistant signalling:

  - Option 1: No need to consider additional RRC signaling for DMRS port

  - Option 2: Introduce the assistant RRC signalling such as upper bound on number of ports of co-scheduled UEs to be detected.

The number of co-UE ports is to be detected by the UE. Depending on the DMRS type and length, the number of co-UE antenna ports to be detected ranges from 3 to 23, considering legacy DMRS and enhanced DMRS in Rel-18.

The performance of any advanced receiver to cancel inter-user interference in MU-MIMO highly depends on how many ports it can detect.

- The performance of advanced receiver to cancel inter-user interference highly depends on how many ports of co-UE it can detect.

- Depending on DMRS type UE might need to detect 3 to 23 ports in the worst case.

It would be very beneficial to the UE to indicate the maximum number of ports of the co-UE so the UE can use that information while detecting co-UE ports. If using MU-MIMO with enhanced DMRS, or DMRS configuration with length 2, this information would be very helpful in UE blind detection. We propose to introduce RRC signaling for the upper bound on number of co-scheduled UE ports

- Introduce RRC signaling for upper bound on number of co-scheduled UE ports

Issue 1-2-1-5: Frequency domain resource allocation type for the co-UE and the target UE

Candidate options

  - Option 1: UE assume the same frequency domain resource allocation type for target and co-UE, and introduce 1-bit RRC signalling to indicate if default assumption not valid

  - Option 2: Not to have this assumption

The frequency domain resource allocation type determines the granularity in which PDSCH resources are allocated, and is different from the default assumption on PRB bundling size and frequency resource allocation - the target UE assumes the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE. If the UE knows the resource allocation type of the co-UE, it would help the UE in determining the granularity it has to detect the co-UEs presence and frequency domain resource allocation.

- Knowledge of resource allocation type of co-UE helps determine the granularity to detect presence and FDRA .

Hence, we propose to introduce default assumption for resource allocation type for the co-UE to be same as target. whether the resource allocation type is same or different than the target UE should be indicated via RRC signaling.

- Introduce default assumption for resource allocation type for co-UE same as targe UE.

- Introduce dedicated RRC signaling to indicate if the default assumption is true or false

Issue 1-2-2-2: The modulation order information of the co-scheduled UE (DCI based assistant signaling)

Candidate options on wording updates to the previous approved LS to RAN1:

  - Option 1:

    - For indexes 1-5, In all the PRGs allocated to the target UE have co-scheduled UE(s), which has the same DMRS sequence as the target UE, scheduled with QPSK/16QAM/… transmission.

    - For indexes 1-6, revise ‘PRB’ to ‘PRG’

We support the proposal above to revise the wording from ‘PRB’ to ‘PRG’ for all the indices in DCI signalling for NWA on modulation order.

- Revise wording – ‘PRB’ to ‘PRG’ for index 1-6 for DCI based NWA.

Issue 1-2-2-3: The modulation order information of the co-scheduled UE (RRC based assistant signaling)

Candidate options on RRC based assistant signaling details:

  - Option 1: 2-bit RRC signaling to indicate MCS table or maximum modulation order of co-UEs

  - Option 2: 1-bit RRC signalling to indicate whether the 1024-QAM MCS table is used or not for the co-scheduled UE

  - Option 3: 1 bit indicates that in the whole cell, max MCS table for all the UEs is below 1024QAM

The maximum modulation order possible is 64QAM, 256QAM or 1024QAM. Indicating the maximum MO of the co-UE would help the UE in modulation order blind detection. At a minimum UE should be informed if 1024QAM MCS table is configured for any of the UEs. While it is common to use 256 QAM MCS table in real deployments, there could be some cases where only 64QAM table is used. Hence, we propose to use 2 bit signalling to indicate maximum MCS table of co-scheduled UEs.

- Introduce 2 bit RRC signaling to indicate MCS table or maximum modulation order of co-UEs

## Reply LS to RAN1

In response to RAN4 LS to RAN1 on DCI based signaling for modulation order of co-UEs, RAN1 has agreed to introduce the requested DCI signaling and sent a reply LS to RAN4 [2] with some additional questions.

Responses to questions from RAN1 are provided below:

Question 1: Whether this new signaling in DCI is introduced in DCI format 1_2 in addition to format 1_1?

DCI format 1_2 is also for scheduling PDSCH, hence the new DCI signaling should also be introduced for DCI format 1_2.

Question 2: Whether this new signaling in DCI is supported for one or more DL multi-TRP schemes?

The single DCI schemes are mainly used to introduce reliability, and would not be suitable for MU-MIMO scheduling. The multi-DCI schemes are used to increasing the user throughput and would also not be suitable for Mu scheduling. Also, single DCI SDM scheme schedules PDSCH on DMRS ports across 2 CDM groups, in which case user doesn’t expect a co-scheduled user. The scenarios for MU-MIMO and mTRP are rather limited in our understanding. But there is not limitation on scheduling MU-MIMO with mTRP transmission. Hence, if DL mTRP is used with MU-MIMO, the new DCI signaling should be supported.

Question 3: Whether this new signaling in DCI is supported when the RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2?

If PDSCH is scheduled with 2 CW, then no MU-MIMO scheduling is allowed. If RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2, UE could only be scheduled with 1 CW and MU-MIMO could be used. This new signaling should be present if RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2 and MU-MIMO is configured.

Question 4: Whether the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured?

Yes, the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured.

Question 5: Whether the new signaling in DCI is supported when Rel-18 DMRS is configured?

Yes, the new signaling should also be supported if Rel-18 DMRS is configured.

Question 6: In the content corresponding to “Bit field mapped to index” =6, whether or not the phrase “In each individual PRB allocated to the target UE, the following condition is satisfied” should be replaced by “In each individual PRB PRG allocated to the target UE, the following condition is satisfied”?

For the new DCI signaling the description for index 1-6 should be updated from ‘PRB’ to ‘PRG’.

Question 7: For “Bit field mapped to index” =1/2/3/4/5, does “empty PRB without co-scheduled UE” is allowed “in all the PRBs” of the target UE.

For advanced receiver with MU-MIMO, the assumption in RAN4 is that UE is expected to perform blind detection of the frequency domain resource allocation of the co-scheduled UE(s). To allow scheduling flexibility at the network and to have the DCI signaling be useful in more scenarios, empty PRB without co-scheduled UE should be allowed with index 1-5.

- Proposed response to RAN1 LS -  
Q1: DCI format 1_2 is also for scheduling PDSCH, hence the new DCI signaling should also be introduced for DCI format 1_2.
Q2: The scenarios for MU-MIMO with mTRP are limited. But there is not limitation on scheduling MU-MIMO with mTRP transmission. Hence, if DL mTRP is used with MU-MIMO, the new DCI signaling should be supported.
Q3: This new signaling should be present if RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2 and MU-MIMO is configured.
Q4: Yes, the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured.
Q5: Yes, the new signaling in DCI should also be supported if Rel-18 DMRS is configured.
Q6: For the new DCI signaling the description for index 1-6 should be updated from ‘PRB’ to ‘PRG’.
Q7: empty PRB without co-scheduled UE should be allowed with bit field mapped to index 1/2/3/4/5.

## UE Capability

UE capability for advanced receiver was discussed and the following options and agreements were captured in [1]:

Issue 1-3-1: Capability signalling for advanced receiver for MU-MIMO

Supporting MU-MIMO advanced receiver is an optional feature with capability signaling

On UE capability signalling details:

| Candidate contents of R-ML capability definition | If defined, by capability signalling or by UE declaration | Note |
| --- | --- | --- |
| R-ML with modulation order blind detection | Option 1: By capability signalling Option 2: By UE declaration |  |
| Maximum number of layers of co-UE or total number of layers for joint detection | Option 1: By capability signalling Other options not precluded |  |
| Maximum number of DMRS ports for blind detection | Option 1: By capability signalling Other options not precluded | If needed, FFS whether can be derived by subtracting the scheduled MIMO layers for the target UE from maxNumberMIMO-LayersPDSCH |
| Maximum modulation orders of interfering DMRS ports supported | Option 1: By capability signalling Other options not plecluded |  |

If UE is capable of R-ML with blind modulation order detection it is not clear how the network would use the information in scheduling. This UE capability would only be needed if RAN4 defines tests with MO blind detection.

- It is not clear how NW would use UE capability of R-ML with MO blind detection in its scheduling.

- This capability would only be needed if requirements with MO blind detection are introduced in RAN4.

In case test cases are defined with MO blind detection, to make the tests not applicable to UEs not supporting this additional feature we have 2 options:

Option 1: Introduce UE optional capability without signaling – similar to R-ML receiver for SU-MIMO

Option 2: Requirements with MO blind detection are based on UE declaration

Both the options would exempt the UEs without the capability of MO blind detection from meeting requirements with MO blind detection if introduced.

- If requirements with  MO blind detection are introduced, then select on of the following options for UE capability or declaration:
Option 1: Introduce UE optional capability without signaling – similar to R-ML receiver for SU-MIMO
Option 2: Requirements with MO blind detection are based on UE declaration

On the UE capability of -  Maximum number of layers of co-UE or total number of layers for joint detection, the additional note of ‘If needed, FFS whether can be derived by subtracting the scheduled MIMO layers for the target UE from maxNumberMIMO-LayersPDSCH’ would be applicable here rather than maximum DMRS ports to be detected.

- The additional note on  ‘If needed, FFS whether can be derived by subtracting the scheduled MIMO layers for the target UE from maxNumberMIMO-LayersPDSCH’ would be applicable to UE capability of -  Maximum number of layers of co-UE or total number of layers for joint detection.

UE should be able to declare capability for maximum number of layers of co-UE or total number of layers for joint detection, separate from maxNumberMIMO-LayersPDSCH. Hence, we propose to include UE capability for max layers of co-UE for joint detection.

- UE should be allowed to indicate capability for Maximum number of layers of co-UE or total number of layers for joint detection separate from maxNumberMIMO-LayersPDSCH.

- Introduce UE capability for Maximum number of layers of co-UE or total number of layers for joint detection.

On UE capability for maximum DMRS ports to be detected -  with different DMRS type and lengths, also considering enhanced DMRS in R18 introduced under MIMO evolution WI, the complexity of DMRS ports detection increases. UE should eb able to declare the number of ports it can detect, so NW can use this information in scheduling MU-MIMO.

- Introduce UE capability for maximum DMRS ports to be detected.

Issue 1-3-2: Capability granularity and details for the R-ML capability signalling

Candidate options:

  - Option 1: Align with the Rel-17 MMSE-IRC for MU-MIMO, i.e., per UE, no FDD/TDD difference, FR1 only.

  - Option 2: Introduce per CC per band per band combination (Per-FSPC) UE capability for Rel-18 MU-MIMO receiver

Advanced receiver for MU-MIMO with joint detection on co-UE layers is highly complex for the UE. This also requires blind detection at the UE for antenna ports, FDRA, channel estimation for co-UE ports which is processing intensive. The UE might be capable of supporting this in smaller bandwidths only. Hence, the UE should be allowed to signal this capability for MU-MIMO advanced receiver per CC per band per band combination (Per-FSPC)

- Introduce per CC per band per band combination (Per-FSPC) UE capability for Rel-18 MU-MIMO advanced receiver.

# 3. Conclusion

In this paper, we provide our views on open issues on on receiver assumptions for advanced receiver considered for mitigating inter- user interference in MU-MIMO. Our observations and proposals are captured below:

RRC based Network Assistance Information for Advanced Receivers

- There would be ambiguity in UE behavior if the RRC signaling is only present when default assumptions are invalid.

- Introduce dedicated RRC signaling to indicate if the default assumption is true or false for the following parameters:
- PRB bundling size and frequency domain resource allocation
- DMRS power boosting for the co-scheduled UE
- Time domain resource allocation information of the co-scheduled UE

- The performance of advanced receiver to cancel inter-user interference highly depends on how many ports of co-UE it can detect.

- Depending on DMRS type UE might need to detect 3 to 23 ports in the worst case.

- Introduce RRC signaling for upper bound on number of co-scheduled UE ports

- Knowledge of resource allocation type of co-UE helps determine the granularity to detect presence and FDRA .

- Introduce default assumption for resource allocation type for co-UE same as targe UE.

- Introduce dedicated RRC signaling to indicate if the default assumption is true or false

- Revise wording – ‘PRB’ to ‘PRG’ for index 1-6 for DCI based NWA.

- Introduce 2 bit RRC signaling to indicate MCS table or maximum modulation order of co-UEs

Reply LS to RAN1

- Proposed response to RAN1 LS -  
Q1: DCI format 1_2 is also for scheduling PDSCH, hence the new DCI signaling should also be introduced for DCI format 1_2.
Q2: The scenarios for MU-MIMO with mTRP are limited. But there is not limitation on scheduling MU-MIMO with mTRP transmission. Hence, if DL mTRP is used with MU-MIMO, the new DCI signaling should be supported.
Q3: This new signaling should be present if RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2 and MU-MIMO is configured.
Q4: Yes, the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured.
Q5: Yes, the new signaling in DCI should also be supported if Rel-18 DMRS is configured.
Q6: For the new DCI signaling the description for index 1-6 should be updated from ‘PRB’ to ‘PRG’.
Q7: empty PRB without co-scheduled UE should be allowed with bit field mapped to index 1/2/3/4/5.

UE capability

- It is not clear how NW would use UE capability of R-ML with MO blind detection in its scheduling.

- This capability would only be needed if requirements with MO blind detection are introduced in RAN4.

- If requirements with  MO blind detection are introduced, then select on of the following options for UE capability or declaration:
Option 1: Introduce UE optional capability without signaling – similar to R-ML receiver for SU-MIMO
Option 2: Requirements with MO blind detection are based on UE declaration

- The additional note on  ‘If needed, FFS whether can be derived by subtracting the scheduled MIMO layers for the target UE from maxNumberMIMO-LayersPDSCH’ would be applicable to UE capability of -  Maximum number of layers of co-UE or total number of layers for joint detection.

- UE should be allowed to indicate capability for Maximum number of layers of co-UE or total number of layers for joint detection separate from maxNumberMIMO-LayersPDSCH.

- Introduce UE capability for Maximum number of layers of co-UE or total number of layers for joint detection.

- Introduce UE capability for maximum DMRS ports to be detected.

- Introduce per CC per band per band combination (Per-FSPC) UE capability for Rel-18 MU-MIMO advanced receiver.

# Reference

- R4-2313993, “WF for Advanced receiver to cancel inter-user interference for MU-MIMO”, China Telecom.

- R1-2308598, “Reply LS on required DCI signalling for advanced receiver on MU-MIMO scenario”, RAN1