3GPP TSG-RAN4 Meeting #108bis	R4-2315509

Xiamen, China, Oct. 9 – 13, 2023

Agenda item:	5.7.2.6

Source:	Apple

Title:   	On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#108, the following agreement was reached on receiving timing difference (RTD), as captured in the WF [1]:

## Receive time difference

Issue 3-1-1: Whether to consider RTD larger than CP in multi-RX WI

Agreement:

  - The requirements defined for multi-Rx are only applicable for receive timing difference (RTD) less than CP in Rel-18 multi-Rx WI.

    - RAN4 understands it is important to support RTD>CP and recommend to consider in future release.

In this contribution, we discuss how to capture the agreement in TS 38.133.

# 2	Discussion

In the endorsed big CR [2], there is no mention of RTD < CP as a condition for the requirements to apply. However, there is a clear need to mention it because of the implications on UE implementation and network deployment.

Proposal 1: RTD < CP should be captured in 38.133 as a condition for the relevant multi-RX RRM requirements to apply.

As to how to capture this important condition in TS38.133, There are two options:

- Option 1: This condition is mentioned for each relevant multi-RX RRM requirement such as scheduling/measurement restriction, dual TCI state switching, L1-RSRP for GBBR.

- Option 2: In clause 3.6	“Applicability of requirements in this specification version,” it is stated that RTD < CP as a condition for the relevant requirements to apply.

Option 1 requires modification of every such mention and is prone to errors while Alt. 2 provides an once-and-for-all modification. Therefore, we prefer Option 2.

Proposal 2: It is proposed to state in clause 3.6	“Applicability of requirements in this specification version.” that RTD < CP is the condition for the relevant multi-RX RRM requirements to apply.

A corresponding R17 CR is provided in [3].

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: RTD < CP should be captured in 38.133 as a condition for the relevant multi-RX RRM requirements to apply.

Proposal 2: It is proposed to state in clause 3.6	“Applicability of requirements in this specification version.” that RTD < CP is the condition for the relevant multi-RX RRM requirements to apply.

# 4	References

R4-2314283, "WF on FR2_multiRx_part2," Moderator (Ericsson)

R4-2312153, "Big CR for RRM requirements for NR FR2 multi-Rx chain DL reception," vivo

R4-2315510, “draft CR on capturing RTD < CP as a condition for multi-RX RRM requirements,” Apple