# 3GPP TSG-RAN WG4 Meeting # 108 bis		R4-2315566

# Xiamen, China, October 9 – 13, 2023

Source:	Sony

Title:	Views on further reduction of UE complexity for eRedcap

Agenda item:	5.31.1

Document for:	Approval

- Background

Support of reduced capability NR devices and support of enhanced reduced capability NR devices (Rel-18) has been discussed in RAN4 for several meetings.

In RAN4 #108 possible solutions to facilitate further reduction of UE complexity such as SAW-less design was discussed. Co-existence requirement between n13 and B14 for eRedcap [2], and NR IBB requirements on eRedCap for n71 [3] were discussed. CRs were submitted but not pursued. This Tdoc further discuss those topics.

- Discussion

The support of industrial sensors, video surveillance, and wearables were the motivations behind Rel-17 RedCap. For Rel-18, some further complexity reduction enhancements were considered. Rel-18 eRedCap should provide NR support for low-tier devices. Possibilities for simplification in HW design was discussed during Rel-17 RedCap and is described in the technical report [4]. For Rel-18 eRedCap further simplification in HW has not been discussed so far. The SAW-less design is one of the main possibilities to achieve HW complexity and cost reduction, especially for a global SKU device. According to TR 38.875 [4] “For Type A HD-FDD, a high proportion of the cost saving occurs because the duplexer can be replaced with a switch and a lowpass filter”. TR 38.875 further states that cost reduction, due to HD-FDD operation, is in the order of 7%. Furthermore, all sourcing companies indicated that the RF cost savings (but not the baseband cost savings) accumulate across supported bands. Thus, for a low-cost global, one-SKU design, a SAW-less design is essential.

Although the SAW-less solution is considered as an important factor for complexity and cost reduction for HD-FDD UEs, the feasibility of SAW-less design in the RedCap HD-FDD UE is still limited. Due to PA nonlinearity and Tx noise outside the operating channel overlapping with the neighboring protected Rx bands makes the SAW-less design very problematic meeting the Tx OOB coexistence requirements and spectrum emissions. This is difficult particularly for wider channel BWs (>5MHz) located at ~10MHz offset from the protected Rx bands where -50dBm/MHz requirement should be fulfilled. For Rel-18 eRedCap class of the devices the above obstacle becomes more relaxed since the BB channel bandwidth is limited by 5MHz and, thus, the OOB Tx emission into the protected Rx bands reduces significantly for most UE-coexistence scenarios. However, for some cases where the Tx band is located very close to the protected Rx bands it is still very challenging. The worst case is the n13/B14 scenario with 9MHz offset between the bands.

Another obstacle for realizing the HD-FDD SAW-less design exists on the Rx side where the Rx IBB spec in some bands (n71 and n105) requires withstanding the modulated blockers from DTV broadcast at very close offsets from the band edges. This still requires band-specific SAW filters to avoid significant Rx de-sense, mainly due to Rx non-linearity effects.

In those cases, additional relaxations in eRedCap specs are required to make the SAW-less design feasible, and to achieve the complexity and cost reductions, targeted by the RedCap and eRedCap WI. In the below sections we discuss this further.

## Co-existence requirement between n13 and B14

TS 38.101-1 requires Tx noise and spectrum emissions into the protected frequency bands not to exceed -50dBm/MHz, specified in TS 38.101-1 section 6.5.3.2, Spurious Emission for UE co-existence [3]. This requires SAW filters for channel BW > 5MHz to reduce PA IMD3 and IMD5 non-linearity products falling into the protected Rx bands. For Rel-18 eRedCap devices, the BB channel BW limitation to 5MHz facilitates the UE-coexistence and makes the SAW-less design possible in most co-existence scenarios. However, still some band combinations are very challenging without SAW filters. The main challenge for SAW-less design is when operating in n13, where the PA spectrum emission (IMD3) is falling into band 14 Rx band since there is very small frequency offset (9MHz) between n13 TX band and band 14 RX. This offset is smaller than the minimum Foob limit (10MHz for channel BW=5MHz). The applicability of UE-coexistence requirement in this case is allowed by special Note15 (in Table 6.5.3.2-1).

Observation 1	n13 Tx noise and spectrum emission into band 14 Rx band is a major problem for a NR eRedCap SAW-less design, since the offset (9MHz) is smaller than the offset in other bands and smaller than the minimum Foob limit (10MHz).

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

Our measurements on a few commercial NR PA types show spurious emission of -45 – -50dBm/MHz at the antenna connector which will be outside the specification. Particularly, one of the measurements is shown below, where emission level (due to IMD7 product) is -48.6dBm/MHz @9.5MHz offset and -51.6dBm/MHz@10.5MHz offset from the lower end of n13.

Figure 1. The measurement emission level of n13 of a commercial NR PA.

Based on our measurement this issue is most critical when the eRedCap device is configured to transmit with starting RB frequency (RBstart) in the lower 1.8MHz frequency range of n13 and the configured RB length (LCRB) is larger than 20. In this case, about 2 dB power backoff in addition to existing MPR values (as defined [7]) is required to ensure the spectrum emission into band 14 Rx is below -50 dBm/MHz.

Observation 2	The NR eRedCap SAW-less design requires additional power backoff to meet the n13/B14 co-existence requirement when LCRB*12*SCS > 3.6MHz and RBstart*12*SCS is < 1.8 MHz which aligns with outer RB allocations.

To enable the possibility for SAW-less eRedCap design for n13 we suggest:

Proposal 1	Apply network signaling to allow additional reduced maximum power (A-MPR) for eRedCap operating in HD-FDD mode in n13 for outer RB allocations.

## Changes to TS 38.101-1

How to introduce this new relaxation in the specification was discussed during RAN4 #108 in off-line sessions and the outcome was a suggestion to introduce a dedicated NS value for eRedCap (NS_08). An NS value shall point to a specific requirement. Although the requirement is already stated in Table 6.5.3.2-1 together with NOTE 15 we chose to repeat the requirement in a new Table 6.5I.3.3.1-1 which we think is natural to place in the dedicated RedCap clause (i.e. 6.5I in TS 38.101-1). A-MPR for NS_08 could be placed among other A-MPR (clause 6.2.3) or it could be placed in a dedicated RedCap clause (e.g. clause 6.2I.3). We think, the latter would be confusing and, therefore, we prefer to place clause “A-MPR for NS_08” under clause 6.2.3, among all other A-MPR. Note that allowed maximum power reduction is defined as A-MPR = max(MPR, A-MPR) [6] and, therefore, the 2dB extra power reduction we propose, has to be added to the MPR as specified in [7]. We have not proposed any modifications to Inner RB allocation but rather put bracket in the proposed A-MPR table. We will come back after further checking the Inner RB allocation. Further, we must ensure the NS_08 is signaled when Rel-18 eRedCap is operating in HD-FDD mode in band n13 and therefore a note is added in Table 6.5.3.2-1. However, we are open to further discuss the specification arrangement. An outline of the proposed specification arrangement is found in a draftCR [9].

## NR IBB requirements on eRedCap for n71

For eRedCap SAW-less design, the in-band blocking Case 3 and Case 5 in TS 38.101-1, Table 7.6.2-2 (see Figure 2) is very problematic to meet. The background of this requirement is found in TR 38.860 [11] clause 6.3.2 also referring to TR 36.755 [12].

“For the 600 MHz band, a similar situation exists with TV broadcast in adjacent channels. However, one significant difference is that power of the TV broadcast at 600 MHz is up to 1 MW. Assuming a similar propagation loss in 600 MHz compared to 700 MHz, the power level reaching the UE receiver is 13 dB higher for 600 MHz band compared to Band 17. However, considering that the propagation is slightly better at 600 MHz than 700 MHz, the maximum Rx power with high probability is 15 dBm higher, resulting in an Rx blocker level of -15 dBm. In FCC R&O 14-50, the television signal strength received by the UE is estimated at -23 dBm. This derivation is based on a received power level of -15 dBm coupled with a UE Rx antenna gain of -8 dBi. However, the 3GPP specifications also account for the case where the antenna gain may be as high as 0 dBi for different types of devices, for example, fixed or nomadic. In TS 36.101, it is stated that a reference antenna with a gain of 0 dBi is assumed for each antenna port. Taking this into account, the received power level is then also -15 dBm, consistent with the proposal here.”

Further from TR 36.755 [12]:

“While the above describes the necessary condition for the UE to withstand TV blocking interference, the feasibility of the UE to meet such requirement should also be considered. It is expected that the Rx filter in the duplexer will be required to provide rejection of the TV blocker.”

The APT600 band is also discussed in [13]. Further, in [15] IBB requirement for n105 is discussed and in the end the IBB requirement was specified to -34dBm Case 5, Note 4 in Table 7.6.2-2, TS38.101-1 [5]. The reasoning was due to limited SAW filter performance.

Firstly, for the eRedCap device the 0dBi antenna may not be relevant. A UE Rx antenna gain of -8 dBi, as used in FCC R&O 14-50 may be more relevant. Secondly, TR 36.755 further discusses the feasibility of SAW filters. Indeed, in a SAW-less implementation there are limitations and, therefore, we suggest adding a note relaxing Case 3 and Case 5 in-band blocker level to -34dBm which is also aligned with the outcome of the APT600 band discussion (refer to NOTE 4). We propose NOTE 6: In-band blocker level is reduced to -34dBm for Rel-18 eRedCap.

Proposal 2	Add a note to TS 38.101-1, Table 7.6.2-2 allowing relaxation of the Case 3 (n71) and Case 5 (n105) in-band blocker level from -15dBm/-22dBm to -34 dBm for Rel-18 eRedCap devices, e.g.: 
“NOTE 6: Pinterferer is -34dBm for Rel-18 eRedCap devices.”

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 | -224 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |  |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n34, n38,n39, n40, n41, n483, n50, n51, n53, n54, n65, n66, n67, n70, n74, n75, n76, n85, n91, n92, n93, n94, n100, n101 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |  |
| n105 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |  | FDL_low – 7 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. NOTE 4: For Band n105 channels overlapping the 612 - 617 MHz frequency range, Pinterferer is modified to -34 dBm. NOTE 4: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |

Figure 2		TS38.101-1, Table 7.6.2-2

An outline of the proposed specification change is found in a draftCR [10].

- Conclusion

In this contribution, we have discussed the feasibility of SAW-less support of band n13 in a Global-SKU design for Rel-18 eRedCap devices. We have made the following observations and proposal:

Observation 1	n13 Tx noise and spectrum emission into band 14 Rx band is a major problem for a NR eRedCap SAW-less design, since the offset (9MHz) is smaller than the offset in other bands and smaller than the minimum Foob limit (10MHz).

Observation 2	The NR eRedCap SAW-less design requires additional power backoff to meet the n13/B14 co-existence requirement when LCRB*12*SCS > 3.6MHz and RBstart*12*SCS is < 1.8 MHz which aligns with outer RB allocations.

Proposal 1	Apply network signaling to allow additional reduced maximum power (A-MPR) for eRedCap operating in HD-FDD mode in n13 for outer RB allocations.

Proposal 2	Add a note to TS 38.101-1, Table 7.6.2-2 allowing relaxation of the Case 3 (n71) and Case 5 (n105) in-band blocker level from -15dBm/-22dBm to -34 dBm for Rel-18 eRedCap devices, e.g.: 
“NOTE 6: Pinterferer is -34dBm for Rel-18 eRedCap devices.”

- References

RP-223544		“Revised WID on Enhanced support of reduced capability NR devices”, Ericsson

R4-2313201 		“Co-existence requirement between n13 and B14 for eRedcap”, Sony

R4-2313202		“Views on NR IBB requirements on eRedCap for n71”, Sony

TR 38.875		“Study on support of reduced capability NR devices”

TS 38.101-1		Clause 6.5.3.2

TS 38.101-1		Clause 6.2.3.1

TS 38.101-1		Table 6.2.2-1

TS 38.101-1		Table 7.6.2-2

R4-235579 		“draftCR to TS 38.101-1: Co-existence for eRedCap”, Sony

R4-235580 		“draftCR to TS 38.101-1: IBB requirements on eRedCap for n71”, Sony

TR 38.860		“Study on Extended 600 MHz NR band”, Section 6.3.2, Section 6.4

TR 36.755		“US 600 MHz Band for LTE”, Section 7.1.2.2

TR 38.892		“APT 600 MHz NR band”

R4-2218824		“Asymmetric Asymmetric bandwidths for APT 600 MHz and the REFSENS requirement”, Section 4, Ericsson

R4-2216646		“In band blocking for APT 600”, Qualcomm Incorporated