# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315574

# Xiamen, China, October 09 – October 13, 2023

# Title: 	Discussion on L1-RSRP measurement requirements

# Source: 	ZTE Corporation

# Agenda item:	5.24.2.1.2

# Document for:	Approval

Introduction

L1-RSRP measurement requirements of L1/L2 based inter-cell mobility have been discussed widely for several meetings. During RAN4 #108 meeting, we have reached lots of consensus on L1/L2 inter-cell mobility delay requirements for inter-cell mobility in WF [1]. There are still some remaining issues that need to be addressed. In this contribution, we want to share further considerations on these open issues.

Discussion

2.1Applicability rule for L1-RSRP measurement

The first open issue is whether to consider L1 measurements on unknown cell:

Issue 2-1-2: Whether to consider L1 measurements on unknown cell <Way Forward> - UE is not required to perform L1 measurements on unknown cell.

Firstly, we would like to express our opinion on whether to consider L1 measurements on unknown cell. In our understanding, the purpose of introducing L1 measurement is to reduce handover delay. If measurements are performed on unknown cell, the advantage of L1 measurement may not be reflected. So we suggest not performing L1 measurements on unknown cell. However, some companies proposed to replace "unknown" with "undetected" or "undetectable".

According to our understanding, "undetected" indicates that the cell is currently undetected but but may be detectable, while "undetectable" indicates that the cell cannot be detected. If a cell is undetectable, it is not possible to measure. If a cell is undetected, it is also not possible for UE to perform L1 measurement as UE has no timing information about the target cell. And using ‘unknown’ in this agreement can avoid performing L1 measurement directly on target cell on which UE doesn’t have valid T/F information. In comparison, we prefer to use "unknown".

Proposal 1: UE is not required to perform L1 measurements on unknown cell.

Issue 2-1-3: known cell condition for L1-RSRP measurement. <Way Forward> FFS the following options - Option 1 (MTK, CATT, xiaomi, CMCC, ZTE, OPPO, vivo, CTC): - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last [5] seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3. - Otherwise, it is unknown - Option 2 (Apple): - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement: - The UE has sent a valid L3 measurement report during the last [5] seconds, and - The SSB from the target cell remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3. - Otherwise, it is unknown - Option 3 (Nokia): - For rel-18 LTM, remove “during the last 5 seconds” from the known cell conditions. - Known condition is not currently visible to the network. - Cells detected more than 5s ago but are still detectable, are considered as known cells for LTM.

The related issue is the known cell condition for L1-RSRP measurement. The known cell condition of Rel-17 ICBM can be used as baseline to define known cell for L1-RSRP measurement for Rel-18 LTM, but update are needed.

As we known, the purpose of R17 ICBM is beam management and not to consider the inter-frequency case. But for R18 LTM, we should consider both intra-frequency and inter-frequency. Therefore, the conditions need to be updated accordingly to cover inter-frequency case. Additional, we have reached the agreement that L3 measurement report is not the prerequisite of L1 measurement configuration for the behaviour of the NW and UE should have performed L3 measurement on the neighbour cell before UE performs L1 measurement on that cell from the view of UE. Consequently, no need to consider the measurement report as one of the conditions.

The company proposes to remove the time limit that “during the last [5] seconds”, but we believe that this time condition is necessary as it can limit L3 measurements to a certain time. Without a time limit, the measurement results are likely to be invalid due to long time intervals. Therefore, 5 seconds condition shall remain which similarly to other known conditions in many places in existing RRM spec.

Proposal 2. Target cell is considered as known if the following conditions are met in this requirement:

- UE has performed L3 measurement on the target cell during the last [5] seconds, and

- The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

       Otherwise, it is unknown

Issue 2-1-4: How to get SSB index information (on which symbols the RS to-be-measured) of the to-be-measured neighbor cell before L1-RSRP measurement? <Way Forward> - Common understanding: If deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is enabled, UE can derive SSB index according to serving cell timing. - FFS: deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is not enabled.

About how to get SSB index information of the to-be-measured neighbor cell before L1-RSRP measurement, we can discuss case by case.

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

deriveSSB-IndexFromCellInter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all inter-frequency neighbour cells on the frequency indicated by the MeasObjectNR. When this field is included, the network should set deriveSSB-IndexFromCell to true.

For intra-frequency L1-RSRP or inter-frequency L1-RSRP on candidate target cell, if network enables deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter, UE can assume SFN and frame boundary alignment across cells on the serving frequency. Meanwhile, we have the common understanding that if deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is enabled, UE can derive SSB index according to serving cell timing and skip the DL synchronization for obtaining frame/slot/symbol level synchronization and SFN level synchronization.

For intra-frequency L1-RSRP or inter-frequency L1-RSRP on candidate target cell, if deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is not enabled, which means that the RTD between the SSBs of serving cell and neighbour cell on the same carrier is larger than 2 SSB symbols, we can discuss it on whether UE has performed L3 measurement with SSB index reading. In our understanding, no additional time is needed if UE has performed L3 measurement with SSB index reading on the to-be-measured neighbor cell, otherwise, additional time for reading SSB index is needed.

Proposal 3: If deriveSSB-IndexFromCellInter-r17 or deriveSSB-IndexFromCellInter is enabled, UE can derive SSB index according to serving cell timing, otherwise,

- If UE only performed L3 measurement without SSB index reading, additional time for reading SSB index is needed.

- If UE has performed L3 measurement and SSB index reading, no additional time is needed.

2.2 Measurement capability

Issue 2-3-2: How to handle the case that the number of cells NW configured to measure exceeds the configuration to exceed UE capability <Way Forward> - If the number of cells/SSB NW configured/activated to measure exceeds UE capability, it is up to UE implementation on how to choose cells/SSB to measure. If any selection rules are further agreed in RAN1/2, the issue can be revisited.

This issue is about how to handle the case that the number of cells NW configured to measure exceeds the configuration to exceed UE capability. It was agreed that the supported maximum number of cells/SSBs configured for L1 measurement is up to UE capability. If configuring more cells than UE capability is not allowed, based on the UE capability, NW may need to reconfigure the cells. In our understanding, it should be allowed that the SSB configured for L1 measurement on neighbour cells exceeds the UE capability to make sure NW can configure equal number of cells for all the UEs which with different capabilities.

If the configuration exceed UE capability, down selection maybe a good choice. We suggest that the dowm selection of the candidate cells to perform L1 measurements should be indicated by NW as NW needs to be able to control whether UE is expected to perform L1 measurement.

Proposal 4. When number of LTM candidate cells configured exceeds the UE capability of number of cells UE can down selection of cells for LTM measurement is based on indication from the NW.

2.3 Intra-frequency L1-RSRP Measurement delay

Issue 2-4-1-2: whether to support the case that SSB periodicity of FR1 intra-frequency neighbour cell equals to SMTC periodicity in R18 LTM. <Way Forward> FFS the following options: - Option 1 (Apple, MTK, Huawei, vivo, Ericsson): Yes. - Option 1a (Ericsson): RAN4 to agree that sharing factor P between L3 for mobility and LTM for L1-RSRP is 1. - Option 1b (QC): For LTM L1 measurement SSB occasions always colliding with SMTC, the LTM L1 measurement requirement is defined assuming the SSB occasions are shared for L3 measurement and LTM L1 measurement with a ratio of two L3 measurements to one LTM L1 measurement. - Option 2 (OPPO): No.

Regarding whether to support the case that SSB periodicity of FR2 intra-frequency neighbour cell equals to SMTC periodicity in R18 LTM, we support option1.

First, according to RAN1 agreement, SSB configuration is up to network, it can be the same or different as SMTC. Therefore in FR2, both cases that SSB periodicity of intra-frequency neighbour cell is the same as SMTC and is different as SMTC shall be considered when defining requirements. Meanwhile, a CR (R4-2310139) is agreed to support the case that SSB of intra-frequency neighbor cell is fully overlapped with SMTC in FR2 in R17 ICBM. Therefore, it’s ok for us to support this case in R18 LTM too [2].

Proposal 5: In FR2, both cases that SSB periodicity of intra-frequency neighbour cell is the same as SMTC and is different as SMTC is supposed to be considered.

Issue 2-4-2-2: Measurement period of intra-frequency L1-RSRP measurement for UE incapable of RTD>CP when RTD of serving cell and neighbour cell is always within CP. < Way Forward >: FFS the following options - Option 1 (xiaomi, Huawei, Ericsson): Only define measurement periods with condition that RTD is no larger than CP. When actual RTD is larger than CP, no requirements or allow accuracy degradation. - Option 1a (xiaomi, Huawei) - when the actual RTD of serving cell and neighbour cell is no larger than CP the legacy measurement, restriction and scheduling restriction defined for non-serving cell in R17 apply for intra-frequency L1-RSRP measurement on neighbour cell. - when actual RTD>CP, there are no requirements. - Option 1b (Ericsson): - UE measures L1-RSRP using single timing - Accuracy may be degraded if the cell RTD is larger than CP - Option 2 (MTK, Nokia, QC): RAN4 to define requirements for both RTD > CP and RTD <= CP - Option 2a (MTK, QC): Define requirements covering all possible RTD values. - In FR1:‘the existing L1-RSRP measurement period (Table 9.5.4.1-1)’ x ‘the number of L1 measurement cells (including non-LTM L1-RSRP measurement cells) having SSBs colliding in the time domain’ - In FR2: same as the measurement period when UE supports RTD>CP Issue 2-4-2-3: Measurement restriction and scheduling restriction of intra-frequency L1-RSRP measurement for UE incapable of RTD>CP or not supporting measuring multiple cells on the same OFDM symbol when RTD>CP < Way Forward>: - Pending on issue 2-4-2-2.

Regarding whether to define the requirement for UE incapable of RTD>CP when the actual of serving cell and neighbour cell is larger than CP, we prefer option 1. If the actual RTD of the serving cell and neighbour cell is greater than CP, but because the UE at this time does not support RTD>CP, the UE follows the serving cell timing and L1-RSRP measurement cannot be performed correctly on neighbour cell. For option2, the UE need to perform L1-RSRP measurement on neighbour cell by tuning UE’s timing to target neighbour, which will cause the interruption on serving cell and UE is not expected to do this. Meanwhile, for UE incapable of RTD>CP, if the actual RTD of the serving cell and neighbour cell is not greater than CP, the traditional measurement and scheduling limitations defined in R17 for non serving cells apply to L1 RSRP measurements within the frequency range of adjacent cells.

Proposal 6: For UE incapable of RTD>CP, only define measurement periods with condition that RTD is no larger than CP. When actual RTD is larger than CP, no requirements or allow accuracy degradation.

2.4 Inter-frequency L1-RSRP measurement requirements

Issue 2-5-2-2: The principles in defining inter-frequency L1-RSRP measurement period with MG in FR1 < Agreement>: - In FR1, within one gap occasion, - If L1-RSRP and L3 measurement of the same frequency layer overlap, they can be counted as same frequency layer when calculating CSSF. - FFS: Otherwise, L1 inter-frequency measurement on a frequency layer is considered as an independent frequency layer when calculating CSSF for other overlapped inter-frequency layers. Issue 2-5-2-5: The principles in defining inter-frequency L1-RSRP measurement period with MG in FR2 < Way Forward >: FFS the following options - Option 1 (xiaomi): In FR2, L1 inter-frequency measurement and L3 inter-frequency measurement are considered as independent frequency layer when calculating CSSF within gap - Option 1a (MTK): Treat L1 inter-frequency measurement with Type-1 MG on [each cell of] each frequency layer as an independent inter-frequency layer in FR2. When calculating CSSF, overlapping with one additional L1 measurement on [each cell of] each frequency layer is equivalent to overlapping with one more L3 frequency layer. - Option 1b (Huawei, Ericsson): In FR2, for inter-frequency L1-RSRP with type 1 gap, legacy CSSFwithingap is supposed to be updated: each cell which is configured for L1-RSRP measurement is regarded as one independent candidate to be measured in a gap. - Option 2 (Apple): In FR2, introduce gap sharing between L1 and L3 gap based measurement. - Option 3 (Nokia): RAN4 is to discuss how to ensure L1 and L3 being measured efficiently in FR2

In last meeting, RAN4 has agreed the principles in defining inter-frequency L1-RSRP measurement period with MG in FR1 that If L1-RSRP and L3 measurement of the same frequency layer overlap, they can be counted as same frequency layer when calculating CSSF. In FR1, if L1-RSRP measurement and L3 measurement are configured on the same frequency, SSB for L1-RSRR measurement and SMTC for L3-RSRP are overlapped within one gap, L1 measurement is able to be simultaneously performed with L3 measurement. For the remaining part, if L1 measurement MO exists and no L3 MO on the same frequency, it is reasonable that the L1-RSRP MO is regarded as one independent candidate to be measured in a gap when calculating CSSF for other overlapped inter-frequency layers.

Proposal 7: In FR1, within one gap occasion, If L1-RSRP and L3 measurement of the same frequency layer overlap, they can be counted as same frequency layer when calculating CSSF. Otherwise, L1 inter-frequency measurement on a frequency layer is considered as an independent frequency layer when calculating CSSF for other overlapped inter-frequency layers.

In FR2, even if L1-RSRP measurement and L3 measurement are configured on the same frequency, SSB for L1-RSRR measurement and SMTC for L3-RSRP are overlapped within one gap, L1 measurement is not able to be simultaneously performed with L3 measurement due to different RX beam sweeping direction. Therefore, inter-frequency L1 measurement needs to share the measurement opportunities with L3 measurement, including other inter-frequency layers and the same inter-frequency layer. In addition, if there are multiple L1-RSRP measurement cells on one frequency, UE shall performL1 measurement by TDM. There are three ways proposed by company,

- Option 1: L1 inter-frequency measurement shares the measurement occasion with the L3 measurement on the same inter-frequency layer; same as the sharing factor between intra-f L3 and intra-f L1 measurement, L3 takes 2/3 measurement opportunities and L1 takes 1/3 measurement opportunities

- Option 2: Treat L1 inter-frequency measurement on each frequency layer as an independent inter-frequency layer

- Option 3: Treat L1 inter-frequency measurement on each cell as an independent inter-frequency layer

Option 1 is not a reasonable choice as other inter-frequency has the shortest delay but the measurement delay of L1 measurement is the longest. Compared to Option 2, the measure delay of L1 measurement in Option 3 is the shorter but Option 3 will lead to the longest measurement delay for L3 measurement. We are open to option2 and option3.

Proposal 8: In FR2, L1 inter-frequency measurement and L3 inter-frequency measurement are considered as independent frequency layer when calculating CSSF within gap.

# 3 Conclusion

In this contribution, we give some discussions on L1/L2 inter-cell mobility delay requirements mobility, The conclusions are:

Proposal 1: UE is not required to perform L1 measurements on unknown cell.

Proposal 2. Target cell is considered as known if the following conditions are met in this requirement:

- UE has performed L3 measurement on the target cell during the last [5] seconds, and

- The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

       Otherwise, it is unknown

Proposal 3: If deriveSSB-IndexFromCellInter-r17 or deriveSSB-IndexFromCellInter is enabled, UE can derive SSB index according to serving cell timing, otherwise,

- If UE only performed L3 measurement without SSB index reading, additional time for reading SSB index is needed.

- If UE has performed L3 measurement and SSB index reading, no additional time is needed.

Proposal 4. When number of LTM candidate cells configured exceeds the UE capability of number of cells UE can down selection of cells for LTM measurement is based on indication from the NW.

Proposal 5: In FR2, both cases that SSB periodicity of intra-frequency neighbour cell is the same as SMTC and is different as SMTC is supposed to be considered.

Proposal 6: For UE incapable of RTD>CP, only define measurement periods with condition that RTD is no larger than CP. When actual RTD is larger than CP, no requirements or allow accuracy degradation.

Proposal 7: In FR1, within one gap occasion, If L1-RSRP and L3 measurement of the same frequency layer overlap, they can be counted as same frequency layer when calculating CSSF. Otherwise, L1 inter-frequency measurement on a frequency layer is considered as an independent frequency layer when calculating CSSF for other overlapped inter-frequency layers.

Proposal 8: In FR2, L1 inter-frequency measurement and L3 inter-frequency measurement are considered as independent frequency layer when calculating CSSF within gap.

# 4	Reference

[1] RP-221799, New WID on Further NR mobility enhancements, MediaTek.

[2] R4-2310160,WF on NR mobility enhancements (part 1), MediaTek.

[3] R4-2308328, Discussion on L1-RSRP measurement requirements, huawei, HiSilicon.