3GPP TSG-RAN WG1 Meeting #108bis	R4-2315605

Xiamen, China, October 9th – 13th 2023

Agenda Item:	6.28.6

Source:	Ericsson

Title:	PDSCH simulation results

Document for:	Information

# 1	Introduction

During RAN4#108, a WF was agreed for NCR demod [1]. This contribution presents results according to the agreed assumptions for PDSCH.

# 2	PDSCH

Results for the PDSCH are as follows:

| TestCase | BW | SCS | MCS | Channel | TxRx | SNRdB for 70% throughput | SNRdBfor1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | -3,1 | 3,5 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | -7,1 | -2,5 |  |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 4,5 | 11,6 |  |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 0,3 | 5,0 |  |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | -3,0 | 2,0 |  |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | -7,3 | -3,4 |  |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 5,0 | 10,2 |  |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 0,4 | 4,5 |  |

# References

R4-2313923, WF on NCR_MT demodulation requirements, ZTE, RAN4#108, August 2023