3GPP TSG-RAN WG4 Meeting # 108bis										R4-2315673

Xiamen, China, October 09 – October 13, 2023

Agenda item:	6.3.2

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for the Extended L-band

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [RP-230790] on introduction of the Extended L-band was approved. Part of the objective is as follows:

- Specify a new E-UTRA NTN FDD band, with initial scope for Region 1, with UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz

  - NOTE: The introduction of this new NTN band through this WID has no impact on existing 3GPP terrestrial band specifications.

- Support UE Categories NB1, NB2, M1;

- Support UE Power class 3 (+23dBm) and Power Class 5 (+20dBm);

- Introduce the corresponding SAN and UE RF core requirements;

- Introduce the corresponding RRM requirements.

- In this contribution, we share some views on UE RF requirement for the Extended L-band.

Discussion

## 2.1 Channel raster, carrier frequency and EARFCN

The following agreement was achieved in the last meeting.

Agreement: - For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.2-1. Table 2.2-1: E-UTRA channel numbers E-UTRA Operating Band; ΔFRaster (kHz); Downlink; Uplink. FDL_low (MHz); NOffs-DL; Range of NDL (First – <Step size> – Last); FUL_low (MHz); NOffs-UL; Range of NUL (First – <Step size> – Last). 253; [100]; 1518; [228501]; [228501-<1>-228570]; 1668; [261269]; [261269-<1>-261338]. NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz..

The channel raster and offset range of band 253 have not been confirmed in the above agreement. Based on TS 36.102 [2] and WF [3], the channel raster of band 256, 255 and 254 are all 100 kHz, and the relation between EARFCN and the carrier frequency of band 256, 255 and 254 are same. We have not seen any technical reasons to change the rules for Band 253. Therefore, the channel raster, carrier frequency and EARFCN can be confirmed for Extended L-band as Table 2.1-1.

Table 2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 253 | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 1: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be confirmed as Table 2.1-1.

## 2.2 Default UE TX-RX frequency separation

In RAN4 #106bis meeting, an agreement was achieved for TX-RX frequency separation as follows:

Agreement: - Consider if Option 1 can be agreed as a starting point, pending further checking of the frequency range. - Proposals - Option 1: For the Extended L-band, the TX–RX frequency separation should be defined as Table 2.4-1 (ZTE) Table 2.4-1: Default UE TX-RX frequency separation E-UTRA Operating Band; TX – RX carrier centre frequency separation. [253]; -150 MHz.

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for the Extended L-band should be confirm as Table 2.2-1.

Table 2.2-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 253 | -150 MHz |

Proposal 2: For the Extended L-band, the TX–RX frequency separation should be confirm as Table 2.2-1.

## 2.3 Reference sensitivity power level

The extended L-band is adjacent to the DL of TN band 24 and NTN band 255 which both support eMTC category M1 and the reference sensitivity power level REFSENS of both band 24 and band 255 are -102.7 dBm. Therefore, the REFSENS of band 24 and band 255 can be reused to the extended L-band as shown in Table 2.3-1 and Table 2.3-2.

Table 2.3-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.7 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 2.3-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -103.5 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

Similarly, the REFSENS of band 255 also can be reused to extended L-band as shown in Table 2.3-3.

Table 2.3-3: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

Proposal 3: For the extended L-band, the reference sensitivity level for eMTC NTN can be defined as Table 2.3-1 and Table 2.3-2 and the reference sensitivity level for NB-IoT can be defined as Table 2.3-3.

## 2.4 Receiver blocking requirement

In the last meeting, an agreement was achieved for receiver blocking requirement as follows:

Agreement: - Specify UE receiver blocking requirements by starting with ECC Report 263. Companies are encouraged to study how to capture these requirements in the specification. - Study the feasibility of specifying UE receiver blocking requirements at the 1518 MHz band edge and at the 1520 MHz channel offset.

According to the following information of ECC report 263 [4], the interference level for MES receiver is -48 dBm with low antenna gain of 3dBi for 1 MHz frequency separation and the interference level for MES receiver is -33 dBm with low antenna gain of 3dBi for 3MHz frequency separation. The interference bandwidth is a 5 MHz broadband signal interferer (LTE).

Table 8: Assumed blocking level for enhanced MES receivers Frequency separation between channel edges; Interference level (at output of receiving antenna). 1 MHz; –55 to –45 dBm. 3 MHz; –35 to –30 dBm. 6 MHz; –30 to –25 dBm. Based on the final results of its compatibility studies, it is concluded that: the minimum in-band blocking characteristic for land mobile earth stations receivers from a 5 MHz broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm above 1520 MHz0,

### 2.4.1 In-band blocking requirement for UE category M1

As shown in figure 2.4-1, we note that the proposed blocker of 5MHz broadband signal interferer with 1 MHz frequency separation from band 253 includes the case 1 and case 2 blockers for which most bands are required to meet at -56 dBm and -44 dBm and the case 1 and case 2 blockers are 1.4 MHz bandwidth. In addition, the interference level for MES receiver is -33 dBm for 3MHz frequency separation which is more challenging than the existing in-band blocking requirement.

Figure 2.1-1 The location of 5MHz broadband signal interferer blocker

Therefore, based on the analysis and agreement above, two sets of UE in-band blocking requirements at the 1518 MHz band edge and at the 1520 MHz channel offset should be introduced and case 3 and case 4 in-band blocking requirement can be introduced as shown in Table 2.4-1.

Table2.4-1: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 | Case 34 | Case 44 |
| --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 | -48 | -33 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |  |  |
| 256, 255 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  |
| 253 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 | FDL_low – 3.5 | FDL_low – 5.5 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: The Interferer bandwidth for Case 3 and Case 4 is a 5 MHz broadband signal interferer (LTE) |  |  |  |  |  |  |

Proposal 4: For the extended L-band, the in-band blocking requirement for UE category M1 can be specified as shown in Table 2.4-1.

### 2.4.2 In-band blocking requirement for UE category NB1 and NB2

For UE category NB1 and NB2, the interference of a 5 MHz broadband signal interferer (LTE) has not overlapping with the IBB1 and IBB2 blocker and the interference level of -48 dBm and -33 dBm with different frequency separation has more challenging than the existing in-band blocking requirement. Similarly, additional in-band blocking requirement should be introduced to cover the conclusion of ECC report 263 and additional in-band blocking requirement for UE category NB1 and NB2 can be specified as shown in Table 2.4-2.

Table 2.4-2: Additional in-band blocking parameters for category NB1 and NB2

| IBB3 test Parameters |  |
| --- | --- |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 48 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | FDL_low -3.5 MHz |
| IBB4 test Parameters |  |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 33 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset range from category NB1 or NB2 channel edge | FDL_low - 5.5 MHz |

Proposal 5: For the extended L-band, additional in-band blocking requirement for UE category NB1 and NB2 can be specified as shown in Table 2.4-2.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be confirmed as Table 2.1-1.

Proposal 2: For the Extended L-band, the TX–RX frequency separation should be confirm as Table 2.2-1.

Proposal 3: For the extended L-band, the reference sensitivity level for eMTC NTN can be defined as Table 2.3-1 and Table 2.3-2 and the reference sensitivity level for NB-IoT can be defined as Table 2.3-3.

Proposal 4: For the extended L-band, the in-band blocking requirement can be specified as shown in Table 2.4-1.

Proposal 5: For the extended L-band, additional in-band blocking requirement for UE category NB1 and NB2 can be specified as shown in Table 2.4-2.

References

- RP-230790, “New WID on Introduction of the Extended L-band (UL 1668-1675, DL 1518-1525) for IoT NTN”, Inmarsat

- 3GPP TS36.102: “User Equipment (UE) radio transmission and reception for satellite access”

- R4-2306579, “WF on IoT_NTN_FDD_LS_band”, MediaTek Inc.

- ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1552 MHz,” Approved 03 March 2017.