3GPP TSG-RAN WG4 Meeting # 108bis										R4-2315675

Xiamen, China, October 09 – October 13, 2023

Agenda item:	6.3.3

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirements for the Extended L-band

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [RP-230790] on introduction of the Extended L-band was approved. Part of the objective is as follows:

- Specify a new E-UTRA NTN FDD band, with initial scope for Region 1, with UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz

  - NOTE: The introduction of this new NTN band through this WID has no impact on existing 3GPP terrestrial band specifications.

- Support UE Categories NB1, NB2, M1;

- Support UE Power class 3 (+23dBm) and Power Class 5 (+20dBm);

- Introduce the corresponding SAN and UE RF core requirements;

- Introduce the corresponding RRM requirements.

- In this contribution, we want to share some initial views on SAN RF requirements for the Extended L-band.

Discussion

2.1. Channel raster, carrier frequency and EARFCN

Based on TS 36.102 [2] and WF [3], the channel raster of band 256, 255 and 254 are all 100 kHz, and the relation between EARFCN and the carrier frequency of band 256, 255 and 254 are same. We have not seen any technical reasons to change the rules for Band 253. Therefore, the channel raster, carrier frequency and EARFCN can be defined as Table 2.1-1.

Table 2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 253 | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 1: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.1-1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.1-1.

References

- RP-230790, “New WID on Introduction of the Extended L-band (UL 1668-1675, DL 1518-1525) for IoT NTN”, Inmarsat

- 3GPP TS36.102: “User Equipment (UE) radio transmission and reception for satellite access”

- R4-2306579, “WF on IoT_NTN_FDD_LS_band”, MediaTek Inc.