# 3GPP TSG-RAN WG4 Meeting # 108bis                              R4-2315697

# Xiamen, China, October 09 – October 13, 2023

# Title: 	Discussion on ATG UE demodulation

# Source: 	ZTE Corporation

# Agenda item:	5.13.7.2

# Document for:	Approval

# 1 Introduction

In RAN#99e meeting[1], the revised work item on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions which need further discussion for ATG DL demodulation are listed below[2].

Issue 2-2-1: Test scope for PDCCH - Option 1: To consider legacy PDCCH requirements for ATG PDCCH requirements. - 2T2R FDD: All test cases in 5.3.2.1.2 - 2T4R FDD: All test cases in 5.3.3.1.2 - 2T2R TDD: All test cases in 5.3.2.2.2 - 2T4R TDD: All test cases in 5.3.3.2.2 - Option 2: Select following legacy test cases for ATG UEs - 1T2R FDD: Test number 2, 3 and 4 in 5.3.2.1.1 - 2T2R FDD: All test cases in 5.3.2.1.2 - 1T2R TDD: Test number 1 and 2 in 5.3.2.2.1 - 2T2R TDD: All test cases in 5.3.2.2.2 - 1T4R FDD: Test number 2, 3 and 4 in 5.3.3.1.1 - 2T4R FDD: All test cases in 5.3.3.1.2 - 1T4R TDD: Test number 1 and 2 in 5.3.3.2.1 - 2T4R TDD: All test cases in 5.3.3.2.2 - Option 3: Select following legacy test cases for ATG UEs - 1T2R FDD: Test number 1, 3 and 5 in 5.3.2.1.1 - 2T2R FDD: Test number 3 in 5.3.2.1.2 - 1T2R TDD: All test cases in 5.3.2.2.1 - 2T2R TDD: All test cases in 5.3.2.2.2 - 1T4R FDD: Test number 1, 3 and 5 in 5.3.3.1.1 - 2T4R FDD: Test number 3 in 5.3.3.1.2 - 1T4R TDD: All test cases in 5.3.3.2.1 - 2T4R TDD: All test cases in 5.3.3.2.2 - Option 4: Do not consider in the ATG scope legacy PDCCH requirements Issue 2-3-1: Test scope for CSI reporting Agreement: - For CQI requirements, reuse the following requirements: - 2T2R FDD: CQI requirements in 6.2.2.1.1.1 - 2T2R TDD: CQI requirements in 6.2.2.2.1.1 - 2T4R FDD: CQI requirements in 6.2.3.1.1.1 - 2T4R TDD: CQI requirements in 6.2.3.2.1.1 - Not introduce RI requirements for ATG UE - FFS for PMI requirements

This contribution shares our views for the issues listed above.

# 2	Discussion

With regard to PDCCH requirements, it was agreed in RAN4 #107 meeting that no new test cases will be defined. Which legacy test cases for ATG UEs are FFS.

As AWGN+Doppler is agreed as the channel model for ATG scenario, the legacy requirement for PDCCH with TDL channel model can be reused for ATG requirements. The following shows some requirements which are copied from 38101-4[3].

Table 5.3.2.1.2-1: Minimum performance for PDCCH with 15 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 10; 24; 2; 4; R.PDCCH. 1-2.2 FDD; TDLC300-100; 2x2 Low; 1; 2.0. 2; 10; 48; 2; 8; R.PDCCH. 1-2.5 FDD; TDLC300-100; 2x2 Low; 1; -1.3. 3; 10; 48; 1; 8; R.PDCCH.1-1.3 FDD; TDLA30-10; 2x2 Low; 1; -0.2. Table 5.3.3.1.2-1: Minimum performance for PDCCH with 15 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 10; 24; 2; 4; R.PDCCH. 1-2.2 FDD; TDLC300-100; 2x4 Low; 1; -1.9. 2; 10; 48; 2; 8; R.PDCCH. 1-2.5 FDD; TDLC300-100; 2x4 Low; 1; -4.5. 3; 10; 48; 1; 4; R.PDCCH.1-1.2 FDD; TDLA30-10; 2x4 Low; 1; -1.0. Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x2 Low; 1; -1.2. Table 5.3.3.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x4 Low; 1; -4.3.

The above requirements can be considered to be reused for ATG PDCCH requirements. The antenna configuration for ATG is 2Tx2Rx and 2Tx4Rx[4]. So the requirements for 2Tx2Rx and 2Tx4Rx need to be considered.

Proposal 1. To consider option1 for ATG PDCCH requirements.

The propagation delay is 1ms assuming the distance between the aircraft and the BS is 300km and the speed is 1200kmph, the distance within 1ms has little effect on the angle which is shown in figure 2-1. Figure 2-1a illustrates the distance relationship for ATG scenario, where point A is the starting position of the cell coverage of BS1 along the flight direction of the aircraft, and point B is the ending position of the cell coverage of BS1. It can be found that the angle variation between UE and BS is very small when the aircraft enters the starting position of the coverage area.

Figure 2-1a, Location and distance relationship of ATG deployment

Figure 2-1b, Angle between UE and BS when the aircraft enters the starting point of the coverage area and the angle between the two after 1ms

From figure 2-1, it can be seen that the change in angle within 1ms (propagation delay) is negligible. And table 2-1 lists some angle variation relative to starting point for different time elapsed.

Table 2-1 time elapse vs angle variation

| Time elapsed | Angle variation(degree) |
| --- | --- |
| 1ms | 0.0001 |
| 10ms | 0.0004 |
| 100ms | 0.0038 |
| 1000ms | 0.0381 |

It can be found that the angle variation within up to 1000ms is negligible based on the above analysis. Therefore the PMI reporting can be considered for ATG deployment.

Observation 1. CSI reporting of PMI is feasible for ATG scenario.

# 3 Conclusion

In this contribution, we give some conclusions for ATG UE demodulation. The conclusions are:

Observation 1. CSI reporting of PMI is feasible for ATG scenario.

Proposal 1. To consider option1 for ATG PDCCH requirements.

# 4	Reference

[1]  RP-230279, Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2313875, WF for ATG demodulation requirements, CMCC

[3]  3GPP TS 38101 v17.8.0

[4]  R4-2305899, WF for ATG demodulation requirements, CMCC