# 3GPP TSG-RAN WG4 Meeting # 108                                   R4-2315699

# Xiamen, China, Oct 9 – Oct 13, 2023

# Title: 	Simulation results on NR BS ATG demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	5.13.7.3

# Document for:	Discussion

# 1 Introduction

During the last RAN4 meeting, some conclusions has been reached for ATG BS demodulation requirements. In this contribution, we want to share some simulation results for ATG BS demodulation from RAN4 perspective.

# 2 Discussion

## 2.1 Simulation assumptions

In RAN4 #107 meeting, simulation assumptions have been approved in [1] which are shown as following Table 2-1.

Table 2-1. Simulation assumptions for PUSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 5 | 10 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
| Channel model |  |  | AWGN+200Hz | AWGN+500Hz |
| MCS/Rank |  |  | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 |
| Antenna configuration |  |  | 1T2R | 1T2R |
| Transform precoding |  |  | CP-OFDM |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 |  |
|  | RV sequence |  | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port |  | {0} |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type |  | A |  |
|  | Start symbol |  | 0 |  |
|  | Allocation length |  | 14 |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| Test metric |  |  | 70% of max throughput at target SNR |  |

  - 2.2 Simulation results

Based on the simulation assumptions in above Table2-1, we did some simulations as shown in following Table 2-2 to Table 2-4.

Table 2-2. Simulation results for PUSCH with FDD and TDD

| Case number | SCS | Bandwidth | MCS | Antenna configuration | Propagation condition | Rank | SNR@70% max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 15kHz | 5MHz | MCS28 | 1x2 | AWGN + 200Hz Doppler | 1 | 16.4 |
| 1-2 | 15kHz | 5MHz | MCS20 | 1x2 | AWGN + 200Hz Doppler | 1 | 15.38 |
| 1-3 | 30kHz | 10MHz | MCS28 | 1x2 | AWGN + 500Hz Doppler | 1 | 16.3 |
| 1-4 | 30kHz | 10MHz | MCS22 | 1x2 | AWGN + 500Hz Doppler | 1 | 17.26 |

Table 2-3. Simulation results for PUSCH with FDD

| - 5M/15K MCS 28(Table 1) 1T2R | - 5M/15K MCS 22(Table 2) 1T2R |
| --- | --- |

  - Table 2-4. Simulation results for PUSCH with FDD

| - 10M/30K MCS 28(Table 1) 1T2R | - 10M/30K MCS 22(Table 2) 1T2R |
| --- | --- |

# 3 Conclusion

In this contribution, we provide our simulation results for NR BS ATG demodulation requirements.

# 4 References

- R4-2309796 WF for ATG demodulation requirements. RAN4#107meeting. CMCC.