# 3GPP TSG-RAN WG4 Meeting # 108bis                                 R4-2315704

# Xiamen, China, Oct 9 – Oct 13, 2023

# Title: 	Simulation results for NCR-MT demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	5.28.6

# Document for:	Discussion

# 1 Introduction

During the last RAN4 meeting[1], some conclusions has been reached for NCR-MT demodulation requirements. In this contribution, we want to share some simulation results for NCR-MT which include PDSCH and PDCCH requirements.

# 2 Discussion

# 2.1 Simulation assumptions

  - 2.1.1  PDSCH Simulation assumptions and results

### 2.1.1.1  FR1

According previous meeting[1][2][3] conclusions, we summarized the following Table 2-1 and Table 2-2 for PDSCH FR1 simulation assumptions and FR2 simulation assumptions.

- Table 2-1. Simulation assumptions for PDSCH FR1

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | FDD | TDD |
| Channel Bandwidth/SCS |  | 10/15 | 40/30 |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| MCS |  | 4/13 | 4/13 |
| Rank |  | 1 | 1 |
| DMRS port allocation |  | Rank 1: Port 1000 | Rank 1: Port 1000 |
| MIMO configuration |  | 2T2R ULA Low and 2T4R ULA Low |  |
| Number of CDM groups without data |  | 1 |  |
| HARQ process number |  | 4 for FDD and 8 for TDD(70% of maximum Throughput) 1( 1% BLER ) |  |
| Precoding model |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Random precoding with Single panel Type 1 per PRB bundling size per slot |
| PDSCH configuration | Mapping type | Type A |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PRB bundling size | 2 |  |
|  | PRB bundling type | Static |  |
| PDSCH DMRS configuration | DMRS Type | DMRS Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |  |
| Propagation conditions |  | TDLA30-10 |  |
| Receiver type |  | MMSE-IRC |  |
| Test metric |  | SNR @ 70% of maximum Throughput SNR @ 1% BLER |  |

- Table 2-2. Simulation assumptions for PDSCH FR2

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 for 70% of maximum Throughput and 1 for 1% BLER |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | DM-RS port(s) | {1000} for Rank 1 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 1 |
|  | Allocation length | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Propagation conditions |  | TDLA30-75 |
| Receiver type |  | MMSE-IRC |
| Test metric |  | SNR @ 70% of maximum Throughput SNR @ 1% BLER |

- Further, according to the RAN4#108 meeting’s WF[1], the PDSCH FR1 simulation results is shown in following Table 2-3.

- ‘

- Table 2-3. Minimum performance for Rank 1

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation conditions | Antenna configuration | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70% maximum throughput | -3.2 |
| 1-1 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 1% BLER | 1.9 |
| 1-2 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70% maximum throughput | -6.0 |
| 1-2 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 1% BLER | -2.4 |
| 1-3 | 10/15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70% maximum throughput | 5.0 |
| 1-3 | 10/15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 1% BLER | 10.2 |
| 1-4 | 10/15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70% maximum throughput | 1.9 |
| 1-4 | 10/15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 1% BLER | 5.6 |
| 1-5 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70% maximum throughput | -3.3 |
| 1-5 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 1% BLER | 2.1 |
| 1-6 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70% maximum throughput | -6.0 |
| 1-6 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 1% BLER | -3.2 |
| 1-7 | 40/30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70% maximum throughput | 5.0 |
| 1-7 | 40/30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 1% BLER | 9.5 |
| 1-8 | 40/30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70% maximum throughput | 2.0 |
| 1-8 | 40/30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 1% BLER | 4.8 |

- Also simulation curves are shown in the following Table 2-4 and Table 2-5.

- Table 2-4. Simulation results for PDSCH FR1

| - 10M/15kHz MCS 4 Rank 1 TDLA30-10 | - 10M/15kHz MCS 13 Rank 1 TDLA30-10 |
| --- | --- |
| - (3) 40M/30kHz MCS 4 Rank 1 TDLA30-10 | - (4) 40M/30kHz MCS 13 Rank 1 TDLA30-10 |

- Table 2-5. Simulation results for PDSCH FR1

| - 10M/15K MCS 4 Rank 1 TDLA30-10 | - 40M/30K MCS 4 Rank 1 TDLA30-10 |
| --- | --- |
| - 10M/15K MCS 13 Rank 1 TDLA30-10 | - 40M/30K MCS 13 Rank 1 TDLA30-10 |

- 2.1.1.2  FR2

- For FR2, PDSCH requirements are needed for 120kHz SCS. Considering the antenna configuration is 2RX Since only 1% BLER and 70% maximum throughput were considered. All required requirements are listed in the following table.

- Table 2-6. Minimum performance for Rank 1

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation conditions | Antenna configuration | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 100/120 | QPSK, 0.30 | TDLA30-75 | 2x2, ULA Low | 70% maximum throughput | -3.2 |
| 2-2 | 100/120 | QPSK, 0.30 | TDLA30-75 | 2x2, ULA Low | 1% BLER | 0.4 |

- Also simulation curves are shown in the following Table 2-7.

- Table 2-7. Simulation results for PDSCH FR2

| - 100M/120kHz MCS 4 Rank 1 TDLA30-75 | - 100M/120kHz MCS 4 Rank 1 TDLA30-75 |
| --- | --- |

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for NCR-MT ,

# 4 References

- R4-2313293 WF for NCR-MT demodulation requirements. ZTE Corporation.

- R4-2309832 WF for NCR-MT demodulation requirements. ZTE Corporation.

- R4-2305934 WF for NCR-MT demodulation requirements. ZTE Corporation.