# 3GPP TSG-RAN WG4 Meeting # 108bis                                R4-2315707

# Xiamen, China, Oct 9 – Oct 13, 2023

# Title: 	Simulation results for PDSCH demodulation requirements for 8Rx

# Source: 	ZTE Corporation

# Agenda item:	5.3.3.1.2

# Document for:	Discussion

# 1 Introduction

- In RAN4#108 meeting, 8Rx UE demodulation and CSI requirements were discussed and a WF was agreed[1]. In this paper, we will provide our simulation results on 8Rx PDSCH CA for FR1.

# 2 Discussion

  - 2.2 Simulation results

  - In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in following Tables.

  - 2.2.1 CA

  - And for 8Rx CA, we also did some simulations as shown in following Table 2-1 and Table 2-2 which covered MCS 17(Table 1) with rank 8.

Table 2-1. Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 10 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.0 |
| 20 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 25 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 30 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 35 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 |
| 40 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 50 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.5 |

Table 2-2. Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 18.5 |
| 10 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 |
| 20 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 25 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 30 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 |
| 35 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.8 |
| 40 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 50 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 |
| 60 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 |
| 70 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 80 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 90 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 |
| 100 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.0 |

# 3 Conclusion

In this contribution, we provide our simulation results for FR1 PDSCH 8Rx .

# 4 References

- R4-2313877, WF for 8Rx UE performance requirements. Huawei, HiSilicon.