3GPP TSG-RAN WG4 Meeting #108bis	R4-2315717

Xiamen, China, Oct 9 – 13, 2023

Agenda item:	5.25.2.3

Source: 	Qualcomm Incorporated

Title: 	On requirements for Rel-17 MUSIM gaps – Network A

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for Rel-17 MUSIM gaps in RAN4#108. The latest agreements and open issues were captured in a WF [1].

In this paper, we will discuss our proposals for defining requirements in Network A.

# Discussion

RAN4 made progress on defining measurement requirements in network A by agreeing to reuse the framework used in Rel-17 to adjust measurement delay when gap collisions occur [1].

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

Agreements:

- Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

Now, RAN4 should agree on the parameters that need to be updated and the details of the scaling factor. Although we agree that some details can be finalized in the CR, we believe it is still useful to reach agreements to simplify the CR review/approval process.

Proposal 1: The following parameters need to be updated to account for collisions with MUSIM gaps:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

Proposal 2: For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 3: For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 4: For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 5: For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 6: For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 7: For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 8: Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 9: Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 10: Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 11: For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

# Conclusions

Proposal 1: The following parameters need to be updated to account for collisions with MUSIM gaps:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

Proposal 2: For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 3: For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 4: For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 5: For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 6: For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 7: For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 8: Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 9: Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 10: Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 11: For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

# References

[1] R4-2314364, WF on R18 MUSIM, RAN4#108