3GPP TSG-RAN WG4 Meeting #108bis	R4-2315721

Xiamen, China, Oct 9 – 13, 2023

Agenda item:	5.22.2.2

Source: 	Qualcomm Incorporated

Title: 	Simulation results for Sidelink positioning

Document for:	Information

# Introduction

RAN4 agreed on simulation assumptions for Sidelink positioning measurements [1]. This paper includes simulation results for the following SL PRS based measurements:

- Rx-Tx time difference

- RSRP

- RSTD

# Simulation results for Rx-Tx time difference

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 2 | 58 | 58 | 60 |  |
|  |  |  |  | 2 | 2 | 58 | 58 | 60 |  |
|  |  |  |  | 3 | 2 | 58 | 59 | 60 |  |
|  |  |  |  | 4 | 2 | 58 | 58 | 59 |  |
|  | 96 |  | 64 | 1 | 2 | 30 | 32 | 31 |  |
|  |  |  |  | 2 | 2 | 30 | 32 | 32 |  |
|  |  |  |  | 3 | 2 | 30 | 32 | 32 |  |
|  |  |  |  | 4 | 2 | 30 | 32 | 32 |  |
|  | 24 | 30 | 128 | 1 | 2 | 57 | 54 | 62 |  |
|  |  |  |  | 2 | 2 | 57 | 54 | 62 |  |
|  |  |  |  | 3 | 2 | 57 | 54 | 62 |  |
|  |  |  |  | 4 | 2 | 57 | 54 | 60 |  |
|  | 48 |  | 64 | 1 | 2 | 29 | 31 | 31 |  |
|  |  |  |  | 2 | 2 | 29 | 30 | 30 |  |
|  |  |  |  | 3 | 2 | 29 | 31 | 30 |  |
|  |  |  |  | 4 | 2 | 29 | 30 | 30 |  |
|  | 96 |  | 32 | 1 | 2 | 16 | 13 | 16 |  |
|  |  |  |  | 2 | 2 | 16 | 13 | 16 |  |
|  |  |  |  | 3 | 2 | 16 | 14 | 16 |  |
|  |  |  |  | 4 | 2 | 16 | 13 | 16 |  |
|  | 24 | 60 | 64 | 1 | 2 | 30 | 28 | 32 |  |
|  |  |  |  | 2 | 2 | 29 | 28 | 31 |  |
|  |  |  |  | 3 | 2 | 30 | 28 | 31 |  |
|  |  |  |  | 4 | 2 | 29 | 28 | 30 |  |

Table 1: Rx-Tx simulation results for partially staggered SL PRS in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 4 | 58 | 60 | 61 |  |
|  |  |  |  | 2 | 4 | 58 | 60 | 60 |  |
|  |  |  |  | 3 | 4 | 60 | 60 | 61 |  |
|  |  |  |  | 4 | 4 | 58 | 60 | 60 |  |
|  | 96 |  | 64 | 1 | 4 | 28 | 28 | 31 |  |
|  |  |  |  | 2 | 4 | 28 | 28 | 31 |  |
|  |  |  |  | 3 | 4 | 30 | 28 | 32 |  |
|  |  |  |  | 4 | 4 | 28 | 28 | 31 |  |
|  | 24 | 30 | 128 | 1 | 4 | 58 | 59 | 60 |  |
|  |  |  |  | 2 | 4 | 58 | 59 | 59 |  |
|  |  |  |  | 3 | 4 | 58 | 59 | 60 |  |
|  |  |  |  | 4 | 4 | 58 | 59 | 58 |  |
|  | 48 |  | 64 | 1 | 4 | 30 | 28 | 32 |  |
|  |  |  |  | 2 | 4 | 30 | 28 | 31 |  |
|  |  |  |  | 3 | 4 | 30 | 28 | 32 |  |
|  |  |  |  | 4 | 4 | 30 | 28 | 30 |  |
|  | 96 |  | 32 | 1 | 4 | 15 | 16 | 17 |  |
|  |  |  |  | 2 | 4 | 15 | 16 | 17 |  |
|  |  |  |  | 3 | 4 | 15 | 17 | 17 |  |
|  |  |  |  | 4 | 4 | 15 | 16 | 16 |  |
|  | 24 | 60 | 64 | 1 | 4 | 29 | 29 | 31 |  |
|  |  |  |  | 2 | 4 | 29 | 29 | 30 |  |
|  |  |  |  | 3 | 4 | 30 | 29 | 31 |  |
|  |  |  |  | 4 | 4 | 29 | 29 | 30 |  |

Table 2: Rx-Tx simulation results for fully staggered SL PRS in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 2 | 105 | 104 | 114 |  |
|  |  |  |  | 2 | 2 | 105 | 103 | 108 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 104 | 103 | 105 |  |
|  | 96 |  | 64 | 1 | 2 | 66 | 66 | 68 |  |
|  |  |  |  | 2 | 2 | 66 | 66 | 67 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 66 | 66 | 67 |  |
|  | 24 | 30 | 128 | 1 | 2 | 104 | 109 | 147 |  |
|  |  |  |  | 2 | 2 | 103 | 107 | 115 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 102 | 106 | 108 |  |
|  | 48 |  | 64 | 1 | 2 | 67 | 68 | 74 |  |
|  |  |  |  | 2 | 2 | 66 | 66 | 68 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 66 | 66 | 68 |  |
|  | 96 |  | 32 | 1 | 2 | 43 | 43 | 44 |  |
|  |  |  |  | 2 | 2 | 43 | 43 | 44 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 43 | 43 | 43 |  |
|  | 24 | 60 | 64 | 1 | 2 | 67 | 68 | 139 |  |
|  |  |  |  | 2 | 2 | 67 | 65 | 78 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 66 | 65 | 71 |  |

Table 3: Rx-Tx simulation results for partially staggered SL PRS in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 4 | 105 | 106 | 107 |  |
|  |  |  |  | 2 | 4 | 105 | 105 | 106 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 105 | 105 | 105 |  |
|  | 96 |  | 64 | 1 | 4 | 70 | 67 | 67 |  |
|  |  |  |  | 2 | 4 | 69 | 67 | 66 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 68 | 67 | 66 |  |
|  | 24 | 30 | 128 | 1 | 4 | 107 | 108 | 115 |  |
|  |  |  |  | 2 | 4 | 106 | 107 | 108 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 107 | 107 | 106 |  |
|  | 48 |  | 64 | 1 | 4 | 65 | 66 | 69 |  |
|  |  |  |  | 2 | 4 | 65 | 66 | 67 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 65 | 66 | 68 |  |
|  | 96 |  | 32 | 1 | 4 | 43 | 43 | 44 |  |
|  |  |  |  | 2 | 4 | 43 | 43 | 44 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 43 | 43 | 43 |  |
|  | 24 | 60 | 64 | 1 | 4 | 67 | 68 | 74 |  |
|  |  |  |  | 2 | 4 | 67 | 67 | 68 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 67 | 67 | 67 |  |

Table 4: Rx-Tx simulation results for fully staggered SL PRS in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 2 | 173 | 176 | 202 |  |
|  |  |  |  | 2 | 2 | 129 | 129 | 131 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 112 | 113 | 114 |  |
|  | 96 |  | 64 | 1 | 2 | 96 | 97 | 99 |  |
|  |  |  |  | 2 | 2 | 87 | 86 | 87 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 83 | 81 | 82 |  |
|  | 24 | 30 | 128 | 1 | 2 | 175 | 187 | 306 |  |
|  |  |  |  | 2 | 2 | 152 | 156 | 187 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 124 | 127 | 131 |  |
|  | 48 |  | 64 | 1 | 2 | 95 | 98 | 107 |  |
|  |  |  |  | 2 | 2 | 92 | 92 | 94 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 84 | 85 | 86 |  |
|  | 96 |  | 32 | 1 | 2 | 79 | 81 | 82 |  |
|  |  |  |  | 2 | 2 | 79 | 80 | 81 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 77 | 77 | 78 |  |
|  | 24 | 60 | 64 | 1 | 2 | 95 | 101 | 317 |  |
|  |  |  |  | 2 | 2 | 93 | 98 | 112 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | 88 | 92 | 94 |  |

Table 5: Rx-Tx simulation results for partially staggered SL PRS in TDL-B.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 4 | 159 | 173 | 189 |  |
|  |  |  |  | 2 | 4 | 124 | 129 | 130 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 113 | 114 | 115 |  |
|  | 96 |  | 64 | 1 | 4 | 90 | 95 | 98 |  |
|  |  |  |  | 2 | 4 | 85 | 86 | 86 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 81 | 81 | 82 |  |
|  | 24 | 30 | 128 | 1 | 4 | 167 | 182 | 213 |  |
|  |  |  |  | 2 | 4 | 149 | 154 | 160 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 125 | 125 | 127 |  |
|  | 48 |  | 64 | 1 | 4 | 95 | 95 | 100 |  |
|  |  |  |  | 2 | 4 | 92 | 91 | 93 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 85 | 85 | 86 |  |
|  | 96 |  | 32 | 1 | 4 | 74 | 79 | 81 |  |
|  |  |  |  | 2 | 4 | 74 | 79 | 80 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 73 | 77 | 78 |  |
|  | 24 | 60 | 64 | 1 | 4 | 96 | 98 | 107 |  |
|  |  |  |  | 2 | 4 | 93 | 96 | 97 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | 90 | 92 | 92 |  |

Table 6: Rx-Tx simulation results for fully staggered SL PRS in TDL-B.

# Simulation results for RSRP

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 2 | ±0.5 | ±0.7 | ±1.6 |  |
|  |  |  |  | 2 | 2 | ±0.4 | ±0.5 | ±1.1 |  |
|  |  |  |  | 3 | 2 | ±0.4 | ±0.5 | ±0.9 |  |
|  |  |  |  | 4 | 2 | ±0.3 | ±0.4 | ±0.8 |  |
|  | 96 |  | 64 | 1 | 2 | ±0.4 | ±0.5 | ±1.2 |  |
|  |  |  |  | 2 | 2 | ±0.3 | ±0.4 | ±0.9 |  |
|  |  |  |  | 3 | 2 | ±0.3 | ±0.3 | ±0.7 |  |
|  |  |  |  | 4 | 2 | ±0.2 | ±0.3 | ±0.6 |  |
|  | 24 | 30 | 128 | 1 | 2 | ±0.7 | ±1.0 | ±1.9 |  |
|  |  |  |  | 2 | 2 | ±0.6 | ±0.8 | ±1.4 |  |
|  |  |  |  | 3 | 2 | ±0.5 | ±0.6 | ±1.2 |  |
|  |  |  |  | 4 | 2 | ±0.4 | ±0.6 | ±1.0 |  |
|  | 48 |  | 64 | 1 | 2 | ±0.5 | ±0.8 | ±1.5 |  |
|  |  |  |  | 2 | 2 | ±0.4 | ±0.6 | ±1.1 |  |
|  |  |  |  | 3 | 2 | ±0.3 | ±0.5 | ±0.9 |  |
|  |  |  |  | 4 | 2 | ±0.3 | ±0.4 | ±0.7 |  |
|  | 96 |  | 32 | 1 | 2 | ±0.4 | ±0.6 | ±1.2 |  |
|  |  |  |  | 2 | 2 | ±0.3 | ±0.4 | ±0.8 |  |
|  |  |  |  | 3 | 2 | ±0.3 | ±0.3 | ±0.7 |  |
|  |  |  |  | 4 | 2 | ±0.2 | ±0.3 | ±0.6 |  |
|  | 24 | 60 | 64 | 1 | 2 | ±0.9 | ±1.4 | ±3.6 |  |
|  |  |  |  | 2 | 2 | ±0.6 | ±1.0 | ±2.7 |  |
|  |  |  |  | 3 | 2 | ±0.5 | ±0.8 | ±2.2 |  |
|  |  |  |  | 4 | 2 | ±0.5 | ±0.7 | ±1.9 |  |

Table 7: RSRP simulation results for partially staggered SL PRS in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 4 | ±0.4 | ±0.5 | ±1.0 |  |
|  |  |  |  | 2 | 4 | ±0.3 | ±0.4 | ±0.7 |  |
|  |  |  |  | 3 | 4 | ±0.3 | ±0.4 | ±0.6 |  |
|  |  |  |  | 4 | 4 | ±0.3 | ±0.3 | ±0.5 |  |
|  | 96 |  | 64 | 1 | 4 | ±0.3 | ±0.4 | ±0.8 |  |
|  |  |  |  | 2 | 4 | ±0.2 | ±0.3 | ±0.5 |  |
|  |  |  |  | 3 | 4 | ±0.2 | ±0.3 | ±0.4 |  |
|  |  |  |  | 4 | 4 | ±0.2 | ±0.2 | ±0.4 |  |
|  | 24 | 30 | 128 | 1 | 4 | ±0.6 | ±0.7 | ±1.3 |  |
|  |  |  |  | 2 | 4 | ±0.5 | ±0.5 | ±0.9 |  |
|  |  |  |  | 3 | 4 | ±0.4 | ±0.5 | ±0.8 |  |
|  |  |  |  | 4 | 4 | ±0.4 | ±0.4 | ±0.7 |  |
|  | 48 |  | 64 | 1 | 4 | ±0.4 | ±0.5 | ±1.0 |  |
|  |  |  |  | 2 | 4 | ±0.3 | ±0.4 | ±0.7 |  |
|  |  |  |  | 3 | 4 | ±0.3 | ±0.4 | ±0.6 |  |
|  |  |  |  | 4 | 4 | ±0.2 | ±0.3 | ±0.5 |  |
|  | 96 |  | 32 | 1 | 4 | ±0.3 | ±0.4 | ±0.8 |  |
|  |  |  |  | 2 | 4 | ±0.2 | ±0.3 | ±0.6 |  |
|  |  |  |  | 3 | 4 | ±0.2 | ±0.3 | ±0.4 |  |
|  |  |  |  | 4 | 4 | ±0.2 | ±0.2 | ±0.4 |  |
|  | 24 | 60 | 64 | 1 | 4 | ±0.6 | ±0.9 | ±2.1 |  |
|  |  |  |  | 2 | 4 | ±0.4 | ±0.6 | ±1.4 |  |
|  |  |  |  | 3 | 4 | ±0.3 | ±0.5 | ±1.1 |  |
|  |  |  |  | 4 | 4 | ±0.3 | ±0.4 | ±1.0 |  |

Table 8: RSRP simulation results for fully staggered SL PRS in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 2 | ±0.7 | ±0.9 | ±2.2 |  |
|  |  |  |  | 2 | 2 | ±0.5 | ±0.7 | ±1.7 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.4 | ±0.5 | ±1.0 |  |
|  | 96 |  | 64 | 1 | 2 | ±0.5 | ±0.7 | ±1.4 |  |
|  |  |  |  | 2 | 2 | ±0.3 | ±0.5 | ±0.9 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.3 | ±0.4 | ±0.7 |  |
|  | 24 | 30 | 128 | 1 | 2 | ±0.9 | ±1.3 | ±2.6 |  |
|  |  |  |  | 2 | 2 | ±0.7 | ±1.0 | ±2.0 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.5 | ±0.7 | ±1.2 |  |
|  | 48 |  | 64 | 1 | 2 | ±0.6 | ±0.9 | ±2.0 |  |
|  |  |  |  | 2 | 2 | ±0.5 | ±0.6 | ±1.5 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.3 | ±0.5 | ±0.9 |  |
|  | 96 |  | 32 | 1 | 2 | ±0.4 | ±0.7 | ±1.3 |  |
|  |  |  |  | 2 | 2 | ±0.3 | ±0.5 | ±0.8 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.2 | ±0.3 | ±0.6 |  |
|  | 24 | 60 | 64 | 1 | 2 | ±1.2 | ±2.1 | ±5.6 |  |
|  |  |  |  | 2 | 2 | ±0.9 | ±1.4 | ±4.5 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.6 | ±0.9 | ±3.7 |  |

Table 9: RSRP simulation results for partially staggered SL PRS in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 4 | ±0.5 | ±0.7 | ±1.4 |  |
|  |  |  |  | 2 | 4 | ±0.4 | ±0.5 | ±1.0 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.3 | ±0.4 | ±0.7 |  |
|  | 96 |  | 64 | 1 | 4 | ±0.3 | ±0.5 | ±1.1 |  |
|  |  |  |  | 2 | 4 | ±0.3 | ±0.3 | ±0.7 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.2 | ±0.3 | ±0.5 |  |
|  | 24 | 30 | 128 | 1 | 4 | ±0.7 | ±0.9 | ±1.7 |  |
|  |  |  |  | 2 | 4 | ±0.5 | ±0.7 | ±1.2 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.4 | ±0.5 | ±0.8 |  |
|  | 48 |  | 64 | 1 | 4 | ±0.5 | ±0.6 | ±1.4 |  |
|  |  |  |  | 2 | 4 | ±0.4 | ±0.5 | ±1.0 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.3 | ±0.4 | ±0.7 |  |
|  | 96 |  | 32 | 1 | 4 | ±0.3 | ±0.5 | ±1.1 |  |
|  |  |  |  | 2 | 4 | ±0.2 | ±0.3 | ±0.8 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.2 | ±0.3 | ±0.6 |  |
|  | 24 | 60 | 64 | 1 | 4 | ±0.7 | ±1.2 | ±3.0 |  |
|  |  |  |  | 2 | 4 | ±0.5 | ±0.8 | ±1.8 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.4 | ±0.5 | ±1.1 |  |

Table 10: RSRP simulation results for fully staggered SL PRS in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 2 | ±0.6 | ±0.9 | ±1.9 |  |
|  |  |  |  | 2 | 2 | ±0.6 | ±0.8 | ±1.5 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.7 | ±0.7 | ±1.1 |  |
|  | 96 |  | 64 | 1 | 2 | ±0.5 | ±0.6 | ±1.2 |  |
|  |  |  |  | 2 | 2 | ±0.5 | ±0.6 | ±0.9 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.5 | ±0.6 | ±0.7 |  |
|  | 24 | 30 | 128 | 1 | 2 | ±0.8 | ±1.2 | ±2.3 |  |
|  |  |  |  | 2 | 2 | ±0.6 | ±0.9 | ±1.8 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.6 | ±0.7 | ±1.2 |  |
|  | 48 |  | 64 | 1 | 2 | ±0.6 | ±0.8 | ±1.7 |  |
|  |  |  |  | 2 | 2 | ±0.4 | ±0.6 | ±1.3 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.4 | ±0.5 | ±0.9 |  |
|  | 96 |  | 32 | 1 | 2 | ±0.4 | ±0.6 | ±1.1 |  |
|  |  |  |  | 2 | 2 | ±0.3 | ±0.4 | ±0.8 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.3 | ±0.4 | ±0.6 |  |
|  | 24 | 60 | 64 | 1 | 2 | ±1.0 | ±1.7 | ±4.7 |  |
|  |  |  |  | 2 | 2 | ±0.7 | ±1.2 | ±3.7 |  |
|  |  |  |  | 3 | 2 |  |  |  |  |
|  |  |  |  | 4 | 2 | ±0.5 | ±0.8 | ±3.0 |  |

Table 11: RSRP simulation results for partially staggered SL PRS in TDL-B.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -3 dB | SINR = -6 dB |  |
| FR1 | 48 | 15 | 128 | 1 | 4 | ±0.7 | ±0.8 | ±1.3 |  |
|  |  |  |  | 2 | 4 | ±0.6 | ±0.7 | ±1.1 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.6 | ±0.6 | ±0.9 |  |
|  | 96 |  | 64 | 1 | 4 | ±0.6 | ±0.6 | ±1.0 |  |
|  |  |  |  | 2 | 4 | ±0.4 | ±0.5 | ±0.8 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.5 | ±0.5 | ±0.7 |  |
|  | 24 | 30 | 128 | 1 | 4 | ±0.7 | ±0.9 | ±1.5 |  |
|  |  |  |  | 2 | 4 | ±0.5 | ±0.7 | ±1.1 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.5 | ±0.6 | ±0.9 |  |
|  | 48 |  | 64 | 1 | 4 | ±0.5 | ±0.6 | ±1.2 |  |
|  |  |  |  | 2 | 4 | ±0.4 | ±0.5 | ±0.9 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.4 | ±0.5 | ±0.7 |  |
|  | 96 |  | 32 | 1 | 4 | ±0.4 | ±0.5 | ±0.9 |  |
|  |  |  |  | 2 | 4 | ±0.3 | ±0.4 | ±0.7 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.3 | ±0.3 | ±0.5 |  |
|  | 24 | 60 | 64 | 1 | 4 | ±0.7 | ±1.0 | ±2.5 |  |
|  |  |  |  | 2 | 4 | ±0.5 | ±0.7 | ±1.5 |  |
|  |  |  |  | 3 | 4 |  |  |  |  |
|  |  |  |  | 4 | 4 | ±0.4 | ±0.5 | ±1.0 |  |

Table 12: RSRP simulation results for fully staggered SL PRS in TDL-B.

# Simulation results for RSTD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB, SINRi = -6 dB | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 48 | 15 | 128 | 1 | 2 | ±62 | ±77 |
|  |  |  |  | 2 | 2 | ±62 | ±77 |
|  |  |  |  | 3 | 2 | ±62 | ±78 |
|  |  |  |  | 4 | 2 | ±62 | ±77 |
|  | 96 |  | 64 | 1 | 2 | ±30 | ±40 |
|  |  |  |  | 2 | 2 | ±30 | ±40 |
|  |  |  |  | 3 | 2 | ±33 | ±40 |
|  |  |  |  | 4 | 2 | ±30 | ±40 |
|  | 24 | 30 | 128 | 1 | 2 | ±69 | ±87 |
|  |  |  |  | 2 | 2 | ±69 | ±87 |
|  |  |  |  | 3 | 2 | ±69 | ±87 |
|  |  |  |  | 4 | 2 | ±69 | ±80 |
|  | 48 |  | 64 | 1 | 2 | ±39 | ±40 |
|  |  |  |  | 2 | 2 | ±39 | ±42 |
|  |  |  |  | 3 | 2 | ±39 | ±42 |
|  |  |  |  | 4 | 2 | ±39 | ±42 |
|  | 96 |  | 32 | 1 | 2 | ±17 | ±20 |
|  |  |  |  | 2 | 2 | ±17 | ±20 |
|  |  |  |  | 3 | 2 | ±18 | ±19 |
|  |  |  |  | 4 | 2 | ±17 | ±19 |
|  | 24 | 60 | 64 | 1 | 2 | ±37 | ±38 |
|  |  |  |  | 2 | 2 | ±37 | ±39 |
|  |  |  |  | 3 | 2 | ±37 | ±39 |
|  |  |  |  | 4 | 2 | ±37 | ±39 |

Table 13: RSTD simulation results for partially staggered SL PRS in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB, SINRi = -6 dB | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 48 | 15 | 128 | 1 | 4 | ±72 | ±81 |
|  |  |  |  | 2 | 4 | ±70 | ±67 |
|  |  |  |  | 3 | 4 | ±75 | ±67 |
|  |  |  |  | 4 | 4 | ±70 | ±67 |
|  | 96 |  | 64 | 1 | 4 | ±40 | ±37 |
|  |  |  |  | 2 | 4 | ±40 | ±37 |
|  |  |  |  | 3 | 4 | ±43 | ±42 |
|  |  |  |  | 4 | 4 | ±40 | ±37 |
|  | 24 | 30 | 128 | 1 | 4 | ±63 | ±71 |
|  |  |  |  | 2 | 4 | ±63 | ±71 |
|  |  |  |  | 3 | 4 | ±63 | ±71 |
|  |  |  |  | 4 | 4 | ±63 | ±71 |
|  | 48 |  | 64 | 1 | 4 | ±38 | ±36 |
|  |  |  |  | 2 | 4 | ±38 | ±36 |
|  |  |  |  | 3 | 4 | ±40 | ±37 |
|  |  |  |  | 4 | 4 | ±38 | ±37 |
|  | 96 |  | 32 | 1 | 4 | ±23 | ±19 |
|  |  |  |  | 2 | 4 | ±22 | ±19 |
|  |  |  |  | 3 | 4 | ±23 | ±19 |
|  |  |  |  | 4 | 4 | ±23 | ±19 |
|  | 24 | 60 | 64 | 1 | 4 | ±30 | ±38 |
|  |  |  |  | 2 | 4 | ±30 | ±37 |
|  |  |  |  | 3 | 4 | ±30 | ±37 |
|  |  |  |  | 4 | 4 | ±30 | ±37 |

Table 14: RSTD simulation results for fully staggered SL PRS in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB, SINRi = -6 dB | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 48 | 15 | 128 | 1 | 2 | ±102 | ±100 |
|  |  |  |  | 2 | 2 | ±99 | ±98 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±96 | ±96 |
|  | 96 |  | 64 | 1 | 2 | ±50 | ±50 |
|  |  |  |  | 2 | 2 | ±49 | ±49 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±48 | ±49 |
|  | 24 | 30 | 128 | 1 | 2 | ±113 | ±104 |
|  |  |  |  | 2 | 2 | ±103 | ±100 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±97 | ±100 |
|  | 48 |  | 64 | 1 | 2 | ±51 | ±51 |
|  |  |  |  | 2 | 2 | ±50 | ±50 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±49 | ±50 |
|  | 96 |  | 32 | 1 | 2 | ±23 | ±23 |
|  |  |  |  | 2 | 2 | ±22 | ±22 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±22 | ±22 |
|  | 24 | 60 | 64 | 1 | 2 | ±61 | ±55 |
|  |  |  |  | 2 | 2 | ±53 | ±52 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±50 | ±51 |

Table 15: RSTD simulation results for partially staggered SL PRS in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB, SINRi = -6 dB | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 48 | 15 | 128 | 1 | 4 | ±100 | ±99 |
|  |  |  |  | 2 | 4 | ±98 | ±99 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±97 | ±98 |
|  | 96 |  | 64 | 1 | 4 | ±53 | ±50 |
|  |  |  |  | 2 | 4 | ±52 | ±50 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±51 | ±49 |
|  | 24 | 30 | 128 | 1 | 4 | ±106 | ±101 |
|  |  |  |  | 2 | 4 | ±101 | ±100 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±99 | ±99 |
|  | 48 |  | 64 | 1 | 4 | ±50 | ±50 |
|  |  |  |  | 2 | 4 | ±49 | ±49 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±49 | ±50 |
|  | 96 |  | 32 | 1 | 4 | ±24 | ±22 |
|  |  |  |  | 2 | 4 | ±23 | ±22 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±23 | ±22 |
|  | 24 | 60 | 64 | 1 | 4 | ±52 | ±52 |
|  |  |  |  | 2 | 4 | ±49 | ±50 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±48 | ±49 |

Table 16: RSTD simulation results for fully staggered SL PRS in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB, SINRi = -6 dB | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 48 | 15 | 128 | 1 | 2 | ±144 | ±146 |
|  |  |  |  | 2 | 2 | ±107 | ±107 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±93 | ±92 |
|  | 96 |  | 64 | 1 | 2 | ±72 | ±69 |
|  |  |  |  | 2 | 2 | ±54 | ±53 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±47 | ±47 |
|  | 24 | 30 | 128 | 1 | 2 | ±198 | ±159 |
|  |  |  |  | 2 | 2 | ±136 | ±127 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±107 | ±108 |
|  | 48 |  | 64 | 1 | 2 | ±78 | ±73 |
|  |  |  |  | 2 | 2 | ±64 | ±64 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±54 | ±53 |
|  | 96 |  | 32 | 1 | 2 | ±62 | ±59 |
|  |  |  |  | 2 | 2 | ±59 | ±55 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±52 | ±51 |
|  | 24 | 60 | 64 | 1 | 2 | ±102 | ±88 |
|  |  |  |  | 2 | 2 | ±78 | ±75 |
|  |  |  |  | 3 | 2 |  |  |
|  |  |  |  | 4 | 2 | ±65 | ±62 |

Table 17: RSTD simulation results for partially staggered SL PRS in TDL-B.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB, SINRi = -6 dB | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 48 | 15 | 128 | 1 | 4 | ±141 | ±136 |
|  |  |  |  | 2 | 4 | ±106 | ±108 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±94 | ±94 |
|  | 96 |  | 64 | 1 | 4 | ±74 | ±68 |
|  |  |  |  | 2 | 4 | ±55 | ±54 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±47 | ±47 |
|  | 24 | 30 | 128 | 1 | 4 | ±147 | ±142 |
|  |  |  |  | 2 | 4 | ±123 | ±123 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±105 | ±104 |
|  | 48 |  | 64 | 1 | 4 | ±72 | ±71 |
|  |  |  |  | 2 | 4 | ±64 | ±63 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±53 | ±53 |
|  | 96 |  | 32 | 1 | 4 | ±68 | ±62 |
|  |  |  |  | 2 | 4 | ±66 | ±58 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±60 | ±54 |
|  | 24 | 60 | 64 | 1 | 4 | ±77 | ±73 |
|  |  |  |  | 2 | 4 | ±69 | ±67 |
|  |  |  |  | 3 | 4 |  |  |
|  |  |  |  | 4 | 4 | ±62 | ±62 |

Table 18: RSTD simulation results for fully staggered SL PRS in TDL-B.

# References

[1] R4-2310077, Simulation assumptions for sidelink positioning, RAN4#107