3GPP TSG RAN WG4 Meeting #108-bis	R4-2315737

Xiamen, China, Oct 09th – Oct 13rd, 2023

Agenda item:	5.24.3

Source: 	vivo

Title: 	Discussion on test cases for R18 NR Mobility Enhancements

Document for:	Discussion

- Introduction

In Rel-18, the RRM core requirements on NR Mobility Enhancements have been introduced, which include the subsequent CPAC, improvement on FR2 SCell/SCG setup delay and enhanced CHO configurations. The latest progress was captured in WF [1]. As per the working plan, the discussion on performance requirements would be triggered in this meeting. Therefore, our views on test cases for NR Mobility Enhancements are provided in this paper.

- Discussion

2.1 Discussion on test case list for NR-DC with selective activation of cell groups via L3 enhancements

As listed in Table 1 below, for the test cases of subsequent CPC, both FR1+FR2 and FR1+FR1 NR-DC scenarios need to be considered. Besides, compared with the current requirement on conditional PSCell addition, conditional PSCell change does not only focus inter-frequency measurement but also intra-frequency measurement. Given this difference, we propose that the test cases for subsequent CPC need to consider both intra-frequency and inter-frequency measurement. In order to reduce the number of cases, we can assign these two cases respectively to FR1-FR1 NR-DC and FR1-FR2 NR-DC scenarios. And for FR1-FR1 to FR1-FR2 case, since the UE behavior is quite similar as FR1-FR1 to FR1-FR1 case except for Tprocessing, we prefer not to verify this case specifically.

Table 1: Test case list for NR-DC with selective activation of cell groups via L3 enhancements in 38.133

| NR-DC with selective activation of cell groups via L3 enhancements |  |  |  |
| --- | --- | --- | --- |
| TC NO. | TC | Description | Whether to test |
| 1 | Subsequent conditional PSCell change delay (FR1-FR2 NR-DC) | Focus on intra-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR2 neighbour cell (Cell 2) on the same frequency as the Cell 1. | Y |
| 2 | Subsequent conditional PSCell change delay (FR1-FR1 NR-DC) | Focus on inter-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR1 neighbour cell (Cell 2) on the different frequency as the Cell 1 | Y |
| 3 | Subsequent conditional PSCell change delay (FR1-FR1 NR-DC) | Focus on inter-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR2 neighbour cell (Cell 2) on the different frequency as the Cell 1 | N UE behaviour is quite similar as TC2 except for Tprocessing. |

Proposal 1: RAN4 to define the test cases for NR-DC with selective activation of cell groups via L3 enhancements as below:

- TC1: Subsequent conditional PSCell change delay (intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC)

- TC2: Subsequent conditional PSCell change delay (inter-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC)

2.2 Discussion on test case list for Enhanced CHO including target MCG and target SCG (obj.3)

For CHO with PSCell change, the current HO with PSCell performance requirements can be the reference on test case design, in which both parallel processing and sequential processing cases are specified. For NR-DC deployment, we list FR1-FR1 NR-DC and FR1-FR2 NR-DC as below.

Note that RAN4 has discussed on how to handle with the testability issues affecting LTE/FR1 + FR2 test cases caused by uncalibrated OTA link in the last meeting, which was triggered by RAN5 LS [2]. Similar as HO with PSCell, there are also some testability issues for CHO with PSCell change(obj.3) and CHO with CPAC (obj.4) in FR1+FR2 scenario. According to latest response in Reply LS [3], RAN4 would further assess on the test cases included in R5-233782. Therefore, our preference is to firstly define the test cases related to FR1+FR2 (as shown in Table 2 and Table 3) and add them to list of tests in TS 38.133 v17.9.0 clause A.3.13A if there are still testability issues after assessment.

Table 2: Test case list for Enhanced CHO including target MCG and target SCG for CPC in 38.133

| Enhanced CHO including target MCG and candidate SCGs for CPC |  |  |  |  |
| --- | --- | --- | --- | --- |
| TC NO. | TC | PCell conditional handover | Conditional PSCell change | Whether to test in R18 |
| 1 | Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR2 | Y Although there are still testability issues in FR1+FR2, we prefer to define the test case firstly and add it to the list of tests in TS 38.133 v17.9.0 clause A.3.13A after assessment |
| 2 | Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR2 | Y Although there are still testability issues in FR1+FR2, we prefer to define the test case firstly and add it to the list of tests in TS 38.133 v17.9.0 clause A.3.13A after assessment |
| 3 | Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR1 | Y |
| 4 | Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR1 | Y |

Proposal 2: RAN4 to define the test cases for Enhanced CHO including target MCG and target SCG (obj.3) as below:

- TC1: Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing (both PCell and PSCell are in FR1)

- TC2: Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing (both PCell and PSCell are in FR1)

- TC3: Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing (PCell is in FR1 and PSCell is in FR2)

- TC4: Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing (PCell is in FR1 and PSCell is in FR2)

Proposal 3: RAN4 to define test cases related to FR1-FR2 joint testing for Enhanced CHO configurations (both Obj.3 and Obj.4) and add them to list of tests in TS 38.133 v17.9.0 clause A.3.13A if there are still testability issues after assessment.

2.3 Discussion on test case list for Enhanced CHO including target MCG and candidate SCGs for CPC (obj.4)

As mentioned in Clause 2.2, there is also testability issue on FR1+FR2 joint testing in CHO with CPC from OTA perspective. Therefore, we prefer to specify all cases below add them to list of tests in TS 38.133 v17.9.0 clause A.3.13A if there are still testability issues after assessment. The details are shown in the following table.

Table 3: Test case list for Enhanced CHO including target MCG and candidate SCGs for CPC in 38.133

| Enhanced CHO including target MCG and candidate SCGs for CPC |  |  |  |  |
| --- | --- | --- | --- | --- |
| TC NO. | TC | PCell conditional handover | Conditional PSCell change | Whether to test |
| 1 | Conditional handover with conditional PSCell change from NR-DC to NR-DC | Inter-frequency conditional handover from FR1 to FR1 | Intra-frequency conditional PSCell change from FR1 to FR1 | Y |
| 2 | Conditional handover with conditional PSCell change from NR-DC to NR-DC | Inter-frequency conditional handover from FR1 to FR1 | Intra-frequency conditional PSCell change from FR1 to FR2 | Y Although there are still testability issues in FR1+FR2, we prefer to define the test case firstly and add it to the list of tests in TS 38.133 v17.9.0 clause A.3.13A after assessment |
| 3 | Conditional handover with conditional PSCell change from NR-DC to NR-DC | Inter-frequency conditional handover from FR1 to FR1 | Intra-frequency conditional PSCell change from FR2 to FR2 | Y Although there are still testability issues in FR1+FR2, we prefer to define the test case firstly and add it to the list of tests in TS 38.133 v17.9.0 clause A.3.13A after assessment |

Proposal 4: RAN4 to define the test cases for Enhanced CHO including target MCG and candidate SCGs for CPC (obj.4) as below:

- TC1: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR1)

- TC2: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR2)

- TC3: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR2-FR2)

- Conclusions

Proposal 1: RAN4 to define the test cases for NR-DC with selective activation of cell groups via L3 enhancements as below:

- TC1: Subsequent conditional PSCell change delay (intra-frequency CPC from FR2 to FR2)

- TC2: Subsequent conditional PSCell change delay (inter-frequency CPC from FR1 to FR1)

Proposal 2: RAN4 to define the test cases for Enhanced CHO including target MCG and target SCG (obj.3) as below:

- TC1: Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing (both PCell and PSCell are in FR1)

- TC2: Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing (both PCell and PSCell are in FR1)

- TC3: Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing (PCell is in FR1 and PSCell is in FR2)

- TC4: Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing (PCell is in FR1 and PSCell is in FR2)

Proposal 3: RAN4 to define test cases related to FR1-FR2 joint testing for Enhanced CHO configurations (both Obj.3 and Obj.4) and add them to list of tests in TS 38.133 v17.9.0 clause A.3.13A if there are still testability issues after assessment.

Proposal 4: RAN4 to define the test cases for Enhanced CHO including target MCG and candidate SCGs for CPC (obj.4) as below:

- TC1: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR1)

- TC2: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR2)

- TC3: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR2-FR2)

- Reference

[1] R4-2314332 WF on NR Mobility Enhancements (Part 2), Apple.

[2] R5-233782 LS on RRM test cases with testability issues, Qualcomm.

[3] R4-2314463 Reply LS on RRM test cases with testability issues, Qualcomm.