3GPP TSG-RAN WG4 Meeting # 108-bis												R4-2315764

Xiamen, China, October 09 – October 13, 2023

Source:				Ericsson

Title:					NTN enhancements – simulation assumptions

Agenda item:		5.26.2

Document for:		Discussion

# Introduction

This document discusses the remaining open issues related to simulation assumptions for the NTN coexistence study.

# Discussion

## Simulation assumptions

One item remains FFS in the simulation assumptions ([2]): the step model for ACLR and ACS, and this for both TN UL and NTN UL.

Similar to NTN FR1, we propose to reuse the ACIR 3-step model for NTN FR2.

For TN UL, RAN4 agreed to schedule 3 UEs in UL, meaning each UE will transmit in one distinct ACIR region.

For NTN UL, RAN4 agreed to schedule 10 UEs in UL. To keep simulation simple and considering  a worst case situation, we would propose to consider 3 UEs transmitting in the ACIR region 1, 3 UEs transmitting in the ACIR region 2 and 4 UEs transmitting in the ACIR region 3.

Proposal1: Reuse the ACIR 3-steps model for TN and NTN in UL.

Proposal2: For NTN, the ACIR 3-steps model will be used according the following: 3 UEs will transmit in ACIR region 1, 3 UEs will transmit in ACIR region 2 and 4 UEs will transmit in ACIR region 3.

# Conclusion

In this contribution, we made the following proposals:

Proposal1: Reuse the ACIR 3-steps model for TN and NTN in UL.

Proposal2: For NTN, the ACIR 3-steps model will be used according the following: 3 UEs will transmit in ACIR region 1, 3 UEs will transmit in ACIR region 2 and 4 UEs will transmit in ACIR region 3.

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2313890, Simulation assumption for NTN co-existence study, Samsung