3GPP TSG-RAN WG4 Meeting #108bis	R4-2315790

# Xiamen, China,  9th – 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on interruption requirements for UE reporting NFG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New interruption requirements apply when a NeedforGapInfoNR capable UE measures inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the interruption length/ratio allowed for a UE measuring inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement is applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.2.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

##### 8.2.2.2.X	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

When a UE supports [NeedForInterruptionInfoNR-R18] measurements and indicates [no-gap-with-interruption] on intra-frequency SSB-based or inter-frequency SSB-based measurements the UE is allowed to cause interruptions while performing measurements on the frequency layer for which [no-gap-with-interruption] is indicated.

The UE is allowed to cause interruption with interruption ratio no more than the requirements specified below upon UE measurements on a specific frequency layer that corresponds to the configured MO, where Tcycle is [FFS].

-	up to [2.50%] probability of missed ACK/NACK when 80ms ≤ Tcycle < 160ms, or

-	up to [1.25%] probability of missed ACK/NACK when 160ms ≤ Tcycle < 320ms, or

-	up to [0.625%] probability of missed ACK/NACK when 320ms ≤ Tcycle.

The interruptions are allowed for all the serving cells in the same FR as NR MO being measured if UE supports per-FR measurement gaps, and all the serving cells if UE does not support per-FR measurement gaps.

Table 8.2.2.2.X-1: Interruption length X

|  | SCS (kHz) | NR Slot | Interruption length X (slots) |
| --- | --- | --- | --- |
|  |  | length (ms) |  |
| 0 | 15 | 1 | TBD |
| 1 | 30 | 0.5 | TBD |
| 2 | 60 | 0.25 | TBD |
| 3 | 120 | 0.125 | TBD |

Editors’ note: Discussion is ongoing on cases where DRX or measurement gap is configured. Further update to this sub-clause subjects to the conclusions of those discussions.

Editors’ note2: Definition of Tcycle resembles measurement cycle definition in the measurement period requirements.

Editors’ note3: Total interruption ratio requirements will be updated subject to further conclusions.

### <<End of Change1>>