3GPP TSG-RAN WG4 Meeting #108bis		R4- 2315797

Xiamen, China, 9th – 13th October, 2023

Agenda item:	5.12.3

Source:	Intel Corporation

Title:	Test cases list for HST FR2 UE multipanel simultaneous reception and CA

Document for:	Discussion

# Introduction

Performance part of the work item starts in this meeting. In this paper we provide our views on the test cases list of multi-panel simultaneous reception and CA part.

# Discussion

## Test cases list for HST FR2 UE simultaneous reception

Table 1 Test cases list for multi-panel receptions

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | Subclause | Responsibility | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | SSB based RLM | RLM IS and OOS tests with UE simultaneous multi-panel reception | FR2 | A.7.5.1.X |  |  |
| 1-2 | SSB based BFD | BFD and LR test with UE simultaneous multi-panel reception | FR2 | A.7.5.5.X |  |  |
| 1-3 | SSB based L1-RSRP reporting | SSB based L1-RSRP measurement test with UE simultaneous multi-panel reception | FR2 | A.7.7.4.X |  |  |
| TBD | [Scheduling restrictions/ measurement availability] |  |  |  |  | TBC |
| … | … |  |  |  |  |  |

## Test cases list for HST FR2 CA

Table 2 Test cases list for inter-RAT measurements without gaps

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | Subclause | Responsibility | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1a | Inter-frequency measurements | Event triggered reporting tests without SSB time index reading | FR2 | A.7.6.2.X |  |  |
| 2-1b |  | Event triggered reporting tests with SSB time index reading | FR2 |  |  |  |
| 2-2 | Idle state mobility | Cell re-selection to FR2 inter-frequency NR case for UE configured with highSpeedMeasFlag-r17 | FR2 | A.7.1.1.X |  |  |
| 2-3 |  | Cell re-selection to FR2 inter-frequency NR case for UE supporting [measurementEnhancementCAInterFreqFR2-r18] | FR2 | A.7.1.1.X |  | Can be merged with 2-2? |
| TBD | [SCell activation delay] |  |  |  |  | TBC |
| … | … |  |  |  |  |  |

Regarding 2-2 and 2-3 in the table, we observe some relevant core spec inconsistencies. We shall first solve core spec problem and after agrees on the test list for this table.

Proposal 1: Endorse the test cases lists in table 1 in this paper.

Proposal 2: RAN4 first solves core spec problem and endorses on the test list for table 2.

# Conclusion

Proposal 1: Endorse the test cases lists in table 1 in this paper.

Proposal 2: RAN4 first solves core spec problem and endorses on the test list for table 2.

# References

# Annex