3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315798

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell, Spark NZ Ltd, Ericsson, CableLabs, Charter Communications Inc., Korea Testing Laboratory

Title:	TP to TR 38.858 Annex E: Comparison of RAN1 and RAN4 simulation methodology and assumptions

Agenda item:	5.19.1 [General aspects (TR)]

Document for:	Approval

Introduction

In 2022, a new study item work on evolution of duplex operation [1] was started, with the target to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation.

The detailed objectives specified in [1] are as follows:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution we discuss general approach that was agreed for simulation assumption work between RAN1 and RAN4.

Discussion

As shown above, RAN1 is responsible, among others, for the study of SBFD performance, inter-gNB, and inter-UE CLI handling and identifying solutions for CLI mitigation, etc. On the other hand, RAN4 is responsible for studying the feasibility of SBFD and the impact of SBFD on the RF requirements. For these tasks, both RAN1 and RAN4 agreed on their respective simulation assumptions and exchanged several LSs during the last year. It is noted that the simulation assumptions are different and may lead to different answers regarding feasibility.

Some concerns on differences between RAN1 and RAN4 simulation assumptions and methodology were already discussed in contributions [2] and [3] without any further actions, with arguments from some companies that both RAN1 and RAN4 have their own specific simulation goals.

Observation 1. It is a common understanding that RAN1 and RAN4 have some differences in simulation assumptions for SBFD.

In our opinion, there are certain aspects of the simulation assumptions and methodology that are key when it comes to conclude on the performance and/or coexistence of SBFD. However, in the current Technical Report skeleton TR 38.858 for SBFD there is no section planned to cover this issue. We think that such a section is needed and will be very useful for all readers of TR who were not in deep details with the RAN1 and RAN4 simulation assumptions.  We propose to do this in a subclause in Annex E.4.

Proposal 1. To include a subclause in Annex E.4 for comparison of RAN1 and RAN4 simulation methodology, assumptions, and potential impacts on the results and conclusions as proposed below.

# 3 Conclusion

In this contribution we discuss issues related to RAN1 and RAN4 methodology for SBFD simulations. We have made following observation and proposal:

Observation 1. It is a common understanding that RAN1 and RAN4 have some differences in simulation assumptions for SBFD.

Proposal 1. To include a subclause in Annex E.4 for comparison of RAN1 and RAN4 simulation methodology, assumptions, and potential impacts on the results and conclusions as proposed below.

# 4 References

- RP-223041, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN #98-e, Electronic Meeting, December 12-16, 2022.

- R4-2300079, Study the feasibility of and impact on RF requirements for NR duplex evolution, Spark NZ;

- R4-2304036, Study the feasibility of and impact on RF requirements for NR duplex evolution, Spark NZ;

<Start of TP to TR 38.858>

# E.4	Comparison of RAN1 and RAN4 simulation methodology and assumptions

## E.4.1 General overview

This annex summarizes essential differences between the sub-band full duplex (SBFD) simulations assumptions performed in RAN1 and RAN4.  Table 1 outlines some essential differences in terms of simulation assumptions with focus on the wide-area FR1 Urban Macro (UMa) scenario with 500 m inter-site distance (ISD). Detailed simulation assumptions in RAN1 and RAN4 for FR1 Urban Macro (UMa) and other scenarios can be found in attachment E.4.5.

Table E.4.1-1: Comparison of RAN1 and RAN4 parameters for SBFD simulations

| Parameter | RAN4 assumption | RAN1 assumption |
| --- | --- | --- |
| Network layout | Single layer with 19 sites, each with three hexagonal sectors (57 cells in total); Wrap around | (Baseline) Single layer with 7 sites, each with three hexagonal sectors (21 cells in total); Wrap around |
| UE deployment | 1 UE per cell per subband, 20% indoor probability as baseline | 10 UEs per cell; 80% indoor probability |
| BS self-interference isolation | Self-interference power is: Noise floor - 6dB (corresponding approximately to 1 dB desense) | Corresponding to 1 dB desense |
| BS Inter-sector interference isolation | Sum of all inter-sector gNB-gNB CLI per site correspond to Noise floor - 6 dB (corresponding approximately to 1 dB desense) Note: for FR1 wide-area, this means the inter-sector isolation should be not less than [144dB] | Total inter-sector isolation is the sum of spatial isolation + BS-BS ACIR. Agreed values for spatial isolation are: - Option 1: 75dB (spatial isolation), 0dB digital isolation - Option 2: 93dB (spatial isolation), 0dB digital isolation - Option 3: 100dB (spatial isolation), 0dB digital isolation - Option 4: 100dB (spatial isolation), 10dB digital isolation (recommended by Moderator) |
| Propagation model | BS-to-UE: UMa see TR 38.803 BS-to-BS: UMa see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.828. - X = [0.75] - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.803. - Option 2: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - Option 2 as 1st priority and option 1 as 2nd priority [Editor’s Note: Agreement 2.4.3 in R4-2302888] UE-to-UE: UMi see TR 36.828, subclause 5.2.2.1. Model is not applicable when 2D distance is less than 10m, instead free space model is applicable. Optional: TR 38.901 model | BS-to-UE: UMa in TR 38.901 BS-to-BS: UMa in TR 38.901 (hUE =25m). If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 75%; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. UE-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For UE-UE penetration loss, Table A.2.1-12 in TR 38.802 is used with modification on the criterion used to determine whether two indoor UEs are in the same building. |
| BS transmission power | 49 dBm or 53 dBm for 100 MHz | Baseline: 53 dBm for 100 MHz; Optional: 49 dBm for 100 MHz |
| Traffic | Full buffer (100% resource load) | FTP3 traffic model consisting of Poisson packet arrival process and packet size: - Option 1 (higher priority): 4Kbytes for DL and 1Kbyte for UL - Option 2 (higher priority): 0.5Mbyte for DL and 0.125 Mbytes for UL Loads: Low (5-10% RB utilization), medium (20-40%) and high (>50%) |
| BS Antenna configuration | Baseline: Reuse TR 38.828 antenna model as in 2.2.1.5 For legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | Companies to report. Recommendation: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization |
| BS antenna element gain | 5 dBi (assuming 1.8dB loss) | 8 dBi |
| UL scheduling and power control | Entire bandwidth scheduled to the UL user (for legacy TDD: 100 MHz, for SBFD {DUD} and {DU}: 20MHz). Transmit power selected to reach a SNR target of 15 dB | One or more UEs scheduled per slot. Open-loop power control where both the number of DL RBs and transmit power per RB is dynamically adjusted according to P0 and alpha parameters. |
| Mechanic tilt | 6° | For calibration: 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | Based on beamforming model in TR 38.828, Section 5.2.3.1 | For calibration: For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

## E.4.2 Simulation objectives in RAN1 and RAN4

RAN4 simulations are focused on coexistence analysis between two networks operating on adjacent channels. Specifically, RAN4 evaluated the impact of a SBFD network towards a (legacy) TDD network and vice versa. The RAN4 coexistence evaluation is conducted considering the different scenarios and cases listed in Annex E.1 in Table E.1-1 and Table E.1-2, respectively.

RAN1 conducted an extensive SBFD performance assessment with the main purpose of understanding SBFD performance benefits over legacy static TDD, as well as to investigate the potential of different techniques to combat the cross-link interference present in SBFD.  Four deployment cases are considered for evaluation in RAN1, as described in Section 7.1. It is worth noting that majority of simulated results are focusing on Deployment Case 1 with single-operator and all the cells using the same SBFD subband configuration, and Deployment Case 4 consisting of two operators with TDD and SBFD, respectively.

## E.4.3 Simulation methodology in RAN1 and RAN4

RAN4 simulations consists of multiple drops (with different UE positions, etc.) where each drop represents a ‘snapshot’ of the network at a particular time. For each drop, RAN4 simulation methodology is outlined in Section E.3.1. Note that RAN4 simulations are fully static (i.e. no time-variant processes, such as fast fading or UE movement, are modeled) so the obtained SINR and throughput statistics intend to capture long-term averaged SINR and throughput performance of the corresponding user. The reported throughput represents the upper bound based on the Shannon capacity as described in Section 4.3.1 . No HARQ or dynamic link adaptation is explicitly simulated.

RAN1 simulations are generally time-driven simulations which try to capture most of the system dynamics that have a big impact on the end-user performance. This includes dynamic traffic models resulting in time-varying load and interference conditions, as well as several radio resource management (RRM) functionalities such as CSI acquisition, HARQ retransmissions, packet scheduler, MIMO (including beamforming and precoding techniques) and link adaptation. For each simulated link, a stochastic 3D channel model is used to generate fast-fading components taking into account the angular and delay spreads of the channel, as specified in TR 38.901.  Given the dynamicity of the underlying system model, throughput and SINR vary over time for each simulated user.

## E.4.4 Modeling of adjacent-channel interference in RAN1 and RAN4

Editor's note: This section requires further updates.

## E.4.5 Simulation assumptions in RAN1 and RAN4 for respective scenarios

Table E.4.5-1 presents details comparison of RAN1 and RAN4 simulations results for Urban Macro scenarios for FR1 and FR2. Table E.4.5-2 presents details comparison of RAN1 and RAN4 simulations results for Indoor Office scenarios for FR1 and FR2.

Table E.4.5-1: Simulation assumptions in RAN1 and RAN4 for Urban Macro (UMa)

|  | RAN1 |  | RAN4 |  |
| --- | --- | --- | --- | --- |
|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) |
| Carrier frequency | 4 GHz | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz | 100MHz | 100MHz | Baseline: 200MHz Optional: 100MHz |
| Numerology | 14 OFDM symbol slot | 14 OFDM symbol slot | 14 OFDM symbol slot, SCS = 30kHz | 14 OFDM symbol slot, SCS = 120kHz |
|  | SCS = 30kHz | SCS = 120kHz |  |  |
| SBFD subband and slot configurations | Alt 1/2/4 in Annex A.7 are used for comparison between baseline legacy TDD network and SBFD deployment case 1. | Alt 1/2/4 in Annex A.7 are used for comparison between baseline legacy TDD network and SBFD deployment case 1. | - | - |
| BS ACLR | 45 dB | 28 dB | 45 dBc | 28 dBc |
| BS ACS | 46 dB | 23.5 dB | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | 24 dBc |
| UE ACLR | 30 dB | 23 dB | Note 1 Note 1: 1. UE ACLR is modelled as 30 dB at max power, and improves 1dB/dB with backoff up to a maximum 10 dB of improvement. So this means at 10 dB backoff the ACLR is 40 dB.(R4-2302977) | Note 1 Note 1: 2. FR2-1 ACLR mode for SBFD sims: 24 dB based value improved 1 dB/dB for up to 10 dB, similar approach as FR1.(R4-2302977) |
| UE ACS | 33 dB | 23 dB | 33 dBc | 23 dBc |
| BS self-interference isolation | corresponding approximately to 1 dB desense | corresponding approximately to 1 dB desense | Self-interference power is: Noise floor - 6dB (corresponding approximately to 1 dB desense) | Self-interference power is: Noise floor - 6dB (corresponding approximately to 1 dB desense) |
| BS Inter-sector interference isolation | • option 1: 75dB for spatial isolation (RAN4 typical value). • option 2: 93dB for spatial isolation (RAN4 best value). • option 3: 100dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. | • option 1: 88dB for spatial isolation (RAN4 typical value). • option 2: 98dB for spatial isolation (RAN4 best value). • option 3: 105dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. | Sum of all inter-sector gNB-gNB CLI per site correspond to Noise floor - 6 dB (corresponding approximately to 1 dB desense) | Sum of all inter-sector gNB-gNB CLI per site correspond to Noise floor - 6 dB (corresponding approximately to 1 dB desense) |
| BS transmit power for SBFD | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | Option 1: TDD: 49 dBm SBFD config 1: 46 dBm SBFD config 2: 49 dBm Option 2: TDD: 53 dBm SBFD config 1: 50 dBm SBFD config 2: 53 dBm | TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |  | SBFD config 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. SBFD config 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm | 23dBm | Peak EIRP should not exceed 22.4 dBm Note 5: Peak EIRP of 22.4 dBm is the baseline assumption, other higher EIRP is optional and companies could provide simulation results with statement of higher EIRP. |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  | UMa Single layer with 19 hexagonal cell with wrap around | UMa Single layer with 19 hexagonal cell with wrap around |
| Inter-BS (2D) distance | 500m | 200m | 500m | 200m |
| Minimum BS-UE (2D) distance | 35m | 35m | 35m | 35m |
| Minimum UE-UE (2D) distance | 1m | 1m | 3m | 3m |
| BS antenna height | 25m | 25m | 25m | 25m |
| UE distribution | UE Clustering | UE Clustering | Uniform | Uniform |
| UE number per macro/indoor TRP (per direction) (M) | 20 | 10 | - | - |
| UE cluster number per macro cell (X) | 2 | 1 | - | - |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 100% outdoor without car penetration loss: 3km/h | Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor | 0% indoor and 100% outdoor |
| Indoor UE height (m) | 1.5m | 1.5m | hUT=3(nfl-1)+1.5 nfl = 1 |  |
| Outdoor UE height (m) | 1.5m | 1.5m | hUT=3(nfl-1)+1.5 nfl~uniform(1,Nfl) where Nfl=1 |  |
| Radius of cluster (R) | 25m | 20m | - | - |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m | 55m | - | - |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m | - | - | - |
| gNB-UE Channel model | Macro-to-UE: UMa in TR 38.901 |  | Macro-to-UE: UMa in TR 38.803 |  |
|  | For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  | O2I penetration loss follows TR 38.803 |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) |  | Macro-to-Macro: UMa in TR 38.803 (hUE =25m) |  |
|  | LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  | Option 1: Reuse the same model as in TR 38.828 with h_UT equals to 25m; Option 2: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.828. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). |  | UE-to-UE: UMi see TR 36.828 + penetration loss see TR 38.803 o UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable. |  |
|  | For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  |  |  |
| BS antenna array configuration for Legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) | (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) | (M,N,P,Mg,Ng)=(8,8,2,1,1) | (M,N,P,Mg,Ng)=(8,16,2,1,1) |
|  | (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization | (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization | (dH,dV)=(0.5,0.8)λ | (dH,dV)=(0.5,0.5)λ |
| BS antenna array configuration for SBFD | - SBFD antenna configuration Option 2 (Method 2-1) | - SBFD antenna configuration Option 2 (Method 2-1) | (M,N,P,Mg,Ng) = (4, 8, 2, 1, 1) (dH,dV)= (0.5λ, 0.5λ) antenna element gain: 3 dBi (assuming 2dB loss) For SBFD antenna config-11: (2, 8, 2, 1, 1); (dH,dV)= (0.5λ, 0.5λ) For SBFD antenna config-22: (4, 8, 2, 1, 1); (dH,dV)= (0.5λ, 0.5λ) • Option 1: both SBFD and NR TDD gNB re-use TR 38.828 model without considering sub-arrays, detailed antenna configurations based on Section 5.2.1.5 for FR1. | (M,N,P, Mg,Ng) =(8, 16, 2, 1, 1) (dH,dV)= (0.5λ, 0.5λ) antenna element gain: 3 dBi (assuming 1.8dB loss) For SBFD antenna config-1: (4, 16, 2, 1, 1); (dH,dV)= (0.5λ, 0.5λ) For SBFD antenna config-2: (8, 16, 2, 1, 1); (dH,dV)= (0.5λ, 0.5λ) Option 1: For FR2, reuse the same as in 38.828 Section 5.2.2.5 for FR2 |
|  | n Two panel groups | n Two panel groups |  |  |
|  | n For each panel group: | n For each panel group: |  |  |
|  | (M,N,P,Mg,Ng) = (8,8,2,1,1). | (M,N,P,Mg,Ng) = (4,16,2,2,2). |  |  |
|  | n Number of TxRUs: same as legacy TDD | n Number of TxRUs: same as legacy TDD |  |  |
|  | (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |  |  |
| BS antenna radiation pattern | reuse Table 9 in Report ITU-R M.2412 | reuse Table 9 in Report ITU-R M.2412 | Both SBFD and NR TDD gNB reuse TR 38.828 model without considering sub-arrays | Both SBFD and NR TDD gNB reuse TR 38.828 model without considering sub-arrays |
| UE antenna configuration | l2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | Omnidirectional antenna | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1), (dH,dV) = (0.5,0.5)λ |
|  | l4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |  |  |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 | Omni-directional with 0 dBi element gain | Reuse TR 38.828 model |
| BS receiver noise figure | 5dB | 7 dB | 5 dB | 10 dB |
| UE receiver noise figure | 9 dB | 13 dB | 9 dB | 10 dB |
| Uplink Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -86 dBm, alpha = 0.9 | Transmission power control as specified in Section E.3.5 | Transmission power control as specified in Section E.3.5 |
| Handover margin (dB) | 3 dB | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. | Cell selection is based on RSRP | Cell selection is based on RSRP |
|  |  | l Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |  |  |
|  |  | l Single gNB panel is used for UE attachment |  |  |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 | Model-1 in clause 7.3.2 in TR 38.901 | - | - |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 6° | 6° |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") | - | - |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): | For direction of TRxP analog beam steering (in LCS): | - | - |
|  | Azimuth angle φi = 0 | Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} |  |  |
|  | Zenith angle θj = pi*102/180 | Zenith angle θj = {5*pi/8, 7*pi/8} |  |  |
|  | NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. | NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |  |  |
|  | Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |  |  |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): | - | - |
|  |  | Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; |  |  |
|  |  | Zenith angle θj = {pi/4, 3*pi/4}; |  |  |
|  |  | NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |  |  |
|  |  | Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |  |  |

Table E.4.5-2: Simulation assumptions in RAN1 and RAN4 for Indoor Office scenario

|  | RAN1 |  | RAN4 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Indoor office (FR1) | Indoor office (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| Carrier frequency | 4 GHz | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot | 14 OFDM symbol slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS = 30kHz | SCS = 120kHz | SCS = 30kHz | SCS = 120kHz |
| SBFD subband and slot configurations | Alt 1/2/4 in Annex A.7 are used for comparison between baseline legacy TDD network and SBFD deployment case 1. | Alt 1/2/4 in Annex A.7 are used for comparison between baseline legacy TDD network and SBFD deployment case 1. | - | - |
| BS ACLR | 45 dB | 28 dB | 45 dBc | 28 dBc |
| BS ACS | 46 dB | 23.5 dB | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | 24 dBc |
| UE ACLR | 30 dB | 23 dB | Note 1 Note 1: 1. UE ACLR is modelled as 30 dB at max power, and improves 1dB/dB with backoff up to a maximum 10 dB of improvement. So this means at 10 dB backoff the ACLR is 40 dB.(R4-2302977) | Note 1 Note 1: 2. FR2-1 ACLR mode for SBFD sims: 24 dB based value improved 1 dB/dB for up to 10 dB, similar approach as FR1.(R4-2302977) |
| UE ACS | 33 dB | 23 dB | 33 dBc | 23 dBc |
| BS self-interference isolation | corresponding approximately to 1 dB desense | corresponding approximately to 1 dB desense | Self-interference power is: Noise floor - 6dB (corresponding approximately to 1 dB desense) | Self-interference power is: Noise floor - 6dB (corresponding approximately to 1 dB desense) |
| BS Inter-sector interference isolation | • option 1: 75dB for spatial isolation (RAN4 typical value). • option 2: 93dB for spatial isolation (RAN4 best value). • option 3: 100dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. | • option 1: 88dB for spatial isolation (RAN4 typical value). • option 2: 98dB for spatial isolation (RAN4 best value). • option 3: 105dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. | Sum of all inter-sector gNB-gNB CLI per site correspond to Noise floor - 6 dB (corresponding approximately to 1 dB desense) | Sum of all inter-sector gNB-gNB CLI per site correspond to Noise floor - 6 dB (corresponding approximately to 1 dB desense) |
| BS transmit power for SBFD | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 23 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 24 dBm is assume for maximum BS transmit power for legacy TDD BS transmit power for SBFD For SBFD antenna configuration 1: 21 dBm For SBFD antenna configuration 2: 24 dBm | 24 dBm is assume for maximum BS transmit power for legacy TDD BS transmit power for SBFD For SBFD antenna configuration 1: 21 dBm For SBFD antenna configuration 2: 24 dBm |
|  |  |  | SBFD config 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. SBFD config 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |  |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm | 23 dBm | Peak EIRP should not exceed 22.4 dBm Note 5: Peak EIRP of 22.4 dBm is the baseline assumption, other higher EIRP is optional and companies could provide simulation results with statement of higher EIRP. |
| Layout | 12 TRPs per 120m x 50m x 3m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS. | 12 TRPs per 120m x 50m x 3m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS | Total 12 BSs (operator A: 6 BSs & operator B: 6 BSs) 120 m x 50 m. | Total 12 BSs (operator A: 6 BSs & operator B: 6 BSs) 120 m x 50 m |
| Inter-BS (2D) distance | 20m |  | 20m | 20m |
| Minimum BS-UE (2D) distance | 0m |  | 0m | 0m |
| Minimum UE-UE (2D) distance | 1m |  | 1m | 1m |
| BS antenna height | 3m |  | 3m | 3m |
| UE distribution | Uniform |  | Uniform | Uniform |
| UE number per macro/indoor TRP (per direction) (M) | 10 |  | - | - |
| UE cluster number per macro cell (X) | - |  | - | - |
| UE outdoor/indoor proportion | 100% indoor in houses: 3km/h |  | 100% indoor | 100% indoor |
| Indoor UE height (m) | 1.5m |  | 1.5m | 1.5m |
| Outdoor UE height (m) | - |  |  |  |
| Radius of cluster (R) | - |  | - | - |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | - |  | - | - |
| Minimum distance between two UE cluster centers (Dinter-cluster) | - |  | - | - |
| gNB-UE Channel model | TRP-to-UE: InH-Office in TR 38.901 |  | BS-to-UE: InH-office see TR 38.803 | BS-to-UE: InH-office see TR 38.803 |
|  | Penetration loss is not modelled. |  | Penetration loss is not modelled. | Penetration loss is not modelled. |
| gNB-gNB Channel model (large-scale) | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) |  | BS-to-BS: InH-office see TR 38.803 | BS-to-BS: InH-office see TR 38.803 |
|  | Penetration loss is not modelled. |  | Penetration loss is not modelled. | Penetration loss is not modelled. |
| UE-UE Channel model (large-scale) | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  | UE-to-UE: InH-office see TR 38.803 | UE-to-UE: InH-office see TR 38.803 |
| BS antenna array configuration for Legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) | (M,N,P,Mg,Ng;Mp,Np) = (16,8,2,1,1; 1,1) | (M,N,P,Mg,Ng)=(4,4,2,1,1) (dH,dV)=(0.5,0.5)λ | (M,N,P,Mg,Ng)=(4,8,2,1,1) (dH,dV)=(0.5,0.8)λ |
|  | (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization | (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization |  |  |
| BS antenna array configuration for SBFD | - SBFD antenna configuration Option 2 (Method 2-1) | - SBFD antenna configuration Option 2 (Method 2-1) |  For SBFD antenna config-1: (M,N,P,Mg,Ng)=(2,4,2,1,1) (dH,dV)=(0.5,0.5)λ  For SBFD antenna config 2: (M,N,P,Mg,Ng)=(4,4,2,1,1) (dH,dV)=(0.5,0.5)λ | • Baseline: Reuse TR 38.828 antenna model as in 2.2.1.5 For SBFD antenna configuration 1: (M,N,P,Mg,Ng)=(4,8,2,1,1) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 2: (M,N,P,Mg,Ng)= (8,8,2,1,1) (dH,dV)=(0.5,0.8)λ Note 1,2,3,4,5 • Optional: Extended AAS model Section 5.2.3.2.4 of TR 38.828 For legacy TDD: TBA For SBFD: TBA |
|  | n Two panel groups | n Two panel groups |  |  |
|  | n For each panel group: | n For each panel group: |  |  |
|  | (M,N,P,Mg,Ng) = (4,4,2,1,1). | (M,N,P,Mg,Ng) = (16,8,2,1,1). |  |  |
|  | n Number of TxRUs: same as legacy TDD | n Number of TxRUs: same as legacy TDD |  |  |
|  | (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |  |  |
| BS antenna radiation pattern | reuse Table 10 in Report ITU-R M.2412 |  | Both SBFD and NR TDD gNB reuse TR 38.828 model | Both SBFD and NR TDD gNB reuse TR 38.828 model |
| UE antenna configuration | l 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | Omnidirectional antenna | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1), (dH,dV) = (0.5,0.5)λ |
|  | l 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |  |  |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 | Omnidirectional antenna | Reuse TR 38.828 model |
| BS receiver noise figure | 5dB | 7 dB | 5 dB | 10 dB |
| UE receiver noise figure | 9 dB | 13 dB | 9 dB | 10 dB |
| Open loop power control parameters | P0= -60 dBm, alpha = 0.6 |  | Transmission power control as specified in Section E.3.5 | Transmission power control as specified in Section E.3.5 |
| Handover margin (dB) | 3 dB | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. | Cell selection is based on RSRP | Cell selection is based on RSRP |
|  |  | l Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |  |  |
|  |  | l Single gNB panel is used for UE attachment |  |  |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 | Model-1 in clause 7.3.2 in TR 38.901 | - | - |
| Mechanic tilt | 180° in GCS (pointing to the ground) | 180° in GCS (pointing to the ground) | 90° (pointing to the ground) | 90° (pointing to the ground) |
| Electronic tilt | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") | - | - |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | - | For direction of TRxP analog beam steering (in LCS): | - | - |
|  |  | Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} | - | - |
|  |  | Zenith angle θj = {pi/4, 3*pi/4} | - | - |
|  |  |  | - | - |
|  |  | NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. | - | - |
|  |  | Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | - |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): | - | - |
|  |  | Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; | - | - |
|  |  | Zenith angle θj = {pi/4, 3*pi/4}; | - | - |
|  |  |  | - | - |
|  |  | NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. | - | - |
|  |  | Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | - |

<End of TP to TR 38.858>