# 3GPP TSG-RAN WG4 Meeting # 108bis														R4-2315810

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for FR2-1 UL 256QAM MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-Core |  |  |  |  |  | Date: |  |  | 2023-09-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce FR2-1 UL 256QAM MPR requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce FR2-1 UL 256QAM MPR requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2-1 UL 256QAM MPR requirements are not available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

6.2.2	UE maximum output power reduction

6.2.2.0	General

The requirements in clause 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in clauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in clause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in clause 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

6.2.2.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as:

MPR = max(MPRWT+△MPR, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.1-1 and 6.2.2.1-2 for FR2-1 and in Tables 6.2.2.1-3 and 6.2.2.1-4 for FR2-2.

Table 6.2.2.1-1 MPRWT for power class 1, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
|  | 256QAM | ≤ 11.0 | ≤ 11.0 | ≤ 11.0 |

Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 MHz

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |
|  | 256QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 |
|  | 256QAM | ≤ 12.5 | ≤ 12.5 | ≤ 12.5 |

Table 6.2.2.1-3 MPRWT for power class 1, BWchannel = 100 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 100 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 2.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |

Table 6.2.2.1-4 MPRWT for power class 1, BWchannel >= 400 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 400, 800, 1600, 2000 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.0 | ≤ 1.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.0 | ≤ 1.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

Table 6.2.2.1-5 △MPR for power class 1, FR2-1

| Modulation |  | Band | △MPR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | BWchannel ≤ 200 MHz | BWchannel = 400 MHz |
| DFT-s-OFDM | 256QAM | n257, n258, n261 | 1.0 | 1.0 |
|  |  | n259, n260 | 3.5 | 3.5 |
| CP-OFDM | 256QAM | n257, n258, n261 | 0.5 | 0.5 |
|  |  | n259, n260 | 3.0 | 3.0 |

Where the following parameters are defined to specify valid RB allocation ranges for the RB allocations regions in Tables 6.2.2.1-1 ,  6.2.2.1-2, 6.2.2.1-3, and 6.2.2.1-4:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

RBStart,Low = Max(1, Floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

An RB allocation is an Outer RB allocation if

RBStart < RBStart,Low OR RBStart > RBStart,High OR LCRB > Ceil(NRB/2)

An RB allocation belonging to table 6.2.2.1-1 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB)

An RB allocation belonging to table 6.2.2.1-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

An RB allocation is a Region 2 inner allocation if it is NOT an Outer allocation AND NOT a Region 1 inner allocation

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.4 apply.

6.2.2.2	UE maximum output power reduction for power class 2

For power class 2, MPR for FR2-1 and FR2-2 as specified in clause 6.2.2.3 applies.

Table 6.2.2.2-1: Void

6.2.2.3	UE maximum output power reduction for power class 3

For power class 3, MPR for contiguous allocations is defined as:

MPR = max(MPRWT+△MPR, MPRnarrow)

For transmission bandwidth configuration less than or equal to 200MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil((2/3NRB)- LCRB) < RBstart ≤ NRB-LCRB:

-	MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz,

-	MPRnarrow = 2.0 dB, when 1.44 MHz < BWalloc,RB <= 4.32 MHz,

-	otherwise MPRnarrow = 0 dB.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined for FR2-1 in Table 6.2.2.3-1.

Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
|  | 256QAM | TBD | TBD |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |
|  | 256QAM | TBD | TBD |
| Note 1: MPRs for 256QAM are applicable for power class 2 and 5 only. |  |  |  |

MPRWT is defined for FR2-2 in Table 6.2.2.3-1b.

Table 6.2.2.3-1b MPRWT for power class 3, BWchannel = 100 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 100 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 0.5 |
|  | QPSK | 0.0 | ≤ 0.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 8.5 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 1.5 | ≤ 1.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2.3-1:

-	RBStart,Low = max(1, LCRB), where max() indicates the largest value of all arguments.

-	RBStart,High = NRB – RBStart,Low – LCRB,

An RB allocation belonging to table 6.2.2.3-1 is a Region 1 inner RB allocation if:

-	RBStart,Low  ≤  RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(NRB/3), where ceil(x) is the smallest integer greater than or equal to x.

For transmission bandwidth configuration equal to 400MHz,

MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined for FR2-1 in Table 6.2.2.3-2.

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 256QAM | TBD | TBD |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |
|  | 256QAM | TBD | TBD |
| Note 1: MPRs for 256QAM are applicable for power class 2 and 5 only. |  |  |  |

MPRWT is defined for FR2-2 in Table 6.2.2.3-2b and 6.2.2.3-2c.

Table 6.2.2.3-2b MPRWT for power class 3, BWchannel = 400 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 | ≤ 3.0 |
|  | QPSK | ≤ 1.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.0 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Table 6.2.2.3-2c MPRWT for power class 3, BWchannel >= 800 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 | ≤ 4.0 |
|  | QPSK | ≤ 1.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.0 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 8.0 |
|  | 64 QAM | ≤ 10.5 | ≤ 10.5 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2.3-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

An RB allocation belonging to table 6.2.2.3-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

For all transmission bandwidth configurations, an RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation.

Table 6.2.2.3-3 △MPR for power class 2,4,5, FR2-1

| Modulation |  | Band | △MPR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | BWchannel ≤ 200 MHz | BWchannel = 400 MHz |
| DFT-s-OFDM | 256QAM | n257, n258, n261 | TBD | TBD |
|  |  | n259, n260 | TBD | TBD |
| CP-OFDM | 256QAM | n257, n258, n261 | TBD | TBD |
|  |  | n259, n260 | TBD | TBD |

6.2.2.4	UE maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2.2.3 applies.

Table 6.2.2.4-1: Void

6.2.2.5	UE maximum output power reduction for power class 5

For power class 5, MPR specified in sub-clause 6.2.2.3 applies.

< end of changes >