3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315832

# Xiamen, China, October 9th– 13th, 2023

Source: 	vivo

Title: 	Remaining issues of lower MSD Signalling

Agenda Item:	5.3.1.4.1

Document for:	Discussion

Introduction

In RAN4#108, significant progress was made on lower MSD signalling including the lower MSD capability class mapping table, and the basic testing criteria. The main progress was documented in the WF [1] and also sent out in the LS [2]. The detailed discussion contents and status can reference to the topic summary in [3].

In this paper, some views and proposals are provided. Particularly, one possible scheme for signalling overhead reduction is raised.

Discussion

Signaling overhead reduction

Though signalling is generally regarded as RAN2 responsibility, and the overhead reduction technique would be most probably defined in RAN2, this kind of scheme is still quite RAN4 related and RAN4’s clear recommendations is needed.

The schemes have been discussed for several meetings, and various schemes had been raised without conclusion. One general way is as following:

“Option 1: allow gNB query UE capability and UE only report certain capability filtered by gNB’s query information.”

In addition, the already agreed TP [4] has been included in TR 38.881 [5] clause 7.1.2.2.3 for some time.

It is believed this is a very flexible and effective scheme, and already receive many supports such as in [6], and related discussions are already started in RAN2.

Proposal 1: RAN4 should provide clear recommendations to RAN2 about the signalling overhead reduction schemes. Among the candidates, the scheme based on gNB query and UE only report certain capability filtered is preferred.

Up till now, the following condition has been introduced into the TR:

“The filter conditions may include: frequency band(s), MSD source (such as IMD order), power class, frequency range, and etc.”

In last RAN4 meeting, the following Lower MSD capability classes has been agreed in the WF [1] which is also listed as following:

| Index | Maximum allowed actual MSD (i.e. Thresholds) | Lower MSD Capability classes | Note |
| --- | --- | --- | --- |
| 0 | 0dB | Ⅰ | No degradation |
| 1 | 3 dB | Ⅱ | Actual MSD ≤ 3dB |
| 2 | 6 dB | Ⅲ | Actual MSD ≤ 6dB |
| 3 | 9 dB | IV | Actual MSD ≤ 9dB |
| 4 | 12 dB | Ⅴ | Actual MSD ≤ 12dB |
| 5 | 15 dB | Ⅵ | Actual MSD ≤ 15dB |
| 6 | 18 dB | Ⅶ | Actual MSD ≤ 18dB |
| 7 | 22dB | Ⅷ | Actual MSD ≤ 22dB |

However, there are still views that this granularity is much finer than the network really needs, depending on how to use this capability by the network. For example, the network may simply use one MSD value as a threshold to set different scheduling method for a particular band / BC, and some minor refinements such as a few dB in relative higher region, e.g 18dB -> 15 or 12 dB, may not that meaningful for the network. In this case, even the UE may be qualified for “Lower MSD” in a certain scenario including band/MSD type etc, the network may simply treat this UE the same to a UE which only satisfy minimum MSD requirement, and the signalling of Lower MSD for this particular scenario, might actually not be needed by the network.

Observation 1: Depending on how the network would utilize Lower MSD capability class, the lower MSD capable UE for a certain scenario may not be differentiated by the network with UE only satisfy minimum MSD requirements, particularly if the lower MSD capability class is relatively large.

In this condition, it is proposed to add the network preferred minimum Lower MSD capability class as another filter parameter for reducing capability reporting. For example, if network would only consider a maximum allowed 6dB MSD for harmonic interference for n3-n78 harmonics interference for n78, it can consider using the lower MSD capability class as a filtering parameter, and set up the maximum allowed report class as III (index 2), which corresponds to a maximum allowed actual MSD of 6dB. In this case, the UE would report the lower MSD class for this scenario if it can satisfy class I or II or III. If the UE support lower MSD but can only achieve lower MSD class larger than III, the UE would not report the capability since anyway the network would treat it the same to UE only support minimum requirements.

In this way, the signalling overhead is saved based on network needs. Though the UE may lose the opportunity to report in the condition, those reporting would anyway be meaningless since network would no use it and explicitly indicted. Furthermore, the whole scheme is well controlled by the network.

Proposal 2: Add the lower MSD class as a filter coefficient, for network to indicate UE a minimum preferred class. For UE with lower MSD class larger than preferred by network, the corresponding signalling of lower MSD support would not be reported by UE to reduce signalling overhead.

A draft TP is also provided in this meeting in [7] based on the previous contents.

lower MSD capability for higher order combination

In last meeting WF, it is still FFS whether further clarification on RAN4’s assumptions on MSD requirements for higher order combinations would be needed to send to RAN2.

After discussion, it was found that RAN4 do not have any real updated technical solution for this. RAN2 was also not asking this question for further feedback, and the related discussion is already on going. So it is believed that no need to do further feedback unless explicitly asked to do. The option 1 from the WF of this issue is supported:

Proposal 3: No need to change or discuss what RAN4 agreed before for lower MSD capability for higher order combination issue, i.e., “lower MSD capability for higher order combination is inherited from lower order fallback combinations”, unless RAN4 receives a LS from RAN2 to ask RAN4 to take a specific action on the agreement.

Others

In last meeting, also there are some more schemes were raised which is not consistent with previous agreements, such as MSD type / MSD order etc. Since the current agreements come from long discussion and the benefit is not clear for multiple sets of different overlapping schemes such as MSD type/order definition for lower MSD, it is proposed to not consider proposals anymore in contradict with current agreements.

Proposal 4: No to consider different overlapping schemes, e.g. new lower MSD type/order definition, in this release.

Conclusion

In this paper, some views were provided for some of the remaining issues. Particularly, one possible scheme for signaling overhead reduction is raised.

Proposal 1: RAN4 should provide clear recommendations to RAN2 about the signalling overhead reduction schemes. Among the candidates, the scheme based on gNB query and UE only report certain capability filtered is preferred.

Observation 1: Depending on how the network would utilize Lower MSD capability class, the lower MSD capable UE for a certain scenario may not be differentiated by the network with UE only satisfy minimum MSD requirements, particularly if the lower MSD capability class is relatively large.

Proposal 2: Add the lower MSD class as a filter coefficient, for network to indicate UE a preferred class. For UE with lower MSD class larger than preferred by network, the corresponding signalling of lower MSD support would not be reported by UE to reduce signalling overhead.

A draft TP is also provided in this meeting in [7] based on the previous contents.

Proposal 3: No need to change or discuss what RAN4 agreed before for lower MSD capability for higher order combination issue, i.e., “lower MSD capability for higher order combination is inherited from lower order fallback combinations”, unless RAN4 receives a LS from RAN2 to ask RAN4 to take a specific action on the agreement.

Proposal 4: No to consider different overlapping schemes, e.g. new lower MSD type/order definition, in this release.

- References

[1] R4-2314923, WF on the lower MSD, Huawei, RAN4#108

[2] R4-2312247, draft LS on lower MSD, Huawei, RAN4#108

[3] R4-2314208, Topic summary for [108][126] FR1_enh2_part1, Moderator (Huawei), RAN4#108

[4] R4-2306593, TP for TR 38.881 on the signalling design for low-MSD capability, Huawei, HiSilicon, RAN4#106bis-e

[5] TR 38.881, v0.6.0

[6] R4-2312554, Discussion of lower MSD Signalling, vivo, RAN4#108

[7] R4-2315833, TP for TR 38.881 on a possible scheme for signaling overhead reduction, vivo, RAN4#108bis