3GPP TSG-RAN WG4 Meeting #108bis                                                       R4-2315853

Xiamen, China, 9th – 13th Oct. 2023

Agenda Item:	5.13.7.2

Source:	Ericsson

Title:	On UE demodulation requirements and applicability for ATG network

Document for:	Discussion

# 1	Introduction

During the last RAN4 #108 meeting’s discussion, following agreements have been made based on the agreed WF [1]:

Issue 2-1-1: MCS Agreement: - Consider MCS24 (Table-2) for defining requirements for 256QAM Issue 2-1-2: Test scope for PDSCH Agreement: - For new incremental requirements: - introduces following test cases for new PDSCH requirements FDD 10 MHz 15kHz SCS; 2T2R, 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). TDD 40MHz 30kHz SCS; 7D1S2U & New TDD pattern for ATG specific scenario e.g. flexible TDD configuration or 30D4S6U(if introduced); 2T2R, 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). Issue 2-3-1: Test scope for CSI reporting Agreement: - For CQI requirements, reuse the following requirements: - 2T2R FDD: CQI requirements in 6.2.2.1.1.1 - 2T2R TDD: CQI requirements in 6.2.2.2.1.1 - 2T4R FDD: CQI requirements in 6.2.3.1.1.1 - 2T4R TDD: CQI requirements in 6.2.3.2.1.1 - Not introduce RI requirements for ATG UE

Regarding to the left open issues, it is not determined whether to select the legacy PDSCH/PDCCH requirements for ATG scenario. Meanwhile, it is still FFS on whether to consider PMI requirements.

In this contribution, we provide our views on the left open issues above and give our proposals on the test applicability rules.

# 2	Discussion

## 2.1 Test cases for PDSCH, PDCCH, PBCH and CSI reporting

PDSCH test case

Companies agreed during the RAN4# 107meeting that for PDSCH, there will be new test cases and new requirements. In addition, we will also choose 1 test case per Modulation Order for PDSCH Mapping Type A only, for FDD and TDD; For PDCCH, there will be no new test cases and requirements, but to select some legacy test cases. Following agreements are captured in the WF: R4-2309796,

Issue 2-1-1: Test scope for PDSCH - New PDSCH test cases for ATG with FDD slot pattern can be as follows. FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - New PDSCH test cases with static TDD slot pattern can be as follows (if introduced): TDD 40MHz 30kHz SCS; FFS on new TDD pattern: 30D4S6U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - For existing PDSCH test cases: - Choose 1 test case per Modulation Order for PDSCH Mapping Type A only, for FDD and TDD, FFS on which test cases to choose. - Consider Single Carrier requirements only

With the respect towards previous agreements, we confirm our support on all above.

For the test cases of PDSCH, we agreed in the last meeting to consider the following new cases for FDD:

FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2).

For TDD, however, we haven’t reached agreement on whether to have new test cases for the new TDD pattern (30D4S6U). We propose to consider using legacy TDD pattern with the following new test cases until it is agreed to define new cases for the new TDD pattern:

TDD 40MHz 30kHz SCS; 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2).

As for the legacy requirements for PDSCH, based on the agreements to choose 1 test case per Modulation Order for PDSCH Mapping Type A only for FDD and TDD, we propose to select following test cases:

- Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD)

- Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD)

- Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD)

- Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD)

Proposal 1: Define following new test cases for PDSCH for ATG UEs:

FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2).

TDD 40MHz 30kHz SCS; 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2).

Select following PDSCH legacy test cases for ATG UEs:

- Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD)

- Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD)

- Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD)

- Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD)

PDCCH test cases

For PDCCH requirements, we have the following proposal:

Proposal 2: Select following PDCCH legacy test cases for ATG UEs:

- 2T2R FDD: All test cases in 5.3.2.1.2

- 2T4R FDD: All test cases in 5.3.3.1.2

- 2T2R TDD: All test cases in 5.3.2.2.2

- 2T4R TDD: All test cases in 5.3.3.2.2

PBCH test cases

Considering the ATG UE is a new type of UE, it should be specified whether there are PBCH requirements need to be passed. We propose to reuse legacy PBCH test cases for ATG UEs and not to define new requirements.

Proposal 3: Select following legacy PBCH test cases for ATG UE:

- Test num 1 in table 5.4.2.1-2 (1T2R, FDD)

- Test num 1 in table 5.4.2.1-3 (1T2R, FDD)

- Test num 1 in table 5.4.2.2-2 (1T2R, TDD)

- Test num 1 in table 5.4.2.2-3 (1T2R, TDD)

- Test num 1 in table 5.4.3.1-2 (1T4R, FDD)

- Test num 1 in table 5.4.3.1-3 (1T4R, FDD)

- Test num 1 in table 5.4.3.2-2 (1T4R, TDD)

- Test num 1 in table 5.4.3.2-3 (1T4R, TDD)

SDR test cases

Same consideration as PBCH test cases. For the SDR requirement, we propose not to consider it since there is no CA scenario for ATG UEs.

Proposal 4: Not to consider SDR requirement for ATG UE

CSI reporting test cases

Regarding to the CSI reporting test cases, companies reached the following agreements:

Issue 2-3-1: Test scope for CSI reporting Agreement: - For CQI requirements, reuse the following requirements: - 2T2R FDD: CQI requirements in 6.2.2.1.1.1 - 2T2R TDD: CQI requirements in 6.2.2.2.1.1 - 2T4R FDD: CQI requirements in 6.2.3.1.1.1 - 2T4R TDD: CQI requirements in 6.2.3.2.1.1 - Not introduce RI requirements for ATG UE - FFS for PMI requirements

It is FFS whether to have PMI requirements. We propose not to consider it because the PMI report may not be useful given the channel condition is AWGN.

Proposal 5: Not to consider PMI requirements for ATG UE

## 2.2 Test applicability rule

ATG is a new deployment scenario which will have newly introduced UE demodulation requirements. Therefore, we should define the corresponding applicability rule.

For PDSCH, as the selected legacy test cases are based on the fading channel, we think it is reasonable to skip the newly define AWGN based requirements.

For PDCCH and CQI reporting tests, selected legacy requirements shall be met.

Hence, we propose the following test applicability rule for PDSCH, PDCCH and CSI reporting tests for ATG UE with different capabilities:

Proposal 6: Proposed tests applicability rule:

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.1.1.x (New tests) Test num 1-3, 1-4 in Table 5.2.2.1.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.2.1.1-3 are passed, all tests in 5.2.1.1.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.2.1.2 |  |
|  |  | PBCH | Test num 1 in Table 5.4.2.1-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.2.1.1.1 |  |
|  | FR1 TDD | PDSCH | Tests in Clause 5.2.1.2.x (New tests) Test num 1-3, 1-4 in Table 5.2.2.2.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.2.1.1-3 are passed, all tests in 5.2.1.1.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.2.2.2 |  |
|  |  | PBCH | Test num 1 in Table 5.4.2.2-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.2.2.1.1 |  |
| ATG UE with 4RX | FR1 FDD | PDSCH | Tests in Clause 5.2.2.1.x (New tests) Test num 1-3, 1-4 in Table 5.2.3.1.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.3.1.1-3 are passed, all tests in 5.2.2.1.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.3.1.2 |  |
|  |  | PBCH | Test num 1 in Tabble 5.4.3.1-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.3.1.1.1 |  |
|  | FR1 TDD | PDSCH | Tests in Clause 5.2.2.2.x (New tests) Test num 1-3, 1-4 in Table 5.2.3.2.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.3.2.1-3 are passed, all tests in 5.2.2.2.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.3.2.2 |  |
|  |  | PBCH | Test num 1 in Table 5.4.3.2-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.3.2.1.1 |  |

# 3	Summary

In summary, we shared our views on the test cases for PDSCH, PDCCH and CSI reporting requirements along with the applicability rule. Therefore, we propose the following:

Proposal 1: Define following new test cases for PDSCH for ATG UEs:

FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2).

TDD 40MHz 30kHz SCS; 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2).

Select following PDSCH legacy test cases for ATG UEs:

- Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD)

- Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD)

- Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD)

- Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD)

Proposal 2: Select following PDCCH legacy test cases for ATG UEs:

- 2T2R FDD: All test cases in 5.3.2.1.2

- 2T4R FDD: All test cases in 5.3.3.1.2

- 2T2R TDD: All test cases in 5.3.2.2.2

- 2T4R TDD: All test cases in 5.3.3.2.2

Proposal 3: Select following legacy PBCH test cases for ATG UEs:

- Test num 1 in table 5.4.2.1-2 (1T2R, FDD)

- Test num 1 in table 5.4.2.1-3 (1T2R, FDD)

- Test num 1 in table 5.4.2.2-2 (1T2R, TDD)

- Test num 1 in table 5.4.2.2-3 (1T2R, TDD)

- Test num 1 in table 5.4.3.1-2 (1T4R, FDD)

- Test num 1 in table 5.4.3.1-3 (1T4R, FDD)

- Test num 1 in table 5.4.3.2-2 (1T4R, TDD)

- Test num 1 in table 5.4.3.2-3 (1T4R, TDD)

Proposal 4: Not to consider SDR requirement for ATG UEs

Proposal 5: Not to consider PMI requirements for ATG UEs

Proposal 6: Propose applicability rule as described above in the section 2.2

# References

[1]	R4-2313875, WF for ATG demodulation requirements, CMCC, RAN4 #108, Toulouse