3GPP TSG-RAN WG4 Meeting #108bis                                                       R4-2315854

Xiamen, China, 9th – 13th Oct. 2023

Agenda Item:	5.13.7.2

Source:	Ericsson

Title:	Updated simulation results for ATG PDSCH demodulation requirements

Document for:	Information

# 1	Introduction

In this contribution, we updated and resubmit our simulation results for all the test cases listed in the result collection sheet [1].

# 2	Discussion

## 2.1 Assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

- AWGN + Doppler for the channel model

  - Channel matrix using:

H = [1 j; 1 -j]

- koffset value = 2ms

- rank = 1

- DMRS: 1 front loaded + 1 additional DMRS

- TDD pattern:

  - For test 2-11, new TDD pattern: 30D4S6U

  - For rest of tests: Normal TDD pattern: 7D1S2U

## 2.2 Simulation result

Here please find our updated simulation results:

Table 2.2-1 PDSCH results for FDD and TDD

| Test number | Duplex mode, SCS and CBW | Propogation condition | Antenna configuration | MCS | Ideal |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz/10MHz | AWGN+220Hz Doppler | 2T2R | 16QAM Table 1 MCS13 | 3,37 |
| 2 |  |  |  | 64QAM Table 1 MCS 22 | 10,44 |
| 3 |  |  |  | 256QAM Table 2 MCS 20 | 15,50 |
| 4 |  |  |  | 256QAM Table 2 MCS 24 | 19,40 |
| 5 |  |  |  | 256QAM Table 2 MCS 27 | 22,52 |
| 6 |  |  | 2T4R | 16QAM Table 1 MCS13 | 0,40 |
| 7 |  |  |  | 64QAM Table 1 MCS 22 | 7,41 |
| 8 |  |  |  | 256QAM Table 2 MCS 20 | 12,48 |
| 9 |  |  |  | 256QAM Table 2 MCS 24 | 16,40 |
| 10 |  |  |  | 256QAM Table 2 MCS 27 | 19,50 |
| 11 | TDD 30kHz/40MHz 7DS2U | AWGN+500Hz Doppler | 2T2R | 16QAM Table 1 MCS13 | 3,39 |
| 12 |  |  |  | 64QAM Table 1 MCS 22 | 10,44 |
| 13 |  |  |  | 256QAM Table 2 MCS 20 | 15,41 |
| 14 |  |  |  | 256QAM Table 2 MCS 24 | 19,41 |
| 15 |  |  |  | 256QAM Table 2 MCS 27 | 22,55 |
| 16 |  |  | 2T4R | 16QAM Table 1 MCS13 | 0,41 |
| 17 |  |  |  | 64QAM Table 1 MCS 22 | 7,41 |
| 18 |  |  |  | 256QAM Table 2 MCS 20 | 12,41 |
| 19 |  |  |  | 256QAM Table 2 MCS 24 | 16,41 |
| 20 |  |  |  | 256QAM Table 2 MCS 27 | 19,48 |
| 21 | TDD 30kHz/40MHz 30D4S6U | AWGN+500Hz Doppler | 2T2R | 16QAM Table 1 MCS13 | 3,39 |
| 22 |  |  |  | 64QAM Table 1 MCS 22 | 10,45 |
| 23 |  |  |  | 256QAM Table 2 MCS 20 | 15,41 |
| 24 |  |  |  | 256QAM Table 2 MCS 24 | 19,41 |
| 25 |  |  |  | 256QAM Table 2 MCS 27 | 22,54 |
| 26 |  |  | 2T4R | 16QAM Table 1 MCS13 | 0,41 |
| 27 |  |  |  | 64QAM Table 1 MCS 22 | 7,41 |
| 28 |  |  |  | 256QAM Table 2 MCS 20 | 12,41 |
| 29 |  |  |  | 256QAM Table 2 MCS 24 | 16,41 |
| 30 |  |  |  | 256QAM Table 2 MCS 27 | 19,48 |

# 3	Summary

In this contribution, we updated and resubmit our simulation results for all the test cases listed in the result collection sheet [1].

# References

[1]	R4-2311795: Summary of simulation results for ATG UE and BS demodulation requirements, CMCC, RAN4 #108, Toulouse

[2]          R4-2313875: WF for ATG demodulation requirements, CMCC, RAN4 #108, Toulouse