| 3GPP TR 38.891 V0.7.0 (2023-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) for Rel-18 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	Study for FR2 UL 256QAM	7

5.1	General	7

5.2	Simulation	7

5.2.1	System level simulation	7

5.2.2	Link level simulation	7

5.2.2.1               Simulation assumptions	8

5.2.3	MPR simulation	8

5.3	Implementation impact for UE	9

5.4	Specification impact	9

5.5	Conclusion	9

6	Study for beam correspondence	9

6.1	General	9

6.2	Implementation impact to UE	9

6.3	Specification Impact	9

6.3.1	RF	9

6.3.2	RRM	9

6.4	Testing impact	9

6.5	Conclusion	9

Annex <A> (informative): Change history	10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document captures finding of the study on FR2 UL 256QAM and beam correspondence for RRC_INACTIVE and initial access. The purpose of this TR is to study the performance gain and implementation aspects for FR2 UL 256QAM and to ensure good UE RACH msg1 performance and UL coverage in FR2 deployments by specifying the beam correspondence requirements for RRC_INACTIVE and initial access as defined in Work Item “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” [2].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-220967, “new WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi

[3]                       3GPP TR 38.803: "Study on new radio access technology: RF and co-existence aspects".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Background

At the 3GPP RAN #95 meeting, the Work Item on “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” was approved for Rel-18. The objectives of the core part in the Work Item are as follows:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

Beam correspondence requirements for RRC_INACTIVE and initial access

- Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

- For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

  - Requirements for other transmission within RRC_INACTIVE state are not precluded.

- For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least)

Study the potential impact on testability aspects (i.e., test time).

This TR aims to study the objectives and further capture the impact on the specification if applicable.

5	Study for FR2 UL 256QAM

Editor’s note: This clause collects the simulation results and implementation aspects for FR2 UL 256QAM study for Rel-18. If necessary, new agenda can be assigned.

5.1	General

5.2	Simulation

5.2.1	System level simulation

The system level simulation as supplementary for link level simulation is to check whether the UE working on FR2 UL 256QAM can achieve target SNR at BS side and to further confirm FR2 UL 256QAM is feasible.

5.2.1.1   Simulation assumptions

Table 5.2.1.1-1 System level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | Uma LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH – Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz, 39GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | First priority: PC1/PC2/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi Second priority: PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| Target SNR at BS side |  | 28dB for 29GHz 30dB for 39GHz |  |
| UE max output power |  | PC1: 35 dBm/PC2: 23dBm/PC3: 23 dBm/PC5: 23 dBm |  |
| UE Min peak EIRP |  | n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm |  |
| Power control |  | Power control parameters set to achieve the target SNR at BS side. |  |

5.2.1.2   Simulation results from Nokia

R4-2218327

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in urban macro scenario (with the target SNR at BS side of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) are shown in figures 5.2.1.2-1 and 5.2.1.2-2, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-1. Urban macro simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-2. Urban macro simulation results at 39 GHz (n260)

It can be observed from figures 5.2.1.2-1 and 5.2.1.2-2 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE.

-	~15% of PC1 UE and ~25% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in indoor scenario (with the target SNR at BS side of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) are shown in figures 5.2.1.2-3 and 5.2.1.2-4, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-3. Indoor simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-4. Indoor simulation results at 39 GHz (n260)

It can be observed from figures 5.2.1.2-3 and 5.2.1.2-4 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

R4-2300193

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in urban macro scenario (with the target SNR at BS side of 27 dB at both 29 GHz and 39 GHz) are shown in figures 5.2.1.2-5 and 5.2.1.2-6, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-5. Urban macro simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-6. Urban macro simulation results at 39 GHz (n260)

It can be observed from figures 5.2.1.2-5 and 5.2.1.2-6 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side of 27 dB can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE.

-	Less than 15% of PC1 UE and less than 30% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in indoor scenario (with the target SNR at BS side of 27 dB at both 29 GHz and 39 GHz) are shown in figures 5.2.1.2-7 and 5.2.1.2-8, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-7. Indoor simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-8. Indoor simulation results at 39 GHz (n260)

Note that the results for PC1 and PC2/5 UE are alike in figures 5.2.1.2-7 and 5.2.1.2-8 as the higher PC1 UE transmit power is not required in indoor scenario. It can be observed from figures 5.2.1.2-7 and 5.2.1.2-8 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side of 27 dB can be achieved by ~70% of PC1 UE and PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

R4-2307231

The UE transmit power and UL SINR CDF at 39 GHz (n260) using the agreed parameters in urban macro scenario (with the target SNR at BS side of 30 dB) are shown in figure 5.2.1.2-9.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-9. Urban macro simulation results at 39 GHz (n260)

It can be observed from figure 5.2.1.2-9 that at 39 GHz (n260):

-	The target SNR at BS side of 30 dB can be achieved by ~60% of PC1 UE and ~50% of PC2/5 UE.

-	~18% of PC1 UE and ~32% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 39 GHz (n260) using the agreed parameters in indoor scenario (with the target SNR at BS side of 30 dB) are shown in figure 5.2.1.2-10.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-10. Indoor simulation results at 39 GHz (n260)

It can be observed from figure 5.2.1.2-10 that at 39 GHz (n260):

-	The target SNR at BS side of 30 dB can be achieved by ~60% of PC1 UE and PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

Therefore, from the observations above, it can be concluded:

-	In all simulated scenarios, a significant percentage of PC1/PC2/PC5 UE can achieve the target SNR at BS side above which 256QAM provides throughput gain over 64QAM at both 29 GHz (n257) and 39 GHz (n260).

5.2.1.3   Simulation results from ZTE

R4-2218595

In order to check whether the UE working on FR2 UL 256QAM can achieve target SNR at BS side we provide some system level simulation results with target SNR of 31dB and proposed Min peak EIRP. Figure 5.2.1.3-1~figure 5.2.1.3-4 show the CDF of SINR for PC1, PC2, PC3 and PC5 in band n257.

Figure 5.2.1.3-1, CDF of SINR for PC1 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-2, CDF of SINR for PC2 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-3, CDF of SINR for PC3 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-4, CDF of SINR for PC5 under Urban Macro with Fc = 29GHz

From figure 5.2.1.3-1~figure 5.2.1.3-4 we can find that 5% UE can achieve above 30dB SINR which show that PC1, PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase, for example, ~10% UE can achieve above 28dB SINR even for PC3.

Observation 3: PC1, PC2, PC3 and PC5 are feasible for UL 256QAM for 29GHz if the operating SNR is not higher than 28dB.

R4-2301235

In last meeting, we provided SLS results for UL 256QAM for PC1/2/3/5 UEs of 29GHz in R4-2218595, in terms of the simulations, which are shown below:

Figure 5.2.1.3-5, CDF of SINR for PC2 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-6, CDF of SINR for PC3 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-7, CDF of SINR for PC5 under Urban Macro with Fc = 29GHz

From figure 5.2.1.3-5, 5.2.1.3-6 and 5.2.1.3-7, we can find that 5% UE can achieve above 30dB SINR which show that PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase. Even ~10% UE can achieve above 28dB SINR even for PC3.

5.2.1.4   Simulation results from vivo

R4-2218869

The SINR distribution for Indoor and Uma are shown in Figure 5.2.1.4-1:

Figure 5.2.1.4-1 SINR distribution

Except for PC1, The SINR of UE is hard to larger than 25 dB at the BS side.

Observation 1:

- For PC1, 20% of UE’s SINR can be larger than 25 dB.

- For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB.

We further counted the percentage of UEs that had used 256QAM in the simulation and the throughput performance gain. The throughput gain is derived from comparing whether the 256QAM is allowed. The results are shown in below:

Figure 5.2.1.4-2 proportion and performance gain for 256 QAM

For the indoor scenario, PC1 UE can have obvious performance gain, and about 70% UE is once scheduled with 256QAM during the simulation time, but for other power classes, the throughput performance even become worse, this is because the SINR for these UE cannot support such high MCS and the BLER is bad. For the Uma scenario, even though there are many UEs scheduled with 256 QAM, the performance gain is tiny.

Observation 2:

- For the indoor scenario, only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM.

- For the Uma scenario, the performance gain of all PC UEs is tiny.

R4-2301569

The simulation assumption for SLS was revised to add minimum EIRP and power control formula, and our simulation results based on such new assumptions are shown in Figure 5.2.1.4-3. The operating SNR is used as the target SNR at the BS side, and the minimum EIRP is used to calculate the conducted power level which means deducting antenna gain (including array gain).

Figure 5.2.1.4-3 SINR distribution based on SLS

For the Indoor scenario, all cases above have more than 10% UE can achieve operating SNR, but for the Uma scenario, the ratio is much lower, especially in PC2/PC5 in the 39GHz case, only about 2% UE can achieve operating SNR.

Observation 1:

- In 29GHz, 14% in Indoor and 5% in Uma PC2/PC5 UE can archive 28 dB SNR.

- In 39GHz, 12% in Indoor and 10% in Uma PC1 UE can archive 30 dB SNR; 10% in Indoor and 2% in Uma PC2/PC5 UE can archive 30 dB SNR.

5.2.1.5  Simulation results from Ericsson

R4-2220036

On Figure 5.2.1.5-1 we show the results of system-level simulations for 29 GHz carrier frequency and FTP traffic model of both 10% and 30%.

Figure 5.2.1.5-1: System-level simulation results on 256QAM feasibility for PC1 UE in FR2-1 for 29 GHz carrier frequency

Note that in the simulations results, no sophisticated model was used for the misalignment between BS and UE beam directions. So, in the simulations the ideal beam management was assumed, and the impairment offset of 10 dB was added in the last stage.

By looking at the results, we can see that around 30% of UEs can achieve the operating SNR of 30 dB for FTP with 30% load and around 35% of UEs can achieve the operating SNR of 30 dB for FTP with 10% load, which means that 256 QAM for PC1 UEs in FR2-1 and 29 GHz carrier frequency is feasible.

R4-2302734

2.1  	29GHz carrier frequency

2.1.1   Indoor scenario for PC2/PC5

Observation 1: For 29GHz carrier frequency, indoor scenario and PC2/PC5, around 60% of users with FTP 10% load and 15% of users with FTP 30% load can reach the operating SNR of 28dB.

2.1.2   UMa scenario for PC2/PC5

Observation 2: For 29GHz carrier frequency, UMa scenario and PC2/PC5, around 30% of users with FTP 10% load and 28% of users with FTP 30% load can reach the operating SNR of 28dB.

2.2  	39GHz carrier frequency

2.2.1   Indoor scenario for PC2/PC5

Observation 3: For 39GHz carrier frequency, indoor scenario and PC2/PC5, around 50% of users with FTP 10% load and 12% of users with FTP 30% load can reach the operating SNR of 30dB.

2.2.2   UMa scenario for PC2/PC5

Observation 4: For 39GHz carrier frequency, UMa scenario and PC2/PC5, around 17% of users with FTP 10% load and 15% of users with FTP 30% load can reach the operating SNR of 30dB.

2.2.3   Indoor scenario for PC1

Observation 5: For 39GHz carrier frequency, indoor scenario and PC1, around 55% of users with FTP 10% load and 12% of users with FTP 30% load can reach the operating SNR of 30dB.

2.2.4   UMa scenario for PC1

Observation 6: For 39GHz carrier frequency, UMa scenario and PC1, around 50% of users with FTP 10% load and 38% of users with FTP 30% load can reach the operating SNR of 30dB.

5.2.1.6  Simulation results from Xiaomi

R4-2301620

In last meeting, it is verified that the UL 256QAM for PC1 UE of 29GHz is feasible based on system simulation, but whether UL 256QAM for PC2/5 UE of 29GHz and PC1/2/5 UE of 39GHz can achieve target SNR at BS side need further confirm by system simulation. Hence, we provide the system level simulation results with the agreed target SNR of 28dB and 30dB for PC1/2/5 UE of 29GHz and 39GHz.

The UE transmit power and UL SINR CDF at 29 GHz and 39 GHz in urban macro scenario are shown in figure 5.2.1.6-1 and figure 5.2.1.6-2, respectively.

- PC1 UE with 35dBm maximum output power (n257)

- PC2/5 UE with 23dBm maximum output power  (n257)

Figure 5.2.1.6-1 Urban macro simulation results at 29 GHz

- PC1 UE with 35 dBm maximum output power (n260)

- PC2/5 UE with 23dBm maximum output power (n259)

Figure 5.2.1.6-2 Urban macro simulation results at 39 GHz

The UE transmit power and UL SINR CDF at 29 GHz and 39 GHz in indoor hotspot scenario are shown in figures 5.2.1.6-3 and 5.2.1.6-4, respectively.

- PC1 UE with 35 dBm maximum output power (n257)

- PC2/5 UE with 23dBm maximum output power (n257)

Figure 5.2.1.6-3 Indoor hotspot simulation results at 29 GHz

- PC1 UE with 35 dBm maximum output power (n260)

- PC2/5 UE with 23dBm maximum output power (n259)

Figure 5.2.1.6-4 Indoor hotspot simulation results at 39 GHz

Observation:

It can be observed from figure 5.2.1.6-1, figure 5.2.1.6-2, figure 5.2.1.6-3 and figure5.2.1.6-4 that at both 29 GHz (n257) and 39 GHz (n259 and n260):

For urban macro scenario:

-    The target SNR of 28dB for 29 GHz at BS side can be achieved by 86.95% of PC1 UE and 73.26% of PC2/5 UE.

-    The target SNR of 30dB for 39 GHz at BS side can be achieved by 82.82% of PC1 UE and 62.31% of PC2/5 UE.

-	~13% of PC1 UE and ~26% of PC2/5 UE are transmitting at maximum output power at 29 GHz (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

-    ~17% of PC1 UE and ~27% of PC2/5 UE are transmitting at maximum output power at 39 GHz (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

For indoor hotspot scenario:

-    The target SNR of 28dB for 29 GHz at BS side can be achieved by 100% of PC1 UE and 100% of PC2/5 UE.

-    The target SNR of 30dB for 39 GHz at BS side can be achieved by 100% of PC1 UE and 99.99% of PC2/5 UE.

-	The transmitting power of all PC1/PC2/PC5 UE are less than 20 dBm at 29 GHz.

-    The transmitting power of ~ 99% PC1/PC2/PC5 UE are less than 13 dBm at 39 GHz.

5.2.1.7  Simulation results from LG Electronics

R4-2300821

-29 GHz UL 256QAM SLS

Fig.5.2.1.7-1 CDF of SINR for PC2/PC5 in urban macro scenario at 29 GHz

Fig. 5.2.1.7-2 CDF of SINR for PC2/PC5 in indoor scenario at 29 GHz

Fig. 5.2.1.7-1 and Fig. 5.2.1.7-2 show CDF of SINR for PC2/PC5 in urban macro scenario and indoor scenario at 29 GHz respectively.

Based on the SLS results, UL 256QAM at 29 GHz is feasible for both PC2 and PC5.

Observation 1: From the 29 GHz 256QAM SLS results, PC2 and PC5 can achieve SINR of 28dB in both urban macro scenario and indoor scenario.

-39 GHz UL 256QAM SLS

Fig. 5.2.1.7-3 CDF of SINR for PC1/PC2/PC5 in urban macro scenario at 39 GHz

Fig. 5.2.1.7-4 CDF of SINR for PC1/PC2/PC5 in indoor scenario at 39 GHz

Fig. 5.2.1.7-3 and Fig. 5.2.1.7-4 show CDF of SINR for PC1/PC2/PC5 in urban macro scenario and indoor scenario at 39 GHz respectively.

Based on the SLS results, UL 256QAM at 39GHz is feasible for PC1, PC2, and PC5 UEs.

Observation 2: From the 39 GHz 256QAM SLS results, PC1, PC2 and PC5 can achieve SINR of 30dB in both urban macro scenario and indoor scenario.

5.2.1.8  Simulation results from Sony

R4-2302240

The SLS simulation results at 29 GHz for the indoor scenario are shown in Fig. 5.2.1.8-1. Since PC1 has been concluded to be feasible, only results of PC2/5 have been provided here.

Figure 5.2.1.8-1 The system level simulation at 29 GHz for the indoor scenario with PC2/5.

As explained, a power control scheme is adopted in the simulation, and thus the CDF plot is “capped” by the target SNR. Based on the simulation results, it can be observed that about 35% of UE can reach the target SNR, which is a significant portion of total devices. Therefore, it can be concluded that it is feasible for PC2 and PC5 to support 256 QAM at 29 GHz.

Observation 1	35% of UE can reach the target SNR condition of PC2 and PC5 at indoor scenario at 29 GHz.

Further simulations have also been performed for 39GHz for the indoor scenario, including PC1 as well, where the simulation results are shown in Fig. 5.2.1.8-2 (PC1) and Fig. 5.2.1.8-3 (PC2/5), respectively.

Figure 5.2.1.8-2 The system level simulation at 39 GHz for the indoor scenario with PC1.

Figure 5.2.1.8-3 The system level simulation at 39 GHz for the indoor scenario with PC2/5.

Based on the results shown in Fig. 5.2.1.8-2 and Fig. 5.2.1.8-3, though the target SNR and path loss have been increased, a significant portion of UE (45% for PC1 and 10% for PC2/5) can still meet the target SINR. Again, such simulation results demonstrate that a system-level gain can be obtained by supporting 256 QAM for uplink, which suggests that it is feasible to support 256 QAM in uplink is feasible for PC1/2/5 at 39 GHz.

Observation 2 	45% of PC1 UE and 10% of PC2/5 UE can meet the target SNR condition for the indoor scenario at 39 GHz.

5.2.1.9   Summary for system level simulation results

| Companies | Urban macro | Indoor |
| --- | --- | --- |
| Nokia (R4-2300193) | The target SNR at BS side of 27 dB can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE. Less than 15% of PC1 UE and less than 30% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). | The target SNR at BS side of 27 dB can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE. Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). |
| LG Electronics (R4-2300821) | For 29GHz: The target SNR at BS side of 28 dB can be achieved by ~77% of PC2 UE and ~65% of PC5 UE. For 39GHz: The target SNR at BS side of 30 dB can be achieved by ~40% of PC1 UE, ~85% of PC2 UE and 70% of PC5. | For 29GHz: The target SNR at BS side of 28 dB can be achieved by ~41% of PC2 UE and ~40% of PC5 UE. For 39GHz: The target SNR at BS side of 30 dB can be achieved by ~37% of PC1 UE, ~75% of PC2 UE and 50% of PC5. |
| ZTE (R4-2301235) | For 29GHz 5% UE can achieve above 30dB SINR for PC2 and PC5. Even ~10% UE can achieve above 28dB SINR even for PC3. Thus we observed PC2 and PC5 are feasible for UL 256QAM for 29GHz, and PC3 is also feasible if the operating SNR is not higher than 28dB. For 39GHz： For PC1, ~55% of UE can reach the operating SNR For PC2, ~8% of UE can reach the operating SNR For PC5 , ~15% of UE can reach the operating SNR For PC3, ~2% of UE can reach the operating SNR | for 39GHz： For PC1, PC2 and PC5, ~86% of UE can reach the operating SNR For PC3, ~75% of UE can reach the operating SNR |
| vivo (R4-2301569) | In 29GHz: 5% in Uma PC2/PC5 UE can archive 28 dB SNR. In 39GHz: 10% in Uma PC1 UE can archive 30 dB SNR 2% in Uma PC2/PC5 UE can archive 30 dB SNR. | In 29GHz: 14% in Indoor PC2/PC5 UE can archive 28 dB SNR. In 39GHz: 12% in Indoor PC1 UE can archive 30 dB SNR; 10% in Indoor PC2/PC5 UE can archive 30 dB SNR. |
| Xiaomi (R4-2301620) | For 29GHz: The target SNR of 28dB for 29 GHz at BS side can be achieved by 86.95% of PC1 UE and 73.26% of PC2/5 UE. ~13% of PC1 UE and ~26% of PC2/5 UE are transmitting at maximum output power at 29 GHz (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). For 39GHz: The target SNR of 30dB for 39 GHz at BS side can be achieved by 82.82% of PC1 UE and 62.31% of PC2/5 UE. ~17% of PC1 UE and ~27% of PC2/5 UE are transmitting at maximum output power at 39 GHz (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). | For 29GHz: The target SNR of 28dB for 29 GHz at BS side can be achieved by 100% of PC1 UE and 100% of PC2/5 UE. The transmitting power of all PC1/PC2/PC5 UE are less than 20 dBm at 29 GHz. For 39GHz: The target SNR of 30dB for 39 GHz at BS side can be achieved by 100% of PC1 UE and 99.99% of PC2/5 UE. The transmitting power of ~ 99% PC1/PC2/PC5 UE are less than 13 dBm at 39 GHz. |
| Sony (R4-2302240) |  | For 29GHz: 35% of UE can reach the target SNR condition of PC2 and PC5 at indoor scenario at 29 GHz. For 39GHz: 45% of PC1 UE and 10% of PC2/5 UE can meet the target SNR condition for the indoor scenario at 39 GHz. |
| Ericsson (R4-2302734) | For 29GHz: UMa scenario and PC2/PC5, around 30% of users with FTP 10% load and 28% of users with FTP 30% load can reach the operating SNR of 28dB. For 39GHz: UMa scenario and PC2/PC5, around 17% of users with FTP 10% load and 15% of users with FTP 30% load can reach the operating SNR of 30dB. UMa scenario and PC1, around 50% of users with FTP 10% load and 38% of users with FTP 30% load can reach the operating SNR of 30dB. | For 29GHz: Indoor scenario and PC2/PC5, around 60% of users with FTP 10% load and 15% of users with FTP 30% load can reach the operating SNR of 28dB. For 39GHz: Indoor scenario and PC2/PC5, around 50% of users with FTP 10% load and 12% of users with FTP 30% load can reach the operating SNR of 30dB. Indoor scenario and PC1, around 55% of users with FTP 10% load and 12% of users with FTP 30% load can reach the operating SNR of 30dB. |

5.2.2	Link level simulation

Link level simulation is targeted as mainstream way to evaluate if FR2 256QAM can achieve benefit by comparing to 64QAM.

5.2.2.1    Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.2.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where UL 256QAM provides performance benefits, does not reflect side conditions for setting the UE RF requirement for UL 256QAM.

Table 5.2.2.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

5.2.2.2   Simulation results from Nokia

R4-2215577

The link level simulation results at 29 GHz (n257) and 39 GHz (n260) using the highlighted parameters in table 1 are shown in figures 5.2.2.2-1 and 5.2.2.2-2, respectively.

Figure 5.2.2.2-1. Simulation results at 29 GHz (n257)

Figure 5.2.2.2-2. Simulation results at 39 GHz (n260)

It can be observed from figure 5.2.2.2-1 that at 29 GHz (n257):

-	If MCS index is 21, 256QAM goes over 64QAM in around 24-25 dB for all simulated EVMs.

-	If MCS index is 23, 256QAM goes over 64QAM in around 28 dB for (transmit and receive) EVM of 3%, around 32 dB for (transmit and receive) EVM of 3.5%, and loses for (transmit and receive) EVM of 4%.

On the other hand, it can be observed from figure 5.2.2.2-2 that at 39 GHz (n260):

-	Only with (transmit and receive) EVM of 0%, MCS21 for 256QAM seems to win 64QAM in around 36 dB.

Note that the PN is challenging case at 39 GHz (n260), and due to small allocation, the number of PTRS symbols is quite low so even higher complexity ICI compensation would not really help here.

R4-2218326

The link level simulation results at 48 GHz (n262) using the highlighted parameters in table 1 with phase noise model option a and option d are shown in figures 5.2.2.2-3 and 5.2.2.2-4, respectively.

Figure 5.2.2.2-3. Simulation results with phase noise model option a

Figure 5.2.2.2-4. Simulation results with phase noise model option d

It can be observed from figures 5.2.2.2-3 and 5.2.2.2-4 that at 48 GHz (n262):

-	256QAM does not provide any throughput gain over 64QAM below 40 dB SNR.

5.2.2.3   Simulation results from LG Electronics

R4-2215920

Table 5.2.2.3-1 shows the cross throughput point, where the cross throughput point indicates the SNR value at which the throughput of 64QAM and 256QAM is same, that is, after the corresponding SNR, the throughput of 256QAM is higher than 64QAM.

Table 5.2.2.3-1 Cross throughput point (29 GHz and 48 GHz, CBW:100 MHz, EVM:Tx=Rx)

Referebce: MCS24(64QAM, PUSCH Table 1, OFDM)

| Propagation MCS Mode | EVM: 2% |  | EVM: 3% |  | EVM: 3.5% |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 29 GHz | 48 GHz | 29 GHz | 48 GHz | 29 GHz | 48 GHz |
| TDL-A MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 24.5 dB | N/A | 27 dB | N/A | 30 dB | N/A |
| TDL-A MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 29 dB | N/A | N/A | N/A | N/A | N/A |
| TDL-D MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 18.5 dB | 30 dB | 20 dB | N/A | 21 dB | N/A |
| TDL-D MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 23 dB | N/A | 24.5 dB | N/A | 27 dB | N/A |
| AWGN MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 16 dB | 22.5 dB | 16.5 dB | 24 dB | 17 dB | 27.5dB |
| AWGN MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 19 dB | N/A | 20 dB | N/A | 21 dB | N/A |

29 GHz (n257), EVM(Tx=Rx):3.5%

| Fig 5.2.2.3-1. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-2. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-3. Throughput performance between 256QAM and 64QAM in AWGN |  |

29 GHz (n257), EVM(Tx=Rx):3%

| Fig 5.2.2.3-4. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-5. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-6. Throughput performance between 256QAM and 64QAM in AWGN |  |

29 GHz (n257), EVM(Tx=Rx):2%

| Fig 5.2.2.3-7. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-8. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-9. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):3.5%

| Fig 5.2.2.3-10. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-11. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-12. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):3%

| Fig 5.2.2.3-13. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-14. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-15. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):2%

| Fig 5.2.2.3-16. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-17. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-18. Throughput performance between 256QAM and 64QAM in AWGN |  |

From the simulation results, the followings are observed for UL 256QAM performance gain,

Observation1: In 29 GHz & TDL-A,

- There is the performance gain in MCS21 when EVM ≥ 3.0%

- There is no performance gain in MCS23

Observation2: In 29 GHz & TDL-D

- There is the perfornace gain when EVM = 3.5%

Observation3: In 29 GHz & AWGN

- There is the perfornace gain at a relatively low SNR when EVM = 3.5%.

Observation4: In 48 GHz,

- There is no performance gain in most cases

R4-2218681

Table 5.2.2.3-2 Cross throughput point (29 GHz and 39 GHz, CBW:100 MHz, EVM:Tx=Rx=3.5%)

Referebce: MCS24(64QAM, PUSCH Table 1, OFDM)

| Propagation Mode | 29 GHz |  |  |  | 39 GHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 20 | MCS 21 | MCS 22 | MCS 23 |
| TDL-A (256 QAM, PDSCH Table 2, OFDM) (dB) | 25.5 | 30 | N/A | N/A | N/A | N/A | N/A | N/A |
| TDL-D (256 QAM, PDSCH Table 2, OFDM) (dB) | 18.5 | 21 | 23 | 27 | 21 | 24 | 30 | N/A |

29 GHz (n257)

Fig5.2.2.3-19. Throughput performance between 256QAM and 64QAM in TDL-A

Fig5.2.2.3-20. Throughput performance between 256QAM and 64QAM in TDL-D

39 GHz (n260)

Fig5.2.2.3-21. Throughput performance between 256QAM and 64QAM in TDL-A

Fig5.2.2.3-22. Throughput performance between 256QAM and 64QAM in TDL-D

Observation 2: In 29GHz TDL-A, the throughput gain of 256QAM is shown in MCS index 20 and 21, and operating SNR can vary from 25.5dB to 30dB depending on the MCS index number.

Observation 3: In 29GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20~23, and operating SNR can vary from 18.5dB to 27dB depending on the MCS index number.

Observation 4: In 39GHz TDL-A, no throughput gain of 256QAM  is shown in MCS index 20~23.

Observation 5: In 39GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20 ~22, and operating SNR can vary from 21dB to 30dB depending on the MCS index number.

Observation 6: If the operating SNR is considered based on TDL-A, SNR of 25.5 dB or more need to be set for operation at 29GHz, however it is not practical. And N/A is shown at 39GHz, so it is also not practical.

Observation 7: If the operating SNR is considered based on TDL-D with limited MCS, an operating SNR value of 18.5dB to 27dB at 29GHz may be considered, and an operating SNR value of 21dB to 30dB at 39GHz may be considered.

5.2.2.4   Simulation results from vivo

R4-2216128

- 29 GHz

- TDL-A

- TDL-D

- AWGN

The simulation results above are summarized as follows.

Observation 1：The UL 256 QAM under 29 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.0 dB | 22.8 dB | 20 dB | 21 dB | 18 dB | 19.2 dB |
| CP-OFDM | 22.5 dB | 23.8 dB | 20.3 dB | 21.5 dB | 19.2 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.2 dB | 23.1 dB | 20.3 dB | 21.2 dB | 18.5 dB | 19.5 dB |
| CP-OFDM | 22.7 dB | 24 dB | 20.8 dB | 21.8 dB | 19.5 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.9 dB | 23.5 dB | 20.6 dB | 21.5 dB | 19.2 dB | 19.7 dB |
| CP-OFDM | 23.2 dB | 24.7 dB | 21.2 dB | 22 dB | 19.8 dB | 19.9 dB |

- 39 GHz

- TDL-A

- TDL-D

- AWGN

Observation 2：The UL 256 QAM under 39 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24 dB | 27 dB | 22.3 dB | 25.8 dB | 19.9 dB | 22 dB |
| CP-OFDM | 28 dB | N/A | 24 dB | N.A. | 21.5 dB | 26 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.4 dB | 27.5 dB | 23 dB | 26 dB | 20 dB | 23 dB |
| CP-OFDM | 29.2 dB | N/A | 25 dB | N.A. | 21.8 dB | 28 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.8 dB | 28.2 dB | 23.3 dB | 28 dB | 20.3 dB | 23 dB |
| CP-OFDM | 34 dB | N/A | 25.8 dB | N.A. | 22 dB | 30 dB |

- 48 GHz

- TDL-A

- TDL-D

- AWGN

Observation 3：The UL 256 QAM is hard to bring performance gain under 48 GHz

5.2.2.5   Simulation results from Huawei

R4-2216245

The link lever simulation results are shown in Figure5.2.2.5-1, Figure 5.2.2.5-2 and Figure 5.2.2.5-3 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 5.2.2.5-1 Throughput comparison of 256QAM and 64QAM for RANK1_29 GHz

Figure 5.2.2.5-2 Throughput comparison of 256QAM and 64QAM for RANK1_39GHz

Figure 5.2.2.5-3 Throughput comparison of 256QAM and 64QAM for RANK1_48GHz

R4-2219144

The link lever simulation results are shown in Figure 5.2.2.5-4, Figure 5.2.2.5-5, Figure 5.2.2.5-6 and Figure 5.2.2.5-7 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 5.2.2.5-4 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_29 GHz

Figure5.2.2.5-5 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_29 GHz

Figure 5.2.2.5-6 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_39GHz

Figure 5.2.2.5-7 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_39GHz

5.2.2.6   Simulation results from Sony

R4-2216251

We performed simulations and compared the throughput between 64QAM and 256QAM for three different transmit and receive EVM values (3%, 3.5%, and 4%). TX EVM has been the same as RX EVM in each simulation. The simulations are done according to the agreed simulation assumptions [3]. Only Rank-1, and AWGN for 29 GHz, respectively 48 GHz, are simulated so far. 100MHz channel BW and no HARQ have been used. Phase noise is added according to option 2 in the simulation assumptions, i.e., example2, at both the UE and BS.

- 29 GHz

We performed simulations to address whether 256-QAM works in FR2-1 and to demonstrate its performance gain. The result is shown in Figure 5.2.2.6-1.

Figure 5.2.2.6-1 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3%, 3.5% and 4%.

Figure 5.2.2.6-1 shows that throughput for 64QAM (red curve) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 22 dB (EVM=3%), 23 dB (3.5%) and 24 dB (EVM=4%) are required to exceed the throughput performance of 64QAM. The simulations have been carried out with a simple receiver, and further studies of advanced receivers are needed to determine whether the above results could be improved.

Observation 1	For 256QAM to exceed throuput performance of 64QAM SNR levels of 22 dB (EVM=3%) to 24 dB (EVM=4%) are required.

R4-2218753

We performed simulations to determine whether 256-QAM works for FR2-1 and demonstrate its performance gain. Figure 5.2.2.6-2 shows the results.

Figure 5.2.2.6-2 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3.5%.

Figure 5.2.2.6-2 shows that throughput for 64QAM (red curves) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 20 dB for MCS21 and 25 dB for MCS23 (EVM 3.5%) are required to exceed the throughput performance of 64QAM. To reach throughput saturation for 256QAM, SNR levels of 21dB for MCS21 and 28 dB for MCS23 are needed.

Observation 1	For 256QAM, to reach saturation in throughput performance, SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz.

Simulations have also been performed for 39GHz with the same setup as for 29GHz. Figure 5.2.2.6-3 shows the results.

Figure 5.2.2.6-3	 Throughput comparing 64QAM and 256QAM with AWGN for 39GHz, EVM 3.5%.

Figure 5.2.2.6-3 shows 256QAM performance for 39GHz compared to 64QAM. SNR levels of 21.5dB are needed to exceed the throughput performance of 64QAM for MCS21. For MCS23, an SNR of 36dB is required to exceed the throughput performance of 64QAM. An SNR level of 24dB is needed to reach throughput saturation for 256QAM for MCS21. For MCS23, we didn’t see saturation within the simulated SNR range.

Observation 2	For 256QAM, to reach saturation in throughput performance, SNR levels of 24dB for MCS21 are needed for 39GHz. To exceed the performance of 64QAM, an SNR of 21.5dBm is required.

5.2.2.7   Simulation results from Xiaomi

R4-2216350

We compare the UL 256QAM and UL 64QAM performance under different channel conditions and carrier frequencies to determine whether UL 256QAM can provide gains over UL 64QAM for FR2-1. For all simulation, we assumed 50MHz channel bandwidth with 120kHz SCS and phase noise model of example2 (UE) + example2(BS) based on above simulation assumptions.

- CP-OFDM

- 29GHz

Figure 5.2.2.7-1 Throughput performance for 29GHz under AWGN, CP-OFDM

Figure 5.2.2.7-2 Throughput performance for 29GHz under TDL-D, CP-OFDM

Figure 5.2.2.7-3 Throughput performance for 29GHz under TDL-A, CP-OFDM

- 39GHz

Figure 5.2.2.7-4 Throughput performance for 39GHz under AWGN, CP-OFDM

Figure 5.2.2.7-5 Throughput performance for 39GHz under TDL-D, CP-OFDM

Figure 5.2.2.7-6 Throughput performance for 39GHz under TDL-A, CP-OFDM

- 48GHz

Figure 5.2.2.7-7 Throughput performance for 48GHz under AWGN, CP-OFDM

- DFT-s-OFDM

- 29GHz

Figure 5.2.2.7-8 Throughput performance for 29GHz under AWGN, DFT-s-OFDM

Figure 5.2.2.7-9 Throughput performance for 29GHz under TDL-D, DFT-s-OFDM

Figure 5.2.2.7-10 Throughput performance for 29GHz under TDL-A, DFT-s-OFDM

- 39GHz

Figure 5.2.2.7-11 Throughput performance for 39GHz under AWGN, DFT-s-OFDM

Figure 5.2.2.7-12 Throughput performance for 39GHz under TDL-D, DFT-s-OFDM

Figure 5.2.2.7-13 Throughput performance for 39GHz under TDL-A, DFT-s-OFDM

- 48GHz

Figure 5.2.2.7-14 Throughput performance for 39GHz under TDL-A, DFT-s-OFDM

From the simulation results, table 5.2.2.7-1 summary the SNR required to achieve gains for 256QAM compare to 64QAM:

Table 5.2.2.7-1 UL 256QAM required SNR to achieve gains compare to 64QAM

| Parameters for UL 256QAM |  |  | SNR required to achieve gains for 256QAM [dB] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 29GHz |  |  |  |  | 39GHz |  |  | 48GHz |  |  |
| Channel | MCS |  |  | 3%+3% | 3.5%+3.5% | 4%+4% | 3%+3% |  | 3.5%+3.5% | 4%+4% | 3%+3% | 3.5%+3.5% | 4%+4% |  |
| AWGN 1x2 RANK1 | MCS21 | CP-OFDM |  | 19.3 | 19.5 | 19.6 | 20 |  | 21.5 | 22 | 24 | 25 | 28 |  |
|  |  | DFT-OFDM |  | 19.3 | 19.5 | 19.6 | 20.1 |  | 21.1 | 21.7 | 22.5 | 23.6 | 25 |  |
|  | MCS23 | CP-OFDM |  | 23 | 23.5 | 25.5 | NA |  | NA | NA | NA | NA | NA |  |
|  |  | DFT-OFDM |  | 22.5 | 23.5 | 24.7 | NA |  | NA | NA | NA | NA | NA |  |
| TDL-D 2x2 RANK2 | MCS21 | CP-OFDM |  | 21.8 | 22.5 | 23.1 | 24.1 |  | 25.2 | 28 | - | - | - |  |
|  |  | DFT-OFDM |  | 22 | 22.6 | 23.3 | 24.1 |  | 25.7 | 28 | - | - | - |  |
|  | MCS23 | CP-OFDM |  | 25.8 | 27.3 | NA | NA |  | NA | NA |  |  |  |  |
|  |  | DFT-OFDM |  | 25.7 | 26.1 | 28 | NA |  | NA | NA | - | - | - |  |
| TDL-A 2x2 RANK2 | MCS21 | CP-OFDM |  | 21.8 | 22.7 | 23.8 | 25.2 |  | 26.7 | NA | - | - | - |  |
|  |  | DFT-OFDM |  | 21.8 | 22.7 | 23.8 | NA |  | NA | NA | - | - | - |  |
|  | MCS23 | CP-OFDM |  | 26.5 | 28 | NA | NA |  | NA | NA | - | - | - |  |
|  |  | DFT-OFDM |  | 25.8 | 27.5 | NA | NA |  | NA | NA | - | - | - |  |
| - means the related simulations haven’t been done due to time limit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation:

And it is shown that supporting UL 256 QAM can provide significant performance gain over UL 64QAM for 3.5% EVM.

For AWGN channel with 3.5% Tx EVM+3.5% Rx EVM, a SNR of >19.5 dB is needed for 29GHz, a SNR of >21.1 dB is needed for 39GHz and a SNR of >23.6 dB is needed for 48GHz.

For TDL-A and TDL-D fading channel with 3.5% Tx EVM+3.5% Rx EVM, a SNR of >22.5 dB is needed for 29GHz, a SNR of >25.2 dB is needed for 39GHz, due to lime limit, the related simulation for 48GHz haven’t been done.

R4-2219075

We further compare the UL 256QAM and UL 64QAM performance under different channel conditions and carrier frequencies to determine whether UL 256QAM can provide gains over UL 64QAM for 48GHz. For all simulations, we assumed 50MHz channel bandwidth with 120kHz SCS and phase noise model of example2 (UE) + example2(BS) based on the agreed simulation assumptions [2].

- CP-OFDM

Figure 5.2.2.7-15 Throughput performance for 48GHz under AWGN, CP-OFDM

Figure 5.2.2.7-16 Throughput performance for 48GHz under TDL-D, CP-OFDM

Figure 5.2.2.7-17 Throughput performance for 48GHz under TDL-D, CP-OFDM

- DFT-s-OFDM

Figure 5.2.2.7-18 Throughput performance for 48GHz under AWGN, DFT-s-OFDM

Figure 5.2.2.7-19 Throughput performance for 48GHz under TDL-D, DFT-s-OFDM

Figure 5.2.2.7-20 Throughput performance for 48GHz under TDL-A, DFT-s-OFDM

From the simulation results for 48GHz, table 5.2.2.7-2 updated the SNR required to achieve gains for 256QAM compare to 64QAM:

Table 5.2.2.7-2 UL 256QAM required SNR to achieve gains compare to 64QAM

| Parameters for UL 256QAM |  |  | 48GHz SNR required to achieve gains for 256QAM [dB] |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel | MCS |  | 3%+3% | 3.5%+3.5% | 4%+4% |
| AWGN 1x2 RANK1 | MCS21 | CP-OFDM | 24 | 25 | 28 |
|  |  | DFT-OFDM | 22.5 | 23.6 | 25 |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
| TDL-D 2x2 RANK2 | MCS21 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
| TDL-A 2x2 RANK2 | MCS21 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |

5.2.2.8   Simulation results from ZTE

R4-2216426

To study the gain and operating SNR, link level simulation approach was adopted to compare 256QAM with 64QAM. The simulation assumptions are from 5.2.2.1, and the parameters used in this simulation are marked in red. The throughput curves for CP-OFDM Rank1 transmission are listed in figure 5.2.2.8-1~ figure  5.2.2.8-9 for reference.

Figure  5.2.2.8-1: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz AWGN channel

Figure 5.2.2.8-2: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz TDLD channel

Figure 5.2.2.8-3: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz TDLA channel

Figure 5.2.2.8-4: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz AWGN channel

Figure 5.2.2.8-5: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz TDLD channel

Figure 5.2.2.8-6: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz TDLA channel

Figure 5.2.2.8-7: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz AWGN channel

Figure 5.2.2.8-8: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz TDLD channel

Figure 5.2.2.8-9: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz TDLA channel

The target SNR values are summarized in table 5.2.2.8-1, where “-” means that 256QAM can not provide better performance in the given SNR range compared to 64QAM.

Table 5.2.2.8-1. Target SNR for CP-OFDM, Rank 1 transmission

| index | frequency | Channel | Test setup (64QAM/256QAM) | Target SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EVM 3.0+3.0 | EVM 3.5+3.5 | EVM 4.0+4.0 |
| 1 | 29GHz | AWGN | MCS23/21 | 21.31 | 21.71 | 22.19 |
| 2 |  |  | MCS24/23 | 25.92 | 27.66 | 31.06 |
| 3 |  | TDL-D | MCS23/21 | 23.42 | 23.73 | 24.16 |
| 4 |  |  | MCS24/23 | 28.44 | 30.10 | 34.28 |
| 5 |  | TDL-A | MCS23/21 | 25.90 | 26.45 | 27.23 |
| 6 |  |  | MCS24/23 | 31.49 | 34.31 | - |
| 7 | 39GHz | AWGN | MCS23/21 | 21.28 | 21.64 | 21.95 |
| 8 |  |  | MCS24/23 | 25.86 | 27.64 | 30.62 |
| 9 |  | TDL-D | MCS23/21 | 26.36 | 27.50 | 29.23 |
| 10 |  |  | MCS24/23 | - | - | - |
| 11 |  | TDL-A | MCS23/21 | 30.48 | 32.35 | 36.62 |
| 12 |  |  | MCS24/23 | - | - | - |
| 13 | 48GHz | AWGN | MCS23/21 | - | - | - |
| 14 |  |  | MCS24/23 | - | - | - |
| 15 |  | TDL-D | MCS23/21 | - | - | - |
| 16 |  |  | MCS24/23 | - | - | - |
| 17 |  | TDL-A | MCS23/21 | - | - | - |
| 18 |  |  | MCS24/23 | - | - | - |

Based on the simulation results, the following observations are given:

Observation 1: For 29GHz:

256QAM performance gain can be expected in the following cases:

- AWGN and TDL-D channel,

- TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected,

- TDL-A channel when MCS23(256QAM)/MCS24(64QAM) and EVM3.0+3.0 or EVM3.5+3.5 are selected

However, 256QAM performance gain can not be expected in the following cases:

- TDL-A channel when MCS23(256QAM)/MCS24(64QAM) and EVM4.0+4.0 are selected.

Observation 2: For 39GHz:

256QAM performance gain can be expected in the following cases:

- AWGN

- TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected

However, 256QAM performance gain can not be expected in the following cases:

- TDL-D and TDL-A channel when MCS23(256QAM)/MCS24(64QAM) are selected.

For 48GHz, using the same phase noise model, the phase noise becomes larger as frequency increases.

Observation 3: For 48GHz:

256QAM performance gain can not be expected for AWGN, TDL-D and TDL-A channel for all the MCS.

R4-2218595

For MCS22 evaluation, the simulation assumptions are 5.2.2.1, and the parameters used in this simulation are marked in red. The throughput curves for MCS22 for CP-OFDM Rank1 transmission are listed in figure5.2.2.8-10~figure 5.2.2.8-12.

Figure 5.2.2.8-10, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz AWGN channel

Figure 5.2.2.8-12, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz TDL-D channel

Figure 5.2.2.8-12, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz TDL-A channel

The updated target SNR values are summarized in table 5.2.2.8-2, where “-” means that 256QAM can not provide better performance in the given SNR range compared to 64QAM. As EVM was reached in last meeting so the simulation results of EVM 3.0% and EVM 4.0% are not provided.

Table 5.2.2.8-2. Target SNR for CP-OFDM, Rank 1 transmission

| index | frequency | Channel | Test setup (64QAM/256QAM) | Target SNR |
| --- | --- | --- | --- | --- |
|  |  |  |  | EVM 3.5%+3.5% |
| 1 | 29GHz | AWGN | MCS23/21 | 20.80 |
| 2 |  |  | MCS24/23 | 27.47 |
| 3 |  | TDL-D | MCS23/21 | 22.42 |
| 4 |  |  | MCS24/23 | 27.71 |
| 5 |  | TDL-A | MCS23/21 | 24.03 |
| 6 |  |  | MCS24/23 | 29.07 |
| 7 | 39GHz | AWGN | MCS23/21 | 22.00 |
|  |  |  | MCS24/22 | 27.55 |
| 8 |  |  | MCS24/23 | - |
| 9 |  | TDL-D | MCS23/21 | 26.00 |
|  |  |  | MCS24/22 | 29.61 |
| 10 |  |  | MCS24/23 | - |
| 11 |  | TDL-A | MCS23/21 | 26.09 |
|  |  |  | MCS24/22 | 30.70 |
| 12 |  |  | MCS24/23 | - |
| 13 | 48GHz | AWGN | MCS23/21 | - |
| 14 |  |  | MCS24/23 | - |
| 15 |  | TDL-D | MCS23/21 | - |
| 16 |  |  | MCS24/23 | - |
| 17 |  | TDL-A | MCS23/21 | - |
| 18 |  |  | MCS24/23 | - |

Based on the simulation results, the following observations are given:

Observation 1: For 39GHz:

UL 256QAM performance gain can be expected in the following cases:

- AWGN, TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected

- AWGN, TDL-D and TDL-A channel when MCS22(256QAM)/MCS24(64QAM) are selected

Observation 2: UL 256QAM performance gain can not be expected for 48GHz.

5.2.2.9   Simulation results from Ericsson

R4-2220036

On Figure 5.2.2.9-1 – Figure 5.2.2.9-8 we provide simulation results for various scenarios described in figure captions and figure legends.

Figure 5.2.2.9-1: 29GHz, 100MHz, No HARQ, AWGN 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-2: 29GHz, 100MHz, No HARQ, Fading 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-3: 29GHz, 100MHz, No HARQ, AWGN 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-4: 29GHz, 100MHz, No HARQ, Fading 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-5: 29GHz, 100MHz, HARQ=8, AWGN 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-6: 29GHz, 100MHz,  HARQ=8, Fading 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-7: 29GHz, 100MHz, HARQ=8, AWGN 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-8: 29GHz, 100MHz, HARQ=8, Fading 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

In Table 5.2.2.9-1 we have summarized results where we indicate two operating SNR points where 256QAM MCS21 outperforms 64QAM MCS23 and 64QAM MCS24, respectively.

| HARQ Type | Channel type | Cross-point between 256QAM MCS21 and 64QAM MCS23 | Cross-point between 256QAM MCS21 and 64QAM MCS24 |
| --- | --- | --- | --- |
| No HARQ | AWGN 2x2 Rank 1 | 26.2 dB | 26.8 dB |
|  | AWGN 2x2 Rank 2 | 28.7 dB | 29 dB |
|  | Fading 2x2 Rank 1 | 27.9 dB | 28.1 dB |
|  | Fading 2x2 Rank 2 | 32.4 dB | 33.2 dB |
| HARQ=8 | AWGN 2x2 Rank 1 | 26 dB | 26.4 dB |
|  | AWGN 2x2 Rank 2 | 28.1 dB | 28.8 dB |
|  | Fading 2x2 Rank 1 | 27.1 dB | 27.8 dB |
|  | Fading 2x2 Rank 2 | 31.2 dB | 32.2 dB |

Table 5.2.2.9-1: Summary of link-level simulation results

We conclude first that between not having HARQ and having HARQ=8 repetitions there are no big differences when it comes to the cross-point (operating SNR) between 256QAM MCS21 and 64QAM MCS23/MCS24, respectively. For the AWGN channel, the operating SNR is lower by 0.3 dB in average in the “No HARQ” case compared with “HARQ=8” case, while for the TDL-D fading channel the operating SNR is lower by 0.8 dB in the “No HARQ” case compared with “HARQ=8” case.

For “No HARQ” case, for AWGN channel the average operating SNR is equal to 27.7 dB.

For “No HARQ” case, for TDL-D fading channel the average operating SNR is equal to 30.4 dB.

5.2.2.10   Summary for 29GHz simulation results

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2215920 R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2216251 R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 | Ericsson R4-2220036 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 100MHz | 50MHz | 100MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none |  | none | none | none | none | none/8 |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | 25.5 |  |  |  |  |  |  |
|  |  | MCS21 |  | 30 | 22.7/24 |  |  | 22.7 | 26.45/24.03 |  |
|  |  | MCS22 |  | NA |  |  |  |  |  |  |
|  |  | MCS23 |  | NA | 26.3/28.4 |  |  | 28 | 34.31/29.07 |  |
|  | TDL-D | MCS20 |  | 18.5 |  |  |  |  |  |  |
|  |  | MCS21 | 24-25 | 21 | 20.8/21.8 | 20.1 | 23 | 22.5 | 23.73/22.42 | 27.9/27.1 |
|  |  | MCS22 |  | 23 |  |  |  |  |  |  |
|  |  | MCS23 | 32 | 27 | 25.7/28 | 23 |  | 27.3 | 30.10/27.71 |  |
|  | AWGN | MCS21 |  | 17 | 19.5/19.7 |  | 20 | 19.5 | 21.71/20.8 | 26.2/26 |
|  |  | MCS23 |  | 21 | 22.2/23.8 |  | 25 | 23.5 | 27.66/27.47 |  |

Average value for 29GHz operating SNR based on TDL-D:

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 25 | - | 32 |
| LGE | 18.5 | 21 | 23 | 27 |
| vivo | - | 21.8 | - | 28 |
| Huawei | - | 20.1 | - | 23 |
| Xiaomi | - | 22.5 | - | 27.3 |
| ZTE | - | 22.42 | - | 27.71 |
| Ericsson | - | 27.9 | - | - |
| Average | 18.5 | 23 | 23 | 27.5 |

5.2.2.11   Summary for 39GHz simulation results

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 50MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none | none | none | none | none | none |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | NA |  |  |  |  |  |
|  |  | MCS21 |  | NA | 29.2/NA |  |  | 26.7 | 32.35/26.09 |
|  |  | MCS22 |  | NA |  |  |  |  | 30.70 |
|  |  | MCS23 |  | NA | NA |  |  | NA | NA |
|  | TDL-D | MCS20 |  | 21 |  |  |  |  |  |
|  |  | MCS21 | 36 | 24 | 25/NA | 21 |  | 25.2 | 27.50/26 |
|  |  | MCS22 |  | 30 |  |  |  |  | 29.61 |
|  |  | MCS23 |  | NA | NA | 25.4 |  | NA | NA |
|  | AWGN | MCS21 |  |  | 21.8/28 |  | 21.5 | 21.5 | 21.64/22 |
|  |  | MCS22 |  |  |  |  |  |  | 27.55 |
|  |  | MCS23 |  |  | NA |  | 36 | NA | 27.64 |

Average value for 39GHz operating SNR based on TDL-D:

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 36 | - | - |
| LGE | 21 | 24 | 30 | NA |
| vivo | - | 25 | - | NA |
| Huawei | - | 21 | - | 25.4 |
| Xiaomi | - | 25.2 | - | NA |
| ZTE | - | 26 | 29.21 | NA |
| Average | 21 | 26.2 | 29.6 | NA |

5.2.2.12   Summary for 48GHz simulation results

| Parameter |  |  | LG Electronics R4-2215920 | vivo R4-2216128 | Huawei R4-2216245 | Xiaomi R4-2216350 R4-2219075 | ZTE R4-2216426 R4-2219595 | Nokia R4-2218326 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz |  |
| CBW |  |  | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 100MHz | 50MHz |  |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |  |
| HARQ |  |  | none | none | none | none | none | none |  |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |  |
| Phase noise model |  |  | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) | Option d): example1 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS21 | NA | NA |  | NA | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |
|  | TDL-D | MCS21 | NA | NA | 22.6 | NA | NA | NA | NA |
|  |  | MCS23 | NA | NA |  | NA | NA | NA | NA |
|  | AWGN | MCS21 | 27.5 | NA |  | 25 | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |

5.2.3 Phase noise profiles evaluation

5.2.3.1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Frequency | 29, 39 GHz |
| SCS | 60kHz, 120kHz |
| BW | 100MHz |
| RB Size | 128RB for 60kHz, 64RB for 120kHz |
| Background AWGN | No additional noise |
| Time offset/Frequency offset | 0 |
| Antenna configuration | 1T1R |
| Modulation | 256QAM |
| Waveform type | CP-OFDM / DFT-s-OFDM |
| DMRS | 3 symbols per slot (UL DMRS add-pos = 2) |
| PTRS configuration | OFF/ON For CP-OFDM: L-PTRS (Time density) = 1 (every 1 symbol) K-PTRS (Freq density) =2 (every 2 RBs) For DFTs-OFDM: L-PTRS (Time density) = 1 (every symbol) N_group = 8, N_samp = 4 Other PTRS configurations can be considered for narrow RB allocations, especially for DFT-s-OFDM. |
| EVM measurement | Data aided EVM calculation, based on ideal data signal |
| Phase noise profiles | Refer to clause 5.2.3.2 |

5.2.3.2 Phase noise profiles

The phase noise profiles in TS 38.803 [3] (“UE example 1” and “UE example 2”) don’t have good enough EVM for higher modulations, i.e., 256QAM with the typical 3.5% EVM requirement. In the discussion progress, some candidate phase noise profiles for 29GHz and 39GHz for UL 256QAM are proposed as following options:

For 29GHz:

- Option1: Example 1 in TR38.803 for 29 GHz.

- Option 2: Example 2 in TR38.803.

- New phase noise model using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

  - Option 3: Parameters from Qualcomm

| PSD0 | 32 dB |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 6e5 | 4.7 | 8e5 | 5.3 |
| 3 | 8e5 | 2.0 | 1.1e6 | 2.5 |

  - Option 4: Parameters from  MTK

For 39GHz:

- Option 1: Example 1-based using following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

  - Parameters from vivo

  - Parameters from Anritsu

- Option 2: Example 2 in TR38.803.

- New phase noise model using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

  - Option 3: Parameters from MTK

- Option 4: Adopt min(example1, example2) as the phase noise profile for UL256QAM, where ‘example2’ refers to the example phase noise profiles in TR38.803 and ‘example1’ refers to Example 1-based for 39GHz.

5.2.3.3 Simulation results from Xiaomi

R4-2308805

Based on the agreed simulation assumptions and candidate phase noise profiles, the EVM floor is further evaluated for CP- OFDM and DFT-s-OFDM.

For 29GHz:

Table 5.2.3.3-1 EVM floor and benefit for with PTRS CP-OFDM at 29GHz

| CP-OFDM with phase noise， 29GHz, 120kHz, 64RB，K-PTRS=2 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1 from TR38.803 | -31.1 | -31.5 | 0.4 |
| Example 2 from TR38.803 | -26.6 | -28.4 | 1.8 |
| Phase noise Profile from Qualcomm | -34.8 | -36.0 | 1.2 |
| Phase noise Profile from MTK | -35.4 | -36.0 | 0.7 |

Table 5.2.3.3-2 EVM floor and benefit for with PTRS DFT-s-OFDM at 29GHz

| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 64RB，N_group = 8, N_samp = 4 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1 from TR38.803 | -31.2 | -30.1 | -1.0 |
| Example 2 from TR38.803 | -26.0 | -30.1 | 4.1 |
| Phase noise Profile from Qualcomm | -34.8 | -34.9 | 0.2 |
| Phase noise Profile from MTK | -35.4 | -34.7 | -0.7 |

From table 5.2.3.3-1 and table 5.2.3.3-2 for 29GHz, we can see no matter for CP- OFDM and DFT-s-OFDM:

- The EVM floors for new phase noise profiles from QC and MTK are reduced obviously compared to Example 1 and Example 2.

- Further compared new phase noise profiles from QC and MTK, we can see the EVM floor of MTK’s phase noise profile is lower than Qualcomm’s when without PTRS correction. And they have similar EVM floor when adding PTRS correction. Since the CPE (common phase error) impacts is closely related to the performance of phase noise profile, it will get less benefit from PTRS correction if the phase noise profile introduce less CPE (common phase error).

Therefore, we can’t only judge the phase noise profile according to the net benefit but also need consider the performance without PTRS correction.

These two new phase noise profiles have similar performance after PTRS correction for both of CP-OFDM and DFT-S-OFDM, from this point, both of them can be used in MPR simulation.

For 39GHz:

Table 5.2.3.3-3 EVM floor and benefit for with PTRS CP-OFDM at 39GHz

| CP-OFDM with phase noise， 39GHz, 120kHz, 64RB，K-PTRS=2 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1:VIVO parameter | -27.8 | -28.1 | 0.3 |
| Example 1:Anritsu parameter | -28.1 | -28.5 | 0.4 |
| Example 2 from TR38.803 | -24.1 | -25.8 | 1.7 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -32.2 | -33.3 | 1.1 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -32.2 | -33.3 | 1.1 |
| Phase noise Profile from MTK | -32.4 | -33.0 | 0.6 |

Table 5.2.3.3-4 EVM floor and benefit for with PTRS DFT-s-OFDM at 39GHz

| CP-OFDM with phase noise， 39GHz, 120kHz, 64RB，N_group = 8, N_samp = 4 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1:VIVO parameter | -27.9 | -26.7 | -1.2 |
| Example 1:Anritsu parameter | -28.2 | -27.0 | -1.2 |
| Example 2 from TR38.803 | -23.6 | -27.5 | 4.0 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -32.1 | -32.1 | 0.0 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -32.1 | -32.2 | 0.1 |
| Phase noise Profile from MTK | -32.4 | -31.6 | -0.8 |

From table 5.2.3.3-3 and table 5.2.3.3-4 for 39GHz, we can see no matter for CP- OFDM and DFT-s-OFDM:

- The EVM floors of Example 1-based from vivo and Anritsu is better than example 2, but still worse than -29.1 dB EVM requirement for 256QAM.

- And the EVM floors of the new phase noise profiles that adopting min(Example 1-based, Example 2) and from MTK are reduced obviously compared to Example 1-based and Example 2.

- Further compared new phase noise profiles from adopting min(Example 1-based, Example 2) and MTK, we can see the EVM floor of MTK’s phase noise profile is better a little than that adopting min(Example 1-based, Example 2) a little when without PTRS correction. But the EVM floor of MTK’s phase noise profile is worse a little than that adopting min(Example 1-based, Example 2) when with PTRS correction.

These two new phase noise profiles that adopting min(Example 1-based, Example 2) and from MTK have better EVM floor than -29.1 dB EVM requirement for 256QAM whether adding PTRS correction or not for both of CP-OFDM and DFT-S-OFDM, from this point, both of them can be used in MPR simulation.

But considering phase noise profiles that adopting min(Example 1-based, Example 2) hasn’t clearly physical description, we can use the new phase noise profile from MTK in MPR simulation.

5.2.3.4 Simulation results from vivo

R4-2308227

In the simulation, distortion other than phase noise is not considered, e.g., PA non-linearity, transceiver impairment, etc., to ensure the results only show the feasibility of different phase noise profiles.

The simulation results for 29GHz bands are shown in Table 5.2.3.4-1

Table 5.2.3.4-1 simulation results for 29GHz band

| 64RB |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- |
| CP-OFDM | Option 1 | -28.3288 | -28.9129 | 0.5841 |
|  | Option 2 | -25.3244 | -25.5883 | 0.2639 |
|  | Option 3 | -31.7997 | -33.0798 | 1.2801 |
|  | Option 4 | -33.0942 | -33.3608 | 0.2666 |
| DFT-s-OFDM | Option 1 | -28.8567 | -27.1551 | -1.7016 |
|  | Option 2 | -25.4507 | -28.9614 | 3.5107 |
|  | Option 3 | -32.8215 | -29.8556 | -2.9659 |
|  | Option 4 | -33.2358 | -30.0408 | -3.195 |

Based on the simulation results, we have the following observations:

Observation 1: The phase noise profile described in option 1 and option 2 is not feasible for the 29GHz band.

Observation 2: CP-OFDM can benefit from PTRS correction in UL 256QAM but DFT-s-OFDM can’t.

The simulation results for 39GHz are shown in Table 5.2.3.4-2.

Table 5.2.3.4-2 simulation results for the 39GHz band

| 64RB |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- |
| CP-OFDM | Option 1 | -25.1733 | -25.3235 | 0.1502 |
|  | Option 2 | -22.7246 | -22.9743 | 0.2497 |
|  | Option 3 | -29.8719 | -30.2049 | 0.333 |
|  | Option 4 | -29.9468 | -30.182 | 0.2352 |
| DFT-s-OFDM | Option 1 | -25.3171 | -23.5189 | -1.7982 |
|  | Option 2 | -22.8672 | -26.3839 | 3.5167 |
|  | Option 3 | -30.0035 | -28.8569 | -1.1466 |
|  | Option 4 | -30.0888 | -28.5366 | -1.5522 |

It is noted that only phase noise is considered in the simulation and when other distortion needs to be counted in MPR evaluation, even under option 3 or option 4 phase noise, it is still challenging to achieve the 29.1 dB EVM requirement for UE.

Observation 5: Either option 3 or option 4 may still be infeasible for MPR evaluation in the 39GHz band.

5.2.3.5 Simulation results from Anritsu Limited

R4-2307043

Figure 5.2.3.5-1 Comparison of simulation results

| Phase noise profile at 29.55GHz | 38_803 Example1 [5] |  | 38_803 Example2 [5] |  | Qualcomm Min (38_803 Example1, 38_803 Example2) using pole-zero model [7] |  | MediaTek [7] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fmin used in integration (Hz) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) |
| 1.00E+03 | 26.05 | 5.0 | 28.2 | 3.9 | 27.8 | 4.1 | 29.4 | 3.4 |
| 1.00E+04 | 28.34 | 3.8 | 28.5 | 3.8 | 31.8 | 2.6 | 32.3 | 2.4 |
| 1.00E+05 | 28.65 | 3.7 | 28.6 | 3.7 | 32.5 | 2.4 | 32.7 | 2.3 |
| 1.00E+06 | 29.08 | 3.5 | 31.9 | 2.5 | 33.7 | 2.1 | 33.3 | 2.2 |
| 1.00E+07 | 33.3 | 2.2 | 41.6 | 0.8 | 41.9 | 0.8 | 37.5 | 1.3 |
| Note: Fmax is fixed to 100MHz in the calculations. |  |  |  |  |  |  |  |  |

Table 5.2.3.5-1 Simulation results for UE example 2 phase noise profile.

The phase noise profiles from MediaTek and Qualcomm improves the SNR by about 3.5dB compare to the case of the Example 1 and Example phase noise profiles.

Observation 1: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 29.55GHz.

Figure 5.2.3.5-2 Comparison of simulation results – 2nd set of results

| Phase noise profile at 39GHz | 38_803 Example1 [5] |  | 38_803 Example2 [5] |  | Qualcomm Min (38_803 Example1, 38_803 Example2) [2] – Option1 [6] |  | MediaTek [7] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fmin used in integration (Hz) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) |
| 1.00E+03 | 23.1 | 7.0 | 25.8 | 5.1 | 27.3 | 4.3 | 26.7 | 4.6 |
| 1.00E+04 | 25.0 | 5.6 | 26.1 | 5.0 | 28.7 | 3.7 | 29.4 | 3.4 |
| 1.00E+05 | 25.2 | 5.5 | 26.2 | 4.9 | 29.3 | 3.4 | 29.7 | 3.3 |
| 1.00E+06 | 25.6 | 5.3 | 29.5 | 3.3 | 30.3 | 3.1 | 30.2 | 3.1 |
| 1.00E+07 | 29.8 | 3.3 | 39.2 | 1.1 | 39.2 | 1.1 | 34.4 | 1.9 |
| Note: Fmax is fixed to 100MHz in the calculations. |  |  |  |  |  |  |  |  |

Table 5.2.3.5-2 Simulation results for UE example 2 phase noise profile.

Using Min() phase noise profile improves the SNR by about 3.5dB compare to the case of the Example 1 and Example phase noise profiles. The phase noise profile by MediaTek is also giving about 2.5dB improvement of the SNR.

Observation 2: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 39GHz.

5.2.3.6 Simulation results from MediaTek Inc.

R4-2307931

To isolate the EVM impact of phase noise, no other types of impairments were added to the waveform. Table 5.2.3.6-1 and Table 5.2.3.6-2 show the initial simulated results.

Table 5.2.3.6-1 Benefit of PTRS in 256QAM, 64RB, 100MHz, SCS= 120kHz, CP-OFDM @29GHz

| CP-OFDM w/ phase noise, 256QAM, 120kHz, 64RB, DMRS based CPE removal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Phase noise model | PTRS configuration | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| TR38.803: UE example 1 | L = 1, K =2 | -28.3 | -27.7 | 0.6 |
| Our proposal | L = 1, K =2 | -32.7 | -31.6 | 1.1 |
| Qualcomm’s proposal | L = 1, K =2 | -32.5 | -30.7 | 1.8 |

Table 5.2.3.6-2 Benefit of PTRS in 256QAM, 64RB, 100MHz, SCS= 120kHz, CP-OFDM @39GHz

| CP-OFDM w/ phase noise, 256QAM, 120kHz, 64RB, DMRS based CPE removal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Phase noise model | PTRS configuration | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| TR38.803: UE example 1 | L = 1, K =2 | -25.7 | -25.3 | 0.4 |
| Our proposal | L = 1, K =2 | -29.6 | -28.7 | 0.9 |

From Table 5.2.3.6-1, we can find that the EVM performance with our proposed phase noise profile is very close to Qualcomm’s proposal at 29GHz after PTRS processing. Therefore, our proposal and Qualcomm’s proposal for the phase noise profile can be used in 256QAM MPR evaluation for FR2-1.

Observation 1: EVM performance with our proposed phase noise profile is very close to Qualcomm’s proposal at 29GHz after PTRS processing.

5.2.3.7 Simulation results from LG Electronics

R4-2308223

Table 5.2.3.7-1 simulation assumption for phase noise

| Parameters | Value |
| --- | --- |
| Frequency | 29, 39 GHz |
| SCS | 120kHz |
| BW | 100MHz |
| RB Size | 64RB for 120kHz |
| Background AWGN | No additional noise |
| Time offset/Frequency offset | 0 |
| Antenna configuration | 1T1R |
| Modulation | 256QAM |
| Waveform type | CP-OFDM |
| DMRS | 3 symbols per slot (UL DMRS add-pos = 2) |
| PTRS configuration | ON For CP-OFDM: L-PTRS (Time density) = 1 (every 1 symbol) K-PTRS (Freq density) =2 (every 2 RBs) |
| EVM measurement | Data aided EVM calculation, based on ideal data signal |
| Phase noise profiles | For 29 GHz Case 1: Example 1 in TR 38.803 for 29 GHz % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 29.35; Case2: Parameters from QC % fz = [3e3 7e5 9e5]; % fp = [1 1e6 1.1e6]; % alphaz = [2.37 4.7 2]; % alphap = [3.3 5.3 2.5]; % PSD0 = 33; Case 3: Prameters from MTK % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 29.35; For 39 GHz Case 4: Parameters from Vivo % fz = [3e3 494e3 418e6]; % fp = [1 1.58e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 34.41; Case 5: Parameters from Anritsu % fz = [3e3 499e3 3.9e8]; % fp = [1 1.54e6 3e7]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 34.4; Case 6: Parameters from MTK % fz = [3e3 620e3 240e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 31.76; |

Table 5.2.3.7-2 CP-OFDM [29 GHz, 120KHZ, 100 MHz, 64RBs, (L=1, K=2)]

|  | EVM (dB) |
| --- | --- |
| Case 1 | -30.75 |
| Case 2 | -31.57 |
| Case 3 | -31.37 |

Table 5.2.3.7-3 CP-OFDM [39 GHz, 120KHZ, 100 MHz, 64RBs, (L=1, K=2)]

|  | EVM (dB) |
| --- | --- |
| Case 4 | -28.56 |
| Case 5 | -28.77 |
| Case 6 | -30.96 |

We simulate EVM according to phase noise profiles and simulation results are shown in table 5.2.3.7-2 and table 5.2.3.7-3.

For 29 GHz simulation results, we can observe the feasibility in case 1, case 2 and case 3. But, case 2 and case 3 phase noise profiles can achieve more EVM margin than case 1.

Observation 1: For 29 GHz simulation results, we can observe the feasibility in case 1, case 2 and case 3. But, case 2 and case 3 phase noise profiles can achieve more EVM margin than case 1.

For 39 GHz simulation results, we can observe the feasibility in only case 6. Case 4 and case 5 can not achieve below 3.5% EVM.

Observation 2: For 39 GHz simulation results, we can observe the feasibility in only case 6. Case 4 and case 5 can not achieve below 3.5% EVM.

5.2.3.8 Simulation results from Sony

R4-2309028

Multiple phase noise models have been proposed in the last few RAN4 meetings, and their contribution to the EVM noise floor based on the agreed simulation assumption in [1] is provided in table 5.2.3.8-1. Only results for 120 kHz SCS (64 RB) at 28 GHz are presented here, but other combinations of numerologies show a similar trend.

Table 5.2.3.8-1 The EVM performance with different phase noise profile in [1] for 120 kHz SCS at 28 GHz

|  | CP-OFDM | DFTs-OFDM |
| --- | --- | --- |
| Ex.1 from TR 38.803 | -28.3 dB | -26.9 dB |
| Model proposed by QC | -32.9 dB | -31.4 dB |
| Model proposed by MTK | -33 dB | -31.4 dB |

Reusing example 1 in TR 38.803 may cause an excessive EVM floor, which is not feasible for MPR evaluation for 256 QAM in our view. In addition, we shall notice that the 256 QAM is mainly targeted for FWA/CPE type of devices, which shall be capable of taking advantage of better-performed RF components than mobile handsets. Meanwhile, to leave sufficient EVM margin for other components, e.g., PA. IQ imbalanced, it is necessary for the device to strive to achieve a better phase noise performance.

Observation 1: the phase noise examples in TR38.803 does not leave enough EVM margin for 256QAM evaluation.

Physical-wise, the phase noise at different frequency offsets is usually dominated by various components inside a PLL, including the VCO, the reference clock, and other associated circuitry. In our view, combining different phase noise profiles is physically viable. Therefore, it is proposed to adopt the improved phase noise model as a baseline for MPR evaluation.

5.2.3.9 Simulation results from Qualcomm

R4-2300707

The penalties from PTRS corrections work against any benefit the PTRS brings in reducing the impact of phase noise. The benefit is a strong function of the phase noise profile, due to the nature of PTRS corrections. The variable benefit cannot be generalized across implementations. The net benefit is the EVM benefit when PTRS processing is enabled despite the penalties discussed previously. The phase noise profiles used below are merely examples for calibration across companies, neither seems particularly worthy of 256QAM.

In the presence of phase noise, the DMRS symbols themselves have some CPE associated with them. The EVM algorithm can either ignore them and find the best channel estimate despite the phase noise, or it can de-rotate each DMRS symbol to nominal phase, along with the PUSCH symbols around the DMRS symbol. This latter variant is termed ‘DMRS based CPE removal’ in the tables below.

Tables 5.2.3.9-1 and -2 shows the net impact of PTRS calculations in the presence of phase noise for CP-OFDM with 64 RBs.

| CP-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile from TR38.803 | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE example 2 @ 45 GHz | -21.8 | -20.6 | 1.2 |
| UE example 1 @ 45 GHz | -24.8 | -24.5 | 0.3 |

Table 5.2.3.9-1: Net benefit for CP-OFDM with PTRS correction, DMRS based CPE removal

| CP-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, no DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile from TR38.803 | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE example 2 @ 45 GHz | -20.8 | -19.1 | 1.7 |
| UE example 1 @ 45 GHz | -23.8 | -23.0 | 0.8 |

Table 5.2.3.9-2: Net benefit for CP-OFDM with PTRS correction

| CP-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile from TR38.803 | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE example 1 @ 30 GHz | -30.1 | -29.7 | 0.4 |
| UE example 2 @ 30 GHz | -25.5 | -24.2 | 1.3 |

Table 5.2.3.9-3: 30 GHz EVM floor in relation to -29.2 dB EVM requirement for 256QAM

Tables5.2.3.9-4 and -5 shows the net impact of PTRS calculations in the presence of phase noise for DFT-s-OFDM. These projections are somewhat revised from earlier estimates due to algorithm development.

| DFT-s-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile from TR38.803 | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE example 2 @ 45 GHz | -24.6 | -20.5 | 4.1 |
| UE example 1 @ 45 GHz | -23.5 | -24.6 | -1.1 |

Table 5.2.3.9-4: Net benefit for DFT-s-OFDM with PTRS correction, DMRS based CPE removal

| DFT-s-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, no DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile from TR38.803 | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE example 2 @ 45 GHz | -23.8 | -19.2 | 4.6 |
| UE example 1 @ 45 GHz | -22.5 | -23.0 | -0.5 |

Table 5.2.3.9-5: Net benefit for DFT-s-OFDM with PTRS correction

| DFT-s-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile from TR38.803 | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE example 1 @ 30 GHz | -28.9 | -29.7 | -0.8 |
| UE example 2 @ 30 GHz | -28.0 | -24.2 | 3.8 |

Table 5.2.3.9-6: 30 GHz EVM floor in relation to -29.2 dB EVM requirement for 256QAM

R4-2304601

Figure 5.2.3.9-1: Proposed phase noise profiles for UL256QAM

In R4-2300707, we proposed an UL256QAM-worthy phase noise profile out of the profiles in TR38.803 as follows:

Fhybrid(f) = min[Fex1(f),Fex2(f)]

The WF R4-2303491 also recorded another phase noise profile as a candidate, referred to simply as ‘option 2’ for brevity. Figures 2.1-1 shows the proposed profiles in relation to the phase noise profile in the TR.

These profiles were evaluated with test signals with no impairment other than phase noise. While impractical, this simplification is useful to isolate the EVM impact due to phase noise and associated processing. Tables below summarize findings. Constellation diagrams are included in the Annex.

| CP-OFDM, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| 30G Hybrid | -33.5 | -32.7 | 0.8 |
| 40G Hybrid | -30.1 | -29.2 | 0.9 |
| Option 2 | -28.4 | -28.1 | 0.3 |

| DFT-s-OFDM, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal |  |  |  |
| --- | --- | --- | --- |
| Phase noise profile | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| 30G Hybrid | -32.7 | -32.8 | -0.1 |
| 40G Hybrid | -29.4 | -29.2 | 0.2 |
| Option 2 | -26.9 | -28.1 | -1.2 |

Given the EVM requirement of -29.1 dB, FR2-1 UEs that can reasonably support UL256QAM seem to be limited to 24-30 GHz. It further appears that the ‘Option 2’ profile has too much high-offset phase noise to support 256QAM. We therefore propose to adopt the only phase noise profile that seems able to support UL256QAM among the WF options. A further consideration is to use the favorable 30 GHz phase noise assumption to establish requirements that advanced UEs in the future can support in higher bands.

Observation 1: UL256QAM looks infeasible with the ‘Option 2’ profile [WF R4-2303491] even for n257, n258 and n261.

5.2.3.10   Simulation results from ZTE

R4-2304689

There were two new phase noise models proposed in last meeting, one is to adopt (example 1, example 2), and the other one is to adopt new parameters based on the multi-pole/zero model. By considering the existing two ones, there are four phase noise model alternatives as below:

Alternative 1. example 1 defined in 38803

Alternative 2. example 2 defined in 38803

Alternative 3. min(example 1, example 2)

Alternative 4. example 1 defined in 38803 with parameters updated

Figure 5.2.3.10-1shows the phase noise profiles for different alternatives.

Figure 5.2.3.10-1. Phase noise profile comparison between different alternatives

From figure 5.2.3.10-1 we can find that lower phase noise can be achieved by alternative 3 and alternative 4 compared to the example 1 and example 2. In table 5.2.3.10-1, we compare the EVM performance for the above alternatives for CP-OFDM@45GHz, which is same conditions as in companies’ paper in last meeting.

Table 5.2.3.10-1 EVM comparison between different alternatives

| CP-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal@45GHz |  |  |  |
| --- | --- | --- | --- |
| Phase noise | EVM(dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE alt 1 | -24.7 | -24.5 | 0.2 |
| UE alt 2 | -21.5 | -20.4 | 1.1 |
| UE alt 3 | -30.4 | -28 | 2.4 |
| UE alt 4 | -27.3 | -27.1 | 0.2 |

From table 5.2.3.10-1 we can find that alt 3 can achieve much smaller phase noise which means much smaller EVM can be expected @ 45GHz. Although the simulation is for 45GHz, it is reasonable to expect that the same results can be applied for other millimeter wave frequency.

For alt 3, although smallest phase noise can be achieved, the logic to minimize both example 1 and example 2 may not be clear. As explained in TS38.803, example 1 is based upon measurements made on a prototype CMOS device, with a larger PLL bandwidth, and example 2 is based on recent research on technology capabilities for UE and BS where CMOS is considered for the UE and GaAs is considered for the BS, so a lower PLL bandwidth than example 1 can be assumed for example2. It is clearly that Example 2 distinguishes UE and BS while example 1 is the common model for both UE and BS, from this perspective, the reasons for directly taking the minimum value of both, min(Ex 1, Ex 2), are not very clear. Nevertheless, we still believe the lower phase noise can not only improve the EVM performance, but also improve the MPR requirements.

Observation 1: Smallest phase noise can be achieved for Alt 3, i.e. min(Ex1, Ex2), and lower EVM caused by phase noise can be expected.

5.2.3.11   Summary the simulation results for 29GHz phase noise profile evaluation

CP-OFDM:

| Companies | Waveform | Phase noise model @29GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -31.1 | -31.5 | 0.4 |
|  |  | UE example 2 |  |  | -26.6 | -28.4 | 1.8 |
|  |  | From Qualcomm |  |  | -34.8 | -36.0 | 1.2 |
|  |  | From MTK |  |  | -35.4 | -36.0 | 0.7 |
| Vivo R4-2308227 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -28.3288 | -28.9129 | 0.5841 |
|  |  | UE example 2 |  |  | -25.3244 | -25.5883 | 0.2639 |
|  |  | From Qualcomm |  |  | -31.7997 | -33.0798 | 1.2801 |
|  |  | From MTK |  |  | -33.0942 | -33.3608 | 0.2666 |
| Anritsu R4-2307043 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -28.65 |  |  |
|  |  | UE example 2 |  |  | -28.6 |  |  |
|  |  | From Qualcomm |  |  | -32.5 |  |  |
|  |  | From MTK |  |  | -32.7 |  |  |
| MTK R4-2307931 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -27.7 | -28.3 | 0.6 |
|  |  | From Qualcomm |  |  | -30.7 | -32.5 | 1.8 |
|  |  | From MTK |  |  | -31.6 | -32.7 | 1.1 |
| LGE R4-2308223 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 |  | -30.75 |  |
|  |  | From Qualcomm |  |  |  | -31.57 |  |
|  |  | From MTK |  |  |  | -31.37 |  |
| Sony R4-2309028 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -28.3 dB |  |  |
|  |  | From Qualcomm |  |  | -32.9 dB |  |  |
|  |  | From MTK |  |  | -33 dB |  |  |

DFT-s-OFDM:

| Companies | Waveform | Phase noise model @29GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB | N_group = 8, N_samp = 4 | -31.2 | -30.1 | -1.0 |
|  |  | UE example 2 |  |  | -26.0 | -30.1 | 4.1 |
|  |  | From Qualcomm |  |  | -34.8 | -34.9 | 0.2 |
|  |  | From MTK |  |  | -35.4 | -34.7 | -0.7 |
| Vivo R4-2308227 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB | N_group = 8, N_samp = 4 | -28.8567 | -27.1551 | -1.7016 |
|  |  | UE example 2 |  |  | -25.4507 | -28.9614 | 3.5107 |
|  |  | From Qualcomm |  |  | -32.8215 | -29.8556 | -2.9659 |
|  |  | From MTK |  |  | -33.2358 | -30.0408 | -3.195 |
| Sony R4-2309028 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB |  | -26.9 dB |  |  |
|  |  | From Qualcomm |  |  | -31.4 dB |  |  |
|  |  | From MTK |  |  | -31.4 dB |  |  |

5.2.3.12   Summary the simulation results for 39GHz phase noise profile evaluation

CP-OFDM

| Companies | Waveform | Phase noise model @39GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | CP-OFDM | Example 1 from VIVO | 120kHz, 64RB | L = 1, K =2 | -27.8 | -28.1 | 0.3 |
|  |  | Example 1from Anritsu |  |  | -28.1 | -28.5 | 0.4 |
|  |  | Example 2 from TR38.803 |  |  | -24.1 | -25.8 | 1.7 |
|  |  | Min(Example1-VIVO, Example2) |  |  | -32.2 | -33.3 | 1.1 |
|  |  | Min(Example1- Anritsu, Example2) |  |  | -32.2 | -33.3 | 1.1 |
|  |  | From MTK |  |  | -32.4 | -33.0 | 0.6 |
| Vivo R4-2308227 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -25.1733 | -25.3235 | 0.1502 |
|  |  | UE example 2 |  |  | -22.7246 | -22.9743 | 0.2497 |
|  |  | From Qualcomm |  |  | -29.8719 | -30.2049 | 0.333 |
|  |  | From MTK |  |  | -29.9468 | -30.182 | 0.2352 |
| Anritsu R4-2307043 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -25.2 |  |  |
|  |  | UE example 2 |  |  | -26.2 |  |  |
|  |  | From Qualcomm |  |  | -29.3 |  |  |
|  |  | From MTK |  |  | -29.7 |  |  |
| MTK R4-2307931 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -25.3 | -25.7 | 0.4 |
|  |  | From MTK |  |  | -28.7 | -29.6 | 0.9 |
| LGE R4-2308223 | CP-OFDM | Example 1 from VIVO | 120kHz, 64RB | L = 1, K =2 |  | -28.56 |  |
|  |  | Example 1 from Anritsu |  |  |  | -28.77 |  |
|  |  | From MTK |  |  |  | -30.96 |  |

DFT-s-OFDM

| Companies | Waveform | Phase noise model @39GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | DFT-s-OFDM | Example 1 from VIVO | 120kHz, 64RB | N_group = 8, N_samp = 4 | -27.9 | -26.7 | -1.2 |
|  |  | Example 1from Anritsu |  |  | -28.2 | -27.0 | -1.2 |
|  |  | Example 2 from TR38.803 |  |  | -23.6 | -27.5 | 4.0 |
|  |  | Min(Example1-VIVO, Example2) |  |  | -32.1 | -32.1 | 0.0 |
|  |  | Min(Example1- Anritsu, Example2) |  |  | -32.1 | -32.2 | 0.1 |
|  |  | From MTK |  |  | -32.4 | -31.6 | -0.8 |
| Vivo R4-2308227 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB | N_group = 8, N_samp = 4 | -25.3171 | -23.5189 | -1.7982 |
|  |  | UE example 2 |  |  | -22.8672 | -26.3839 | 3.5167 |
|  |  | From Qualcomm |  |  | -30.0035 | -28.8569 | -1.1466 |
|  |  | From MTK |  |  | -30.0888 | -28.5366 | -1.5522 |

5.2.4	MPR simulation

5.2.4.1   Simulation assumptions

Antenna configuration and PA calibration point for MPR simulation

      - The MPR evaluation was performed by using 32 PAs, 16 for each polarizations within an antenna array for PC1/2/5 keeping align with the antenna configuration agreed in system level simulation.

      - PA calibration point should follow current definition in Spec 38.101-2:

  - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

  - Calculate MPR as total backoff needed for 256QAM from this calibration point.

Emission requirements for MPR simulation

Follow the current Spec 38.101-2, otherwise specified

      - Occupied bandwidth (Table 6.5.1-1 in TS 38.101-2)

      - General NR spectrum emission mask for FR2-1 (Table 6.5.2.1-1 in TS 38.101-2)

      - NR ACLR1 for FR2-1 (Table 6.5.2.3-1 in TS 38.101-2)

      - General in-band emissions limit for FR2-1 (Tables 6.4.2.3.2-1 for PC1, 6.4.2.3.3-1 for PC2, 6.4.2.3.6-1 for PC5 in TS 38.101-2)

      - General NR spurious emission limits for FR2-1 (Table 6.5.3-2 in TS 38.101-2)

      - Maximum error vector magnitude (EVM) 3.5% for 29GHz and 39GHz as agreed in RAN4 #104-bis-e meeting.

EVM budget for MPR evaluation:

      - Only consider the total value of 3.5% for Tx EVM

  - Companies need to clarify the components of Tx EVM in their simulation results, including

    - Phase noise

    - Value for IQ imbalance

    - PA and transmitter non-linearity

Phase noise profiles for MPR simulation:

- For 29GHz:

  - Both of new phase noise profiles from Qualcomm and MTK for 29GHz are feasible for MPR simulation.

    - New phase noise profiles using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

    - Parameters from Qualcomm

| PSD0 | 32 dB |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 6e5 | 4.7 | 8e5 | 5.3 |
| 3 | 8e5 | 2.0 | 1.1e6 | 2.5 |

    - Parameters from MTK

- For 39GHz:

  - The new phase noise profile from MTK for 39GHz is feasible for MPR simulation.

    - New phase noise profile using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

    - Parameters from MTK

PTRS configuration for MPR simulation:

- For CP-OFDM

  - Adopt L-PTRS = 1 K-PTRS =2 as PTRS configuration.

- For DFT-s-OFDM

  - No PTRS configuration for  wider RB allocations as starting point

Companies are encouraged to simulate the difference between with/without PTRS configuration under narrow RBs (<20 RB) allocations.

5.2.4.2   Simulation results from LGE

R4-2304634 MPR simulation for PC1 and PC2

Clarification of EVM budge, PA model, Phase noise profile, PTRS configuration for CPE correction:

EVM budget

- For CP-OFDM (L=1, K=2), (64 RB,120 kHz)

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise+IQ Imbalance | 2.95 | 30.6 |
| PA Non-linearity & Transmitter | 1.88 | 34.5 |
| Total | 3.50 | 29.1 |

- For DFT-s-OFDM (no PTRS correction), (64 RB,120 kHz)

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise+IQ Imbalance | 2.30 | 32.8 |
| PA Non-linearity & Transmitter | 2.64 | 31.6 |
| Total | 3.50 | 29.1 |

PA

- Using the 39 dBm PA model considering the Tx post loss for PC1

- Using the 27 dBm PA model considering the Tx post loss for PC2

Phase noise profile

- Using the example 1 in TR38.803.

PTRS configuration for CPE correction

- Using the PTRS (K=2, L=1) for CP-OFDM

- No PTRS for DFT-s-OFDM

Table 5.2.4.2-1 RB allocations

For PC1 ■Considered RB allocation for 100 MHz in FR2-1 256QAM MPR simulation ■Region 1 ●DFT-s-OFDM : [Start RB position:22, allocated RB: 20] ●CP-OFDM : [Start RB position:22, allocated RB: 22] ■Region 2 ●DFT-s-OFDM : [Start RB position:16, allocated RB: 32] ●CP-OFDM : [Start RB position:16, allocated RB: 32] ■Outer RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 64], [Start RB position:0, allocated RB: 10]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 66], [Start RB position:0, allocated RB: 10]) ■Considered RB allocation for 400 MHz in FR2-1 256QAM MPR simulation ■Region 1 ●DFT-s-OFDM : [Start RB position:60, allocated RB: 60] ●CP-OFDM : [Start RB position:66, allocated RB: 60] ■Region 2 ●DFT-s-OFDM : [Start RB position:60, allocated RB: 120] ●CP-OFDM : [Start RB position:66, allocated RB: 132] ■Outer RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 264], [Start RB position:0, allocated RB: 40]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 264], [Start RB position:0, allocated RB: 40]) For PC2 ■Considered RB allocation for 100 MHz in FR2-1 256QAM MPR simulation ■Region 1 inner RB allocation ●DFT-s-OFDM : [Start RB position:22, allocated RB: 20] ●CP-OFDM : [Start RB position:23, allocated RB: 20] ■Edge RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 64], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 20]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 66], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 20]) ■Considered RB allocation for 400 MHz in FR2-1 256QAM MPR simulation ■Region 1 inner RB allocation ●DFT-s-OFDM : [Start RB position:88, allocated RB: 80] ●CP-OFDM : [Start RB position:88, allocated RB: 88] ■Edge RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 240], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 80]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 264], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 80])

Table  5.2.4.2-2 Simulated MPR results for PC1 UL256QAM

| CBW (100 MHz) | Region 1 [22, 20] | Limiting factor | Region 2 [16, 32] | Limiting factor | Outter 1 [0, 64] | Limiting factor | Outter 2 [0, 10] | Limiting factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 8.04 | EVM | 8.05 | EVM | 8.38 | EVM | 6.68 | SEM |
| CBW (100 MHz) | Region 1 [22, 22] | Limiting factor | Region 2 [16, 32] | Limiting factor | Outter 1 [0, 66] | Limiting factor | Outter 2 [0, 10] | Limiting factor |
| CP-OFDM | 10.44 | EVM | 9.86 | EVM | 10.31 | EVM | 9.01 | EVM |
| CBW (400 MHz) | Region 1 [60, 60] | Limiting factor | Region 2 [60, 120] | Limiting factor | Outter 1 [0, 264] | Limiting factor | Outter 2 [0, 40] | Limiting factor |
| DFT-s-OFDM | 8.44 | EVM | 9.28 | EVM | 9.40 | EVM | 8.44 | EVM |
| CBW (400 MHz) | Region 1 [66, 60] | Limiting factor | Region 2 [66, 132] | Limiting factor | Outter 1 [0, 264] | Limiting factor | Outter 2 [0, 40] | Limiting factor |
| CP-OFDM | 10.86 | EVM | 11.96 | EVM | 10.76 | EVM | 11.96 | EVM |

[start RB position, allocated RB number]

Table  5.2.4.2-3 Simulated MPR results for PC2 UL256QAM

| CBW (100 MHz) | Region 1 [22, 20] | Limiting factor | Outter1 [0, 64] | Limiting factor | Outter 2 [0, 1] | Limiting factor | Outter 3 [0, 20] | Limiting factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 7.54 | EVM | 7.54 | EVM | 6.21 | EVM | 6.50 | EVM |
| CBW (100 MHz) | Region 1 [23, 20] | Limiting factor | Outter1 [0, 66] | Limiting factor | Outter 2 [0,1] | Limiting factor | Outter 3 [0, 20] | Limiting factor |
| CP-OFDM | 9.48 | EVM | 9.50 | EVM | 8.27 | EVM | 9.05 | EVM |
| CBW (400 MHz) | Region 1 [88, 80] | Limiting factor | Outter1 [0, 240] | Limiting factor | Outter 2 [0, 1] | Limiting factor | Outter 3 [0, 80] | Limiting factor |
| DFT-s-OFDM | 8.91 | EVM | 8.91 | EVM | 7.19 | EVM | 7.84 | EVM |
| CBW (400 MHz) | Region 1 [88, 88] | Limiting factor | Outter1 [0,264] | Limiting factor | Outter 2 [0,1] | Limiting factor | Outter 3 [0, 80] | Limiting factor |
| CP-OFDM | 10.96 | EVM | 10.84 | EVM | 9.04 | EVM | 10.09 | EVM |

[start RB position, allocated RB number]

Table  5.2.4.2-4 Proposed MPRWT for power class 1, BWchannel ≤ 200 MHz, FR2-1

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | 256 QAM | ≤ 10 | ≤ 9.5 | ≤ 9.5 |
| CP-OFDM | 256QAM | ≤ 12 | ≤ 12 | ≤ 11.5 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |  |

Table  5.2.4.2-5 Proposed MPRWT for power class 1, BWchannel = 400 MHz

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | 256 QAM | ≤ 11 | ≤ 10 | ≤ 11 |
| CP-OFDM | 256 QAM | ≤ 13.5 | ≤ 12.5 | ≤ 13.5 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |  |

Table  5.2.4.2-6 Proposed MPRWT for power class 2, BWchannel ≤ 200 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 256 QAM | ≤ 9 | ≤ 9 |
| CP-OFDM | 256 QAM | ≤ 11 | ≤ 11 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |

Table  5.2.4.2-7 Proposed MPRWT for power class 2, BWchannel = 400 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel ≤ 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 256 QAM | ≤ 10.5 | ≤ 10.5 |
| CP-OFDM | 256 QAM | ≤ 12.5 | ≤ 12.5 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |

R4-2308223 MPR simulation for PC2

Table 5.2.4.2-8 Simulated MPR results for 29 GHz PC2 UL256QAM

| CBW (100 MHz) | PN profile | Region 1 [22, 20] | Outer1 [0, 64] | Outter 2 [0, 1] | Outter 3 [0, 20] |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Case 1 | 7.54 | 7.54 | 6.21 | 6.50 |
|  | Case 2 | 7.22 | 7.34 | 5.96 | 6.18 |
|  | Case 3 | 7.22 | 7.22 | 6.00 | 6.19 |
| CBW (100 MHz) | PN profile | Region 1 [23, 20] | Outter1 [0, 66] | Outter 2 [0,1] | Outter 3 [0, 20] |
| CP-OFDM | Ex1 | 9.48 | 9.50 | 8.27 | 9.05 |
|  | QC’s | 9.26 | 9.27 | 8.04 | 8.72 |
|  | MDK’s | 9.26 | 9.26 | 7.74 | 8.83 |
| CBW (400 MHz) | PN profile | Region 1 [88, 80] | Outter1 [0, 240] | Outter 2 [0, 1] | Outter 3 [0, 80] |
| DFT-s-OFDM | Ex1 | 8.91 | 8.91 | 7.19 | 7.84 |
|  | QC’s | 8.06 | 8.06 | 7.10 | 7.33 |
|  | MDK’s | 8.48 | 8.06 | 7.11 | 7.44 |
| CBW (400 MHz) | PN profile | Region 1 [88, 88] | Outter1 [0,264] | Outter 2 [0,1] | Outter 3 [0, 80] |
| CP-OFDM | Ex1 | 10.96 | 10.84 | 9.04 | 10.09 |
|  | QC’s | 10.30 | 10.30 | 9.10 | 9.76 |
|  | MDK’s | 10.19 | 10.30 | 8.96 | 9.75 |

[start RB position, allocated RB number]

Table 5.2.4.2-9 MPR results for 29 GHz PC2 UL256QAM with margin

| CBW (100 MHz) |  | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- |
| DFT-s-OFDM | Example1 | 9 | 9 |
|  | QC | 8.5 | 9 |
|  | MDK | 8.5 | 8.5 |
| CP-OFDM | Example1 | 11 | 11 |
|  | QC | 11 | 11 |
|  | MDK | 11 | 11 |
| CBW (400 MHz) |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Example1 | 10.5 | 10.5 |
|  | QC | 9.5 | 9.5 |
|  | MDK | 10 | 9.5 |
| CP-OFDM | Example1 | 12.5 | 12.5 |
|  | QC | 12 | 12 |
|  | MDK | 11.5 | 12 |

In the last meeting, there was a concern that EVM 3.5% could not be achieve due to phase noise, or even if it was achieved, it would have a very large MPR, so new phase noise profiles that are more relaxed than the exisiting phase noise profile were proposed. In this clause, the simulated MPR values of PC2 UL 256QAM according to the phase noise profile are shown in Table 5 and Table 6. It was found that the MPR value for 29 GHz can be improved by 0~1 dB by using the phase noise profile of case 2 and case 3 compared to the exisiting case1.

Observation 3: The MPR value for 29 GHz can be improved by 0~1 dB by using the phase noise profile of case 2 and case 3 compared to the case 1.

R4-2312237 MPR simulation for PC2 with low AM-PM distortion PA

Table 5.2.4.2-10 phase noise profiles

For 29 GHz phase noise profile - option 1 (TR 38.803 Example 1) % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 32; - option 2 (QC’s) % fz = [3e3 7e5 9e5]; % fp = [1 1e6 1.1e6]; % alphaz = [2.37 4.7 2]; % alphap = [3.3 5.3 2.5]; % PSD0 = 33; - option 3 (MTK’s) % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 29.35; For 39 GHz phase noise profi - option 4 (MTK’s) % fz = [3e3 620e3 240e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 31.76;

We recognize the AM-PM distortion is the key factor for the MPR value based on our mmWave transceiver system. So, we performed MPR simulation with low AM-PM distortion PA. The low AM-PM distortion PA has only few degrees distortion until the AM-AM drops. The MPR result comparison between general PA and Low AM-PM distortion PA are showed in table 2 for 29 GHz and table 3 for 39 GHz.

-DFT-s-OFDM for 100 MHz CBW

MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 1~1.5 dB over MPR with Option2 phase noise profile only.

-CP-OFDM for 100 MHz CBW

MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 0.5~1 dB over MPR using only Option2 phase noise profile.The CP-OFDM waveform has a high PAPR, so the AM-AM distortion is more critical to the MPR value. That’s why the MPR improvement is less than DFT-s-OFDM when the lower AM-PM distortion PA is applied.

-DFT-s-OFDM for 400 MHz CBW

The MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 0~0.5 dB. The limiting factor for 400 MHz DFT-s-OFDM is IBE&EVM, so it has low improvement.

-CP-OFDM for 400 MHz CBW

The MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 0~1 dB over MPR using only Option2 phase noise profile.

Observation 1: The lower MPR value with low AM-PM distortion PA than MPR value with general PA is observed.

Table 5.2.4.2-11 Simulated MPR results for 29 GHz PC2 UL256QAM

|  | PA model | PN Profile | Region 1 [22, 20] | Outer1 [0, 64] | Outer 2 [0, 1] | Outer 3 [0, 20] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM (CBW: 100 MHz) | General | Option 1 | 7.54 | 7.54 | 6.21 | 6.50 |
|  | General | Option 2 | 7.22 | 7.34 | 5.96 | 6.18 |
|  | General | Option 3 | 7.22 | 7.22 | 6.00 | 6.19 |
|  | Low AM-PM distortion | Option 2 | 5.86 | 5.97 | 5.60 | 5.78 |
| CP-OFDM (CBW: 100 MHz) | General | Option 1 | 9.48 | 9.50 | 8.27 | 9.05 |
|  | General | Option 2 | 9.26 | 9.27 | 8.04 | 8.72 |
|  | General | Option 3 | 9.26 | 9.26 | 7.74 | 8.83 |
|  | Low AM-PM distortion | Option 2 | 8.39 | 8.29 | 7.51 | 8.18 |
|  | PN profile | Region 1 [88, 80] | Outer1 [0, 240] | Outer 2 [0, 1] | Outer 3 [0, 80] | Outer 3 [0, 20] |
| DFT-s-OFDM (CBW: 400 MHz) | General | Option 1 | 8.91 | 8.91 | 7.19 | 7.84 |
|  | General | Option 2 | 8.48 | 8.06 | 7.10 | 7.33 |
|  | General | Option 3 | 8.48 | 8.06 | 7.11 | 7.44 |
|  | Low AM-PM distortion | Option 2 | 8.26 | 7.54 | 7.06 | 7.23 |
| CP-OFDM (CBW: 400 MHz) | General | Option 1 | 10.96 | 10.84 | 9.04 | 10.09 |
|  | General | Option 2 | 10.30 | 10.30 | 9.10 | 9.76 |
|  | General | Option 3 | 10.19 | 10.30 | 8.96 | 9.75 |
|  | Low AM-PM distortion | Option 2 | 9.44 | 9.86 | 8.76 | 9.44 |

[start RB position, allocated RB number]

Table 5.2.4.2-12 Simulated MPR results for 39 GHz PC2 UL256QAM

|  | PA model | PN Profile | Region 1 [22, 20] | Outer1 [0, 64] | Outer 2 [0, 1] | Outer 3 [0, 20] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM (CBW: 100 MHz) | General | Option 4 | 7.44 | 7.33 | 6.01 | 6.37 |
|  | Low AM-PM distortion | Option 4 | 5.97 | 6.17 | 5.54 | 5.86 |
| CP-OFDM (CBW: 100 MHz) | General | Option 4 | 9.27 | 9.48 | 8.00 | 8.83 |
|  | Low AM-PM distortion | Option 4 | 8.37 | 8.39 | 7.49 | 8.31 |
|  | PN profile | Region 1 [88, 80] | Outer1 [0, 240] | Outer 2 [0, 1] | Outer 3 [0, 80] | Outer 3 [0, 20] |
| DFT-s-OFDM (CBW: 400 MHz) | General | Option 4 | 8.47 | 8.48 | 7.00 | 7.54 |
|  | Low AM-PM distortion | Option 4 | 8.48 | 7.64 | 6.84 | 7.33 |
| CP-OFDM (CBW: 400 MHz) | General | Option 4 | 10.73 | 10.85 | 9.13 | 9.82 |
|  | Low AM-PM distortion | Option 4 | 10.19 | 10.30 | 8.78 | 9.54 |

[start RB position, allocated RB number]

Table 5.2.4.2-13 MPR results for 29 GHz PC2 UL256QAM with margin

| CBW (100 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | General | Option 1 | 9 | 9 |
|  | General | Option 2 | 8.5 | 9 |
|  | General | Option 3 | 8.5 | 8.5 |
|  | Low AM-PM distortion | Option 2 | 7.5 | 7.5 |
| CP-OFDM | General | Option 1 | 11 | 11 |
|  | General | Option 2 | 11 | 11 |
|  | General | Option 3 | 11 | 11 |
|  | Low AM-PM distortion | Option 2 | 10 | 10 |
| CBW (400 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | General | Option 1 | 10.5 | 10.5 |
|  | General | Option 2 | 9.5 | 9.5 |
|  | General | Option 3 | 10 | 9.5 |
|  | Low AM-PM distortion | Option 2 | 10 | 9 |
| CP-OFDM | General | Option 1 | 12.5 | 12.5 |
|  | General | Option 2 | 12 | 12 |
|  | General | Option 3 | 11.5 | 12 |
|  | Low AM-PM distortion | Option 2 | 11 | 11.5 |

[start RB position, allocated RB number]

Table 5.2.4.2-14 MPR results for 39 GHz PC2 UL256QAM with margin

| CBW (100 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | General | Option 4 | 9 | 9 |
|  | Low AM-PM distortion | Option 4 | 7.5 | 7.5 |
| CP-OFDM | General | Option 4 | 11 | 11 |
|  | Low AM-PM distortion | Option 4 | 10 | 10 |
| CBW (400 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | General | Option 4 | 10 | 10 |
|  | Low AM-PM distortion | Option 4 | 10 | 9 |
| CP-OFDM | General | Option 4 | 12 | 12.5 |
|  | Low AM-PM distortion | Option 4 | 11.5 | 12 |

[start RB position, allocated RB number]

Table 4 and table 5 show the MPR value with the margin. We observed the MPR values using Low AM-PM distortion PA are lower than MPR value with general PA model. But, the characteristic of low AM-PM distortion PA is from our mmWave products, so we need to more margin for the MPR valu with low AM-PM distortion. So, we propose the MPR value as the average of the MPR results using the general PA and the MPR results using the low AM-PM distortion PA.

5.2.4.3   Simulation results from Nokia

R4-2311665 simulation results for PC1/PC2/PC5

To simulate the MPR we used the maximum allocation placed at the center of the channel. The phase noise is expected to increase with increasing size of the allocation, and the effect of the IQ imbalance is maximized at the center of the channel.

In the simulations we use the multi-zero/pole model with Qualcomm’s parameters.

In all cases in the tables below we see the transmit power is limited by the EVM. This explains why the MPR is very similar for all the simulated power classes.

The MPR simulation results with different PCs are provided in Tables 1-3, and the EVM budget is provided in Table 4.

Table 5.2.4.3-1. MPR simulation results for 256QAM with PC1.

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| DFT-S-OFDM |  | 60 | 7.1 | 7.1 | 7.2 |  | 7.2 |
|  |  | 120 | 7.1 | 7.0 | 7.1 | 7.2 |  |

Table 5.2.4.3-2. MPR simulation results for 256QAM with PC2.

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| DFT-S-OFDM |  | 60 | 7.0 | 7.0 | 7.1 |  | 7.1 |
|  |  | 120 | 7.0 | 6.9 | 7.0 | 7.0 |  |

Table 5.2.4.3-3. MPR simulation results for 256QAM with PC5.

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| DFT-S-OFDM |  | 60 | 7.1 | 7.3 | 7.3 |  | 7.3 |
|  |  | 120 | 7.0 | 7.0 | 7.1 | 7.1 |  |

Table 5.2.4.3-4. EVM budget for DFT-s-OFDM at 29 GHz. RB start position 0, number of RBs 64, SCS 120 kHz.

| Tx EVM contributor | EVM (%) |
| --- | --- |
| Phase Noise+IQ Imbalance | 2.79 |
| PA Non-linearity & Transmitter | 2.11 |
| Total | 3.50 |

5.2.4.4   Simulation results from ZTE

R4-2314619 simulation results for PC1

Some initial MPR simulation results for 29GHz PC1 100MHz 256QAM DFT-s-OFDM and CP-OFDM are shown in Fig 5.2.4.4-1.

Fig  5.2.4.4-1. 29GHz PC1 MPR for 256QAM 100MHz, DFT-s-OFDM(Left) and CP-OFDM (Right)

The maximum MPR@outer RB is 8.375dB and the maximum MPR@inner RB is 8.875dB for DFT-s-OFDM and 12.125dB for CP-OFDM.

The EVM budget for the 64RB0 is provided in Table  5.2.4.4-2.

Table  5.2.4.4-2 EVM budget for PC1 256QAM 100MHz, DFT-s-OFDM, 64RB0

| Tx EVM contributor | EVM (%) |
| --- | --- |
| Phase Noise+IQ Imbalance | 2.93 |
| PA Non-linearity & Transmitter | 1.90 |
| Total | 3.50 |

Therefore, the MPR requirements for 29GHz PC1 256QAM DFT-s-OFDM are proposed:

Table  5.2.4.4-3 MPR requirements for 29GHz PC1 256QAM DFT-s-OFDM (BWchannel ≤ 200 MHz)

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | # QPSK | # ≤ 6.5 | # 0.0 | # ≤ 3.0 |
|  | # 16 QAM | # ≤ 6.5 | # ≤ 4.0 | # ≤ 4.0 |
|  | # 64 QAM | # ≤ 6.5 | # ≤ 5.0 | # ≤ 5.0 |
|  | # 256QAM | # ≤8.5 | # ≤ 9.0 | # ≤ 9.0 |
| # CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
|  | # 256QAM | # ≤ 12.5 | # ≤ 12.5 | # ≤ 12.5 |

5.2.4.5   Simulation results from vivo

R4-2312574 simulation results for PC1

The EVM budget for this simulation is shown in Table 5.2.4.5-1:

Table 5.2.4.5-1 EVM budget for 256QAM MPR evaluation

| Tx EVM budget | EVM (%) |
| --- | --- |
| Phase Noise & IQ Imbalance | 2.99 |
| PA Non-linearity &Transmitter | 1.82 |
| Total | 3.50 |

In our simulation, the PTRS is only configured for CP-OFDM with L =1, K = 2, and the 29GHz phase noise profile from MTK in [1] is used for all waveforms.

- BW = 200 MHz SCS = 120kHz

The simulation results are shown in Figure 5.2.4.5-1

Figure 5.2.4.5-1 MPR simulation results for 256QAM when BW = 200MHz

- BW = 400 MHz SCS = 120kHz

Due to time constraints, we don’t finish all cases and as an alternative, we only give simulation results for cases that need to be verified in the test, which are specified in TS38.521-2 Table 6.2.2.4.1-3.

| MPR (dB) | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| Outer_Full (DFT-s-OFDM: 256@0 CP-OFDM: 264@0) | 7.94 | 10.12 |
| Inner_Full_Region2(DFT-s-OFDM: 128@66 CP-OFDM: 132@66) | 8.51 | 10.64 |
| 8@0 | 7.76 | 9.18 |
| 8@NRB-8 | 7.74 | 9.64 |

In our simulation, we find that the sole gating factor for 256QAM is EVM, and it is hard to further differentiate regions for MPR value since the inner region may require higher MPR due to the impact of the phase noise, carrier leakage, etc., so in our view, same MPR value can be applied to all regions which is similar to 64QAM with CP-OFDM.

Observation 2: The gating factor for 256 QAM is EVM only.

Another thing we noticed in the simulation results is that the MPR results are insensitive to the channel bandwidth. However, considering the challenge for real wideband device design, e.g., the droop effect at the bandwidth edge, the EVM of 400MHz may be further degraded which leads to higher MPR than previous simulation results, so it is proposed to reserve 1.5 dB margin for 400MHz case compared to 200MHz.

Observation 3: The 256QAM MPR simulation results are insensitive to the channel bandwidth but the challenge of wideband device design should be considered in MPR requirement.

5.2.4.6   Simulation results from Xiaomi

R4-2312686 simulation results for PC1

And the EVM budget is shown in Table 5.2.4.6-1:

Table 5.2.4.6-1 EVM budget in MPR simulation for 256QAM

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise | 1.41 | 37 |
| IQ Imbalance(w/ compensation) | 1.70 | 35.4 |
| PA Non-linearity & Transmitter | 2.70 | 31.4 |
| Total | 3.49 | 29.1 |

Based on above simulation parameters, we simulated different configurations:

- PC1 100MHz without PN and PTRS.

- PC1 100MHz with PN and PTRS.

- PC1 100MHz with PN and without PTRS.

The simulation results for DFT-s-OFDM are shown in figure 5.2.4.6-1 to figure 5.2.4.6-3:

Figure 5.2.4.6-1 PC1 100MHz MPR for 256QAM without PN and PTRS for DFT-s-OFDM

Figure 5.2.4.6-2 PC1 100MHz MPR for 256QAM with PN and PTRS for DFT-s-OFDM

Figure 5.2.4.6-3 PC1 100MHz MPR for 256QAM with PN without PTRS for DFT-s-OFDM

The simulation results for CP-OFDM are shown in figure 5.2.4.6-4 to figure 5.2.4.6-6:

Figure 5.2.4.6-4 PC1 100MHz MPR for 256QAM without PN and PTRS for CP-OFDM

Figure 5.2.4.6-5 PC1 100MHz MPR for 256QAM with PN and PTRS for CP-OFDM

Figure 5.2.4.6-6 PC1 100MHz MPR for 256QAM with PN without PTRS for CP-OFDM

Table 5.2.4.6-2 shows the largst MPR values in each regions based on above figure 5.2.4.6-1 to figure 5.2.4.6-6.

Table 5.2.4.6-2 MPRWT for power class 1, BWchannel =100 MHz in FR2-1

| Modulation |  | MPRWT (dB), BWchannel =100 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | w/o PN | 11.7 | 11.7 | 11.4 |
|  | w/ PN w/ PTRS | 12.8 | 12.3 | 11.4 |
|  | w/ PN w/o PTRS | 13 | 12.8 | 12.1 |
| CP-OFDM | w/o PN | 12.4 | 13.2 | 12.5 |
|  | w/ PN w/ PTRS | 14.9 | 14.5 | 14.1 |
|  | w/ PN w/o PTRS | 14.5 | 13.9 | 13.9 |

From Table 5.2.4.6-2, we can see the impact of phase noise to MPR value is about 1dB. And for DFT-s-OFDM, we can see about 0.5dB benefit when introducing the PTRS correction. But for CP-OFDM, there is no the benefit when introducing PTRS correction.

5.2.4.7   Simulation results from Sony

R4-2313190 simulation results for PC1

To investigate the feasibility of confining the MPR for UL 256QAM, the MPR for CP-OFDM for 100 MHz and 400 MHz BW are simulated for PC1 with 120 kHz SCS. The power amplifier is modeled based on a couple different III-V semiconductor PA models and the RB allocation is referred from contributions in last RAN4 meeting [2]. Other simulation assumptions are listed below and the simulated MPR is shown in Table 5.2.4.7-1.

- The phase noise model proposed by QC has been adopted in the simulation [1], where the EVM budget for Phase noise + IQ imbalance is -30.8 dB.

- L-PTRS = 1 K-PTRS =2 are adopted as PTRS configuration for CP-OFDM.

Table 5.2.4.7-1. The MPR simulation results for CP-OFDM with 120 kHz SCS for PC1

| Waveform | BWchanne | Outer RB allocations | Inner RB allocations |  |
| --- | --- | --- | --- | --- |
|  |  |  | Region 1 | Region 2 |
| CP-OFDM | 100 MHz | 10.4 | 11.4 | 11.2 |
|  | 400 MHz | 11 | 10.2 | 10.8 |

Based on the simulated results, it can be observed that for 400 MHz, the MPR values are well below the “64QAM MPR + 3dB” range. Though not all cases (inner region of 100 MHz) can be well-fitted within the proposed range, the values are not far from the proposed limits for narrower BW. In addition, it is worth mentioning that no advanced linearity technologies, e.g., DPD or APD, have been included in such a simulation. Therefore, it can be expected that the MPR confinement, as mentioned above, is feasible to be used to define the MPR values for 256 QAM.

5.2.4.8   Simulation results from MediaTek

R4-2313417 simulation results for PC2

In the Tx, the EVM performance is determined by many factors including baseband clipping and quantization, transmitter non-linearity, IQ imbalance, phase noise, PA non-linearity, etc. In RAN4 #106, the approved agreement for EVM budget is listed below:

EVM budget for MPR evaluation:

      - Only consider the total value of 3.5% for Tx EVM

  - Companies need to clarify the components of Tx EVM in their simulation results, including

    - Phase noise

    - Value for IQ imbalance

    - PA and transmitter non-linearity

Table 5.2.4.8-1 EVM budget for FR2-1 UL 256QAM MPR at 29GHz and 39GHz

|  | 29GHz |  | 39GHz |  |
| --- | --- | --- | --- | --- |
| EVM Contributor | EVM(%) | SNR(dB) | EVM(%) | SNR(dB) |
| Transmitter +IQ Imbalance | 1.7 | 35.3 | 1.7 | 35.3 |
| Phase Noise | 2.2 | 33.15 | 2.95 | 30.6 |
| PA Non-linearity | 2.1 | 33.56 | 0.8 | 41.93 |
| Total | 3.5 | 29.1 | 3.5 | 29.1 |

In the FR2-1 UL 256QAM MPR simulation, we propose the EVM budget with approved phase noise profiles in the last meeting summarized in Table 1. With current 39GHz phase noise profile, it should be noted that the phase noise performance of 39GHz dominates the EVM budget for 3.5% EVM. PA requires more back-off power to compensate the quality of the transmitting signal. In our initial simulations, MPR for 29GHz UL 256QAM could achieve reasonable MPR value. However, the MPR value for 39GHz UL 256QAM could be too large and this will result in insufficient dynamic range.

Observation 1: With current 39GHz phase noise profile, it should be noted that the phase noise performance of 39GHz dominates the EVM budget for 3.5% EVM. PA requires more back-off power to compensate the quality of the transmitting signal. In our initial simulations, MPR for 29GHz UL 256QAM could achieve reasonable MPR value. However, the MPR value for 39GHz UL 256QAM could be too large and this will result in insufficient dynamic range.

In FR1, the MPR of 256QAM is entirely determined by the EVM. This could be also expected for FR2-1. Therefore, in this contribution, we provide our initial MPR simulation for the EVM by using our phase noise model and Qualcomm’s phase noise model with full RB allocation in 100 MHz BW with 120 kHz SCS.

Table 5.2.4.8-2. PC2 MPR simulated results

| Modulation |  | PTRS CPE compensation | MPR (dB) |  |
| --- | --- | --- | --- | --- |
| 100MHz (full RB) |  |  | MTK’s model @29GHz | QC’s model @29GHz |
| DFT-s-OFDM | 256 QAM | Off | 7.2 | 7.4 |
| DFT-s-OFDM | 256 QAM | On | 8 | 7.8 |
| CP-OFDM | 256 QAM | Off | 9.9 | 10.1 |
| CP-OFDM | 256 QAM | On | 9.7 | 9.7 |

5.2.5	Simulation for PTRS configurations

5.2.5.1   Simulation results from Qualcomm

R4-2304601

The desired waveform was 256QAM of various allocation sizes and 120 kHz SCS. To isolate the EVM impact of phase noise, no AWGN, image, LO or non-linear distortion products accompanied the signal. The phase noise profile is chosen as the hybrid 30G profile (Proposal 1: Adopting min(example1, example2) as the phase noise profile for UL256QAM, where ‘example’ refers to the example phase noise profiles in TR38.803 for n257, n258 and n261.)

CP-OFDM

For CP-OFDM, PTRS density ranged from no correction to frequency density k=2 and k = 4. Time density was fixed at L = 1. Table 5.2.5.1-1 summarizes the EVM

| ‘hybrid 30’ profile |  | EVM from floor (dBc) |  |  |
| --- | --- | --- | --- | --- |
|  |  | No PTRS | K=4 | K=2 |
| LCRB | 4 | -33.7 | -34.0 | -34.4 |
|  | 16 | -33.0 | -33.6 | -33.9 |
|  | 64 | -32.7 | -33.5 | -33.5 |
|  | 256 | -32.5 | -33.3 | -33.4 |

Table 5.2.5.1-1: EVM penalty for CP-OFDM with PTRS correction

Results show that PTRS corrections seem to help the EVM floor in proportion to the density of PTRS. The effectiveness tapers off for wider allocations, so there is room for optimization in the field.

Observation 2: A fixed PTRS configuration of K=2, L=1 benefits EVM for CP-OFDM waveforms with the phase noise profile of proposal 1.

DFT-s-OFDM

For DFT-s-OFDM, the configuration ([Nsampgroup, NPT-RSgroup]) are chosen amongst this set: {[4 2], [4 4], [4 8]}. The # of groups was maximized while ensuring there were fewer PTRS data symbols than PDSCH symbols. For all cases, number of PTRS samples per group was limited to 4.The group count is recorded in that tables below .

| ‘hybrid 30’ profile |  | # PTRS groups | EVM from floor (dBc) |  |
| --- | --- | --- | --- | --- |
|  |  |  | No PTRS | PTRS enabled |
| LCRB | 4 | 4 | -33.8 | -35.3 |
|  | 16 | 8 | -32.9 | -33.6 |
|  | 64 | 8 | -32.7 | -32.7 |
|  | 256 | 8 | -32.5 | -32.3 |

Table 5.2.5.1-2: EVM penalty for DFT-s-OFDM with PTRS correction

Results show that PTRS corrections in the configuration above only help narrow allocations (~ 20 or less) of DFT-s-OFDM. PTRS seems like wasted overhead for medium sized allocations (20-64) and can be detrimental for very wide allocations. Here too there is room for optimization in the field.

Observation 3: Assuming the phase noise profile of proposal 1, only narrow allocations of DFT-s-OFDM waveforms (~20RB or narrower) benefit from PTRS.

Observation 4: Assuming the phase noise profile of proposal 1, the network is better off not configuring PTRS for allocations wider than 20 RBs.

The analysis assumed a certain strategy of how the number of PTRS groups must be reduced in keeping with narrower allocations. Specifically, the total number of symbols available per OFDM symbol is 12*LCRB. Of these, Nsampgroup, * NPT-RSgroup symbols are occupied by PTRS symbols. When the latter product gets comparable to the total number of symbols available per OFDM symbol, PTRS starts to represent a significant overhead. It is also self-evident that 4 symbols per group and 8 groups per OFDM symbol is physically impossible to fit into a 2RB allocation. RAN4 must therefore determine this strategy.

Observation 5: Unlike CP-OFDM, it is not clear how to adjust PTRS parameters when the number of symbols in each OFDM symbol (12*LCRB) starts to become comparable to the product Nsampgroup, * NPT-RSgroup.

5.2.5.2   Simulation results from vivo

R4-2308227

To further investigate PTRS configuration under different RB allocations, the simulation is performed and the results are shown in Table 5.2.5.2-1. Considering that Option 1 and Option 2 are not feasible, only Option 3 and Option 4 are evaluated.

Table 5.2.5.2-1 PTRS correction benefit evaluation for different RB allocation

|  |  |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- | --- |
| 32RB | CP-OFDM | Option 3 | -32.5213 | -34.0762 | 1.5549 |
|  |  | Option 4 | -33.2888 | -34.1761 | 0.8873 |
|  | DFT-s-OFDM | Option 3 | -32.6484 | -30.7554 | -1.893 |
|  |  | Option 4 | -33.4813 | -31.4505 | -2.0308 |
| 16RB | CP-OFDM | Option 3 | -33.4905 | -33.9333 | 0.4428 |
|  |  | Option 4 | -33.9908 | -34.2379 | 0.2471 |
|  | DFT-s-OFDM | Option 3 | -35.5055 | -33.3672 | -2.1383 |
|  |  | Option 4 | -34.1009 | -32.5789 | -1.522 |
| 8RB | CP-OFDM | Option 3 | -33.3388 | -33.9874 | 0.6486 |
|  |  | Option 4 | -34.4442 | -34.8899 | 0.4457 |
|  | DFT-s-OFDM | Option 3 | -33.3038 | -32.9937 | -0.3101 |
|  |  | Option 4 | -34.5513 | -34.0163 | -0.535 |
| 4RB | CP-OFDM | Option 3 | -33.9173 | -35.0024 | 1.0851 |
|  |  | Option 4 | -35.6699 | -36.7056 | 1.0357 |
|  | DFT-s-OFDM | Option 3 | -34.7217 | -34.3119 | -0.4098 |
|  |  | Option 4 | -36.4809 | -35.7798 | -0.7011 |

Observation 3: Even under narrow RB allocation, the DFT-s-OFDM still hard to benefit from PTRS correction.

Take the 8RB case as an example, we further evaluated DFT-s-OFDM with different PTRS configurations, and the simulation results are shown in Table 5.2.5.2-2:

Table 5.2.5.2-2 PTRS correction benefit evaluation for DFT-s-OFDM with narrow RB allocation under different PTRS configurations.

| PTRS configuration |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- |
| L-PTRS = 1 N_group = 4, N_samp = 4 | Option 3 | -33.3038 | -33.946 | 0.6422 |
|  | Option 4 | -34.5513 | -34.7963 | 0.245 |
| L-PTRS = 1 N_group = 2, N_samp = 4 | Option 3 | -33.3038 | -32.8345 | -0.4693 |
|  | Option 4 | -34.5513 | -34.022 | -0.5293 |
| L-PTRS = 1 N_group = 2, N_samp = 2 | Option 3 | -33.3038 | -32.9937 | -0.3101 |
|  | Option 4 | -34.5513 | -34.0163 | -0.535 |

Observation 4: Only configuration L-PTRS = 1, (N_group = 4, N_samp = 4) can make narrow RB allocation benefit from PTRS correction.

5.2.5.3   Simulation results from Xiaomi

R4-2308805

To further evaluate the effects of different PTRS configurations for CP- OFDM and DFT-s-OFDM, the simulations are done for different RBs allocation (4RBs, 16RBs, 64RBs) separately.

Table 5.2.5.3-1 and table 5.2.5.3-2 show the simulation results for CP- OFDM at 29GHz and 39GHz. Table 5.2.5.3-3 and table 5.2.5.3-4 show the simulation results for DFT-s- OFDM at 29GHz and 39GHz.

For CP- OFDM:

Table 5.2.5.3-1 EVM floor and benefits with different PTRS configurations for CP-OFDM at 29GHz

| CP-OFDM with phase noise， 29GHz, 120kHz, 4RB， |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1 from TR38.803 | -34.0 | -34.2 | -33.6 | 0.2 | -0.6 |
| Example 2 from TR38.803 | -26.9 | -28.1 | -27.6 | 1.2 | -0.5 |
| Phase noise Profile from Qualcomm | -36.0 | -36.9 | -36.4 | 0.9 | -0.5 |
| Phase noise Profile from MTK | -37.9 | -38.4 | -37.8 | 0.5 | -0.6 |
| CP-OFDM with phase noise， 29GHz, 120kHz, 16RB， |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1 from TR38.803 | -31.9 | -32.3 | -32.1 | 0.4 | -0.2 |
| Example 2 from TR38.803 | -26.7 | -28.3 | -28.0 | 1.6 | -0.3 |
| Phase noise Profile from Qualcomm | -35.2 | -36.3 | -36.0 | 1.1 | -0.2 |
| Phase noise Profile from MTK | -36.1 | -36.7 | -36.5 | 0.6 | -0.2 |
| CP-OFDM with phase noise， 29GHz, 120kHz, 64RB， |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1 from TR38.803 | -31.1 | -31.5 | -31.5 | 0.4 | -0.1 |
| Example 2 from TR38.803 | -26.6 | -28.4 | -28.3 | 1.8 | -0.1 |
| Phase noise Profile from Qualcomm | -34.8 | -36.0 | -36.0 | 1.2 | -0.1 |
| Phase noise Profile from MTK | -35.4 | -36.0 | -35.9 | 0.7 | -0.1 |

Table  5.2.5.3-2 EVM floor and benefits with different PTRS configurations for CP-OFDM at 39GHz

| CP-OFDM with phase noise， 39GHz, 120kHz, 4RB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1: VIVO parameter | -30.9 | -30.9 | -30.2 | 0.0 | -0.6 |
| Example 1:Anritsu parameter | -31.1 | -31.1 | -30.5 | 0.0 | -0.6 |
| Example 2 from TR38.803 | -24.4 | -25.5 | -25.1 | 1.2 | -0.5 |
| Phase noise Profile from Qualcomm; Min(Example1-VIVO, Example2) | -33.3 | -34.1 | -33.5 | 0.8 | -0.5 |
| Phase noise Profile from Qualcomm; Min(Example1- Anritsu, Example2) | -33.3 | -34.1 | -33.6 | 0.8 | -0.5 |
| Phase noise Profile from MTK | -35.1 | -35.5 | -34.9 | 0.4 | -0.6 |
| CP-OFDM with phase noise， 39GHz, 120kHz, 16RB |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1:VIVO parameter | -28.6 | -28.9 | -28.7 | 0.2 | -0.2 |
| Example 1:Anritsu parameter | -29.0 | -29.2 | -29.0 | 0.3 | -0.2 |
| Example 2 from TR38.803 | -24.2 | -25.7 | -25.5 | 1.5 | -0.3 |
| Phase noise Profile from Qualcomm; Min(Example1-VIVO, Example2) | -32.5 | -33.5 | -33.3 | 1.0 | -0.2 |
| Phase noise Profile from Qualcomm; Min(Example1- Anritsu, Example2) | -32.5 | -33.6 | -33.3 | 1.0 | -0.2 |
| Phase noise Profile from MTK | -33.2 | -33.7 | -33.5 | 0.5 | -0.2 |
| CP-OFDM with phase noise， 39GHz, 120kHz, 64RB |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1:VIVO parameter | -27.8 | -28.1 | -28.1 | 0.3 | -0.1 |
| Example 1:Anritsu parameter | -28.1 | -28.5 | -28.4 | 0.4 | -0.1 |
| Example 2 from TR38.803 | -24.1 | -25.8 | -25.7 | 1.7 | -0.1 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -32.2 | -33.3 | -33.2 | 1.1 | -0.1 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -32.2 | -33.3 | -33.2 | 1.1 | -0.1 |
| Phase noise Profile from MTK | -32.4 | -33.0 | -32.9 | 0.6 | -0.1 |

From Table 5.2.5.3-1 and table  5.2.5.3-2 for CP-OFDM, we can see although the benefits are different for different phase noise profile:

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously increased with the increase of the density of PTRS configurations.

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously increased with the number of RBs allocation. For the narrow RBs allocation, the benefits with PTRS correction is comparable with 64RBs allocation for 120kHz SCS.

For DFT-S-OFDM:

Table  5.2.5.3-3 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 29GHz

| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 4RB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -34.1 | -33.6 | -34.4 | -34.3 | -0.4 | 0.3 | 0.2 |
| Example 2 from TR38.803 | -26.8 | -30.9 | -29.0 | -26.8 | 4.1 | 2.1 | 0.0 |
| Phase noise Profile from Qualcomm | -36.0 | -36.6 | -37.0 | -36.6 | 0.6 | 1.0 | 0.6 |
| Phase noise Profile from MTK | -37.9 | -38.1 | -38.6 | -38.4 | 0.2 | 0.7 | 0.5 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 10RB |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -32.5 | -32.1 | -32.2 | -32.1 | -0.4 | -0.3 | -0.4 |
| Example 2 from TR38.803 | -26.2 | -29.8 | -27.9 | -26.1 | 3.6 | 1.7 | -0.1 |
| Phase noise Profile from Qualcomm | -35.3 | -35.7 | -35.6 | -35.2 | 0.4 | 0.3 | -0.1 |
| Phase noise Profile from MTK | -36.5 | -36.6 | -36.6 | -36.4 | 0.1 | 0.1 | -0.2 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 16RB |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -32.0 | -31.4 | -31.4 | -31.3 | -0.6 | -0.6 | -0.7 |
| Example 2 from TR38.803 | -26.2 | -29.9 | -27.9 | -26.0 | 3.7 | 1.7 | -0.2 |
| Phase noise Profile from Qualcomm | -35.1 | -35.4 | -35.2 | -34.8 | 0.3 | 0.1 | -0.3 |
| Phase noise Profile from MTK | -36.1 | -35.9 | -35.9 | -35.7 | -0.2 | -0.2 | -0.4 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 64RB |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -31.2 | -30.1 | -30.0 | -29.9 | -1.1 | -1.2 | -1.3 |
| Example 2 from TR38.803 | -26.0 | -30.1 | -27.8 | -26.0 | 4.1 | 1.8 | 0.0 |
| Phase noise Profile from Qualcomm | -34.8 | -34.9 | -34.6 | -34.1 | 0.1 | -0.2 | -0.7 |
| Phase noise Profile from MTK | -35.4 | -34.7 | -34.5 | -34.3 | -0.7 | -0.9 | -1.1 |

Table  5.2.5.3-4 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 39GHz

| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 4RB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections |  | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1:VIVO parameter | -30.9 |  | -30.2 | -31.0 | -31.0 | -0.7 | 0.1 | 0.1 |
| Example 1: Anritsu parameter | -31.1 |  | -30.5 | -31.3 | -31.3 | -0.6 | 0.2 | 0.2 |
| Example 2 from TR38.803 | -24.2 |  | -28.4 | -26.4 | -24.3 | 4.2 | 2.2 | 0.1 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -33.3 |  | -33.6 | -34.0 | -33.8 | 0.3 | 0.7 | 0.5 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -33.3 |  | -33.6 | -34.1 | -33.8 | 0.3 | 0.8 | 0.5 |
| Phase noise Profile from MTK | -35.1 |  | -35.1 | -35.7 | -35.5 | 0.0 | 0.6 | 0.4 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 10RB |  |  |  |  |  |  |  |  |
| Phase noise model |  | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1:VIVO parameter | -29.3 |  | -28.7 | -28.8 | -28.8 | -0.6 | -0.4 | -0.5 |
| Example 1: Anritsu parameter | -29.5 |  | -29.0 | -29.2 | -29.1 | -0.5 | -0.4 | -0.5 |
| Example 2 from TR38.803 | -23.6 |  | -27.2 | -25.3 | -23.5 | 3.6 | 1.7 | -0.1 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -32.6 |  | -32.8 | -32.7 | -32.4 | 0.2 | 0.1 | -0.2 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -32.6 |  | -32.8 | -32.7 | -32.4 | 0.2 | 0.1 | -0.2 |
| Phase noise Profile from MTK | -33.7 |  | -33.6 | -33.6 | -33.4 | -0.1 | -0.1 | -0.3 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 16RB |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections |  | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1: VIVO parameter | -28.7 |  | -27.9 | -28.0 | -28.0 | -0.8 | -0.7 | -0.7 |
| Example 1: Anritsu parameter | -29.0 |  | -28.3 | -28.4 | -28.3 | -0.7 | -0.6 | -0.7 |
| Example 2 from TR38.803 | -23.7 |  | -27.5 | -25.3 | -23.5 | 3.8 | 1.6 | -0.2 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -32.4 |  | -32.5 | -32.3 | -32.0 | 0.1 | -0.1 | -0.4 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -32.5 |  | -32.6 | -32.4 | -32.0 | 0.1 | -0.1 | -0.5 |
| Phase noise Profile from MTK | -33.2 |  | -32.9 | -32.9 | -32.7 | -0.3 | -0.3 | -0.5 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 64RB |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections |  | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1: VIVO parameter | -27.9 |  | -26.7 | -26.6 | -26.5 | -1.2 | -1.3 | -1.4 |
| Example 1: Anritsu parameter | -28.2 |  | -27.0 | -26.9 | -26.8 | -1.2 | -1.2 | -1.4 |
| Example 2 from TR38.803 | -23.6 |  | -27.5 | -25.3 | -23.3 | 4.0 | 1.7 | -0.2 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -32.1 |  | -32.1 | -31.7 | -31.3 | 0.0 | -0.3 | -0.7 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -32.1 |  | -32.2 | -31.8 | -31.4 | 0.1 | -0.3 | -0.7 |
| Phase noise Profile from MTK | -32.4 |  | -31.6 | -31.5 | -31.3 | -0.8 | -0.9 | -1.1 |

From Table 5.2.5.3-3 and table 5.2.5.3-4 for DFT-s-OFDM at 29GHzand 39GHz, we can see although the benefits are different for different phase noise profile:

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously increased with the increase of the density of PTRS configurations.

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously reduced with the number of RBs allocation.

- There is obvious benefit with PTRS correction only for example 2 new phase noise profiles.

- There is no obvious benefits with PTRS correction for the RBs allocation larger than 10RBs for new phase noise profiles from Qualcomm and MTK, even some PTRS configurations have the penalty for the new phase noise profiles.

- There is obvious benefits with PTRS correction for the RBs allocation less than 10RBs for new phase noise profiles from Qualcomm and MTK, especially for (N_group = 4, N_samp = 4) PTRS configuration, i.e., 1dB and 0.7dB benefits for 29GHz for 4RBs allocation for 120kHz SCS, 0.8dB and 0.6dB for 39GHz for 4RBs allocation for 120kHz SCS.

As we know, introducing PTRS will increase the data code rate, especially for narrow RB allocation. In order to check the impact of PTRS, we further compare the BLER performance between PTRS off and PTRS on with different RB allocation for DFT-S-OFDM waveform. The simulation assumption is same with the agreed for evaluation of phase noise profiles in lasting meeting and QC phase noise profile at 29GHz is applied. The background noise is set to -35dB. As we can see from above simulation results, the BLER increase for both 4RB and 64RB allocation when PTRS is configured shown as Table 5.2.5.3-5. For 4RB allocation, although there is EVM benefit as previous analyzation, the BLER increase due to high PTRS overhead. For 64RB, the BLER increase due to the joint effect of EVM penalty and overhead of PTRS. For 10RB, maybe there is a trade-off between PTRS overhead and EVM benefit, the BLER is same for PTRS on/off. In short, except for the case that example 2 phase noise profile is applied for MPR simulation, it may not be recommended to configure PTRS.

Table  5.2.5.3-5 the BLER for different RBs allocation with/without PTRS corrections

| BLER | 4 RBs | 10 RBs | 64 RBs |
| --- | --- | --- | --- |
| Without PTRS corrections | 0.5 | 0.5 | 0.6680 |
| With PTRS correction | 0.75 | 0.5 | 0.7075 |

R4-2312686

For CP-OFDM, based on the submitted simulation results in last meeting [R4-2308805], we can see the benefits with PTRS correction are monotonously increased with the number of RBs allocation. If adopting the fixed PTRS configuration of L-PTRS = 1 K-PTRS =2, it is good for full RBs allocation in EVM test, but the overhead for partial RBs allocation need consider.

Table 5.2.5.3-6: PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

For DFT-s-OFDM, based PTRS group pattern in Table 5.2.5.3-6, we introduced more simulation cases and the simulation results for some PTRS configurations under different RB allocations are shown in Table 5.2.5.3-7 and Table 5.2.5.3-8.

Table 5.2.5.3-7 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 29GHz

| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 4RB |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -36.0 | -36.2 | -36.6 | -36.6 | -37.0 | -36.6 | 0.2 | 0.6 | 0.6 | 1 | 0.6 |
| Phase noise Profile from MTK | -37.9 | -38.0 | -38.4 | -38.3 | -38.6 | -38.1 | 0.1 | 0.5 | 0.4 | 0.7 | 0.2 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 8RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -35.5 | -35.1 | -35.5 | -35.5 | -35.9 | -35.9 | -0.4 | 0.0 | 0.0 | 0.4 | 0.4 |
| Phase noise Profile from MTK | -36.9 | -36.3 | -36.8 | -36.5 | -37.1 | -37.0 | -0.6 | 0.0 | -0.4 | 0.2 | 0.1 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 10RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -35.3 | -34.9 | -35.2 | -35.2 | -35.6 | -35.7 | -0.4 | -0.1 | -0.1 | 0.3 | 0.4 |
| Phase noise Profile from MTK | -36.5 | -35.9 | -36.4 | -36.1 | -36.6 | -36.6 | -0.6 | -0.1 | -0.4 | 0.1 | 0.1 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 16RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -35.1 | -34.4 | -34.8 | -35.0 | -35.2 | -35.4 | -0.7 | -0.3 | -0.1 | 0.1 | 0.3 |
| Phase noise Profile from MTK | -36.1 | -35.1 | -35.7 | -35.4 | -35.9 | -35.9 | -1.0 | -0.4 | -0.7 | -0.2 | -0.2 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 20RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -35.1 | -34.4 | -34.7 | -34.8 | -35.0 | -35.3 | -0.7 | -0.4 | -0.3 | -0.1 | 0.2 |
| Phase noise Profile from MTK | -36.0 | -34.8 | -35.4 | -35.1 | -35.6 | -35.6 | -1.1 | -0.6 | -0.8 | -0.4 | -0.3 |

Table 5.2.5.3-8 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 39GHz

| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 4RB |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -35.1 | -35.1 | -35.5 | -35.3 | -35.7 | -35.1 | 0.0 | 0.4 | 0.2 | 0.6 | 0.0 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 8RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -34.0 | -33.3 | -33.9 | -33.5 | -34.1 | -34.0 | -0.7 | -0.1 | -0.5 | 0.1 | 0.0 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 10RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -33.7 | -32.9 | -33.4 | -33.1 | -33.6 | -33.6 | -0.8 | -0.3 | -0.6 | -0.1 | -0.1 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 16RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -33.2 | -32.1 | -32.7 | -32.4 | -32.9 | -32.9 | -1.1 | -0.5 | -0.8 | -0.4 | -0.3 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 20RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -33.1 | -31.8 | -32.4 | -32.1 | -32.5 | -32.6 | -1.2 | -0.7 | -1.0 | -0.5 | -0.5 |

From above tables for DFT-s-OFDM at 29GHzand 39GHz, we can see:

- The benefits with PTRS correction are monotonously increased with the increase of the density of PTRS configurations.

- The benefits with PTRS correction are monotonously reduced with the number of RBs allocation.

- There is no obvious benefits with PTRS correction for the RBs allocation larger than 10RBs, even some PTRS configurations have the penalty.

- There is obvious benefits with PTRS correction for the RBs allocation less than 10RBs, especially for (N_group = 4, N_samp = 4) PTRS configuration.

5.2.5.4   Simulation results from ZTE

R4-2314619

For DFT-s-OFDM waveform, the results for EVM comparison between different RB allocations are summarized in Table 5.2.5.4-1, in which the phase noise model for 29GHz (i.e. option 1.) agreed in the WF R4-2310260 is used.

Table 5.2.5.4-1 EVM comparison between different configurations for DFT-s-OFDM waveform

| DFT-s-OFDM w/ phase noise, 256QAM, 120k SCS, w/ CPE vs w/o CPE @29GHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scheduled BW (RB) | PTRS config [PTRS/group,PTRS group] | EVM(dB) with CPE | EVM (dB) without CPE | Net benefit of CPE |
| 8 | [2, 2] | -31.4 | -31.9 | -0.5 |
| 20 | [4, 4] | -31.5 | -31.6 | -0.1 |
| 20 | [2, 4] | -30.8 | -30.7 | 0.1 |
| 66 | [4, 8] | -30.9 | -30.8 | 0.1 |

From table 5.2.5.4-1, it can be seen that the net benefit of CPE is only 0.1dB for full RB allocation, however, for partial RB allocation, even negative net benefits is found. It would be foreseen that this tendency can be applied to all the start allocation for the partial RB. Therefore, we think PTRS for CPE will not provide EVM benefit for all RB allocation cases for DFT-s-OFDM.

Observation 1: Introducing PTRS for CPE for DFT-s-OFDM may not improve EVM or the improvement is very small.

With regard to the PTRS configuration for EVM test for CP-OFDM, we can see that the difference between the two options is whether to configure PTRS for EVM test based on the UEs declaration or to configure the same PTRS without considering UE declaration. Currently, how UEs declare whether they need PTRS or not is FFS. In our contribution[R4-2304689], we compared net benefit of PTRS for CPE for different UE phase noise model alternatives for CP-OFDM, we copy the results in table 5.2.5.4-2 for convenience.

Table 5.2.5.4-2. EVM comparison between different alternatives

| CP-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal@45GHz |  |  |  |
| --- | --- | --- | --- |
| Phase noise | EVM(dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE alt 1 | -24.7 | -24.5 | 0.2 |
| UE alt 2 | -21.5 | -20.4 | 1.1 |
| UE alt 3 | -30.4 | -28 | 2.4 |
| UE alt 4 | -27.3 | -27.1 | 0.2 |

The four phase noise options in the above table are:

Alt 1. example 1 defined in 38803

Alt 2. example 2 defined in 38803

Alt 3. min(example 1, example 2)

Alt 4. example 1-based

In table 5.2.5.4-2, when the low phase noise model (i.e. alt 3.) is adopted, the net benefit of PTRS is promising. But companies have different views of the PTRS configuration, e.g., some companies prefer to use PTRS for all device and others prefer to use PTRS if UE declare to use it. From the contributions [3][4]provided by companies we can see that PTRS based CPE can improve EVM at least for CP-OFDM waveform. If most companies believe that PTRS based CPE can improve EVM for CP-OFDM waveform, then we tend to use the same PTRS configuration for all devices.

Observation 2: Introducing PTRS for CPE can improve EVM for CP-OFDM waveform.

5.2.5.5   Summary the CP-OFDM simulation results for PTRS configuration

For CP-OFDM at 29GHz:

|  |  | Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm R4-2304601 | 120kHz, 4RBs | min(example1, example2) | -33.7 | -34.4 | -34.0 | 0.7 | 0.3 |
|  | 120kHz, 16RBs |  | -33.0 | -33.9 | -33.6 | 0.9 | 0.6 |
|  | 120kHz, 64RBs |  | -32.7 | -33.5 | -33.5 | 0.8 | 0.8 |
|  | 120kHz, 256RBs |  | -32.5 | -33.4 | -33.3 | 0.9 | 0.8 |
| Xiaomi R4-2308805 | 120kHz, 4RBs | UE example 1 | -34.0 | -34.2 | -33.6 | 0.2 | -0.6 |
|  |  | UE example 2 | -26.9 | -28.1 | -27.6 | 1.2 | -0.5 |
|  |  | From Qualcomm | -36.0 | -36.9 | -36.4 | 0.9 | -0.5 |
|  |  | From MTK | -37.9 | -38.4 | -37.8 | 0.5 | -0.6 |
|  | 120kHz, 16RBs | UE example 1 | -31.9 | -32.3 | -32.1 | 0.4 | -0.2 |
|  |  | UE example 2 | -26.7 | -28.3 | -28.0 | 1.6 | -0.3 |
|  |  | From Qualcomm | -35.2 | -36.3 | -36.0 | 1.1 | -0.2 |
|  |  | From MTK | -36.1 | -36.7 | -36.5 | 0.6 | -0.2 |
|  | 120kHz, 64RBs | UE example 1 | -31.1 | -31.5 | -31.5 | 0.4 | -0.1 |
|  |  | UE example 2 | -26.6 | -28.4 | -28.3 | 1.8 | -0.1 |
|  |  | From Qualcomm | -34.8 | -36.0 | -36.0 | 1.2 | -0.1 |
|  |  | From MTK | -35.4 | -36.0 | -35.9 | 0.7 | -0.1 |
| Vivo R4-2308227 | 120kHz, 4RBs | From Qualcomm | -33.9173 | -35.0024 |  | 1.0851 |  |
|  |  | From MTK | -35.6699 | -36.7056 |  | 1.0357 |  |
|  | 120kHz, 8RBs | From Qualcomm | -33.3388 | -33.9874 |  | 0.6486 |  |
|  |  | From MTK | -34.4442 | -34.8899 |  | 0.4457 |  |
|  | 120kHz, 16RBs | From Qualcomm | -33.4905 | -33.9333 |  | 0.4428 |  |
|  |  | From MTK | -33.9908 | -34.2379 |  | 0.2471 |  |
|  | 120kHz, 32RBs | From Qualcomm | -32.5213 | -34.0762 |  | 1.5549 |  |
|  |  | From MTK | -33.2888 | -34.1761 |  | 0.8873 |  |
|  | 120kHz, 64RBs | From Qualcomm | -31.7997 | -33.0798 |  | 1.2801 |  |
|  |  | From MTK | -33.0942 | -33.3608 |  | 0.2666 |  |

5.2.5.5   Summary the DFT-s-OFDM simulation results for PTRS configuration

For DFT-s-OFDM at 29GHz:

|  |  | Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 2 | Net benefit of PTRS N_group = 8, N_samp = 4 | Net benefit of PTRS N_group = 4, N_samp = 4 | Net benefit of PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 2, N_samp = 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm R4-2304601 | 120kHz, 4RBs | min(example1, example2) | -33.8 |  | -35.3 |  |  |  | 1.5 |  |  |
|  | 120kHz, 16RBs |  | -32.9 | -33.6 |  |  |  | 0.7 |  |  |  |
|  | 120kHz, 64RBs |  | -32.7 | -32.7 |  |  |  | 0 |  |  |  |
|  | 120kHz, 256RBs |  | -32.5 | -32.3 |  |  |  | -0.2 |  |  |  |
| Xiaomi R4-2308805 | 120kHz, 4RBs | UE example 1 | -34.1 | -33.6 | -34.4 | -34.3 |  | -0.4 | 0.3 | 0.2 |  |
|  |  | UE example 2 | -26.8 | -30.9 | -29.0 | -26.8 |  | 4.1 | 2.1 | 0.0 |  |
|  |  | From Qualcomm | -36.0 | -36.6 | -37.0 | -36.6 |  | 0.6 | 1.0 | 0.6 |  |
|  |  | From MTK | -37.9 | -38.1 | -38.6 | -38.4 |  | 0.2 | 0.7 | 0.5 |  |
|  | 120kHz, 10RBs | UE example 1 | -32.5 | -32.1 | -32.2 | -32.1 |  | -0.4 | -0.3 | -0.4 |  |
|  |  | UE example 2 | -26.2 | -29.8 | -27.9 | -26.1 |  | 3.6 | 1.7 | -0.1 |  |
|  |  | From Qualcomm | -35.3 | -35.7 | -35.6 | -35.2 |  | 0.4 | 0.3 | -0.1 |  |
|  |  | From MTK | -36.5 | -36.6 | -36.6 | -36.4 |  | 0.1 | 0.1 | -0.2 |  |
|  | 120kHz, 16RBs | UE example 1 | -32.0 | -31.4 | -31.4 | -31.3 |  | -0.6 | -0.6 | -0.7 |  |
|  |  | UE example 2 | -26.2 | -29.9 | -27.9 | -26.0 |  | 3.7 | 1.7 | -0.2 |  |
|  |  | From Qualcomm | -35.1 | -35.4 | -35.2 | -34.8 |  | 0.3 | 0.1 | -0.3 |  |
|  |  | From MTK | -36.1 | -35.9 | -35.9 | -35.7 |  | -0.2 | -0.2 | -0.4 |  |
|  | 120kHz, 64RBs | UE example 1 | -31.2 | -30.1 | -30.0 | -29.9 |  | -1.1 | -1.2 | -1.3 |  |
|  |  | UE example 2 | -26.0 | -30.1 | -27.8 | -26.0 |  | 4.1 | 1.8 | 0.0 |  |
|  |  | From Qualcomm | -34.8 | -34.9 | -34.6 | -34.1 |  | 0.1 | -0.2 | -0.7 |  |
|  |  | From MTK | -35.4 | -34.7 | -34.5 | -34.3 |  | -0.7 | -0.9 | -1.1 |  |
| Vivo R4-2308227 | 120kHz, 4RBs | From Qualcomm | -34.7217 | -34.3119 |  |  |  | -0.4098 |  |  |  |
|  |  | From MTK | -36.4809 | -35.7798 |  |  |  | -0.7011 |  |  |  |
|  | 120kHz, 8RBs | From Qualcomm | -33.3038 | -32.9937 | -33.946 | -32.8345 |  | -0.3101 | 0.6422 | -0.4693 |  |
|  |  | From MTK | -34.5513 | -34.0163 | -34.7963 | -34.022 |  | -0.535 | 0.245 | -0.5293 |  |
|  | 120kHz, 16RBs | From Qualcomm | -35.5055 | -33.3672 |  |  |  | -2.1383 |  |  |  |
|  |  | From MTK | -34.1009 | -32.5789 |  |  |  | -1.522 |  |  |  |
|  | 120kHz, 32RBs | From Qualcomm | -32.6484 | -30.7554 |  |  |  | -1.893 |  |  |  |
|  |  | From MTK | -33.4813 | -31.4505 |  |  |  | -2.0308 |  |  |  |
|  | 120kHz, 64RBs | From Qualcomm | -32.8215 | -29.8556 |  |  |  | -2.9659 |  |  |  |
|  |  | From MTK | -33.2358 | -30.0408 |  |  |  | -3.195 |  |  |  |
| ZTE R4-2314619 | 120kHz, 8RBs | From MTK | -31.9 |  |  |  | -31.4 |  |  |  | -0.5 |
|  | 120kHz, 20RBs |  | -31.6 |  | -31.5 |  |  |  | -0.1 |  |  |
|  | 120kHz, 20RBs |  | -30.7 |  |  | -30.8 |  |  | 0.1 |  |  |
|  | 120kHz, 66RBs |  | -30.8 | -30.9 |  |  |  |  | 0.1 |  |  |

5.3	EVM test

5.3.1	CPE compensation methods

To compensate CPE for 256QAM EVM testing, the DMRS based channel estimation and PTRS extraction shall be as below:

- The DMRS based channel estimate in the PTRS-ready EVM calculator shall utilize CPE-corrected DMRS symbols.

- The PTRS extraction and correction stage in the PTRS-ready EVM calculator is the final refinement of the received signal.

5.3.2	PTRS correction methods

- For CP-OFDM

  - PTRS correction is implemented by de-rotation of each sub carrier in an OFDM symbol. The de-rotation angle is estimated as the frequency domain average of the phase rotation of all the PTRS tones in the allocation.

- For DFT-s-OFDM:

  - PTRS correction is implemented by de-rotation of each time-domain symbol by the estimated instantaneous phase deviation.

  - The instantaneous phase deviation impacting a data symbol due to DUT phase noise is estimated by linearly interpolating between the phase deviations determined for the nearest neighbouring PTRS groups. The phase deviation for each PTRS group is determined as the time domain arithmetic mean phase deviation of all PTRS symbols in the group.

5.3.3	EVM calculation flow with PTRS

Figure 5.3.3-1 shows a generalized EVM calculator signal flow with PTRS processing. In the flow chart, for CP-OFDM, the DFT and IDFT blocks are replaced by unity blocks; For DFT-s-OFDM, all blocks pertaining to layer 2 signals are disabled due to RAN1 has not introduced rank 2 UL. And Figure 5.3.3-1 shall be included in the annex of 38.101-2 as normative content.

Figure5.3.3-1: A generalized 2-Rx EVM calculator with PTRS processing

5.3	Implementation impact for UE

5.4	Specification impact

5.5	Conclusion

6	Study for beam correspondence

6.1	General

3GPP defined 5G NR Frequency Range 2 bands with huge bandwidths which can cater 5G NR use cases requiring higher data rates. However, these bands are also subject to challenging propagating conditions such as high path loss, absorption from the environment, penetration losses to name a few. To overcome these, beam management procedures have been defined in 3GPP and beam correspondence is one of the main aspects of beam management. Beam correspondence is the ability of the UE to determine its uplink transmit beam based on downlink measurements on one or more Rx beams. UE beam correspondence functionality for RRC_CONNECTED, RRC_INACTIVE and initial access in IDLE have already been specified in the RAN1 and RAN2 specifications in Release 15 but FR2 UE beam correspondence requirements have not yet been defined for RRC_INACTIVE and initial access in IDLE.

Without UE beam correspondence requirements for initial access and RRC_INACTIVE it is not possible to ensure good UE Random access Msg1/Msg A performance and uplink coverage in FR2 deployments due to varying UE performances.

Rel-15 RRC_INACTIVE and Rel-17 Small data transmission (SDT) have a large potential in UE power efficiency, latency and signalling overhead reduction. RRC_INACTIVE allows for reduced latency and UE power saving, while SDT further enhances this for small data sessions. Considering that UE power savings are especially important for successful FR2 operations and good end-user experience, it would be important that the networks could efficiently utilize RRC_INACTIVE and Small Data Transmissions for FR2 as well.

Small data transmission procedures have been defined in 3GPP Rel-17 for SDT-RA for both two step and four step RA during RRC Inactive with a view of providing power saving opportunities and reducing overhead signalling. Without well performing UE beam correspondence support, wide usage of RRC_INACTIVE and Small Data Transmission may not be feasible in practical FR2 deployments. To enable efficient use of RRC_INACTIVE and Small Data Transmission in FR2 deployments to save UE power with reasonable latencies it is important to develop FR2 UE beam correspondence requirements for RRC_INACTIVE in Rel-18.

6.2	Implementation impact to UE

One of the main characteristics of 5G NR beam management procedures is using directive beams hence requiring beam alignment between the UE and the base station. In FR2, the UE uses the beam refinement procedure to have narrow highly directive beams towards the gNodeB thereby increasing the uplink coverage. During a Random access procedure, the UE could use both the rough and fine beams. An example of UE simulated at 28 GHz (band n257) with 1x4 array placed on the sides of a smart phone in free space (no user presence). We assume all the UE surroundings, UE model, and antenna model are the same, only change the beam width. The simulation results in Figure 6.2-1 and Figure 6.2-2 show the EIRP coverage curves (using different simulation assumptions, UE surroundings, UE model, and antenna model may cause different results).

Figure 6.2-1. EIPR spherical coverage for single and multiple broad beams (Nokia)

Figure 6.2-2: EIRP spherical coverage for envelope of narrow beams (Nokia)

In this example, the current requirements in RRC_CONNETED mode of 11.5 dBm EIRP at 50MHz CBW and at 50%-ile is represented with the grey box. In Figure 6.2-1, the blue curve, purple curve, and red curve represent the spherical coverage of a broad beam from one panel, the sum of spherical coverages of 2 broad beams from two 2 panels, and the sum of spherical coverages of 3 broad beams from two 3 panels. In Figure 6.2-2, the blue curve represents the envelope of narrow beams with a single panel with 4 elements active. The purple curve represents the sum of envelopes of narrow beams from two 2 panels, and each with 4 elements active. The red curve represents the sum of envelopes of narrow beams from two 3 panels, and each with 4 elements active.

As illustrated above, the spherical coverage at the 50%-ile is 16.5 – 19 dBm in Figure 6.2-1 and 21.4 – 24.2 dBm in Figure 6.2-2. The choice of using narrow beams and using broad beams has a big impact to the EIRP spherical coverage of a UE. We can also observe that the EIPR spherical coverage increases as the number of panels increases.

During random access procedure for Initial access or even handovers, cell reselection and other scenarios where Random access is required, the 3GPP standards do not mandate the beam type the UE used. This is up to UE implementation if a rough beam or a narrow beam is to be used and how to achieve beam refinement.

6.3	Specification Impact

6.3.1	RF

In RRC connected state, beam correspondence requirement is considered as fulfilled if the UE meets the corresponding minimum peak EIRP and spherical coverage requirement for that power class with its chosen UL beams with or without relying on uplink beam sweeping. Beam correspondence tolerance requirement could also be considered for UE supporting beam management relying on UL beam sweeping.

While in initial access and RRC_INACTIVE state, UL beam sweeping is not available. The focus is how to specify UE beam correspondence requirements for only SSB-based beam correspondence without UL beam sweeping as well as the implication to legacy UE capabilities.

RAN4 discussion is mainly focused on minimum peak EIRP and spherical coverage requirement for beam correspondence in initial access and RRC_INACTIVE state in R18. Since the UE behavior for beam management in initial access, RA-SDT and CG-SDT in RRC_INACTIVE state is the same, common set of beam correspondence requirement can be defined to well accommodate these scenarios. To save test effort, the beam correspondence requirement only needs to be tested for initial access based on Msg1 which is thought to be the most critical scenario.

- Minimum peak EIRP

Only SSB based beam correspondence requirement for minimum peak for RRC_CONNECTED state is based on fine beam implementation which is considered as too restrictive and not applicable for initial access from the perspective of addressing different beam type implementation.

Minimum peak EIRP will not be considered for initial access.

- Spherical coverage requirement at 50%-tile

TBA later

6.3.2	RRM

No RRM specification impact is identified.

6.4	Testing impact

Unlike traditional RF requirements defined in the RRC_CONNECTED state, beam correspondence for initial access needs to be verified in RRC_IDLE, and several potential testability issues were raised during the discussion. The following aspects of test related issues were identified:

- Verification in RRC_IDLE

In general, the power test needs to be averaged over a period, which requires the UE to remain in RRC_IDLE and multiple PRACH transmission need to be triggered, so testing mode including holding RAR should be considered. The accumulative period of measurement for PRACH transmission shall be at least 1 ms.

- Maximum output power for PRACH

To guarantee stable test results, the requirement is defined and verified under maximum output power (MOP). Since the open loop power control is used for PRACH, how to ensure that the UE reaches and maintains MOP is a problem that has to be faced. During the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. The required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax if configured. The power control for PRACH is defined in [1] as:

[dBm],

where  is a pathloss calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, and referenceSignalPower is provided by ss-PBCH-BlockPower. The parameter  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers as specified in clause 5.1.3 of [2]:

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

UE can ramp up its power when RAR is held since it will trigger PRACH retransmission, and adjusting each individual RA response window may affect the overall test time. UE may also achieve MOP quickly by setting the preambleReceivedTargetPower properly with as few as possible PRACH re-transmission. Based on the analysis above, The PRACH can achieve MOP by holding RAR with well-defined parameters. In addition, during the initial access, UE might start transmitting preamble once it detects the SSB RSRP higher than preconfigured threshold on one RX beam, even if this RX beam is not the optimal one that UE could form, so the test function need to be considered. The details of test function and the details of the parameter configured in the test can be left to RAN5.

- Beam lock function

Currently, the beam lock function is only available in the RRC_CONNECTED state, and if the beam lock function is absent during the verification in RRC_IDLE, many complex issues will appear, e.g., when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test. It also will be difficult to measure the θ (or φ) component EIRP under φ (or θ) link polarization without beam lock function. In [3], the LS from RAN5 concludes it is feasible to introduce beam lock function for RRC_IDLE, so the concern above can be solved. RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.

- Impact of power control accuracy

TBA

6.5	Conclusion

Editor’s note:

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104-e | R4-2212590 |  |  |  | TR skeleton | 0.0.1 |
| 2022-10 | RAN4 #104-bis-e | R4-2217730 |  |  |  | TP for TR 38.891 on link level simulation assumptions for FR2 UL 256QAM | 0.1.0 |
| 2022-11 | RAN4 #105 | R4-2219122 |  |  |  | TP for TR 38.891 on link level simulation results and system level simulation assumption for FR2 UL 256QAM | 0.2.0 |
| 2023-02 | RAN4 #106 | R4-2303490 |  |  |  | TP for TR 38.891 on simulation results and system level simulation assumptions for FR2 UL 256QAM | 0.3.0 |
| 2023-04 | RAN4 #106bis-e | R4-2305067 |  |  |  | TP for TR 38.891 on system level simulation results for FR2 UL 256QAM | 0.4.0 |
| 2023-04 | RAN4 #106bis-e | R4-2306602 |  |  |  | TP for TR 38.891: General and specification impact | 0.4.0 |
| 2023-05 | RAN4 #107 | R4-2308804 |  |  |  | TP for TR 38.891 on simulation assumptions for phase noise profiles evaluation and MPR simulation results for FR2 UL 256QAM | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2307231 |  |  |  | TP to TR 38.891: System level simulation results for FR2-1 UL 256QAM | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2310291 |  |  |  | TP for TR 38.891: General and Specification impact | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2310292 |  |  |  | TP for TR 38.891: Beam type and DRX implications | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2310293 |  |  |  | TP for test related issue of beam correspondence in initial access | 0.5.0 |
| 2023-08 | RAN4 #108 | R4-2312684 |  |  |  | TP for TR 38.891 on simulation results of phase noise profiles evaluation and MPR for FR2 UL 256QAM | 0.6.0 |
| 2023-10 | RAN4 #108bis | R4-2315435 |  |  |  | TP for TR 38.891 on simulation results of phase noise profiles evaluation and MPR for FR2 UL 256QAM | 0.7.0 |

Change history of this template:

| 2001-07 | Copyright date changed to 2001; space character added before TTC in copyright notification; space character before first reference deleted. | 1.3.3 |
| --- | --- | --- |
| 2002-01 | Copyright date changed to 2002. | 1.3.4 |
| 2002-07 | Extra Releases added to title area. | 1.3.5 |
| 2002-12 | "TM" added to 3GPP logo. | 1.3.6 |
| 2003-02 | Copyright date changed to 2003. | 1.3.7 |
| 2003-12 | Copyright date changed to 2004. Chinese OP changed from CWTS to CCSA | 14.0 |
| 2004-04 | North American OP changed from T1 to ATIS | 1.5.0 |
| 2005-11 | Stock text of clause 3 includes reference to 21.905. | 1.6.0 |
| 2005-11 | Caters for new TSG structure. Minor corrections. | 1.6.1 |
| 2006-01 | Revision marks removed. | 1.6.2 |
| 2008-11 | LTE logo line added, © date changed to 2008, guidance on keywords modified; acknowledgement of trade marks; sundry editorial corrections and cosmetic improvements | 1.7.0 |
| 2010-02 | 3GPP logo changed for cleaner version, with tag line; LTE-Advanced logo line added; © date changed to 2010; editorial change to cover page footnote text; trade marks acknowledgement text modified; additional Releases added on cover page; proforma copyright release text block modified | 1.8.0 |
| 2010-02 | Smaller 3GPP logo file used. | 1.8.1 |
| 2010-07 | Guidance note concerning use of LTE-Advanced logo added. | 1.8.2 |
| 2011-04-01 | Guidance of use of logos on cover page modified; copyright year modified. | 1.8.3 |
| 2013-05-15 | Changed File Properties to MCC macro default. Removed R99, added Rel-12/13. Modified Copyright year. Guidance on annex X Change history. | 1.8.4 |
| 2014-10-27 | Updated Release selection on cover. In clause 3, added "3GPP" to TR 21.905. | 1.8.5 |
| 2015-01-06 | New Organizational Partner TSDSI added to copyright block. Old Releases removed. | 1.9.0 |
| 2015-12-03 | Provision for LTE Advanced Pro logo Update copyright year to 2016 | 1.10.0 |
| 2016-03-08 | Standarization of the layout of the Change History table in the last annex.(Unreleased) | 1.11.0 |
| 2016-06-15 | Minor adjustment to Change History table heading | 1.11.1 |
| 2017-03-13 | Adds option for 5G logo on cover | 1.12.0 |
| 2017-05-03 | Smaller 5G logo to reduce file size | 1.12.1 |
| 2019-02-25 | Replacement of frames on cover pages by in-line text. Clarification of help text on when to use 5G logo. Removal of defunct keywords frame on page 2. Add Rel-16, Rel-17 options, eliminated earlier, frozen, Releases (cover page, below title) Corrections to some guidance text, addition of guidance text concerning automatic page headers under Word 2016 ff. Use of modal auxiliary verbs added to Foreword. More explicit guidance on Bibliography and Index annexes. Converted to .docx format. | 1.13.0 |