3GPP TSG-RAN WG4 Meeting #108bis	R4-2315912

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE demodulation performance and CSI requirements – simulation results

Agenda item:	5.14.6.1

Document for:	Discussion

Introduction

This contribution contains Nokia’s initial simulation results for the <5MHz WID [1]. The results are provided to aid in determining where new requirements can/should be introduced.

Simulation results

PDSCH simulation results

All simulation results below are done with the following:

- Carrier: 900MHz

- SCS: 15kHz

- CBW: 10MHz

- FDD

Table 1:PDSCH simulation results

| Test num. | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |  |
|  |  |  |  |  | 52PRB | 15PRB | 12PRB |
| RANK 1 |  |  |  |  |  |  |  |
| 1 | QPSK, 0.30 | TDLB100-400 | 2x4, ULA Low | 70 | -6.1 | -5.7 | -5.8 |
| 2 | QPSK, 0.30 | TDLC300-100 | 2x4, ULA Low | 70 | -5.7 | -5.5 | -5.5 |
| 3 | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 30 | -4.1 | -4.1 | -4.1 |
| RANK 2 |  |  |  |  |  |  |  |
| 4 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | 6.7 | 6.4 | 6.45 |

As expected, only minor changes in SNR are seen when going from 52PRB to 12 PRB when keeping the same CBW. We do however expect more significant changes in SNR when reducing the CBW for 15 and 12 PRB to 3MHz as we see the UE performance will be more susceptible to the edge effect of the implementation in particular related to the channel smoothening and filter implementation.

With 10MHz CBW and reduced PRB there is minor changes in the 70% SNR level. We expect more significant changes in SNR when the CBW is 3MHz due to edge-effect of the PRBs will have more impact on throughput.

PDCCH simulation results

As there do not exist requirements for 5MHz and since CCE puncturing is expected to have some impact on simulation results, we have focused our simulation on showing the impact with relation to:

- Number of RBs reduced to 15 and in 1 case 12

- Use of both 2 and 3 symbols for coreset duration

- AL4 and AL8

- With and without interleaving

We have included Figure 1 which shows how CCE puncturing will occur for different configurations.

Figure 1: CCE to REG mapping with interleaved and non-interleaved options in 15 RB CORESET#0

Our simulation results are shown in the following tables:

Table 2: PDCCH simulation results for 1Tx 2Rx / 15RB

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 5 | 15 | 2 | 4 | TDLA30-10 | 1x2 Low | no | 1 | 2.2 |
| 1-2 | 5 | 15 | 2 | 8 | TDLA30-10 | 1x2 Low | no | 1 | 1.2 |
| 1-3 | 5 | 15 | 3 | 8 | TDLA30-10 | 1x2 Low | no | 1 | -0.5 |
| 1-4 | 5 | 15 | 2 | 8 | TDLA30-10 | 1x2 Low | yes | 1 | 1.2 |
| 1-5 | 5 | 15 | 3 | 8 | TDLA30-10 | 1x2 Low | yes | 1 | 0.4 |

Table 3: PDCCH simulation results for 1Tx 2Rx / 12RB

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 5 | 12 | 2 | 4 | TDLA30-10 | 1x2 Low | yes | 1 | 2.2 |

Table 4: PDCCH simulation results for 2Tx 2Rx / 15RB

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | 5 | 15 | 2 | 4 | TDLC300-100 | 2x2 Low | no | 1 | -2 |
| 3-2 | 5 | 15 | 2 | 8 | TDLC300-100 | 2x2 Low | no | 1 | -3 |
| 3-3 | 5 | 15 | 3 | 8 | TDLC300-100 | 2x2 Low | no | 1 | -4.7 |
| 3-4 | 5 | 15 | 2 | 8 | TDLC300-100 | 2x2 Low | yes | 1 | -3 |
| 3-5 | 5 | 15 | 3 | 8 | TDLC300-100 | 2x2 Low | yes | 1 | -3.6 |

Table 5: PDCCH simulation results for 1Tx 4Rx / 15RB

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-1 | 5 | 15 | 2 | 4 | TDLA30-10 | 1x4 Low | no | 1 | -4 |
| 4-2 | 5 | 15 | 2 | 8 | TDLA30-10 | 1x4 Low | no | 1 | -4.8 |
| 4-3 | 5 | 15 | 3 | 8 | TDLA30-10 | 1x4 Low | no | 1 | -6.5 |
| 4-4 | 5 | 15 | 2 | 8 | TDLA30-10 | 1x4 Low | yes | 1 | -4.8 |
| 4-5 | 5 | 15 | 3 | 8 | TDLA30-10 | 1x4 Low | yes | 1 | -5.3 |

Table 6: PDCCH simulation results for 2Tx 4Rx / 15RB

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-1 | 5 | 15 | 2 | 4 | TDLC300-100 | 2x4 Low | no | 1 | -6 |
| 5-2 | 5 | 15 | 2 | 8 | TDLC300-100 | 2x4 Low | no | 1 | -6.8 |
| 5-3 | 5 | 15 | 3 | 8 | TDLC300-100 | 2x4 Low | no | 1 | -8.3 |
| 5-4 | 5 | 15 | 2 | 8 | TDLC300-100 | 2x4 Low | yes | 1 | -6.8 |
| 5-5 | 5 | 15 | 3 | 8 | TDLC300-100 | 2x4 Low | yes | 1 | -7.3 |

We have the following observations based on our simulation results when using 5MHz CBW:

For AL4, 2 symbols, non-interleaved, there is no puncturing as all CCEs can be fully included in the available 12/15 PRB. The results correspond to existing requirements in 38.101-4 when taking impairments and margin into account. Since there is no puncturing, no change will be seen with interleaving.

For AL8, 2 symbols, non-interleaved, 3 CCEs are punctured which results in 0.8 to 1dB lower SNR compared to AL4.

For AL8, 3 symbols, non-interleaved, only 0.5CCE is punctured which results in 1.5 dB lower SNR compared to AL8 with only 2 symbols.

For AL8, 2 symbols, interleaved provides the same result as non-interleaved as there is no change in the number of punctured CCEs.

For AL8, 3 symbols, interleaved there is an increase in SNR of 0.5dB due to the puncturing of 2 additional CCEs compared to the non-interleaved case.

PBCH simulation results

All simulation results below are done with the following:

- Block index known

- SCS: 15KHz

- CBW: 10Mhz

- Propagation conditions: TDLC300-100

Table 7 PBCH simulation results with Block index known

| Test Number | 4Ghz CC, SNR [dB] | 900Mhz CC SNR [dB] | 900MHz CC 12PRB SNR [dB] |
| --- | --- | --- | --- |
| 1Tx2Rx |  |  |  |
| 1 | -11.2 | -11.2 | -6.2 |
| 1Tx4Rx |  |  |  |
| 2 | -13.9 | -13.9 | -9.7 |

As expected, we do not see any degradation when going from 4GHz to 900MHz CC when keeping the 10MHz CBW. However, when puncturing the PBCH down to 12PRB a significant degradation is seen.

For PBCH, significant impact is seen when reducing to 12PRB.

Conclusion

Within this contribution we have provided our simulation results PDSCH, PDCCH and PBCH related to <5MHz CBW.

We have made the following observations:

PDSCH

With 10MHz CBW and reduced PRB there is minor changes in the 70% SNR level for PBSCH. We expect more significant changes in SNR when the CBW is 3MHz due to edge-effect of the PRBs will have more impact on throughput.

PDCCH

For AL4, 2 symbols, non-interleaved, there is no puncturing as all CCEs can be fully included in the available 12/15 PRB. The results correspond to existing requirements in 38.101-4 when taking impairments and margin into account. Since there is no puncturing, no change will be seen with interleaving.

For AL8, 2 symbols, non-interleaved, 3 CCEs are punctured which results in 0.8 to 1dB lower SNR compared to AL4.

For AL8, 3 symbols, non-interleaved, only 0.5CCE is punctured which results in 1.5 dB lower SNR compared to AL8 with only 2 symbols.

For AL8, 2 symbols, interleaved provides the same result as non-interleaved as there is no change in the number of punctured CCEs.

For AL8, 3 symbols, interleaved there is an increase in SNR of 0.5dB due to the puncturing of 2 additional CCEs compared to the non-interleaved case.

PBCH

For PBCH, significant impact is seen when reducing to 12PRB.

- References

- RP-213603, “New WI: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, UIC, Anterix, Qualcomm