3GPP TSG-RAN WG4 # 108-bis	R4-2315938

Xiamen, China, October 9 – October 13, 2023

Agenda item:	5.7.4.2

Source:	MediaTek Inc.

Title:	Simulation results of PDSCH requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#108, the Way Forward document was approved [1]. We provide some simulation results of PDSCH requirements for simulation alignment for the Rel-18 work in this contribution.

# Simulation results

In this chapter we provide some simulation results of PDSCH requirements for simulation alignment.

We have simulated both Single-DCI and Multi-DCI transmission schemes for this document. Multi-DCI transmission scheme is simulated with both fully overlapping and non-overlapping modes.

In Single-DCI transmission scheme, we have simulated both joint and separate processing receiver assumptions. In Multi-DCI transmission schemes, we have simulated only joint processing receiver assumption.

Channel model is TDLA30-75 for 100 MHz/120 kHz. The correlation matrix model follows agreement from the previous meeting [1]. MCS13 is used in all simulations. TX EVM is 6% as agreed. For phase noise modelling we are using Example 1 model from TR38.803 [2] assuming carrier frequency parameters for 29.55 GHz. We have simulated both time and frequency offset pairs (0.25ns, 0Hz) and (-0.0625us, 600Hz) between TRPs. In Single-DCI SDM simulations we have used single port PT-RS configuration because it seems to be feasible.

First, in Figures 1-8 there are simulation curves of Multi-TRP Single-DCI SDM scheme with Rank 2 and 4 using both joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 1.

Next, in Figures 9-12 there are simulation curves of Multi-TRP Multi-DCI fully overlapping scheme with Rank 1+1 and 2+2 using joint processing receiver assumption. Simulation results from all these figures are collected to Table 2.

Finally, in Figures 13-16 there are simulation curves of Multi-TRP Multi-DCI non-overlapping scheme with Rank 1+1 and 2+2 using joint processing receiver assumption. Simulation results from all these figures are collected to Table 3.

The simulated cross-talk power ratios ρ are 0dB, -3dB, -6dB, -9dB, -12dB and -15dB. Here, ρ in [dB] is defined as 10·log10(ρ).

For Single-DCI SDM and Multi-DCI fully overlapping we have not assumed additional 1/sqrt(2) scaling to TRPs because this scaling is embedded in the correlation matrix model. Instead, we have scaled TRPs in Multi-DCI non-overlapping by sqrt(2) to compensate the total power loss caused by the reduced per-TRP transmission bandwidth.

Figure 1: Multi-TRP Single-DCI SDM Rank 4, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 2: Multi-TRP Single-DCI SDM Rank 4, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 3: Multi-TRP Single-DCI SDM Rank 4, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 4: Multi-TRP Single-DCI SDM Rank 4, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 5: Multi-TRP Single-DCI SDM Rank 2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 6: Multi-TRP Single-DCI SDM Rank 2, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 7: Multi-TRP Single-DCI SDM Rank 2, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 8: Multi-TRP Single-DCI SDM Rank 2, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 9: Multi-TRP Multi-DCI fully overlapping Rank 2+2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 10: Multi-TRP Multi-DCI fully overlapping Rank 2+2. joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 11: Multi-TRP Multi-DCI fully overlapping Rank 1+1, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 12: Multi-TRP Multi-DCI fully overlapping Rank 1+1, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 13: Multi-TRP Multi-DCI non-overlapping Rank 2+2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 14: Multi-TRP Multi-DCI non-overlapping Rank 2+2. joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 15: Multi-TRP Multi-DCI non-overlapping Rank 1+1, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 16: Multi-TRP Multi-DCI non-overlapping Rank 1+1, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 14.0dB | 14.0dB | 13.8dB | 13.6dB | 13.4dB | 13.3dB |
| Rank 4 / separate processing 0.25us, 0Hz | N/A | N/A | N/A | N/A | 16.2dB | 14.1dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 14.0dB | 13.9dB | 13.8dB | 13.6dB | 13.4dB | 13.3dB |
| Rank 4 / separate processing -0.0625us, 600Hz | N/A | N/A | N/A | N/A | 16.2dB | 14.1dB |
| Rank 2 / joint processing 0.25us, 0Hz | 7.3dB | 7.2dB | 7.1dB | 7.0dB | 6.9dB | 6.9dB |
| Rank 2 / separate processing 0.25us, 0Hz | N/A | 10.6dB | 9.0dB | 7.7dB | 6.9dB | 6.4dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 7.3dB | 7.2dB | 7.1dB | 7.0dB | 7.0dB | 6.9dB |
| Rank 2 / separate processing -0.0625us, 600Hz | N/A | 10.6dB | 9.0dB | 7.7dB | 6.9dB | 6.4dB |

Table 1: Simulation result collection of Single-DCI SDM scenarios
showing SNR needed to achieve 70% of maximum throughput.

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 13.9dB | 13.6dB | 13.1dB | 12.6dB | 12.2dB | 12.0dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 13.8dB | 13.6dB | 13.1dB | 12.6dB | 12.3dB | 11.9dB |
| Rank 2 / joint processing 0.25us, 0Hz | 6.6dB | 6.4dB | 6.1dB | 5.9dB | 5.7dB | 5.6dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 6.5dB | 6.4dB | 6.1dB | 5.9dB | 5.7dB | 5.6dB |

Table 2: Simulation result collection of Multi-DCI fully overlapping scenarios
showing SNR needed to achieve 70% of maximum throughput.

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 6.8dB | 6.9dB | 7.1dB | 7.4dB | 7.8dB | 8.1dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 6.8dB | 6.9dB | 7.1dB | 7.5dB | 7.9dB | 8.1dB |
| Rank 2 / joint processing 0.25us, 0Hz | 2.1dB | 2.0dB | 2.0dB | 2.0dB | 2.0dB | 2.1dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 2.1dB | 2.0dB | 2.0dB | 2.0dB | 2.0dB | 2.0dB |

Table 3: Simulation result collection of Multi-DCI non-overlapping scenarios
showing SNR needed to achieve 70% of maximum throughput.

From simulation results collection in Tables 1-3, we can do following observations:

Observation #1: In fully overlapping schemes separate processing receiver assumption requires high isolation to work.

Observation #2: In fully overlapping schemes joint processing receiver assumption is robust with all simulated ρ values.

Observation #3: In fully overlapping schemes joint processing receiver assumption performance is better with higher isolation.

Observation #4: In non-overlapping schemes joint processing receiver assumption in low isolation gives diversity gain.

# Conclusion

In this paper we provided simulation results on the PDSCH requirements of FR2 multipanel RX downlink demodulation requirements for performance alignment. The following observations are made:

Observation #1: In fully overlapping schemes separate processing receiver assumption requires high isolation to work.

Observation #2: In fully overlapping schemes joint processing receiver assumption is robust with all simulated ρ values.

Observation #3: In fully overlapping schemes joint processing receiver assumption performance is better with higher isolation.

Observation #4: In non-overlapping schemes joint processing receiver assumption in low isolation gives diversity gain.

# Reference

- R4-2313977, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm

- 3GPP TR 38.803 V14.3.0 (2022-03), “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”