3GPP TSG-RAN WG4 Meeting #108bis	R4-2315967
Xiamen, China, 9 – 13 Oct, 2023

Title:	R18 Tx requirement for 3Tx inter-band combinations

Source:	OPPO

Agenda item:	5.5.3.1

Document for:	Approval

- Introduction

Rel-18 Tx requirements for inter-band 3Tx has been discussed in last meeting with WF [1] approved. In this paper, the RF requirement left over issues for 3Tx will be discussed.

Issue 4-3-1: How to indicate the 3Tx band combinations in the CA configuration tables - WF: FFS whether below updates to the NOTEs of CA configuration tables is agreeable in next meeting. Issue 4-4-1: Introduce new per FS TxD capability - Proposals (R4-2312249) - WF: FFS whether new per FS TxD capability is needed for a band under a band combination that support TxD.

- Discussion

2.1 TxD capability

It was pointed out that the current TxD capability is per band in 38.306, and to support inter-band UL CA+TxD, a per band per BC TxD capability is needed. RAN4 can inform RAN2 on this and whether to define new capability or reuse the existing capability is up to RAN2.

Proposal 1:         Inform RAN2 to define capability for Tx diversity for a band under band combination.

2.2 Indicate 3Tx in CA configuration tables

For the clarification of applicable 3Tx power class capabilities in configuration tables. The changes can be as below by using general descriptions and refer to the MOP table to avoid inconsistency.

#### 5.5A.3.1 Configurations for inter-band CA (two bands) <<<unchanged part omitted >>> The following notes are applied to the above tables: NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE 2: The minimum requirements for intra-band contiguous or non-contiguous CA apply. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration NOTE 7: Limited to operation at 3450-3550 MHz and 3700–3980 MHz. NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 10: Only single uplink carriers with power class other than PC3 are listed. NOTE 11: The CA configurations are given in Table 5.5A.1-1 or Table 5.5A.2-1 in this specification NOTE 12: UL configurations are for non-simultaneous Rx/Tx operation. NOTE X: Minimum requirements for Power Class 2 or Power Class 1.5 are applicable for this inter-band uplink CA with UL MIMO supported in one of the two bands as specified in Table 6.2H.3.1-1. NOTE Y: Minimum requirements for Power Class 2 or Power Class 1.5 are applicable for this inter-band uplink CA with Tx diversity supported in one of the two bands as specified in Table 6.2J.3.1-1.

Proposal 2:         Clarify 3Tx power class capabilities in inter-band CA configuration with general notes referring to MOP table.

2.3 Release independent issue

From which release the 3Tx band combinations can be used hasn’t been discussed, and in our understanding except the above new per band per BC TxD capability, the 3Tx band combinations can be release independent from Rel-16.

And before meeting there was comment that 38.307 is not part of the WID. To solve this concern, the WID can be updated in next RAN plenary, however, considering there is no RAN4 R18 meeting available it is proposed to focus on technical discussion and endorse CR then update the WID.

Proposal 3:         3Tx band combinations without TxD can be release independent from Rel-16. And 3Tx band combinations with TxD should be from Rel-18.

Proposal 4:         Propose to focus on technical discussion and endorse 38.307 CR then update the WID in next RAN plenary to include 38.307 spec.

- References

- R4-2314705, WF on 3Tx and 4Rx requirements, OPPO, RAN4#108, Aug 2023

- R4-2314915, CR for TS38101-1 Clarifying applicable power classes for NR CA, Huawei, RAN4#108, Aug 2023