3GPP TSG-RAN WG4 Meeting # 108	R4-2315979

# Toulouse, France, August 21 – August 25, 2023

Title: 	Test parameters for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	5.18.2

Document for:	Discussion

Background

At last meeting, a WF [1] was agreed and one part is how to define test parameters for advanced receiver for MU-MIMO. In this contribution, we provide our corresponding views

Discussions

Co-scheduled UE number

Based on our understanding, target UE is agnostic to number of co-scheduled UEs when performing interference cancellation. There are no difference of baseband processing between one co-scheduled UE and two co-scheduled UEs. We propose to only consider one scheduled UE.

Frequency domain resource allocation

We don’t see the necessity to introduce case with partial CHBW of co-scheduled UE since FDRA has no impact on baseband processing. Meanwhile, full CHBW of co-scheduled has more interfered RBs which can lead to more performance gain.

Precoder selection for co-scheduled UE

We propose to reuse the precoder selection of Rel-17 MMSE-IRC test. I.e. Random selection of Rank 1+1 and Orthogonal selection of Rank 2+2. RAN4 didn’t run the simulation for orthogonal PMI selection for Rank 1+1 in phase I, the R-ML performance gain is unclear.

Test setting for the RAN4 agreed network default assumptions

Based on our understanding, RRC signalling should be configured as following for R-ML test. (All information are true)

- For target and co-scheduled UE in different CDM groups, the precoding and resource allocation of any co-scheduled UE  with same DMRS sequence as target UE are same in the PRG-level grid configured to the target UE when PRG=2 or 4.

- Target and any co-scheduled UE with same DMRS sequence as target UE has same parameter “Number of CDM groups without data ”

- Target and any co-scheduled UE with same DMRS sequence as target UE has same time domain allocation in a slot.

Modulation order for co-scheduled UE

We propose to use QPSK to maximum the performance gain

MCS

For MCS, our simulation results captured in Appendix shows that MCS4 for Rank 1+1 and MCS17 for Rank 2+2 have sufficient performance gain. We propose to select MCS4 for Rank 1+1 and MCS17 for Rank 2+2

Modulation order blind detection

To trade off the performance and complexity and make it convenient to align the simulation results, we propose only consider Rank1+1

Others

For other parameters, we propose to reuse the Rel-17 MMSE-IRC test setup.

Based on the above discussion, we propose the following:

Proposal 1: Consider following test setup:

- Duplex mode: FDD, TDD

- CBW/SCS: 10MHz/15kHz for FDD and 40MHz/30kHz for TDD

- Number of co-scheduled UE: 1 co-scheduled UE with full overlapping

- Rank allocation: Rank 1+1 and Rank 2+2

- Antenna configuration: 2T2R and 2T4R for Rank 1+1, 4T4R for Rank 2+2

- Propagation conditions: TDLC300-100 medium for Rank 1+1 and TDLA30-10 low for Rank 2+2

- PMI selection: Random for Rank 1+1 and Orthogonal for Rank 2+2

- MCS: 13 for Rank 1+1 and 17 for Rank 2+2

- Modulation order of co-scheduled UE” QPSK

- DCI bitmap mapped to index 1 is indicated.

- RRC signalling should is indicated:

  - For target and co-scheduled UE in different CDM groups, the precoding and resource allocation of any co-scheduled UE  with same DMRS sequence as target UE are same in the PRG-level grid configured to the target UE when PRG=2 or 4.

  - Target and any co-scheduled UE with same DMRS sequence as target UE has same parameter “Number of CDM groups without data ”

  - Target and any co-scheduled UE with same DMRS sequence as target UE has same time domain allocation in a slot.

Proposal 2:  Choose Rank 1+1 for requirements definition with modulation order detection

Conclusion

In this contribution, we provide our views on test parameters on advanced receiver for MU-MIMO, the proposals and observations are:

Proposal 1: Consider following test setup:

- Duplex mode: FDD, TDD

- CBW/SCS: 10MHz/15kHz for FDD and 40MHz/30kHz for TDD

- Number of co-scheduled UE: 1 co-scheduled UE with full overlapping

- Rank allocation: Rank 1+1 and Rank 2+2

- Antenna configuration: 2T2R and 2T4R for Rank 1+1, 4T4R for Rank 2+2

- Propagation conditions: TDLC300-100 medium for Rank 1+1 and TDLA30-10 low for Rank 2+2

- PMI selection: Random for Rank 1+1 and Orthogonal for Rank 2+2

- MCS: 13 for Rank 1+1 and 17 for Rank 2+2

- Modulation order of co-scheduled UE” QPSK

- DCI bitmap mapped to index 1 is indicated.

- RRC signalling should is indicated:

  - For target and co-scheduled UE in different CDM groups, the precoding and resource allocation of any co-scheduled UE  with same DMRS sequence as target UE are same in the PRG-level grid configured to the target UE when PRG=2 or 4.

  - Target and any co-scheduled UE with same DMRS sequence as target UE has same parameter “Number of CDM groups without data ”

  - Target and any co-scheduled UE with same DMRS sequence as target UE has same time domain allocation in a slot.

Proposal 2:  Choose Rank 1+1 for requirements definition with modulation order detection

Reference

[1]    R4-2313993 WF on advanced receiver for MU-MIMO scenario. China Telecom

Appendix

Table 2-1: Summary of simulation results for genie aided knowledge of required information

| Rank allocation | Propagation Conditions | PMI selection | MCS for Serving UE | Interference UE | MMSE-IRC | E-MMSE-IRC | R-ML | E-MMSE-IRC(Gain) | R-ML(Gain) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1+1 | TDLC300-100 Medium | Random | 4 | QPSK | 5.6 | 5.3 | 4.2 | 0.3 | 1.1 |
|  |  |  | 13 | QPSK | 21.9 | 20.8 | 13.7 | 1.1 | 8.2 |
| Rank 2+2 | TDLA30-10 Low | Orthogonal | 13 | QPSK | 12.1 | 12.1 | 9.3 | 0.0 | 2.8 |
|  |  |  |  | 16QAM | 12.1 | 12.0 | 11.3 | 0.1 | 0.8 |
|  |  |  |  | 64QAM | 12.1 | 12.1 | 11.7 | 0.0 | 0.4 |
|  |  |  | 17 | QPSK | 16.4 | 16.4 | 13.1 | 0.0 | 3.3 |
|  |  |  |  | 16QAM | 16.5 | 16.4 | 15.1 | 0.1 | 1.4 |
|  |  |  |  | 64QAM | 16.5 | 16.4 | 16.2 | 0.1 | 0.2 |
|  | TDLC300-100 Low | Random | 13 | QPSK | 22.3 | 18.3 | 13.7 | 4.0 | 8.6 |
|  |  |  |  | 64QAM | 22.3 | 18.3 | 17.4 | 4.0 | 4.9 |
|  |  | Orthogonal |  | QPSK | 14.5 | 13.3 | 10.7 | 1.2 | 3.8 |
|  |  |  |  | 64QAM | 14.5 | 13.3 | 12.9 | 1.2 | 1.6 |