3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2315982

# Xiamen, China, Oct.9th – Oct.13th, 2023

Title: 	Remain issues on 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.3.3.1.1

Document for:	Discussion

Background

A WF for 8Rx UE performance requirements was approved in last meeting and there are some remaining open issues. This contribution provides our views on these open issues.

Discussions

HARQ feedback mode

Two HARQ feedback modes are supported for 2 codewords: spatial HARQ bundling enabled and spatial HARQ bundling disabled. For the former, two ACK/NACK bits are bundled as one bit with logic “AND” operation, that means UE shall retransmit the two TBs corresponding to two codewords together if at least one TB is failed. However, RAN4 didn’t discuss this parameter and simply reuse the Rel-15 parameter specified in common test parameters table (Table 5.2-1 in TS 38.101-4):

| HARQ ACK/NACK bundling |  | Multiplexed |
| --- | --- | --- |

That means RAN4 assumed that spatial HARQ bundling is enabled for Rank 8 test in the simulation assumption.

Observation 1: RAN4 assumed that spatial HARQ bundling is enabled in the simulation assumption for Rank 8 test

We provide our simulation results for case with “Spatial HARQ bundling enabled” and “Spatial HARQ bundling disabled” for Rank 8 as below:

Figure 2-1:  Simulation results for HARQ bundling enabled and disabled.

We can find that there is 0.7dB performance degradation for case with spatial HARQ bundling enabled compared to case with HARQ bundling disabled.

Observation 2: 0.7dB performance degradation can be observed if spatial HARQ bundling is enabled compared to spatial HARQ bundling disabled.

Furthermore, according to 38.306, this feature is a mandatory feature with UE capability signalling:

spatialBundlingHARQ-ACK Indicates whether the UE supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation.

We don’t think it's necessary to bundle two ACK/NACK bits during the Rank8 test since it is a redundant operation which causes  unnecessary retransmission for some TBs and degrade the performance.

Proposal 1: Don't configure “Multiplexed” for “HARQ ACK/NACK bundling”. Use “independence” or “Disabled” instead.

CA for PDSCH requirements

CBW for PDSCH CA requirements

One issue is CBW for PDSCH CA requirements. The candidate options [1] are:

CBW for PDSCH CA requirements - Option 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e. - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz - Option 2: Single bandwidth for each CC: 15kHz/10MHz for FDD and 30kHz/40MHz for TDD separately

Following are agreements on example band combination from [2]:

Issue 2-2-1: Whether example band combinations are needed. <Agreement in Main session in Tue.> - Agree on Option 1, meaning that - RAN4 does not define example band combinations for 8Rx CA/DC - 8Rx requirements should apply to the band(s) implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC)

RAN4 RF session agreed not define example band combination for 8Rx CA/DC, if RAN4 only defines 8Rx CA demodulation performance requirements only for one certain bandwidth combination, such as the one proposed in Option 2 [1], firstly we can’t guarantee the UE under test will support that bandwidth combination. Secondly maybe it isn’t the largest aggregated bandwidths supported by the UE. One of main test purpose of CA demodulation requirements is to test the soft buffer capability that can be supported by the UE, this will not be properly verified by following Option 2 [1], also this is the logic behind for the test rules defined in TS 38.101-4 Table 5.1.1.7.2-1 and Table 5.1.1.7.2-2 for CA test. The SNR span across different bandwidth is not the key factor for CA requirements definition. So, we prefer Option 1.

Proposal 2: Follow the existing methodology for CA requirements defined for 2Rx and 4Rx:

FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Intra band contiguous CA, Intra band non-contiguous CA, Inter band CA

Test scenario

One issue is how to select the MIMO layers, the candidate options are shown as follows:

One single test scenario to be defined, with same test configurations as single carrier case except the bandwidth: - Option 1: Rank8 case - Option 2: Rank2 case

8Rx UE support up to 8MIMO layers, which is introduced as a main feature in the WI scope. Hence RAN4 should consider Rank8 as much as possible for requirements definition to verify whether 8Rx UE is capable of this feature in various scenarios. Rank2 is a old feature which has been considered in many legacy cases, which should be precluded for the new tests.

Proposal 3: Consider Rank8 for CA test

Duplex mode

One issue is duplex mode, the candidate options are shown as follows:

- Option 1: Consider the following duplex model for 8Rx CA - FDD 15kHz + TDD 30kHz - FDD 15kHz + FDD 15kHz - TDD 30kHz + TDD 30kHz - Other options not precluded

We prefer to preclude TDD 15kHz since this configuration is rare in the real deployment. I.e. We prefer Option 1.

Proposal 4: Consider following duplex model for 8Rx CA:

FDD 15kHz + TDD 30kHz

FDD 15kHz + FDD 15kHz

TDD 30kHz + TDD 30kHz

Test applicability

One issue is whether to consider hybrid Rx for CA such as 2Rx+8Rx, 4Rx+8Rx. Based on our understanding, RAN4 should focus on UE’s performance for 8 Rx rather than 2Rx. Therefore, hybrid Rx for CA should be precluded.

Proposal 5: Don’t consider hybrid Rx combination for 8Rx CA performance requirements

Regarding the applicability of selecting CA configuration, we propose the procedure listed in Table 2-1:

Table 2-1: Proposed selection of CA configuration for 8Rx (demodulation test)

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C or CA_N | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is 8 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is 8. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is 8. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |
| NOTE 1: For CA_AX capability, if CA configuration from step 2 is CA configuration with the largest number of bands then Step 3 and Step 4 are skipped. Otherwise, the two CA configurations selected from Step 2 and Step 4 are used for testing. NOTE 2: Maximum supported data rate for Step 2 and Step 4 is calculated based clause 4.1.2 of TS 38.306 [14]. NOTE 3: Tested data rate for Step 2 and Step 4 is calculated based on the equation DataRate=10-3j=1JTBSj2μj and FRCs used in the test. |  |  |  |  |

Proposal 6: Consider the test applicability rules in Table 2-1

CA for CQI requirements

CA test for CQI reporting

One issue is whether to define CQI requirements for CA, the candidate options are shown as follows:

CA test for CQI reporting - Option 1: Not consider CQI reporting test for CA - Option 2: Define CQI test for CA

The motivation of introducing CA test for CQI reporting is to verify whether UE performs CQI computation for each CC independently. Large SNR span is set across different CCs and the test metric is the reported CQI difference between CCs. We don’t see any reason to preclude the CQI CA requirements for 8Rx since it has been introduced for 2Rx/4Rx.

Test parameter:

We can reuse the test parameters of single CC for CA. I.e. The test parameters listed in Table 2-2:

Table 2-2: Proposed test parameters for CA CQI test

| Parameter |  |  | Unit | FDD CC | TDD CC |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 5MHz 10MHz, 15MHz, 20MHz 25MHz, 30MHz, 35MHz, 40MHz, 45MHz and 50MHz | 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz ,50MHz, 60MHz, 80MHz, 90MHz and 100MHz |
| SCS |  |  | kHz | 15 | 30 |
| TDD UL-DL pattern |  |  |  | N/A | 7D1S2U S=6D+4G+4U |
| Propagation channel |  |  |  | static channel model with 4T8R |  |
| Antenna configuration |  |  |  | XP 4×8 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 2 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 for 5MHz and 10MHz, 16 for 15MHz, 20MHz and 25MHz, 32 for 30MHz, 35MHz, 40MHz, 45MHz and 50MHz | 8 for 10MHz, 15MHz, 20MHz and 25MHz, 16 for 30MHz, 40MHz and 50MHz, 32 for 60MHz, 80MHz, 90MHz and 100MHz |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00000001 |  |
|  |  | RI Restriction |  | 00001000 |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Number of HARQ Processes |  |  |  | 4 | 8 |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-3 | As specified in Table A.4-3, TBS.3-4 |

SNR configuration:

The SNR configuration can refer to the simulation results of single CC. We provide our simulation results as shown in Figure 2-2:

Figure 2-2:  CQI simulation results

Based on the simulation results, the CQI difference corresponds to 12dB, 6dB and 0dB is 3, which is feasible be used. we propose to select {12dB, 6dB} for {Pcell, Scell} for 2DL CA and select {12dB, 6dB, 0dB} for {Pcell, Scell1, Scell2,3,…} for 3 or more DL CA.

Test metric:

Based on our simulation results and proposed the tested SNR points, considering some margin, we propose the following:

For CA with 2 DL CC, the difference between the wideband CQI indices of PCell and SCell reported shall be such that wideband CQIPCell – wideband CQISCell ≥ 2 for more than 90% of the time. For CA with 3 or more DL CC, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, 3… reported shall be such that wideband CQIPCell – wideband CQISCell1 ≥ 2 wideband CQISCell1 – wideband CQISCell2, 3… ≥ 2 for more than 90% of the time.

Others

- We propose the following for CA. I.e.

FDD 15kHz + TDD 30kHz

FDD 15kHz + FDD 15kHz

TDD 30kHz + TDD 30kHz

FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Intra band contiguous CA, Intra band non-contiguous CA, Inter band CA

Only consider CA configuration with 8Rx and don’t consider hybrid Rx configuration

Test applicability

We propose to reuse the test applicability of demodulation test, except for selecting the CA for which the supported maximum number of MIMO layers is higher than 4 instead of 8.

Table 2-3: Proposed selection of CA configuration for 8Rx (CQI test)

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C or CA_N | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is higher than 4 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is higher than 4. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is higher than 4. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |
| NOTE 1: For CA_AX capability, if CA configuration from step 2 is CA configuration with the largest number of bands then Step 3 and Step 4 are skipped. Otherwise, the two CA configurations selected from Step 2 and Step 4 are used for testing. NOTE 2: Maximum supported data rate for Step 2 and Step 4 is calculated based clause 4.1.2 of TS 38.306 [14]. NOTE 3: Tested data rate for Step 2 and Step 4 is calculated based on the equation DataRate=10-3j=1JTBSj2μj and FRCs used in the test. |  |  |  |  |

Proposal 7: RAN4 to define the CA CQI reporting requirements for 8Rx with following:

Test parameters listed in Table 2-2

FDD 15kHz + TDD 30kHz

FDD 15kHz + FDD 15kHz

TDD 30kHz + TDD 30kHz

FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Intra band contiguous CA, Intra band non-contiguous CA, Inter band CA

Only consider CA configuration with 8Rx and don’t consider hybrid Rx configuration

SNR configuration:

  - {12dB, 6dB} for {Pcell, Scell} for 2DL CA , {12dB, 6dB, 0dB} for {Pcell, Scell1, Scell2,3,…} for 3 or more DL CA.

- Test metric

  - For CA with 2 DL CC, the difference between the wideband CQI indices of PCell and SCell reported shall be such that: wideband CQIPCell – wideband CQISCell ≥ 2 for more than 90% of the time.

  - For CA with 3 or more DL CC,  the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, 3… reported shall be such that, wideband CQIPCell – wideband CQISCell1 ≥ 2, wideband CQISCell1 – wideband CQISCell2, 3… ≥ 2 for more than 90% of the time.

- Consider test applicability rules listed in Table 2-3

Simulation results for CA

We provide our simulation results for CA in sub-section 3.1 and 3.2.

FDD 15kHz SCS

Table 2-4: Simulation results for 8Rx PDSCH CA (FDD 15kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.1 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.3 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.6 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 35 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.7 |
| 45 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.7 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.4 |

TDD 30kHz SCS

Table 2-5: Simulation results for 8Rx PDSCH CA (TDD 30kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.4 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.3 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.9 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.8 |
| 60 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.2 |
| 80 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 |
| 90 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.8 |
| 100 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.1 |

Conclusion

In this contribution, we provide our views on open issues for 8Rx performance requirements. The observations and proposals are:

Observation 1: RAN4 assumed that spatial HARQ bundling is enabled in the simulation assumption for Rank 8 test

Observation 2: 0.7dB performance degradation can be observed if spatial HARQ bundling is enabled compared to spatial HARQ bundling disabled.

Proposal 1: Don't configure “Multiplexed” for “HARQ ACK/NACK bundling”. Use “independence” or “Disabled” instead.

Proposal 2: Follow the existing CA requirements defined for 2Rx and 4Rx:

FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Intra band contiguous CA, Intra band non-contiguous CA, Inter band CA

Proposal 3: Consider Rank8 for CA test

Proposal 4: Consider following duplex model for 8Rx CA:

FDD 15kHz + TDD 30kHz

FDD 15kHz + FDD 15kHz

TDD 30kHz + TDD 30kHz

Proposal 5: Don’t consider hybrid Rx for CA

Proposal 6: Consider the test applicability rules in Table 2-1

Proposal 7: RAN4 to define the CA CQI reporting requirements for 8Rx with following:

Test parameters listed in Table 2-2

FDD 15kHz + TDD 30kHz

FDD 15kHz + FDD 15kHz

TDD 30kHz + TDD 30kHz

FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Intra band contiguous CA, Intra band non-contiguous CA, Inter band CA

Only consider CA configuration with 8Rx and don’t consider hybrid Rx configuration

SNR configuration:

  - {12dB, 6dB} for {Pcell, Scell} for 2DL CA , {12dB, 6dB, 0dB} for {Pcell, Scell1, Scell2,3,…} for 3 or more DL CA.

- Test metric

  - For CA with 2 DL CC, the difference between the wideband CQI indices of PCell and SCell reported shall be such that: wideband CQIPCell – wideband CQISCell ≥ 2 for more than 90% of the time.

  - For CA with 3 or more DL CC,  the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, 3… reported shall be such that, wideband CQIPCell – wideband CQISCell1 ≥ 2, wideband CQISCell1 – wideband CQISCell2, 3… ≥ 2 for more than 90% of the time.

- Consider test applicability rules listed in Table 2-3

Reference

[1]   R4-2313877  WF for 8Rx UE performance requirements. Huawei, HiSilicon

[2]   R4-2314695  WF on support of 8Rx UE RF. Huawei, HiSilicon