3GPP TSG-RAN WG4 Meeting #108bis-e	  R4-2315996

Xiamen, China, October 9 - 13, 2023

Agenda Item:	5.21.3

Source:	Huawei, HiSilicon

Title:	Discussion on Interoperability and Testability Aspect

Document for:	Discussion and Decision

# Introduction

At RAN4 #106bis-e [1], a new WF on AI/ML RAN4 studies has been achieved.

At RAN4 #107 [2] and #108 [3], some potential test metrics related to interoperability and testability are studied.

In this paper, we continue discussing the remaining issues.

# Discussion

### 2.1 2-sided framework

RAN4 #108 offline discussion: Table for description of 2-sided model testing options The table below should be used to summarize issues to be discussed in describing the options for the testing of 2-sided model. The options in the table are the options for 2-sided model testing as agreed in WF R4-2310433 Issue 3-3 with replacing “reference decoder” by “test decoder”. Interested companies are invited to provide inputs based on the structure of this table in RAN4#108Bis Table below is meant for the UE testing, similar table is needed for testing BS with replacing test decoder by test encoder. The table below is not exhaustive, it can be further updated in the future based on companies’ inputs. Option 1; Option 2; Option 3; Option 4. Clarification of options. Source of the test decoder. Source of decoder training data. DUT vendor knowledge of the test decoder. Supported training collaboration type (source of training data should be consistent with the collaboration type). Test decoder verification procedure at TE and/or DUT. Feasibility of test decoder verification procedure. Pros/Cons analysis. Reflection on the real deployment (knowledge of model, training type, etc.). TE requirements to deploy the decoder (e.g. training, complexity, interoperability). Specification Effort (e.g. test decoder). Confidentiality/IP issues. Applicability to different scenarios/conditions/ configurations. Complexity of actual testing procedure for the ecosystem. Note: training data should be consistent with the collaboration procedure

As discussed in our companion contribution [4], how to test the UE-side model depends on the method of model transfer/delivery mechanism specified by RAN1/2. See the following table from [4].

|  | Testing goal 1: verify that the model is properly conducted | Testing goal 2: verify the performance of the model |
| --- | --- | --- |
| Case a: Model under test is transferred from the opposite side with air-interface signaling | √ | - |
| Case b: Model under test is delivered w/o air interface signaling | - | √ |

In the above table, we notice that it is waiting for RAN1/2 to study whether to specify model transfer with air interface signaling or not. And whether it is testable or not for selected testing goal needs a separate discussion.

In the following, the analysis for the aspect listed in the table is provided, taking account of different model transfer/delivery mechanism.

- Source of test decoder training data

  - It is noticed that there is still no consensus on training data exchange over air interface signaling in RAN1/2. If there is no specified signaling/procedure for training data exchange in WI stage, then the source of decoder training data is spec transparent for options 1 and 2. Determining the source of decoder training data is out of RAN4 scope.

  - However, for options 3 and 4, since the test model is specified or partially specified in RAN4 spec, then the training data is also needed to be captured in RAN4 spec.

- DUT vendor knowledge of the test decoder

  - For options 1 and 3, it is obvious that the DUT has a full knowledge of the test decoder.

  - For option 2, if the test decoder is not specified to be transferred from TE to DUT via dedicated signaling, then there is no way to identify that the DUT knows or not knows the test decoder.  It is noticed that until now, there is no consensus on nominal decoder transfer (for UE-side model training) over the air interface in RAN1/2. Therefore, if during test, the test decoder is transferred from gNB vendor or TE, then it may be a signaling introduced only for testing.

  - For option 4, the RAN4 specified part is known at DUT. However, the RAN4 unspecified part may and may not be known at DUT. It depends on which entity provides the unspecified part, if DUT, then yes. Otherwise, it is out of RAN4 scope.

- Supported training collaboration type

  - For option 1, if it is Type 1 UE-side joint training or Type 3 UE-first sequential training, then UE knows the nominal decoder which matches well with the model at UE. Then the test decoder can be derived by the nominal decoder. Otherwise, if the test decoder is not specified to be transferred from TE to DUT via dedicated signaling, then there is no way to identify that the DUT knows or not knows the test decoder.

  - For option 2, if it is Type 1 NW-side joint training or Type 3 NW-first sequential training, then gNB knows the decoder which matches well with the model at UE. Then the test decoder can be derived by the gNB decoder.

  - For option 3, only if the test decoder can match well with models where the model structure is supported at UE. Otherwise, option 3 is not supported for any training type.

  - For option 4, it depends on which part of the test decoder is specified and which entity provides the unspecified part.

- Test decoder verification procedure at TE and/or DUT

  - For all options, the verification of test decoder is needed. Even for RAN4 specified test decoder, different TE implementations may result in totally different model performance. We noticed that from the open format AI model to the run-time binary image, a sequence of optimization operations is required in order to fit with the target device’s software environment and hardware environment. This is totally different from RAN4 legacy test.

Proposal 1: Table for description of 2-sided model testing options.

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | UE vendor | gNB vendor | RAN4 Spec | RAN4 Spec |
| Source of test decoder training data | Out of RAN4 scope | Out of RAN4 scope | RAN4 Spec | RAN4 Spec |
| DUT vendor knowledge of the test decoder | YES | Out of RAN4 scope | YES | Partially (Whether the unspecified part is known is out of RAN4 scope) |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | At least for - Type 1 UE-side joint training - Type 3 UE-first sequential training | At least for - Type 1 NW-side joint training - Type 3 NW-first sequential training | Only if the test decoder can match well with models where the model structure is supported at UE | Depend on which part of the test decoder is specified and which entity provides the unspecified part |
| Test decoder verification procedure at TE and/or DUT | Need | Need | Need | Need |
| Feasibility of test decoder verification procedure | No consensus | No consensus | No consensus | No consensus |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | It depends on training dataset | It depends on training dataset | It depends on training dataset | It depends on training dataset |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | It depends on the total number of the test model | It depends on the total number of the test model | It depends on the total number of the test model | It depends on the total number of the test model |
| Specification Effort (e.g. test decoder) | - Procedure for verifying the decoder | - Procedure for verifying the decoder | - Align on assumptions for both for model structure and for model parameters - It also depends on whether model structure per use case or per configuration/scenario, and whether model parameters per configuration/scenario | Depend on which part of the test decoder is specified and which entity provides the unspecified part |
| Confidentiality/IP issues | YES (if model exchange between TE vendor and UE vendor) | YES (if model exchange between TE vendor and NW vendor) | NO | NO |
| Applicability to different scenarios/conditions/ configurations |  |  |  |  |
| Complexity of actual testing procedure for the ecosystem |  |  |  |  |

### 2.2 Interoperability aspects

The interoperability should be considered if the model outputs have an impact on the other side, or the other side is involved in model management. For example, for direct AI/ML positioning using UE-side model, there is no interoperability involved. For another example, in AI/ML CSI feedback enhancement, the core requirements of latency/interruption due to model switching at UE as indicated by NW may be needed.

Proposal 2: The interoperability is verified via core requirements and performance requirements.

# Conclusions

Proposal 1: Table for description of 2-sided model testing options.

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | UE vendor | gNB vendor | RAN4 Spec | RAN4 Spec |
| Source of test decoder training data | Out of RAN4 scope | Out of RAN4 scope | RAN4 Spec | RAN4 Spec |
| DUT vendor knowledge of the test decoder | YES | Out of RAN4 scope | YES | Partially (Whether the unspecified part is known is out of RAN4 scope) |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | At least for - Type 1 UE-side joint training - Type 3 UE-first sequential training | At least for - Type 1 NW-side joint training - Type 3 NW-first sequential training | Only if the test decoder can match well with models where the model structure is supported at UE | Depend on which part of the test decoder is specified and which entity provides the unspecified part |
| Test decoder verification procedure at TE and/or DUT | Need | Need | Need | Need |
| Feasibility of test decoder verification procedure | No consensus | No consensus | No consensus | No consensus |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | It depends on training dataset | It depends on training dataset | It depends on training dataset | It depends on training dataset |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | It depends on the total number of the test model | It depends on the total number of the test model | It depends on the total number of the test model | It depends on the total number of the test model |
| Specification Effort (e.g. test decoder) | - Procedure for verifying the decoder | - Procedure for verifying the decoder | - Align on assumptions for both for model structure and for model parameters - It also depends on whether model structure per use case or per configuration/scenario, and whether model parameters per configuration/scenario | Depend on which part of the test decoder is specified and which entity provides the unspecified part |
| Confidentiality/IP issues | YES (if model exchange between TE vendor and UE vendor) | YES (if model exchange between TE vendor and NW vendor) | NO | NO |
| Applicability to different scenarios/conditions/ configurations |  |  |  |  |
| Complexity of actual testing procedure for the ecosystem |  |  |  |  |

Proposal 2: The interoperability is verified via core requirements and performance requirements.

# References

- R4-2306299, “RAN4 #106bis-e WF on AIML RAN4 studies”, 3GPP RAN4 #106bis-e, 17 April –26 April, 2023.

- R4-2310433, “WF on RAN4 requirements for NR AI/ML”, 3GPP RAN4 #107, May 22nd – May 26th, 2023.

- R4-2314740, “WF on RAN4 requirements for NR AI/ML”, 3GPP RAN4 #108, Aug 21nd – Aug 28th, 2023.

- R4-2315994, “General Aspects for RAN4 R-18 SI on AI/ML for NR air interface”, 3GPP RAN4 #108bis, Huawei, Hisilicon, October 9 - 13, 2023.