3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316006

# Xiamen, 9th - 13th Oct, 2023

Title: 	Simulation results on UE CA demodulation requirements for HST FR2

Source: 	Huawei, HiSilicon

Agenda item:	5.12.4.2

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] for FR2 HST demodulation and simulation assumption [2] for FR2 HST demodulation was approved. In this contribution, we provide our updated simulation results about UE CA demodulation requirements for HST FR2.

Simulation

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 9.6 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.2 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.4 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 9.6 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.2 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.4 |

Conclusion

In this contribution, we provide our updated simulation results about UE CA demodulation requirements for HST FR2.

Reference

R4-2313919, WF for FR2 HST demodulation requirements, RAN4#108, Samsung

R4-2313920, Simulation assumption for FR2 HST demodulation, RAN4#108, Ericsson