3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316014

# Xiamen, 9th - 13th Oct, 2023

Title: 	Simulation results on NR BS ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.13.7.3

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we provide our updated simulation results about NR BS ATG demodulation requirements.

Simulation

The ideal simulation results for NR BS ATG demodulation requirements are shown as following Table 2-1.

Table 2-1 Ideal simulation results for NR BS ATG demodulation requirements

| Case number | SCS | Bandwidth | Antenna configuration | Propagation condition | Rank | SNR@70% max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MCS28 | MCS22 (Table2) |
| 1 | 15kHz | 5MHz | 1x2 | AWGN + 200Hz Doppler | 1 | 17.6 | 17.3 |
| 2 | 30kHz | 10MHz | 1x2 | AWGN + 500Hz Doppler | 1 | 16.6 | 17.5 |

Conclusion

In this contribution, we provide our updated simulation results about NR BS ATG demodulation requirements.

Reference

R4-2313875, WF for ATG demodulation requirements, RAN4#108, CMCC

- Appendix: Simulation assumption

Simulation assumption for PUSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 5 | 10 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
| Channel model |  |  | AWGN+200Hz | AWGN+500Hz |
| MCS/Rank |  |  | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 |
| Antenna configuration |  |  | 1T2R | 1T2R |
| Transform precoding |  |  | CP-OFDM |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 |  |
|  | RV sequence |  | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port |  | {0} |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type |  | A |  |
|  | Start symbol |  | 0 |  |
|  | Allocation length |  | 14 |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| Test metric |  |  | 70% of max throughput at target SNR. |  |