3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316016

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	5.7.4.1

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Receiver assumption

- Further evaluate the performance for 2+2 with fully overlapping case with joint processing and sperate processing - Pending on the performance comparison between different receiver assumption, RAN4 plan to decide in 2023 Q4 for this case including receiver assumption. - If test case with joint processing assumption is introduced, it shall be with UE capability as an optional feature.

For the UE processing, there are mainly two assumptions with the difference that UE perform joint processing with 4x4 channel matrix for two TRPs or UE perform separate processing with 2x2 channel matrix per TRP. From our understanding, it is up to UE implementation to select which processing method, however, it is not suitable to use UE joint processing assumption for the FR2 multi-Rx demodulation requirements definition. Firstly, it is more complexity for the UE to perform joint processing comparing to separate processing. Basic processing method should be considered since we are defining the minimum requirements. Secondly, the detailed UE panel deployment is up to UE implementation and it is not possible to perform joint processing in some cases, e.g. when two UE panels are deployed on the left and right side of the UE. Finally, there is also an agreement achieved in RF session that only two TRP to RX pairs TRP#1-RX#1 and TRP#2-RX#2 are considered. The signal from TRP#2 to RX#1 and TRP#1 to RX#2 is treated as interference. It is reasonable for follow RF agreement. Therefore, we propose to only consider UE perform separate processing with 2x2 channel matrix per TRP for FR2 multi-Rx demodulation requirements definition.

Only consider UE perform separate processing with 2x2 channel matrix per TRP for FR2 multi-Rx demodulation requirements definition.

The simulation results are shown as following Table 2.1-1, Table 2.1-2 and Table 2.1-3.

Table 2.1-1 Ideal simulation results for sDCI mTRP SDM

| Case number | MCS | frequency offset[Hz] | timing offset[us] | Rank | UE processing | SNR@70%maximum throughput |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ρ = -15dB | ρ = -12dB | ρ = -9dB | ρ = -6dB |
| 1 | 13 | 600 | -0.0625 | 1+1 | independent | 8.6 | 9.1 | 9.8 | 10.9 |
| 2 | 17 | 600 | -0.0625 | 1+1 | independent | 12.2 | 12.8 | 13.7 | 14.9 |
| 3 | 13 | 600 | -0.0625 | 1+1 | joint | 9.1 | 9.2 | 9.3 | 9.5 |
| 4 | 17 | 600 | -0.0625 | 1+1 | joint | 12.3 | 12.4 | 12.6 | 12.8 |
| 5 | 13 | 600 | -0.0625 | 2+2 | independent | 16.0 | 18.3 | N/A | N/A |
| 6 | 17 | 600 | -0.0625 | 2+2 | independent | 22.5 | N/A | N/A | N/A |
| 7 | 13 | 600 | -0.0625 | 2+2 | joint | 15.3 | 15.5 | 15.9 | 16.4 |
| 8 | 17 | 600 | -0.0625 | 2+2 | joint | 19.0 | 19.4 | 19.9 | 20.6 |
| 9 | 13 | 0 | 0.25 | 1+1 | independent | 8.6 | 9.1 | 9.8 | 10.9 |
| 10 | 17 | 0 | 0.25 | 1+1 | independent | 12.2 | 12.8 | 13.7 | 14.9 |
| 11 | 13 | 0 | 0.25 | 1+1 | joint | 9.1 | 9.2 | 9.3 | 9.5 |
| 12 | 17 | 0 | 0.25 | 1+1 | joint | 12.3 | 12.4 | 12.6 | 12.8 |
| 13 | 13 | 0 | 0.25 | 2+2 | independent | 16.0 | 18.3 | N/A | N/A |
| 14 | 17 | 0 | 0.25 | 2+2 | independent | 22.5 | N/A | N/A | N/A |
| 15 | 13 | 0 | 0.25 | 2+2 | joint | 15.3 | 15.5 | 15.9 | 16.4 |
| 16 | 17 | 0 | 0.25 | 2+2 | joint | 19.0 | 19.4 | 19.9 | 20.6 |

Table 2.1-2 Ideal simulation results for mDCI mTRP fully-overlapping

| Case number | MCS | frequency offset[Hz] | timing offset[us] | Rank | UE processing | SNR@70%maximum throughput |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ρ = -15dB | ρ = -12dB | ρ = -9dB | ρ = -6dB |
| 1 | 13 | 600 | -0.0625 | 1+1 | independent | 8.5 | 8.9 | 9.6 | 10.7 |
| 2 | 17 | 600 | -0.0625 | 1+1 | independent | 11.9 | 12.5 | 13.4 | 14.6 |
| 3 | 13 | 600 | -0.0625 | 1+1 | joint | 9.0 | 9.1 | 9.2 | 9.4 |
| 4 | 17 | 600 | -0.0625 | 1+1 | joint | 12.1 | 12.2 | 12.4 | 12.6 |
| 5 | 13 | 600 | -0.0625 | 2+2 | independent | 16.0 | 18.3 | N/A | N/A |
| 6 | 17 | 600 | -0.0625 | 2+2 | independent | 23.0 | N/A | N/A | N/A |
| 7 | 13 | 600 | -0.0625 | 2+2 | joint | 15.1 | 15.3 | 15.7 | 16.2 |
| 8 | 17 | 600 | -0.0625 | 2+2 | joint | 19.0 | 19.4 | 20.1 | 20.7 |
| 9 | 13 | 0 | 0.25 | 1+1 | independent | 8.5 | 8.9 | 9.6 | 10.7 |
| 10 | 17 | 0 | 0.25 | 1+1 | independent | 11.9 | 12.5 | 13.4 | 14.6 |
| 11 | 13 | 0 | 0.25 | 1+1 | joint | 9.0 | 9.1 | 9.2 | 9.4 |
| 12 | 17 | 0 | 0.25 | 1+1 | joint | 12.1 | 12.2 | 12.4 | 12.6 |
| 13 | 13 | 0 | 0.25 | 2+2 | independent | 16.0 | 18.3 | N/A | N/A |
| 14 | 17 | 0 | 0.25 | 2+2 | independent | 23.0 | N/A | N/A | N/A |
| 15 | 13 | 0 | 0.25 | 2+2 | joint | 15.1 | 15.3 | 15.7 | 16.2 |
| 16 | 17 | 0 | 0.25 | 2+2 | joint | 19.1 | 19.4 | 20.1 | 20.7 |

Table 2.1-3 Ideal simulation results for mDCI mTRP non-overlapping

| Case number | MCS | frequency offset[Hz] | timing offset[us] | Rank | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | 13 | 600 | -0.0625 | 2+2 | 11.0 |
| 2 | 17 | 600 | -0.0625 | 2+2 | 14.6 |

Based on the simulation results, also considering the feasibility for different UE processing, we propose to select the following cases:

Select {MCS17, ρ = -9dB or -6dB, rank 1} and/or {MCS13, ρ = -15dB or -12dB, rank 2} for mDCI fully-overlapping and sDCI SDM cases.

Select {MCS17, ρ = -9dB or -6dB, rank 1} and/or {MCS13, ρ = -15dB or -12dB, rank 2} for sDCI SDM cases.

Select MCS17, rank 2 for mDCI non-overlapping cases.

Proposals

In this contribution, we discuss on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Only consider UE perform separate processing with 2x2 channel matrix per TRP for FR2 multi-Rx demodulation requirements definition.

Select {MCS17, ρ = -9dB or -6dB, rank 1} and/or {MCS13, ρ = -15dB or -12dB, rank 2} for mDCI fully-overlapping and sDCI SDM cases.

Select {MCS17, ρ = -9dB or -6dB, rank 1} and/or {MCS13, ρ = -15dB or -12dB, rank 2} for sDCI SDM cases.

Select MCS17, rank 2 for mDCI non-overlapping cases.

Reference

R4-2313977, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#108, Qualcomm