3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316018

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	5.7.4.3

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception.

Discussion

TDD pattern

<way forward> - Proposals - Option 1: Consider PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 as given by TS 38.101-4 Table A.3.2.2.5-8 with 1.375ms delay. - Option 2: Use DDDSU TDD pattern. - Option 2a: DDDSU TDD pattern with CQI/RI/PMI with CQI/RI/PMI delay of 1.75ms.

For the TDD pattern, we think FR2.120-1 (DDDSU) TDD pattern can be selected since it is more typically used in the real network, and the corresponding CQI/RI/PMI delay should be 1.75ms that is same as the legacy PMI reporting case (i.e. Test 2 in clause 8.3 in TS 38.101-4) with the same TDD pattern.

Select DDDSU TDD pattern with CQI/RI/PMI with CQI/RI/PMI delay of 1.75ms for PMI reporting requirement for sDCI SDM scheme.

Channel model

<way forward> - Proposals - Option 1: Use TDLA30-35 propagation channel for PMI reporting requirement for sDCI SDM scheme.

For the channel model, we propose to select TDLA30-35 that is same as the existing PMI reporting cases.

Select TDLA30-35 for PMI reporting requirement for sDCI SDM scheme.

Antenna Configuration

<way forward> - Proposals - Option 1: Use 2x2 ULA low antenna configuration for PMI reporting requirement for sDCI SDM scheme. - Option 2: 2x4 XP (4 antenna at UE across 2 panels/ 2 Rx at each panel) - Option 3: 2x2 XP

For the antenna configuration, considering that we have achieve the agreements to use 2 port per TRP, it is reasonable to consider 2x2 XP per panel for PMI reporting requirement for sDCI SDM scheme that is same as PDSCH requirements.

Consider 2x2 XP per panel for PMI reporting requirement for sDCI SDM scheme.

Simulation parameters for PMI reporting requirement for sDCI SDM scheme

<way forward> All tables related to this issue can be found in R4-2314258. - Proposals: - Option 1: Consider the parameters in Table 7 to define PMI reporting requirements for sDCI SDM based scheme. - Option 1a: Table 7 with updated number of transmit antennas to 2 - Option 2: Define new reference channel for PMI reporting requirement for sDCI SDM scheme as presented in Table 8. - Option 3: Decide on simulation parameters for PMI reporting for sDCI SDM scheme following alignment of sDCI fundamental in the PDSCH discussion, and all current agreements for PMI shall be conditioned on the outcome of the sDCI concerning inclusion of sDCI test cases in PDSCH. - Option 4: - SCS/CBW: 120KHz/ 100MHz - TDD Pattern: DDDSU (10D+2G+2U) - Propagation channel: TDLA30-35 - Antenna configuration per TRxP: 2x4 XP (4 antenna at UE across 2 panels) - Cross talk levels: -15,-9 dB - Cross correlation coefficient (g): 0.125

From our understanding, the new reference channel should be defined rather than reuse the existing one since at least total two layer (1+1) is expected for the new PMI reporting requirement for sDCI SDM scheme.

Define new reference channel for PMI reporting requirement for sDCI SDM scheme as following:

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | Modified from R.PDSCH.5-7.1 TDD |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 12 |
| Allocated slots per 2 frames |  | 63 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 2 |
| Number of DMRS REs (Note 3) |  | 24 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 28680 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A |
| For Slot i = 80 | CBs | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 57648 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |

For the gamma value, we think it is better to align with the value agreed in PDSCH requirements discussion, i.e. 0.0625.

Select the gamma value 0.0625 for PMI reporting requirement for sDCI SDM scheme.

Proposals

In this contribution, we discuss on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Select DDDSU TDD pattern with CQI/RI/PMI with CQI/RI/PMI delay of 1.75ms for PMI reporting requirement for sDCI SDM scheme.

Select TDLA30-35 for PMI reporting requirement for sDCI SDM scheme.

Consider 2x2 XP per panel for PMI reporting requirement for sDCI SDM scheme.

Define new reference channel for PMI reporting requirement for sDCI SDM scheme as following:

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | Modified from R.PDSCH.5-7.1 TDD |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 12 |
| Allocated slots per 2 frames |  | 63 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 2 |
| Number of DMRS REs (Note 3) |  | 24 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 28680 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A |
| For Slot i = 80 | CBs | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 57648 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |

Select the gamma value 0.0625 for PMI reporting requirement for sDCI SDM scheme.

Reference

R4-2313977, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#108, Qualcomm