3GPP TSG-RAN WG4 Meeting #108bis	R4-2316033

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion on RRM test cases for R18 SL evolution

Source: 	Huawei, HiSilicon

Agenda item:	5.30.4

Document for:	Discussion

Introduction

In RAN4, the impacted RRM core requirements for R18 SL evolution have been identified and discussed. In this contribution, we will discuss the impacts on RRM test cases for R18 SL evolution.

Discussion

Currently, RAN4 has identified that at least the following RRM requirements will be impacted due to R18 SL evolution:

- SL-U operation - UE transmit timing - Initiation/cease of SLSS transmission - Selection/reselection of V2X synchronization reference source - SL CA operation - Interruptions to WAN due to SL CA. - SL component carrier addition/release delay - Selection/reselection of V2X synchronization reference source for SL CA - Co-channel coexistence for LTE SL and NR SL - Applicability rule for solution of TDM-based semi-static resource pool partitioning and dynamic resource sharing

For SL-U operations, the requirements on UE transmit timing and initiation/cease of SLSS transmission are impacted only when SyncRef UE is used as synchronization reference source. For UE transmit timing requirements, the applicable conditions are introduced for SL-U operation. For initiation/cease of SLSS transmission requirements, the evaluation period is extended for SL-U operation. New test cases need to be introduced to verify the UE transmit timing requirements and initiation/cease of SLSS transmission requirements under SL-U operation when SyncRef UE as synchronization reference source. For selection/reselection of V2X synchronization reference source requirements, the SyncRef UE detection time for both sync case and async case are also extended for SL-U operation. Two test cases need to be defined to verify the extended detection time for sync case and async case separately.

Proposal 1: RAN4 to define the following test cases under SL-U operation:

- Test for V2X UE transmit timing with SyncRef UE as synchronization reference source under SL-U

- Test for initiation/cease of SLSS transmission with SyncRef UE as synchronization reference source under SL-U

- Test for V2X synchronization reference selection/reselection with GNSS as the highest priority under SL-U

- Test for V2X synchronization reference selection/reselection with gNB as the highest priority under SL-U

For SL CA operation, RAN4 agreed to define the interruption requirements due to SL carrier addition/release but not to specify the exact interruption location. Since only mode 2 is considered in R18 SL CA operation, the SL carrier addition/release delay is up to UE implementation. In RRM, there is no specific test case for interruption requirements and the interruption is usually verified in relevant delay test case. However, the SL carrier addition/release delay and the interruption location are up to UE implementation, which makes the interruption and delay requirements untestable. So, we suggest not to define the test case for SL carrier addition/release in R18.

For selection/reselection of V2X synchronization reference source, RAN4 agreed to reuse LTE SL CA requirements for NR SL CA, which means that the SyncRef UE detection time will be scaled by the number of SL carriers configured for synchronization when the selected synchronization reference source is not available. In R16/R17, there are two test cases for synchronization reference source selection/reselection including GNSS as the highest priority and gNB as the highest priority. We suggest to choose one of them under SL CA operation and the scenario “GNSS as the highest priority” is suggested, same as LTE SL CA operation.

Proposal 2: RAN4 to define the following test case under SL CA operation:

- Test for V2X synchronization reference selection/reselection with GNSS as the highest priority under  for SL CA

For co-channel coexistence for LTE SL and NR SL, the existing RRM requirements are reused and only applicability rules need to be introduced for clarification. There is no new RRM requirement to be defined. So, no new test case is needed for co-channel coexistence for LTE SL and NR SL in R18.

Proposal 3: RAN4 not to define new test case for co-channel coexistence for LTE SL and NR SL in R18.

Conclusions

In this paper we provided our analysis on the impacts on RRM tests for R18 SL evolution. The followings are provided.

Proposal 1: RAN4 to define the following test cases under SL-U operation:

- Test for V2X UE transmit timing with SyncRef UE as synchronization reference source under SL-U

- Test for initiation/cease of SLSS transmission with SyncRef UE as synchronization reference source under SL-U

- Test for V2X synchronization reference selection/reselection with GNSS as the highest priority under SL-U

- Test for V2X synchronization reference selection/reselection with gNB as the highest priority under SL-U

Proposal 2: RAN4 to define the following test case under SL CA operation:

- Test for V2X synchronization reference selection/reselection with GNSS as the highest priority under  for SL CA

Proposal 3: RAN4 not to define new test case for co-channel coexistence for LTE SL and NR SL in R18.

Reference

- R4-2314349, “WF on R18 NR SL RRM requirements (part 1)”, LG Electronics

- R4-2314350, “WF on R18 NR SL RRM requirements (part 2)”, OPPO