3GPP TSG-RAN WG4 Meeting #108bis	R4-2316081

Xiamen, China, 9 – 13 October 2023

Agenda Item:	5.7.4.2

Source:	Ericsson

Title:	PDSCH demodulation requirements for FR2 UE multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#108 agreed with the way forward on UE demodulation and CSI requirements for FR2 multi-Rx DL reception [1]. This contribution discusses the remaining open issues on the PDSCH requirements with FR2 UE multi-Rx reception scenario.

# 2	Discussion

## 2.1	sDCI-based transmission

### 2.1.1	Simulation results

For the sDCI-based SDM transmission scheme, according to the WF, there are two candidates for evaluation: 1+1 and/or 2+2. Since RAN4 are also discussing the time/frequency offset between two TRPs, we have 4 candidates in total as listed in Table 1.

Table 1	Test case list for sDCI-based SDM transmission schemes for FR2 multi-Rx panel reception.

| Test number | CBW/SCS (MHz/kHz) | MCS per TRP | Rank | Propagation | Antenna configuration per TRP per Rx panel | Cross talk power ratio (ρ) | Time / Frequency offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | MCS17 | 1+1 | TDLA30-75 | 2x2 XP (α=β=0, γ=0.0625) | -6dB | -0.0625us, 600Hz |
| 1-2 | 100/120 | MCS17 | 1+1 | TDLA30-75 | 2x2 XP (α=β=0, γ=0.0625) | -6dB | 0.25us, 0Hz |
| 2-1 | 100/120 | MCS13 | 2+2 | TDLA30-75 | 2x2 XP (α=β=0, γ=0.0625) | -12dB | -0.0625us, 600Hz |
| 2-2 | 100/120 | MCS13 | 2+2 | TDLA30-75 | 2x2 XP (α=β=0, γ=0.0625) | -12dB | 0.25us, 0Hz |

Table 2	Test parameters for sDCI-based SDM transmission schemes for FR2 multi-Rx panel reception.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 for 1+1 1000-1001 for 2+2 | 1002 for 1+1 1002-1003 for 2+2 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

| (a) With PN | (b) Without PN |
| --- | --- |

Figure 1	Simulation result of sDCI-based SDM schemes.

Figure 1 shows our simulation results for sDCI-based SDM schemes, where we evaluated the cases with and without the phase noise with fc=30GHz. The common simulation parameters are listed in Table 2. Table 3 compares the simulation results about the SNR to achieve 70% of the peak rate.

Table 3	Summary of simulation results of sDCI-based SDM scheme.

|  | SNR (dB) to achieve 70% of the peak rate |  |
| --- | --- | --- |
| Test case options | With PN | Without PN |
| 1+1, MCS17, ρ=-6dB, TO/FO=-0.0625us/600Hz | 14.5 | 14.4 |
| 1+1, MCS17, ρ=-6dB, TO/FO=0.25us/0Hz | 14.3 | 14.2 |
| 2+2, MCS13, ρ=-12dB, TO/FO=-0.0625us/600Hz | 16.5 | 16.3 |
| 2+2, MCS13, ρ=-12dB, TO/FO=0.25us/0Hz | 17.2 | 17.0 |

### 2.1.2	PTRS configuration

One of the opens issues on sDCI-based transmission is the PTRS configuration.

- Further evaluate both cases: one PTRS port across TRPs and one PTRS port for each TRP - Companies are encouraged to evaluate the performance impact and decide suitable parameters for each case. - Test applicable rules can be considered based on UE capability.

From the simulation results in Table 3, the degradation due to the phase noise is 0.1-0.2dB. Considering the impairment margin provided by companies and additional margin on top of the average of impairment results, we think the impact of PN to the final PDSCH demodulation requirements is negligible. Moreover, since supporting 2-port DL PT-RS for sDCI-based SDM scheme is optional capability, we propose to configure one PTRS port across TRPs.

Proposal 1: Configure one PTRS port across TRP for sDCI-based SDM scheme.

### 2.1.3	Test cases

Although we have evaluated 4 configuration combinations, RAN4 may reduce the test cases further. Considering the impairment margin and achievable SNR in OTA environment, we propose to consider two test cases. But we are fine to reduce further since RAN4 will also define mDCI-based requirements.

Proposal 2: RAN4 consider the following PDSCH demodulation requirements with sDCI-based SDM scheme.

- 1+1, MCS17, ρ=-6dB, TO/FO=0.25us/0Hz

- 2+2, MCS13, ρ=-12dB, TO/FO=-0.0625us/600Hz

## 2.2	mDCI-based transmission

### 2.2.1	Simulation results

For the mDCI-based transmission schemes, in our understanding, there are three candidates for evaluation, 1+1 with fill-overlapping, 2+2 full-overlapping, and 2+2 non-overlapping, as shown in Table 4.

Table 4	Simulation parameters for mDCI-based transmission schemes for FR2 multi-Rx panel reception.

|  | Full-overlapping, 1+1 | Full-overlapping, 2+2 | Non-overlapping, 2+2 |
| --- | --- | --- | --- |
| UE processing | Option 1: Independent processing Option 2: Joint processing |  | Independent processing |
| CBW/SCS | 100MHz/120kHz | 100MHz/120kHz | 100MHz/120kHz |
| PDSCH resource allocation | 66 per TRP | 66 per TRP | 33 per TRP |
| DMRS ports | TRP1: 1000 TRP2: 1002 | TRP1: 1000,1001 TRP2: 1002,1003 | TRP1: 1000,1001 TRP2: 1002,1003 |
| Propagation | TDLA30-75 | Option 1: TDLA30-75 [Option 2: TDLA30-300] | TDLA30-75 |
| Antenna configuration | 2x2, ULA Low per TRP |  | 2x2, ULA Low per TRP |
| MCS index (MCS table 1) | 64QAM, 0.43 (MCS17) | 16QAM, 0.48 (MCS13) | 16QAM, 0.48 (MCS13) |
| Time offset / Frequency offset | Option 1: (-0.0625us, 600Hz) Option 2: (0.25us, 0Hz) [Option 3: (0.25us, 600Hz)] |  | (-0.0625us, 600Hz) |
| Cross talk power ratio (ρ) | -6 dB | -12 dB | None |
| TDD pattern | DDDSU | DDDSU | DDDSU |

#### 2.2.1.1	mDCI-based full-overlapping 1+1

Figure 2	Simulation results of mDCI-based full-overlapping transmission 1+1, cross talk: -6dB

Figure 2 shows the simulation results of mDCI-based full-overlapping 1+1 scenario. According to the simulation results, it is clearly observed the performance difference between independent processing (Ind Proc.) and joint processing (Joint Proc.) about SNRs to achieve 70% of the peak rate.

Since the inter-TRP cross talk factor comes from the network deployment scenario and UE could have different implementation for FR2 multi-Rx reception, we propose to introduce UE capability for FR2 multi-Rx processing, and define two sets of requirements.

Observation 1: The performance difference between independent processing and joint processing about SNRs to achieve 70% of the peak rate is significant (~2dB) for mDCI-based full-overlapping 1+1 scenario under cross talk factor of -6dB.

Proposal 3: Introduce UE receiver capability for FR2 multi-Rx reception, i.e., independent processing and joint processing, and define two sets of UE demodulation requirements for mDCI-based full-overlapping 1+1 scenario, according to the UE receiver capability.

#### 2.2.1.2	mDCI-based full overlapping 2+2

| (a) TDLA30-35 | (b) TDLA30-300 |
| --- | --- |

Figure 3	Simulation results of mDCI-based full-overlapping transmission 2+2 TDL30-35, cross talk: -12dB.

Figure 3 shows the simulation results of mDCI-based full-overlapping 2+2 scenario. Regarding the channel model options, RAN4 has another WI called FR2 HST enhancements, where RAN4 has already agreed to define the multi-Rx simultaneous reception scenario with very high Doppler shift. Therefore we are fine to focus on TDLA30-35 in this WI.

Proposal 4: Set TDLA30-35 for mDCI-based full-overlapping transmission 2+2.

If we focus on TDLA30-35, unlike 1+1 with cross talk factor of -12dB, we don’t observe the significant performance difference between independent processing and joint processing, because of the lower cross talk impacts. For this scenario, we think a single test case is enough reduce the total number of test cases. If we review the simulation results TO/FO=0.025us/0Hz is a good option.

Proposal 5: Consider single UE demodulation requirements for mDCI-based full-overlapping 2+2 scenario.

#### 2.2.1.3	mDCI-based non-overlapping 1+1

Figure 4	Simulation results of mDCI-based non-overlapping transmission 2+2 TDL30-35.

Figure 4 shows the simulation results of mDCI-based non-overlapping 2+2 scenario. Since PDSCH REs are not overlapped between two TRPs, theoretically we should not expect any performance difference between independent processing and joint processing.

Proposal 6: Consider single UE demodulation requirements for mDCI-based non-overlapping 2+2 scenario.

Regarding the requirements we propose to define the following test case considering the UE capability and minimizing the number of test cases.

Proposal 7: Define the PDSCH demodulation requirements for FR2 multi-Rx reception as follows.

| Test number | Transmission scheme | MCS, Rank | Inter-TRP cross talk factor (ρ) | Time offset, Frequency offset | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | sDCI-based SDM | 1+1, MCS17 | -6dB | (0.25us, 0Hz) |  |
| 2 | sDCI-based SDM | 2+2, MCS13 | -12dB | (-0.0625us, 600Hz) |  |
| 3 | mDCI-based full-overlapping | 1+1, MCS17 | -6dB | (-0.0625us, 600Hz) | Two sets of requirements according to UE receiver capability. |
| 4 | mDCI-based full-overlapping | 2+2, MCS13 | -12dB | (0.025us, 0Hz) |  |
| 5 | mDCI-based non-overlapping | 2+2, MCS13 | N/A | (-0.0625us, 600Hz) | Applicable only when UE does not support full-overlapping in mDCI transmission. |

# 3	Summary

Proposal 1: Configure one PTRS port across TRP for sDCI-based SDM scheme.

Proposal 2: RAN4 consider the following PDSCH demodulation requirements with sDCI-based SDM scheme.

- 1+1, MCS17, ρ=-6dB, TO/FO=0.25us/0Hz

- 2+2, MCS13, ρ=-12dB, TO/FO=-0.0625us/600Hz

Observation 1: The performance difference between independent processing and joint processing about SNRs to achieve 70% of the peak rate is significant (~2dB) for mDCI-based full-overlapping 1+1 scenario under cross talk factor of -6dB.

Proposal 3: Introduce UE receiver capability for FR2 multi-Rx reception, i.e., independent processing and joint processing, and define two sets of UE demodulation requirements for mDCI-based full-overlapping 1+1 scenario, according to the UE receiver capability.

Proposal 4: Set TDLA30-35 for mDCI-based full-overlapping transmission 2+2.

Proposal 5: Consider single UE demodulation requirements for mDCI-based full-overlapping 2+2 scenario.

Proposal 6: Consider single UE demodulation requirements for mDCI-based non-overlapping 2+2 scenario.

Proposal 7: Define the PDSCH demodulation requirements for FR2 multi-Rx reception as follows.

| Test number | Transmission scheme | MCS, Rank | Inter-TRP cross talk factor (ρ) | Time offset, Frequency offset | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | sDCI-based SDM | 1+1, MCS17 | -6dB | (0.25us, 0Hz) |  |
| 2 | sDCI-based SDM | 2+2, MCS13 | -12dB | (-0.0625us, 600Hz) |  |
| 3 | mDCI-based full-overlapping | 1+1, MCS17 | -6dB | (-0.0625us, 600Hz) | Two sets of requirements according to UE receiver capability. |
| 4 | mDCI-based full-overlapping | 2+2, MCS13 | -12dB | (0.025us, 0Hz) |  |
| 5 | mDCI-based non-overlapping | 2+2, MCS13 | N/A | (-0.0625us, 600Hz) | Applicable only when UE does not support full-overlapping in mDCI transmission. |

# References

- R4-2313977, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm.