3GPP TSG-RAN WG4 Meeting #108bis	R4-2316084

Xiamen, China, 9 – 13 October 2023

Agenda Item:	5.14.6.1

Source:	Ericsson

Title:	Discussion on UE demodulation and CSI reporting requirements for NR less than 5MHz

Document for:	Discussion

# 1	Introduction

This contribution discusses our initial views on the UE demodulation and CSI reporting requirements for NR less than 5MHz [1].

# 2	Feature summary

## 2.1	Support 3MHz CBW

RAN1 agreed to introduce new UE feature ‘NR_FR1_lessthan_5MHz_BW’, where gNB operates NR with 5MHz or less channel bandwidth [2]. According to the WID, this feature is applicable only for several FDD bands less than 1GHz (e.g., n100, n106, n26, n28 and n85) and SCS is limited to 15kHz [1]. RAN#101 also agreed a new WI to introduce new frequency bands n31 (DL 462.5 – 467.5 MHz) and n72 (DL 461 – 466 MHz) for NR and these frequency bands support both 3MHz/5MHz CBW [3].

For 3MHz and 5MHz CBW, RAN4 RF has defined the maximum transmission bandwidths with 15 PRBs and 25 PRBs, respectively [4], but the new 3MHz CBW does not have enough PRBs to transmit PBCH and CORESET#0. RAN1 accordingly updated the specification to adjust PBCH/CORESET#0 within the limited PRBs. At the initial cell access, UE does not know PBCH and CORESET#0 transmission bandwidth. RAN4 RF therefore introduced new sync raster (a.k.a. GSCN) to indicate the data transmission bandwidth in TS38.101-1 and TS 38.104.

RAN4 RF also considered to support 12 PRBs for 3MHz and 20 PRBs for 5MHz according to the band n100 FRMCS migration plan. To support the migration scenario, RAN4 RF introduced the dedicated GSCN to support 12/20 PRBs for n100 (See Table 5.4.3.1-3 in [4]). However please note RAN4 RF still keeps the maximum transmission bandwidth for 3MHz/5MHz to 15/25 PRBs (See Table 5.3.2-1 in [3]).

As of Rel-18, RAN4 RF has not specified any CA/DC combinations with the bands with these new channel bandwidths.

## 2.2	PBCH transmission

In Rel-15, PBCH requires 20PRBs. To transmit PBCH within 12/15PRBs for 3MHz, RAN1 agreed that gNB transmits PBCH by puncturing the upper 4PRBs and lower 4PRBs of 20PRBs. UE knows whether PBCH is punctured or not by GSCN at the initial cell search.

## 2.3	CORESET#0 configuration

In Rel-15, CORESET#0 (Control channel resources used for common control channel such as PDCCH scheduling SIB1, RAR or paging) is assumed to configure at least 24 PRBs for FDD SCS=15kHz according to TS 38.213 clause 13. To configure CORESET#0 within 12/15/20PRBs, RAN1 agreed to define new CORESET#0 configuration tables, where CORESET#0 PRBs can be configured to 12 or 24PRBs according to the channel bandwidth. If the available transmission channel bandwidth is less than the configured PRBs, for example, NW configure 24RPBs, but the actual transmission channel bandwidth is 20PRBs, gNB punctures upper 4PRBs to adjust to 20PRBs.

On the other hand, RAN1 does not assume the puncturing for CORESETs other than CORESET#0. This means CORESET used for UE-specific search space is NOT punctured in 3MHz cell.

## 2.4 	Summary

Table 1 summarizes the PBCH/CORESET#0 transmissions for all the transmission bandwidth configurations assumed for NR_FR1_lessthan_5MHz_BW.

Table 1	Summary of PBCH/CORESET#0 transmission for UE supporting NR_FR1_lessthan_5MHz_BW.

| Channel Bandwidth (maximum transmission PRBs with SCS=15kHz) | Maximum available transmission PRBs | PBCH transmission | CORESET#0 transmission | GSCN | Notes |
| --- | --- | --- | --- | --- | --- |
| 3MHz (15 PRBs) | 12 PRBs | 12 PRBs (Puncturing) | No puncturing (Only 12 PRBs can be configured) | 41367 (New) | Applicable only for n100, to address FRMCS migration plans |
| 3MHz (15 PRBs) | 15 PRBs | 12 PRBs (Puncturing) | Punctured to 15 (When 24 PRBs are configured) [No puncturing (When 12 PRBs are configured)] | 26640 – 31634 (New) |  |
| 5MHz (25 PRBs) | 20 PRBs | 20 PRBs (Rel-15 Legacy) | Punctured to 20 PRBs (Only 24 PRBs can be configured) | 41368 (New) | Applicable only for n100, to address FRMCS migration plans |
| 5MHz (25 PRBs) | 25 PRBs | 20 PRBs (Rel-15 Legacy) | No puncturing (Rel-15 Legacy) | Rel-15 Legacy GSCN |  |

Other than PBCH and CORESET#0, in our understating, there is no physical layer design changes due to the feature NR_FR1_lessthan_5MHz_BW.

Observation 1: RAN1/RAN4 added new maximum transmission channel bandwidths with 15 PRBs for 3MHz with FDD SCS=15kHz in FR1 below 1GHz.

Observation 2: As of Rel-18, RAN4 RF does not assume CA/DC with the new 3/5MHz bands.

Observation 3: RAN1 introduced PBCH puncturing to adapt to the new transmission bandwidth of 12/15 PRBs.

Observation 4: RAN1 introduced new CORESET#0 formats and puncturing to adapt to the new transmission bandwidth of 12/15/20 PRBs.

Observation 5: No physical layer channel/signals updates other than PBCH and CORESET#0.

# 3	UE demodulation and CSI reporting requirements

## 3.1	General

RAN4 NR UE demodulation and CSI reporting requirements are usually defined with 10MHz for FDD SCS=15kHz, i.e., 52 PRBs. Since the new feature limits the maximum available transmission bandwidth to 15 or 25 PRBs, RAN4 need to define new UE demodulation and CSI reporting requirements to verify UE supporting NR_FR1_lessthan_5MHz_BW.

Observation 6: The existing NR UE demodulation and CSI reporting requirements for FDD SCS=15kHz are defined based on the 10MHz CBW, that is, 52PRBs.

According to the RAN1 feature list [2], all the UEs supporting NR_FR1_lessthan_5MHz_BW should support feature index 51-x, requiring the support of puncturing PBCHs and new maximum available transmission bandwidths (15/25PRBs). To minimize the simulation work and consider the minimum requirements, we propose to define the new UE demodulation and CSI reporting requirement only with 15 PRBs with FR1 FDD SCS=15kHz.

Proposal 1: Define new UE demodulation and CSI reporting requirements with 15 PRBs with FR1 FDD SCS=15kHz for UE supporting NR_FR1_lessthan_5MHz_BW.

RAN4 defines the UE demodulation and CSI reporting requirements with 2Rx and 4Rx in FR1 except for RedCap. Although the assumed operating bands are less than 1GHz and 4Rx UEs are not mandated for these bands, we propose to define the requirements with both 2Rx and 4Rx to keep the test coverage.

Proposal 2: Define new UE demodulation and CSI reporting requirements with 2Rx and 4Rx for UE supporting NR_FR1_lessthan_5MHz_BW.

## 3.2	 UE demodulation requirements

### 3.2.1	PDSCH demodulation performance

From PDSCH demodulation point of view, the difference from the existing PDSCH demodulation requirements is only the channel bandwidth, which is similar to RedCap, whose maximum channel bandwidth is limited to 20MHz in FR1. To ensure the minimum PDSCH demodulation performance, we propose to define PDSCH demodulation requirements with the following scenario, which is same configuration as Rel-17 RedCap.

Proposal 3: Define PDSCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW by reusing TS 38.101-4 5.2.2.1.17.

- QPSK, 1/3, TDLB100-400, Rank 1, 2Rx/4Rx.

- 16QAM, 0.48, TDLC300-100, Rank 1, 2Rx/4Rx.

- 64QAM, 0.5, TDLA30-10, Rank 2, 2Rx/4Rx.

- 256QAM, 0.82, TDLA30-10, Rank 1, 2Rx/4Rx.

In addition, RAN4 has received LS from RAN plenary that UE speeds up to 500km/h should be targeted for band n100 without impact to RAN1. We therefore propose to define one PDSCH demodulation requirements by reusing HST-972 channel model.

Proposal 4: Define PDSCH demodulation requirements for UE speed 500km/h with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW by reusing TS 38.101-4 Table 5.2.2.1.1-3 Test 1-6.

- 64QAM, 0.43, HST-972, Rank 1, 1Tx, 2Rx/4Rx.

As we discussed above, RAN4 RF only assumes the single carrier operation for NR less than 5MHz, we propose to define PDSCH demodulation requirements with single carrier only, i.e., no PDSCH CA demodulation requirements are defined.

Proposal 5: Not define PDSCH CA demodulation requirements with CBW 15PRBs.

### 3.2.2	PDCCH demodulation performance

As we discussed before, RAN1 introduced CORESET#0 puncturing for Type0-PDCCH CSS sets, however no puncturing is applied for CORESETs used for user-specific search space (USS). Since PDCCH demodulation requirements defined in TS38.101-4 use USS because the TE needs feedback to count the number of HARQ-ACKs to derive PDCCH BLER, RAN4 cannot not define PDCCH demodulation requirements with punctured CORESET#0.

Proposal 6: Not to define PDCCH demodulation requirements with punctured PDCCH for CORESET#0.

According the RAN1 specification, the possible AL (CCE) configurations with 15PRBs are 1, 2, or 4. However RAN5 test setup needs to schedule at least 2 PDCCHs in one slot (one for DL scheduling, another for UL grant), RAN4 effectively only defines AL1 or AL2 for 15PRBs. Considering the existing TS38.101-4, we propose to define PDCCH demodulation requirements with AL2 only.

Proposal 7: Define (non-punctured) PDCCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, 2 symbols, AL2, DCI 1_0 (35 bits for 15 PRBs), TDLA30-10, 2Rx/4Rx.

### 3.2.3	PBCH demodulation performance

One of the impacts due to this feature is the PBCH puncturing as we discussed before. We propose to define the punctured PBCH demodulation requirements with 12 PRBs by reusing the existing test setup. Since this 3MHz cell can be used for initial cell search, RAN4 need to define both the scenarios where SS/PBCH block index is known and unknown.

Proposal 8: Define punctured PBCH demodulation requirements with 12 PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 12PRBs, TDLC300-100, 2Rx/4Rx, SS/BPCH block index is known.

- 12PRBS, TDLC300-100, 2Rx/4Rx, SS/BPCH block index is unknown.

Although we expect the performance degradation due to the PBCH puncturing, RAN4 should also reuse the test metric of PBCH demodulation requirements; that is BLER is calculated over MIB TTI of 80ms.

Proposal 9: Reuse the Rel-15 PBCH demodulation test metric for punctured PBCH demodulation requirements:

- Probability of miss-detection of the PBCH (Pm-bch): 1%

- Pm-bch := 1 – A/B , where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs. The Pm-bch is derived with the assumption UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms).

### 3.2.4	SDR performance

Sustained data rate (SDR) requirements are to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. Since RAN4 has already defined SDR tests for 5MHz CBW (cf. TS 38.101-4 Table 5.5A-4), we think RAN4 should define SDR performance for 3MHz CBW also.

Proposal 10: Apply SDR tests for 3MHz CBW. Update TS 38.101-4 Tables 5.5A-1 and 5.5A-4 to support 3MHz CBW.

## 3.3	CSI reporting requirements

### 3.3.1	CQI reporting performance

RAN4 defines two types of CQI reporting requirements: one for the CQI definition tests in static channel condition, and another for the CQI reporting tests in fading channel condition. Since both tests are important to verify the CQI estimation performance, we propose to define both the CQI reporting requirements for 15 PRBs.

Proposal 11: Define CQI definition test under static channel condition with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, CQI Table 2, 2Tx Rank 2, 2Rx/4Rx, 2 SNR test points.

- Reuse the same metric as Rel-15 CQI definition test in static condition.

Proposal 12: Define CQI reporting test under fading channel condition with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, CQI Table 2, 2Tx Rank 1, TDLA30-5, 2Rx/4Rx, 2 SNR test points.

- Reuse the same metric as Rel-15 CQI reporting test in fading condition.

### 3.3.2	PMI reporting performance

Since the 3MHz CBW is only defined for operating bands less than 1GHz, we think 4 CSI-RS ports are reasonable to define PMI reporting requirements.

Proposal 13: Define PMI reporting test with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRB, Single PMI, Type I, MCS13 (16QAM, 0.48), Rank 1, 4 CSI-RS ports, 4Tx XPOL, TDLA30-5, 2Rx/4Rx.

- Reuse the same metric as Rel-15 4TX PMI reporting requirements.

### 3.3.3	RI reporting performance

Since UE supporting NR_FR1_lessthan_5MHz_BW should support at least 2Rx, it is important to ensure the rank indication performance even if the CBW is 15RPB. We therefore propose to define RI reporting requirements by reusing the exiting RI reporting test framework.

Proposal 14: Define RI reporting test with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, CQI Table 2, SNR=0dB, 2Tx, low antenna correlation, TDLA30-5, 2Rx/4Rx, fixed RI=2 vs. follow RI

- 15PRBs, CQI Table 2, SNR=20dB, 2Tx, low antenna correlation, TDLA30-5, 2Rx/4Rx, fixed RI=1 vs. follow RI

- 15PRBs, CQI Table 2, SNR=20dB, 2Tx, high antenna correlation, TDLA30-5, 2Rx/4Rx, fixed RI=1 vs. follow RI

- Reuse the same metric as Rel-15 RI reporting requirements.

## 3.4	Applicability of requirements

Since we are going to revisit all the demodulation and CSI reporting requirements with 15PRBs, RAN4 should create the requirement applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

Proposal 15: Create the UE demodulation requirement applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

| Supported RX antenna port | Test type |  | Test list |
| --- | --- | --- | --- |
| 2R only | FR1 FDD | PDSCH | TBD |
|  |  | PDCCH | TBD |
|  |  | PBCH | TBD |
|  |  | SDR | TBD |
| 4Rx only or | FR1 FDD | PDSCH | TBD |
| both 2Rx and 4Rx |  | PDCCH | TBD |
|  |  | PBCH | TBD |
|  |  | SDR | TBD |

Proposal 16: Create the CSI reporting requirement applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

| Supported RX antenna port | Test type |  | Test list |
| --- | --- | --- | --- |
| 2R only | FR1 FDD | CQI | TBD |
|  |  | PMI | TBD |
|  |  | RI | TBD |
| 4Rx only or | FR1 FDD | CQI | TBD |
| both 2Rx and 4Rx |  | PMI | TBD |
|  |  | RI | TBD |

# 4	Summary

Observation 1: RAN1/RAN4 added new maximum transmission channel bandwidths with 15 PRBs for 3MHz with FDD SCS=15kHz in FR1 below 1GHz.

Observation 2: As of Rel-18, RAN4 RF does not assume CA/DC with the new 3/5MHz bands.

Observation 3: RAN1 introduced PBCH puncturing to adapt to the new transmission bandwidth of 12/15 PRBs.

Observation 4: RAN1 introduced new CORESET#0 formats and puncturing to adapt to the new transmission bandwidth of 12/15/20 PRBs.

Observation 5: No physical layer channel/signals updates other than PBCH and CORESET#0.

Observation 6: The existing NR UE demodulation and CSI reporting requirements for FDD SCS=15kHz are defined based on the 10MHz CBW, that is, 52PRBs.

Proposal 1: Define new UE demodulation and CSI reporting requirements with 15 PRBs with FR1 FDD SCS=15kHz for UE supporting NR_FR1_lessthan_5MHz_BW.

Proposal 2: Define new UE demodulation and CSI reporting requirements with 2Rx and 4Rx for UE supporting NR_FR1_lessthan_5MHz_BW.

Proposal 3: Define PDSCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW by reusing TS 38.101-4 5.2.2.1.17.

- QPSK, 1/3, TDLB100-400, Rank 1, 2Rx/4Rx.

- 16QAM, 0.48, TDLC300-100, Rank 1, 2Rx/4Rx.

- 64QAM, 0.5, TDLA30-10, Rank 2, 2Rx/4Rx.

- 256QAM, 0.82, TDLA30-10, Rank 1, 2Rx/4Rx.

Proposal 4: Define PDSCH demodulation requirements for UE speed 500km/h with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW by reusing TS 38.101-4 Table 5.2.2.1.1-3 Test 1-6.

- 64QAM, 0.43, HST-972, Rank 1, 1Tx, 2Rx/4Rx.

Proposal 5: Not define PDSCH CA demodulation requirements with CBW 15PRBs.

Proposal 6: Not to define PDCCH demodulation requirements with punctured PDCCH for CORESET#0.

Proposal 7: Define (non-punctured) PDCCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, 2 symbols, AL2, DCI 1_0 (35 bits for 15 PRBs), TDLA30-10, 2Rx/4Rx.

Proposal 8: Define punctured PBCH demodulation requirements with 12 PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 12PRBs, TDLC300-100, 2Rx/4Rx, SS/BPCH block index is known.

- 12PRBS, TDLC300-100, 2Rx/4Rx, SS/BPCH block index is unknown.

Proposal 9: Reuse the Rel-15 PBCH demodulation test metric for punctured PBCH demodulation requirements:

- Probability of miss-detection of the PBCH (Pm-bch): 1%

- Pm-bch := 1 – A/B , where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs. The Pm-bch is derived with the assumption UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms).

Proposal 10: Apply SDR tests for 3MHz CBW. Update TS 38.101-4 Tables 5.5A-1 and 5.5A-4 to support 3MHz CBW.

Proposal 11: Define CQI definition test under static channel condition with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, CQI Table 2, 2Tx Rank 2, 2Rx/4Rx, 2 SNR test points.

- Reuse the same metric as Rel-15 CQI definition test in static condition.

Proposal 12: Define CQI reporting test under fading channel condition with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, CQI Table 2, 2Tx Rank 1, TDLA30-5, 2Rx/4Rx, 2 SNR test points.

- Reuse the same metric as Rel-15 CQI reporting test in fading condition.

Proposal 13: Define PMI reporting test with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRB, Single PMI, Type I, MCS13 (16QAM, 0.48), Rank 1, 4 CSI-RS ports, 4Tx XPOL, TDLA30-5, 2Rx/4Rx.

- Reuse the same metric as Rel-15 4TX PMI reporting requirements.

Proposal 14: Define RI reporting test with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW.

- 15PRBs, CQI Table 2, SNR=0dB, 2Tx, low antenna correlation, TDLA30-5, 2Rx/4Rx, fixed RI=2 vs. follow RI

- 15PRBs, CQI Table 2, SNR=20dB, 2Tx, low antenna correlation, TDLA30-5, 2Rx/4Rx, fixed RI=1 vs. follow RI

- 15PRBs, CQI Table 2, SNR=20dB, 2Tx, high antenna correlation, TDLA30-5, 2Rx/4Rx, fixed RI=1 vs. follow RI

- Reuse the same metric as Rel-15 RI reporting requirements.

Proposal 15: Create the UE demodulation requirement applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

| Supported RX antenna port | Test type |  | Test list |
| --- | --- | --- | --- |
| 2R only | FR1 FDD | PDSCH | TBD |
|  |  | PDCCH | TBD |
|  |  | PBCH | TBD |
|  |  | SDR | TBD |
| 4Rx only or | FR1 FDD | PDSCH | TBD |
| both 2Rx and 4Rx |  | PDCCH | TBD |
|  |  | PBCH | TBD |
|  |  | SDR | TBD |

Proposal 16: Create the CSI reporting requirement applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

| Supported RX antenna port | Test type |  | Test list |
| --- | --- | --- | --- |
| 2R only | FR1 FDD | CQI | TBD |
|  |  | PMI | TBD |
|  |  | RI | TBD |
| 4Rx only or | FR1 FDD | CQI | TBD |
| both 2Rx and 4Rx |  | PMI | TBD |
|  |  | RI | TBD |

In appendix, we summarize all the proposed test cases for UE supporting NR_FR1_lessthan_5MHz_BW.

# References

- RP-231425, “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia.

- R1-2308521, “Updated RAN1 UE features list for Rel-18 NR after RAN1#114”, RAN1.

- RP-232691, “New WID on introduction of NR bands n31 and n72”, Nokia.

- R4-2314764, “CR to 38.101-1 Introduction of dedicated spectrum less than 5MHz for FR1”, Nokia, Ericsson, ZTE Corporation, Apple, Qualcomm Inc, Huawei, Mediatek.

- RR-230781, “Response to LS from RAN4 on spectrum less than 5 MHz”, RAN.

# Appendix (for information)

## A.1	UE demodulation requirements

### PDSCH demodulation

| CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table |
| --- | --- | --- | --- | --- | --- |
| 15PRB / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low 2x4 ULA low | 70% of peak rate | Table 5.2.2.1.17-3 Test 1-1 |
| 15PRB / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low 2x4 ULA low | 70% of peak rate | Table 5.2.2.1.17-3 Test 1-2 |
| 15PRB / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low 2x4 ULA low | 70% of peak rate | Table 5.2.2.1.17-4 Test 2-1 |
| 15PRB / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low 2x4 ULA low | 70% of peak rate | Table 5.2.2.1.17-3 Test 1-3 |
| 15PRB / 15kHz | 64QAM 0.43 | HST-972 | 1x2 1x4 | 70% of peak rate | Table 5.2.2.1.1-3 Test 1-6 |

### PDCCH demodulation

| RB size | duration | AL | Propagation condition | Antenna configuration | DCI | Metric (Pm-dsg) | Reference from TS38.101-4 Table |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 2 | 2 | TDLA30-10 | 1x2 low 1x4 low | 1_0 (35 bits for 15 PRB) | 1% | Table 5.3.2.1.1-1 Test 1 |

### Punctured PBCH demodulation

| RB size / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 15RB / 15kHz | TDLC300-100 | 1x2 low 1x4 low | 1% | Unknown | Table 5.4.2.1-2 Test 1 |
| 15RB / 15kHz | TDLC300-100 | 1x2 low 1x4 low | 1% | Known | Table 5.4.2.1-3 Test 1 |

## A.2	CSI reporting requirements

### CQI reporting

| Reference | CQI test type | RB size / SCS | CQI table | Rank | Antenna configuration | Propagation condition |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 15 RB / 15 kHz | Table 2 | 2 | 2x2 static 2x4 static (Annex B.1 in TS 38.101-4) | Static |
| Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 15 RB / 15 kHz | Table 2 | 1 | 2x2 ULA high 2x4 ULA high | TDLA30-5 |

### PMI reporting

| Reference | PMI test | RB size / SCS | Tx ports | Antenna Configuration | Propagation condition |
| --- | --- | --- | --- | --- | --- |
| Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 15 RB / 15 kHz | 4 | 4x2 XP high 4x4 XP high | TDLA30-5 |

### RI reporting

| Reference | RI configuration | RB size / SCS | CQI table | Antenna Configuration | Propagation condition | SNR test point |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.4.2.1-1 Test 1 | Fix RI=2 and follow RI | 15 RB / 15 kHz | Table 2 | 2x2 ULA low 2x4 ULA low | TDLA30-5 | 0dB |
| Table 6.4.2.1-1 Test 2 | Fix RI=1 and follow RI | 15 RB / 15 kHz | Table 2 | 2x2 ULA low 2x4 ULA low | TDLA30-5 | 20dB |
| Table 6.4.2.1-1 Test 3 | Fix RI=1 and follow RI | 15 RB / 15 kHz | Table 2 | 2x2 ULA high 2x4 ULA high | TDLA30-5 | 20dB |

# B.1	UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-x | Support for 3 MHz channel bandwidth | 1) Reception of 12 RB PBCH based on RB-level puncturing 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 3) Reception of 15 RB and CORESET0 |  | Yes | N/A | UE is not able to support 3 MHz channel bandwidth | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only | Optional with capability signalling |
| 51. NR_FR1_lessthan_5MHz_BW | 51-y | Support 12 RB CORESET0 | 1) Reception of 12 RB CORESET0 | 51-x | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 12 RB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only [This FG is applicable to the case when transmission BW is limited within 12 PRB] | Optional with capability signalling |