3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2316101

Xiamen Meeting, October 9th – October 13th, 2023

Title:	TP to TR 37.718-11-21: Addition of DC_3_n40-n105

Source:	Nokia, Spark

Agenda item:	4.6.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 37.718-11-21 to add DC_3_n40-n105.

************************************* Start of TP*****************************************

## 6.x	DC_3_n40-n105

### 6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n40-n105 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.x.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n40A-n105A | DC_3A_n40A DC_3A_n105A |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n40A-n105A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n40 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n105 |  | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3			Co-existence studies

### This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

### A study of the uplink combination affecting the downlink of the third band is shown in Table 6.x.3-1 for DC_3A_n40A into band n105 downlink.

Table 6.x.3-1: IMD analysis for DC_3A_n40A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2300 | 2400 |
| DL Frequency [MHz] | 1805 | 1880 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 515 | 690 | 4010 | 4185 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1020 | 1270 | 2815 | 3090 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2730 | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1380 | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7430 | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 | 10770 | 9730 | 10155 |

The analysis shows the 2nd and 5th order IMD products fall inside band n105 downlink.

### A study of the uplink combination affecting the downlink of the third band is shown in Table 6.x.3-2 for DC_3A_n105A into band n40 downlink.

Table 6.x.3-2: IMD analysis for DC_3A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 663 | 703 |
| DL Frequency [MHz] | 1805 | 1880 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

The analysis shows the 2nd order IMD product falls inside band n105 downlink.

Based on Table 6.x.3-1 and Table 6.x.3-2:

- The 2nd and 5th order IMD products of DC_3_n40 falls inside band n105 downlink.

- The 2nd order IMD product of DC_3_n105 fall inside band n40 downlink.

### 6.x.4		∆TIB and ∆RIB values

For DC_3_n40-n105, the TIB,c and RIB,c values are reused from CA_n3-n40-n105 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n40-n105 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n40-n105 | - | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.x.5	REFSENS requirements

MSD values are reused from CA_n3A-n40A-n105A.

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3_n40-n105 | 3 | 1745 | 5 | 25 | 1840 | N/A | N/A |
|  | n40 | 2380 | 10 | 50 | 2380 | N/A | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 26.0 | IMD24 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2388 | 10 | 50 | 2388 | 26.0 | IMD2 |
|  | n105 | 668 | 5 | 25 | 617 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

************************************* End of TP******************************************