3GPP TSG-RAN WG4 Meeting # 108bis						R4-2316119

Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.30.2.1

Source: 	OPPO

Title: 	on the SL-U MRP simulation

Document for:	Approval

- Introduction

During the last RAN4 meeting, companies has submitted initial SL-U MPR simulation results. There are some divergence on the simulation results hence some further alignment has been made during the meeting, i.e. the image rejection are set to 34dBc, SEM for wide-band operation has been aligned offline, etc. Hence we provide some further simulation results after the alignment and try to finalize the SL-U MPR requirement.

- Discussion

The updated agreement for SL-U MPR simulation assumption is captured below and the MPR simulation are captured below based on the agreed WF[1] in the last meeting.

Issue 3-1-1: Simulation assumption and cases for PSSCH/PSCCH

- WF

- Waveforms: CP-OFDM for SL-U

- Supported CBW: 20/40/60/80/100MHz.

- Modulation: QPSK/16-QAM/64-QAM/256-QAM

- SCS: 15/30/60kHz

- C-IMD: 60dBc

Moderator note: For the sake of simplicity of simulation, 60kHz is removed at this meeting.

Issue 3-1-2: Image rejection for simulation

- Proposals

  - Option 1: For image rejection, it is proposed to use 30dBc for QPSK/16QAM/64QAM and 34dBc for 256QAM. (OPPO)

  - Option 2: In NR a 28dBc carrier and image is specified but in NR-U, to meet the wideband operation mask of 28dBr outside the allocated sub-band, there are case which would be marginal even without added spectral regrowth. (Skyworks)

Moderator note: Based on the online discussion, it seems 34dBc can be agreed. Furthermore, based on moderator understanding, even the in-band image should not be ignored since the SEM for in-band still apply and need to take the image into account.

- WF

  - To use 34dBc image rejection for QPSK/16QAM/64QAM/256QAM

With the agreement above, the simulation cases are updated with 60kHz deleted. For each case, the QPSK, 16QAM, 64QAM and 256QAM will be simulated.

Table 1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 5 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 6 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 8 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 9 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 10 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 11 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 13 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 14 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation contiguous RB allocation | 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 18 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 23 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band operation Interlaced RB allocation | 32 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 34 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 35 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 39 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 40 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 47 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 48 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Corresponding MPR results are shown as below for case 1 to 14:

Table 2 MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.4 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.4 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.1 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 3 MPR for Wideband operation

|  | Bitmap | 10 | 100 | 1000 | 10000 | 010 | 0100 | 01000 | 00100 | 110 | 1100 | 11000 | 0110 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 1, Contiguous and Interlaced Single CC MPR

Figure 2, Contiguous and Interlaced Wideband MPR

Some observation can be made when considering these simulation results.

Firstly. for full RB allocation, the MPR for QPSK, 16QAM and 64QAM are quite stable. For different simulation cases the numbers are within 0.5dB difference and the largest number for each modulation is 3.2dB for QPSK; 3.7dB for 16QAM and 5.2dB for 64QAM. The above numbers apply for both the single CC and wideband cases.

Observation 1: The MPR for QPSK, 16QAM and 64QAM for different simulation cases the numbers are within 0.5dB difference.

Observation 2: The largest number of MPR for each modulation is 3.2dB for QPSK; 3.7dB for 16QAM and 5.2dB for 64QAM and they apply for both the single CC and wideband cases.

From NR-U requirement perspective, the NR-U MPR requirement has been set as shown below captured from TS 38.101-1 while only the CP-OFDM part has been captured.

Table 4: NR-U MPR requirement

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |

| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 |
| --- | --- | --- | --- | --- |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |  |

In such cases, the full RB allocation limit as 3.5dB for QPSK; 4.0dB for 16QAM and 5.5dB for 64QAM are quite near to our simulation results when considering the observation 2. In this case, it is proposed to re-use the full RB allocation MPR for NR-U to SL-U for QPSK, 16QAM and 64QAM.

Proposal 1: It is proposed to re-use the full RB allocation MPR for NR-U to SL-U for QPSK, 16QAM and 64QAM.

However, for 256QAM, the MPR simulation result for single CC full RB allocation are also quite aligned for different cases and the largest number is 8.8. In this case, we propose 9dB for single CC full RB allocation cases. Together consider the full RB allocation for wideband operation, it is observed that for case 15 to case 31, the 256QAM result vary a little from 7.3dB to 8.5dB. In this case, two limits can be set with 8dB MPR limit and 9dB MPR limit. The sub-band configuration exceptions that currently the NR-U requirement is differentiated can apply at this case.

Observation 3: For 256QAM, the MPR simulation result vary between 1. dB from 7.3 to 8.5dB.

Proposal 2: It is proposed to set two limit as 8dB and 9dB MPR for 256QAM full RB allocation.

Proposal 3: The 8dB limit can be used with bitmap exception method that the NR-U apply.

In current TS 38.101-1, the full and partial are defined besides the full RB and interlaced RB allocation together with the wide-band operation bit-maps. Table 5 below captures the exception MPR mapping for wide-band operation of different bit-maps.

Table 5: Exception MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | None |
| 80 | 1100, 0011, 0100, 0010 |
| 100 | 00111, 11100, 00011, 11000 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Void. |  |

The case 15, 16, 17, 19, 20, 22, 24, 25, 26, 28, 29 can apply 8dB limit while the rest of the cases should apply 9dB limit.

So the exception bitmap can be listed as below:

Table 6 exception cases mapping to bitmap

| 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
| --- | --- | --- | --- | --- |
| 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
| 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
| 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
| 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
| 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
| 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
| 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
| 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
| 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
| 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |

The Exception MPR mapping for wideband operation is proposed as below:

Proposal 4: The exception bitmaps are proposed as:

Table 7: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | 110, 011 |
| 80 | 1000, 1100, 0100, 0010, 0011, 0001 |
| 100 | 10000, 11000, 11100, 01000, 01100, 00110, 00010, 00111, 00011, 00001 |

Secondly. for Inter-laced RB allocation, similar to full RB allocation, for single CC, MPR for QPSK, 16QAM and 64QAM are quite stable. For different simulation cases the numbers are within 0.5dB difference. A table 8 below shows the comparison between single cc and wide-band operation.

Table 8 largest simulation results for single CC and Wideband

|  | Single CC Interlaced | Wide band Interlaced | NR-U limit |
| --- | --- | --- | --- |
| QPSK | 3.4 | 2.8 | 3.5 |
| 16QAM | 3.4 | 3.0 | 4.0 |
| 64QAM | 4.4 | 4.2 | 5.5 |

The difference between single CC and wideband is at most 0.6dB. Also the NR-U limit covers the simulation results with around 1dB margin.

Observation 4: For inter-laced RB allocation, the simulation result for single CC and wide-band are near and the NR-U limit can cover with 1dB margin.

In this case, we believe to re-use the NR-U MPR for SL-U for QPSK, 16QAM, 64QAM for interlaced RB allocation are quite reasonable.

Proposal 5: It is proposed to re-use the Inter-laced RB allocation MPR for NR-U to SL-U for QPSK, 16QAM and 64QAM.

For interlaced RB allocation 256QAM, the simulation result for single CC with largest number as 7.9dB and for wide-band with largest number as 6.8dB. For the exception defined in proposal 3 and 4, 8dB MPR limit is proposed and hence it can be used for inter-laced RB allocation MPR.

Proposal 6: For interlaced RB allocation, use 8dB MPR for 256QAM for SL-U.

With the above proposals, the SL-U MPR is proposed to be as below:

Proposal 7: The SL-U MPR table is as below:

Table 9: NR-U MPR requirement

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |

| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
| --- | --- | --- | --- |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 8.0 |

3	Conclusions

In this contribution, we give initial discussion on the sidelink evolution and the observation and proposals are shown as below:

Observation 1: The MPR for QPSK, 16QAM and 64QAM for different simulation cases the numbers are within 0.5dB difference.

Observation 2: The largest number of MPR for each modulation is 3.2dB for QPSK; 3.7dB for 16QAM and 5.2dB for 64QAM and they apply for both the single CC and wideband cases.

Observation 3: For 256QAM, the MPR simulation result vary between 1. dB from 7.3 to 8.5dB.

Observation 4: For inter-laced RB allocation, the simulation result for single CC and wide-band are near and the NR-U limit can cover with 1dB margin.

Proposal 1: It is proposed to re-use the full RB allocation MPR for NR-U to SL-U for QPSK, 16QAM and 64QAM.

Proposal 2: It is proposed to set two limit as 8dB and 9dB MPR for 256QAM full RB allocation.

Proposal 3: The 8dB limit can be used with bitmap exception method that the NR-U apply.

Proposal 4: The exception bitmaps are proposed as:

Table 7: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | 110, 011 |
| 80 | 1000, 1100, 0100, 0010, 0011, 0001 |
| 100 | 10000, 11000, 11100, 01000, 01100, 00110, 00010, 00111, 00011, 00001 |

Proposal 5: It is proposed to re-use the Inter-laced RB allocation MPR for NR-U to SL-U for QPSK, 16QAM and 64QAM.

Proposal 6: For interlaced RB allocation, use 8dB MPR for 256QAM for SL-U.

Proposal 7: The SL-U MPR table is as below:

Table 9: NR-U MPR requirement

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |

| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
| --- | --- | --- | --- |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 8.0 |

4 References

[1] R4-2314737		WF on NR_SL_enh2_UERF_part1, OPPO