3GPP TSG-RAN WG4 Meeting # 108bis													R4-2316133

# Xiamen, China, October 09 – October 13, 2023

Source: 	OPPO

Title: 	TP to draft TR for IoT NTN bands

Agenda Item:	6.4.3

Document for:	Approval

Introduction

In the RAN4 #108 meeting, there are some agreements for IoT NTN bands and this TP is to capture the agreed requirements into the draft TR. Also the out of band blocking requirement has been discussed in this meeting and we try to capture the agreement in this TP.

# References

[1] R4-2314716	WF on IoT_NTN_FDD_LS_band, MediaTek Inc.

# Text proposal

## <<Start of Change >>

- 6	FDD band B254 (L+S band)

- 6.1	Regulation

Existing regulatory information concerning regulatory rules applicable to this NTN L-/S-band can be found in Clause 5 in [2].

- 6.2	UE requirements

- 6.2.1	UE transmitter characteristics

6.2.1.1	Maximum output power for category M1 and NB1/NB2

Table 6.2.1.1-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 6.2.1.1-2: UE Power Class for category NB1/NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 254 | 23 | +/-2 | 20 | +/-2 |

6.2.1.2	Emission requirements and NS values

- 6.2.1.2.1         Spurious emission for category M1

Table 6.2.1.2.1-1: Requirements for spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 254 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n54, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.1.1-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.2.1.3	Configured TX power

It is agreed the current TS 36.102 requirements can be re-used for band n254.

6.2.1.4	Power control

It is agreed the current TS 36.102 requirements can be re-used for band n254.

6.2.1.5	Frequency error

It is agreed the current TS 36.102 requirements can be re-used for band n254.

6.2.1.6	Transmit modulation quality

It is agreed the current TS 36.102 requirements can be re-used for band n254.

- 6.2.2	UE receiver characteristics

- 6.2.2.1	Reference sensitivity for category M1 and NB1/NB2

Table 6.2.2.1-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 6.2.2.1-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

Table 6.2.2.1-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 254 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

Table 6.2.2.1-4: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

- 6.2.2.2	Blocking requirements

For Out-of-Band Blocking requirement, it is agreed to reuse the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, for band b254 category M1 and NB1/NB2.

## <<End of Change>>