3GPP TSG-RAN WG4 Meeting # 108-bis                               R4-2316138

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.8.2.1

Source:	China Telecom

Title:	Discussion on enhancement for FR2 SCell activation

Document for:	Discussion

- Introduction

In RAN4 #108 meeting, FR2 SCell activation delay reduction solutions were discussed further and the WF [1] was approved.

In this paper, we present our views about enhancement for FR2 SCell activation issues captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 2-1-1: delay requirement or time margin for “L3 measurement reporting after SCell activation command”? Agreements: - Define delay requirement or time margin for L3 measurement reporting after SCell activation command: - UE shall report L3-RSRP report with SSB index within [Y]ms after receiving SCell activation command MAC CE if the UL grant is available. - UE is not required to report L3-RSRP report after exceeding [Y]ms - Where Y= THARQ + 3ms + [M]ms, M is FFS.

In last meeting, it was agreed to define delay requirement or time margin for L3 measurement reporting after SCell activation command, UE shall report L3-RSRP report with SSB index within [Y]ms after receiving SCell activation command MAC CE if the UL grant is available, while UE is not required to report L3-RSRP report after exceeding [Y]ms, where Y= THARQ + 3ms + [M]ms, M is FFS. In our views, M value shall be less than the delay of L3 part to get the substantial benefits from L3 measurement reporting enhancement.

Proposal 1: M value shall be less than the delay of L3 part to get the substantial benefits from L3 measurement reporting enhancement.

Status in the WF [1]:

Issue 2-1-4: Condition the UE shall trigger the L3 report when receiving SCell activation command - FFS: - Option 1 (Nokia, Huawei): RAN4 to discuss in what condition the UE shall trigger the L3 report when receiving SCell activation command to make the solution testable. - Option 1a (Huawei): following aspects shall be considered in option 1: - MeasCycleSCell - DRX cycle - CSSF - Option 2: No need to add side condition in core requirement, but setup testing environment (e.g., good SNR condition and sufficient time period) in the test case for UE to get valid L3 measurement result to report.

For condition the UE shall trigger the L3 report when receiving SCell activation command, in our views, since it has been agreed to define the delay requirement or time margin for L3 measurement reporting after SCell activation command, the L3 measurement reporting can be testable. Furthermore, it was agreed that the report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133. Therefore, we prefer to setup testing environment in the test case for UE to get valid L3 measurement result to report.

Proposal 2: Setup testing environment in the test case for UE to get valid L3 measurement result to report.

- Conclusion

The contribution presents our views on the solutions of unknown SCell activation delay reduction, with the following proposals:

Proposal 1: M value shall be less than the delay of L3 part to get the substantial benefits from L3 measurement reporting enhancement.

Proposal 2: Setup testing environment in the test case for UE to get valid L3 measurement result to report.

- References

- R4-2314336, WF on NR RRM enhancements – FR2 SCell activation, Apple, RAN4 #108, August 2023.