3GPP TSG-RAN WG4 Meeting # 108-bis                               R4-2316139

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.8.2.2

Source:	China Telecom

Title:	Discussion on other enhancements for FR2 SCell activation

Document for:	Discussion

- Introduction

In RAN4 #108 meeting, FR2 SCell activation delay reduction solutions were discussed further and the WF [1] was approved.

In this paper, we present our views about UE capability and other enhancements for FR2 SCell activation issues captured in the WF [1].

- Discussion

UE capability and others

Status in the WF [1]:

Issue 4-1-1: UE capability for FR2 unknown SCell activation enhancement - FFS: - to introduce three UE capabilities for FR2 unknown SCell activation enhancement: - (1) One L3 measurement enhancement capability for unknown SCell activation, to indicate supporting the new L3 report after SCell activation command and using SSB periodicity instead of SMTC - (2) Two beam sweeping factor reduction capability for L3 and L1, respectively, to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

As for UE capability for FR2 unknown SCell activation enhancement, we support to introduce L3 measurement enhancement capability and beam sweeping factor reduction capability.

For L3 measurement enhancement capability, the solution of new L3 report after SCell activation command and the solution of using SSB periodicity instead of SMTC are two different solutions, if L3 measurement result is reported after SCell activation command, in our understanding, it’s no need to consider using SSB periodicity instead of SMTC because L3 part and L1 part would be skipped. Therefore, for L3 measurement enhancement capability, two separate UE capabilities for the solution of new L3 report after SCell activation command and the solution of using SSB periodicity instead of SMTC may be more suitable.

For beam sweeping factor reduction capability, two beam sweeping factor reduction capability for L3 and L1 can be introduced respectively to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

Proposal 1: Introduce L3 measurement enhancement UE capability and beam sweeping factor reduction UE capability.

Observation 1: The solution of new L3 report after SCell activation command and the solution of using SSB periodicity instead of SMTC are two different solutions, if L3 measurement result is reported after SCell activation command, it’s no need to consider using SSB periodicity instead of SMTC because L3 part and L1 part would be skipped.

Proposal 2: For L3 measurement enhancement capability, two separate UE capabilities for the solution of new L3 report after SCell activation command and the solution of using SSB periodicity instead of SMTC may be more suitable.

Proposal 3: For beam sweeping factor reduction capability, two beam sweeping factor reduction capability for L3 and L1 can be introduced respectively to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

Status in the WF [1]:

Issue 4-2-3: FR1 unknown SCell activation enhancement - FFS: - the enhancement with L3 measurement report after SCell activation command can be used for FR1 SCell unknown activation delay. - with L3 measurement report after SCell activation command, the L3 part (Trs) and L1 part (TL1-RSRP,measure + TL1-RSRP,report) can be removed from FR1 SCell activation delay requirements. - if it is agreed to use SSB periodicity instead of SMTC periodicity for FR2 SSB periodicity instead of SMTC periodicity, this can also be used for FR1 SCell activation delay.

In our views, the enhancement with L3 measurement report after SCell activation command can be extended to FR1 and can be determined when FR2 case is concluded. When L3 measurement report after SCell activation command, L3 part and L1 measurement for FR1 SCell activation can be skipped which is similar with FR2 case.

Proposal 4: The enhancement with L3 measurement report after SCell activation command can be extended to FR1 and can be determined when FR2 case is concluded.

Proposal 5: When L3 measurement report after SCell activation command, L3 part and L1 measurement for FR1 SCell activation can be skipped which is similar with FR2 case.

- Conclusion

The contribution presents our views on the solutions of unknown SCell activation delay reduction, with the following observation and proposals:

Proposal 1: Introduce L3 measurement enhancement UE capability and beam sweeping factor reduction UE capability.

Observation 1: The solution of new L3 report after SCell activation command and the solution of using SSB periodicity instead of SMTC are two different solutions, if L3 measurement result is reported after SCell activation command, it’s no need to consider using SSB periodicity instead of SMTC because L3 part and L1 part would be skipped.

Proposal 2: For L3 measurement enhancement capability, two separate UE capabilities for the solution of new L3 report after SCell activation command and the solution of using SSB periodicity instead of SMTC may be more suitable.

Proposal 3: For beam sweeping factor reduction capability, two beam sweeping factor reduction capability for L3 and L1 can be introduced respectively to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

Proposal 4: The enhancement with L3 measurement report after SCell activation command can be extended to FR1 and can be determined when FR2 case is concluded.

Proposal 5: When L3 measurement report after SCell activation command, L3 part and L1 measurement for FR1 SCell activation can be skipped which is similar with FR2 case.

- References

- R4-2314336, WF on NR RRM enhancements – FR2 SCell activation, Apple, RAN4 #108, August 2023.