3GPP TSG-RAN WG4 Meeting #108bis              	R4-2316152

Xiamen, China, 9th Oct-13th Oct, 2023

Agenda item:	5.29.4.2

Source:	Samsung

Title:	View on BS demodulation requirements for Rel-18 FeMIMO

Document for:	Discussion

- Introduction

In RAN#94-e meeting, the WI on MIMO Evolution for Downlink and Uplink was approved. The related objective of this WI was presented as following [1]

- CSI enhancement - Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 - Multi-TRP enhancement - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed - Reference signal enhancement - Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS - Enhanced uplink transmission - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will ve identified - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC..

According to the meeting agenda, this meeting is the first meeting to discuss the performance part of FeMIMO WI. As presented in our accompany contributions [2], the overall RAN4 performance scope was analyzed with potential impact on BS demod:

| Objectives | BS demodulation |
| --- | --- |
| Objective 1:CSI enhancement for high/medium UE velocities | No impact |
| Objective 2: Unified TCI enhancement on multi-TRP use case | No impact |
| Objective 3: DMRS enhancement for downlink and uplink MU-MIMO | FFS on PUSCH requirement (FR1 only) |
| Objective 4: CSI enhancement for Coherent-JT | No impact |
| Objective 5: Uplink 8Tx transmision | Not consider in Rel-18 |
| Objective 6: FR2 STxMP for non-handheld UE | FFS on PUSCH/PUCCH requirements |
| Objective 7: Uplink enhancement for Multi-TRP operation | No impact |

In this contribution, the view on test setup of BS demodulation requirement for FeMIMO if the related requirement agreed to introduce is provided.

# 2	Test scope of BS demodulation requirement

In this section, the detail analysis for test scope of BS demodulation for FeMIMO is provided.

## 2.1	PUSCH requirement for DMRS enhancement

Compared with Rel-15, the maximum number of enhanced DMRS ports in Rel-18 is doubled to support MU-MIMO scenario. To enable new DMRS patten without increasing the DM-RS overhead, the following enhanced FD-OCC length was agreed in RAN1#110b-e meeting as following

Agreement For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group

Meanwhile, the related FD-OCC sequence was agreed as

Agreement For FD-OCC length 4 for PDSCH/PUSCH, select the following: - Opt.1-1 (Walsh matrix) for PDSCH - Opt.1-2 (Cyclic shift) for PUSCH FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +j; -1; -j. 3; +1; -j; -1; +j.

Compared with Rel-15 type 1 DMRS, new FD-OCC length and new FD-OCC sequence have been introduced. It will have impact on baseband processing for channel estimation to derive the channel information per each DMRS port, which is different with FD-OCC length of 2.

Observation 1: New FD-OCC length 4 will have impact on the baseband processing for channel estimation to derive the channel information per each DMRS port.

Meanwhile, how to indicate the port combination, the following was agreed in RAN1#111 meeting

Agreement - For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated: - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11) - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23) - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case, - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE. Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0. 1; 1; 1. 2; 2; 0. 3; 2; 1. 4; 2; 2. 5; 2; 3. 6; 1; 8. 7; 1; 9. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12-15; Reserved; Reserved. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0,1. 1; 2; 0,1. 2; 2; 2,3. 3; 2; 0,2. 4; 1; 8,9. 5; 2; 8,9. 6; 2; 10,11. [7]; [2]; [8,10]. 8-15; Reserved; Reserved. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-2. [1]; [2]; [8-10]. 2; 1; 0,1,8. 3; 2; 0,1,8. 4; 2; 2,3,10. 5-15; Reserved; Reserved. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-3. [1]; [2]; [8-11]. 2; 1; 0,1,8,9. 3; 2; 0,1,8,9. 4; 2; 2,3,10,11. 5-15; Reserved; Reserved.

Based on the above RAN1 agreement, the legacy port indexes and new port indexed can be multiplexed within the same CDM group, e.g., up to 4 ports from {0,1, 8,9} for type 1 with maxlength =1, where only one CDM group is used per UE

Observation 2: The legacy port indexes and new port indexed can be multiplexed within the same CDM group, up to 4 ranks, only one CDM group is used.

In current BS demodulation requirement, RAN 4 has introduced PUSCH requirements with 1Tx, 2Tx and 4Tx based on Rel-15 DMRS pattern with FD-OCC length 2, where one CDM group is configured for 1 Tx and 2Tx, and 2 CDM group is configured for 4Tx, separately.

As analyzed above, due to the new FD-OCC length and new FD-OCC sequence introduced, there is a difference baseband processing for channel estimation, and difference performance foreseen.

For 1 Tx and 2Tx based on Rel-18 DMRS pattern, only one CDM is configured. From performance requirement perspective, the performance is not expected to be different, if the channel frequency selectivity is low.

For 4Tx based on Rel-18 DMRS pattern, in case the legacy port indexes and new port indexed multiplexed within the same CDM group. e.g., {0,1,8,9}, only one CDM requirement is configured. From baseband processing and performance perspective, there will be different compared with 2 CDM group.

In summary, from both baseband processing and performance perspective, it is necessary to introduce PUSCH demodulation requirement with new DMRS pattern for functionality verification.

For 1Tx and 2Tx, considering there is minor performance for 1Tx and 2Tx, and reduce the simulation effort, the existing requirement can be applied for PUSCH with Rel-18 DMRS pattern configured with ports {0} and {0,1}, separately

For 4Tx, new additional PUSCH requirement with DMRS port {0,1,8,9} could be introduced.

Proposal 1: PUSCH requirement with Rel-18 DMRS pattern to support less than 4 layers could be introduced

- Reuse the existing requirement for PUSCH with 1Tx and 2Tx, with DMRS port {0} and DMRS ports {0, 1}, separately

- New additional PUSCH requirement with 4Tx, with DMRS ports {0,1,8,9}

For more than 4 layers SU-MIMO, it also related with uplink 8Tx transmission. It is mainly targeting CPE/FWA/vehicle/Industrial devices to further improve the UL performance, where the corresponding physical layer enhancement is designed to support UL 8Tx transmission, such as 2CW, UCI multiplexing and 8Tx codebook. Given these enhancement RAN1 feature, the related baseband performance should be introduced to verify basic functionality of UL 8Tx.

As our preference in [], given RF session have already concluded to postpone the related discussion to later release in RAN4 from RAN4 RF core requirements perspective, to be consistent with RF conclusion, performance requirements also need to be postponed to future release. Therefore, there is no PUSCH requirements introduced to support more than 4 layers SU-MIMO with new enhanced DMRS pattern in Rel-18

Proposal 2: No PUSCH requirement introduced to support more than 4 layers SU-MIMO with new enhanced DMRS patten in Rel-18.

## 2.2	BS requirement for FR2 STxMP

Compared with Rel-17 UL panel selection, CPE/FWA/vehicle/Industrial devices can benefit from higher UL coverage and average throughput with simultaneous UL multi-panel transmission. Uplink multi-panel transmission was introduced based per TRP per uplink panel basis in Rel-18.

To support UL multi -panel transmission, different UL transmission schemes were considered for evaluation in RAN1, similar as DL transmission in Rel-16 eMIMO WI, including both single-DCI based multiple TRP and multi-DCI based multiple TRP for both PUSCH and PUCCH.

Cross-link interference between 2 panels

In Rel-18 multi-Rx WI, the cross-link interference between 2 panels is modeling considering different cross-talk power ratio for evaluation, the target scenario is handheld UE. For Rel-18 FeMIMO WI, UE with two uplink panels will transmission signal simultaneously per TRP, whether to model cross-link should be further discussed. Since the FR2 STxMP is targeting CPE/FWA, instead of handled UE, it should have good isolation between 2 panels, considering there is no limitation on size. Therefore, we think cross-link power ratio should be minor, Similar as FR2 HST, targeting PC6 UE, RAN4 has assumed that no interference modeling between 2 panels. Therefore, we think it can be considered as a starting point, assuming no cross-link interference between 2 panels. While, we are also open to further discuss whether the cross-link interference between 2 panels is needed

Proposal 3: FFS on whether to model cross-link interference between 2 panels. Assuming no cross-link interference targeting CPE/FWA with UL multi-panel transmission as a starting point.

Receiver assumption

Both joint processing and separately processing receiver assumption were under discussion in Rel-18 multi-Rx WI. Joint processing requires high complexity, better performance achieved is expected considering the layer interference can be regarded as signal, while separately processing require less complexity, and suffer the performance degradation due to interference. Since the uplink multi-panel transmission was introduced based on TRP per uplink panel basis. Different with DL, multi TRPs are deployed in non-collocated manner, with joint processing, the large overhead and data transfer among multi-TRP, which is not a practical scenario with high complexity. Therefore, we think separately processing per each TRP may be more suitable, should be taken as baseline receiver assumption.

Proposal 4: Take separately processing as a baseline receiver assumption per TRP for Uplink multi-panel transmission

Scope of PUSCH

The following transmission schemes for single DCI-based was agreed in RAN1, where the detail

Agreement Confirm the working assumption with the following updates: Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with one CW Agreement Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18.

As illustrated in Fig.1, two (or two sets of) layers of PUSCH with two beams are transmitted from two UE panels towards two TRPs. In addition, we think SFN based MTRP PUSCH can be considered equally

Fig.1 Single DCI based on MTRP PUSCH with SDM and SFN scheme

- For SDM scheme:  different layers/DMRS ports of one PUSCH are separately precoded and transmitted from different UE panels simultaneously. This method can exploit spatial multiplexing as the different layers are transmitted from different panels. This method can provide efficient resource utilization and increase the UE uplink throughput.

- For SFN-based transmission scheme: all the same layer/DMRS ports of one PUSCH are transmitted from different UE panels simultaneously. This method can improve the reliability of transmission since the TB can still be decoded when beam blockage occurs in one link

For SDM scheme, UE will transmit the different layer of one PUSCH with across 2 panels on the same time-frequency resource. According to RAN1 agreement, only 1 CW is supported for single-DCI based multi-TRP, which means that different layers is generated from 1 CW, it requires two TRPs to combine different layers LLR information into one decoder. Different with existing single panel for multi-layer transmission, there is no interference among layers transmitted from different UE panels. Therefore, the UE uplink throughput expected to be increased, Therefore, the related PUSCH requirement could be considered

Proposal 5: PUSCH requirement with SDM scheme could be introduced

For SFN scheme, UE will transmit the same layer(s) one PUSCH with across 2 panels on the same time-frequency resource. Since there is no interference assumption across 2 panels, so, the TB can still be decoded when beam blockage occurs in one link, which can improve the reliability of transmission.  Pending on whether combination the signal across TRPs required, it is FFS on whether PUSCH requirement with SFN scheme could be introduced.

Proposal 6: Pending on whether combination across TRPs required, it is FFS on whether PUSCH requirement with SFN scheme could be introduced

The following transmission schemes for multi- DCI-based was agreed in RAN1,

Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2.

As illustrated in Fig.2, two separate PUSCHs with two beams are transmitted from two UE panels towards two TRPs.

Fig.2 multi- DCI based on MTRP PUSCH

In multi-DCI based multi-TRP transmission, each TRP schedules the DL and UL transmission separately through its own PDCCH transmissions. In Rel-15/16, it is required that PUSCH 1 and PUSCH 2 are not overlapped in time domain. However, for a UE capable of simultaneous multi-panel transmissions, it is feasible to schedule PUSCH 1 and PUSCH 2 in resources that are overlapped in time domain. As agreed, the following resource allocation can be supported for multi-DCI based PUSCH transmission.

Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain

Since two independent PUSCHs associated with different TRPs transmitted by UE for multi-DCI, where separately processing for each PUSCH is required. If there is no interference assumption across 2 panels, the multi-DCI based STxMP PUSCH could be verified based on existing PUSCH requirement in FR2

Proposal 7: No PUSCH requirement introduced if no interference assumption across 2 panels.

Scope of PUCCH

Excepting for PUSCH, PUCCH is also enhanced with supporting both single-DCI based multi-TRP and multi-TCI based multi-TRP transmission in FR2.

The following transmission schemes for single DCI-based was agreed in RAN1

Agreement Support the SFN scheme for single-DCI based STxMP PUCCH transmission.

From the receiver processing perspective, the SFN scheme for single-DCI based STxMP PUCCH transmission can be verified by existing PUCCH requirement if no combination across TRPs required. If combination across TRPs required, the multi-slot PUCCH can be used to verify the performance of single-DCI based STxMP PUCCH. Given there is no related PUCCH repetition requirement in FR2, PUCCH requirement with SFN scheme could be considered if combination across TRP required. Therefore, whether to introduce PUCCH requirement with SFN transmission pending on the receiver assumption discussion.

Proposal 8: PUCCH requirement with SFN scheme could be considered if combination across TRPs required. No PUCCH requirement with SFN scheme if no combination across TRPs required. FFS on whether combination across TRP required.

# 3	Test setup of BS demodulation requirement

## 3.1	Test setup of PUSCH requirement for DMRS enhancement

In this section, the detail test setup of PUSCH requirement for DMRS enhancement is provided.

SCS&CBW

Regarding SCS &BW, since the DMRS enhancement is mainly targeting for FR1, to reduce the test effort, we would like to specify the minimum CBW per SCS, e.g., 15KHz SCS with 5MHz, and 30KHz SCS with 10MHz

Antenna configuration

Regarding the antenna configuration, to reduce the test effort, the following antenna configuration as 1Tx2Rx, 2Tx2Rx and 4Tx4Rx can be considered as starting point for specifying the PUSCH requirement with DMRS enhancement.

MCS

Regarding MCS, MCS 16 can be taken as a starting point

Channel Model

Regarding channel model, the baseline for delay spread is 30ns or 300ns in RAN1 evaluation of DMRS enhancement.  Based on existing test parameters for PUSCH requirement, TDLC 300-100 can be considered for specifying PUSCH requirement with DMRS enhancement

Mapping type

Regarding mapping type, type A is selected for DMRS enhancement evaluation. Therefore, DMRS mapping type A can be selected as a starting point for PUSCH requirement definition, which can meet test purpose of Rel-18 DMRS pattern

Number of DMRS symbol

Regarding the number of DMRS symbol, typical UE velocity is around 30km/h as RAN1 evaluation for DMRS enhancement. Therefore, 2 DMRS should be enough, it can be selected as a starting point for PUSCH requirement definition, which can meet test purpose of Rel-18 DMRS pattern.

DM-RS port

Since the test purpose is to verify functionality of Rel-18 DMRS pattern and FD-OCC length 4, DMRS port index with {0,1, 8,9} can be considered for 4Tx, {0,1} for 2Tx ad {0} for 1Tx, separately.

Waveform

Regarding the waveform, RAN4 has specified PUSCH requirement with both CP-OFDM and DFT-s-OFDM waveform in Rel-15. According to the WID, the DMRS enhancement is mainly targeting for CP-OFDM as following

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

Other

For other test parameters, exiting test parameters for specifying PUSCH requirement can be considered as following

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0,1, 8,9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

In summary, the following test parameters can be considered for PUSCH requirement with DMRS enhancement.

Proposal 9:  The following simulation assumption can be considered for PUSCH requirement with DMRS enhancement if agreed to introduced

- SCS&CBW

  - 15KHz SCS ,5MHz

  - 30KHz SCS, 10MHz

- Antenna configuration: 1Tx2Rx, 2Tx2Rx and 4Tx4Rx

- MCS:  MCS 16

- Channel Model: TDLC300-100

- Mapping type: Type A

- Number of DMRS symbol: 2

- DM-RS port:

  - {0} for 1Tx, {0, 1} for 2Tx and {0,1, 8, 9} for 4Tx

- Waveform: CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0,1, 8,9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

## 3.2	Test setup of PUSCH requirement for STxMP

In this section, the detail test setup of PUSCH requirement for STxMP is provided. For this section, we will focus on the test setup for PUSCH requirements with SDM firstly.

## 3.2.1	PUSCH requirements with SDM

Layer combination and DMRS port

Regarding the layer combination, the following is agreed in RAN1#110-e meeting.

Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least.

Agreement For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2: - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

Based on RAN1 agreement, up to {2+2} layer combination can be supported. The main purpose of SDM scheme is to improve UL throughput, and assuming no cross-link interference between 2 panels, we think {2+2} layer combination should be feasible for SDM scheme single-DCI based STxMP transmission.

Proposal 10: Specify PUSCH requirements with layer combination 2+2 for SDM scheme single-DCI based STxMP transmission

- DMRS port {0, 1} and DMRS port {2, 3} for each panel

PTRS

The targeting scenario of UL multi-panel transmission is FR2. For FR2, considering phase noise impact per TRP, it is beneficial to configure 2 PTRS port separately. The following is agreed in RAN1, up to 2 PTRS ports for SDM supported

Agreement Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, Ports 0 and 1 are associated with one DMRS port of two different panels respectively Study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme.

Proposal 11: Configure 2 PTRS ports, K_PTRS=2, L_PTRS=1 for specifying PUSCH requirement with SDM scheme

Number of DMRS

Since targeting scenario of UL multi-panel transmission is not for high-speed scenario, we think 2 DMRS should be enough.

Proposal 12: Configure 2 symbol of DMRS for specifying PUSCH requirement with SDM scheme

Timing and Frequency offset assumption

Regarding the timing offset and frequency offset, since 2TA for UL multi-DCI for multi-TRP operation is supported, each TRP will process the PUSCH signal transmitted per panel separately, we think there is no need to model timing and frequency offset. We are also open to further discussion whether it is needed.

Proposal 13: FFS on Timing and Frequency offset modeling for specify PUSCH requirement with SDM scheme

Channel Model

For channel model, TDLA30-300 was introduced for PUSCH requirement with 2Tx2Rx in current spec for FR2. Then, TDLA30-300 can be taken as a starting point for PUSCH requirement with SDM scheme.

Proposal 14: Introduce TDLA30-300 for PUSCH requirement with SDM scheme.

MCS

For current PUSCH requirement in FR2 with 2Tx2Rx, up to MCS16 was considered. For each TRP per panel, the processing is same with current PUSCH with 2Tx2Rx. Therefore, MCS 16 can be taken as a stating point

Proposal 15: Introduce MCS 16 for PUSCH requirement with SDM scheme

SCS&BW

Regarding the SCS and BW, both 60KHz and 120KHz SCS with different CBW are considered for performance definition. For PUSCH requirement with SDM scheme, we would like to select the typical configuration as 120KHz SCS with 100MHz.

Proposal 16: Introduce 120KHz SCS and 100MHz for PUSCH requirement with SDM scheme

Other test parameters

For other test parameters, the exiting test parameters for PUSCH requirements in FR2 can be reused as following

Proposal 17: The exiting test parameters for PUSCH requirements in FR2 can be reused as following

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1} for TRP1 {2,3} for TRP 2 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission per each TRP |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

# 4	Conclusion

In this contribution, the view on the test scope of BS demodulation requirement for FeMIMO was provided.

Observation 1: New FD-OCC length 4 will have impact on the baseband processing for channel estimation to derive the channel information per each DMRS port.

Observation 2: The legacy port indexes and new port indexed can be multiplexed within the same CDM group, up to 4 ranks, only one CDM group is used.

Proposal 1: PUSCH requirement with Rel-18 DMRS pattern to support less than 4 layers could be introduced

- Reuse the existing requirement for PUSCH with 1Tx and 2Tx, with DMRS port {0} and DMRS ports {0, 1}, separately

- New additional PUSCH requirement with 4Tx, with DMRS ports {0,1,8,9}

Proposal 2: No PUSCH requirement introduced to support more than 4 layers SU-MIMO with new enhanced DMRS patten in Rel-18.

Proposal 3: FFS on whether to model cross-link interference between 2 panels. Assuming no cross-link interference targeting CPE/FWA with UL multi-panel transmission as a starting point.

Proposal 4: Take separately processing as a baseline receiver assumption per TRP for Uplink multi-panel transmission

Proposal 5: PUSCH requirement with SDM scheme could be introduced

Proposal 6: Pending on whether combination across TRPs required, it is FFS on whether PUSCH requirement with SFN scheme could be introduced

Proposal 7: No PUSCH requirement introduced if no interference assumption across 2 panels.

Proposal 8: PUCCH requirement with SFN scheme could be considered if combination across TRPs required. No PUCCH requirement with SFN scheme if no combination across TRPs required. FFS on whether combination across TRP required.

Proposal 9:  The following simulation assumption can be considered for PUSCH requirement with DMRS enhancement if agreed to introduced

- SCS&CBW

  - 15KHz SCS ,5MHz

  - 30KHz SCS, 10MHz

- Antenna configuration: 1Tx2Rx, 2Tx2Rx and 4Tx4Rx

- MCS:  MCS 16

- Channel Model: TDLC300-100

- Mapping type: Type A

- Number of DMRS symbol: 2

- DM-RS port:

  - {0} for 1Tx, {0, 1} for 2Tx and {0,1, 8, 9} for 4Tx

- Waveform: CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0,1, 8,9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Proposal 10: Specify PUSCH requirements with layer combination 2+2 for SDM scheme single-DCI based STxMP transmission

- DMRS port {0, 1} and DMRS port {2, 3} for each panel

Proposal 11: Configure 2 PTRS ports, K_PTRS=2, L_PTRS=1 for specifying PUSCH requirement with SDM scheme

Proposal 12: Configure 2 symbol of DMRS for specifying PUSCH requirement with SDM scheme

Proposal 13: FFS on Timing and Frequency offset modeling for specifying PUSCH requirement with SDM scheme

Proposal 14: Introduce TDLA30-300 for PUSCH requirement with SDM scheme

Proposal 15: Introduce MCS 16 for PUSCH requirement with SDM scheme

Proposal 16: Introduce 120KHz SCS and 100MHz for PUSCH requirement with SDM scheme

Proposal 17: The exiting test parameters for PUSCH requirements in FR2 can be reused as following

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1} for TRP1 {2,3} for TRP 2 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission per each TRP |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

# References

RP-213598, WID on MIMO Evolution for Downlink and Uplink

R4-2315885, RAN4 performance requirements work scope for Rel-18 MIMO Evolution WI