# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316159

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.7.2.4

Source: 	OPPO

Title: 	On scheduling and measurement restriction for FR2_mulitRx_DL

Document for:	Approval

1	Introduction

A WF on Rel-18 FR2_multiRX_DL was approved in RAN4#108 meeting [1].

In this contribution, we further discuss the remaining issues related to scheduling and measurement restriction for FR2_mulitRx_DL.

2	Discussion

- Scheduling restriction

#### Issue 3-1-1: Update conditions/cases for scheduling restriction that can be relaxed for CSI-RS based L1 measurements for multi-Rx - Discussed with CR by taking agreements on Issue 2-2-1 into consideration. #### Issue 3-1-2: Cases whether scheduling restriction can be relaxed for L1 measurements for multi-Rx - Introduce scheduling restriction relaxation (enhancement) for CSI-RS based RLM, BFD which includes cell specific BFD and TRP specific BFD, and non-group-based and group-based L1-RSRP measurements with repetition OFF. Detailed conditions are FFS. - FFS whether scheduling restrictions are needed for UEs supporting simultaneous reception with mixed numerologies - No scheduling restriction relaxation (enhancement) for other cases. #### Issue 3-1-3: Conditions/cases that scheduling restriction for L3 measurements can be relaxed for multi-Rx - Scheduling restriction/measurement restriction relaxation (enhancement) for L3 measurements are not further discussed in Rel-18 multi-Rx WI.

The conditions of scheduling restriction requirements for cell specific and TRP specific RLM, BFD and CBD requirements should be aligned when drafting CRs.

Take BFD as example, the following scheduling restriction applies due to beam failure detection.

-	For the case where no RSs are provided for BFD, or when CSI-RS is configured for BFD is explicitly configured and is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON, or

-	For FR2-1 for PCell, for UE supporting [TBD - multi-rx capability] and activated with multi-Rx operation, when resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair, and CSI-RS for BFD has same QCL source as the active TCI state of one PDCCH or PDSCH, and has different QCL-TypeD from the other PDCCH, PDSCH, and the CSI-RS is configured in the same or overlapping OFDM symbols in the PCell, and the CSI-RS is not in a CSI-RS resource set with repetition ON,

-	There are no scheduling restrictions due to beam failure detection performed based on the CSI-RS.

No additional scheduling restrictions or measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies. And no scheduling restriction relaxation (enhancement) for other cases.

- Measurement restriction

#### Issue 3-2-1: Cases whether measurement restriction for SSB based L1 measurements can be relaxed for multi-Rx <Online agreement> - Introduce measurement restriction relaxation (enhancement) for CSI-RS based RLM, BFD which includes cell specific BFD and TRP specific BFD, and non-group-based and group-based L1-RSRP measurements with repetition OFF. Detailed conditions are FFS. - FFS whether measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies - No measurement restriction relaxation (enhancement) for other cases. #### Issue 3-2-2: Conditions that measurement restriction for CSI-RS based L1 measurements can be relaxed for multi-Rx - Reuse the same principle of scheduling restriction, if possible, and necessary revisions and additional conditions could be considered.

Similarly, no additional measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies. The conditions of measurement restriction requirements for cell specific and TRP specific RLM, BFD and CBD requirements should be aligned when drafting CRs.

Proposal 1: No additional scheduling restrictions or measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies. And no scheduling restriction relaxation (enhancement) for other cases.

Proposal 2: The conditions of scheduling and measurement restriction requirements for cell specific and TRP specific RLM, BFD and CBD requirements should be aligned when drafting CRs.

3	Conclusion

Proposal 1: No additional scheduling restrictions or measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies. And no scheduling restriction relaxation (enhancement) for other cases.

Proposal 2: The conditions of scheduling and measurement restriction requirements for cell specific and TRP specific RLM, BFD and CBD requirements should be aligned when drafting CRs.

4	Reference

[1] R4-2314477, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo