3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316183

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.25.2.2

Source: 	OPPO

Title: 	Discussion on collision between gap and priority rules

Document for:	Approval

1	Introduction

MUSIM gaps were discussed and introduced in Rel-17 and the corresponding requirements will be discussed in Rel-18. For MUSIM gap collision, some agreements as well as open issues were discussed and captured in [1]. We will discuss the following topics case by case, and provide our considerations.

- MUSIM gap priority configuration

- Collision between different MUSIM gaps

- Collision between MUSIM and legacy gaps

2	Discussion

2.1	MUSIM gap priority configuration

Issue 2-1-4-2: Constraints on MUSIM gap request from UE side Proposals - P1: There shall be a minimum MGRP defined for the requested MUSIM gap pattern (Nokia) - P2: When UE requests the MUSIM gaps, the MGRP of highest priority gap should be larger than 160ms; When UE requests only one MUSIM gap, the MGRP should be larger than 80ms (Ericsson ZTE) - P3: Do not define constraints on MUSIM gap request from UE side (Huawei Apple Qualcomm vivo oppo MTK) - P4: Network A will configure the MUSIM gap priority requested by the UE under the following conditions (Qualcomm) - If the UE requests multiple MUSIM gaps, the MUSIM gap that the UE requests with the highest priority has MGRP larger than 160 ms. - If the UE requests only one MUSIM gap, the MUSIM gap has MGRP larger than 80 ms.

It was proposed to introduce some constraints on MUSIM gap request from UE side, such as the MGRP should be larger than 80ms. We don’t think such constraints are useful since anyway the MUSIM configuration is controlled by NW-A. If an unreasonable MUSIM gap is requested, NW-A could reject it.

Proposal 1: Do not define constraints on MUSIM request from UE side.

Issue 2-1-7: Further considerations on MUSIM gap priority - Proposals: - P1: The priorities among all configured gaps shall be comparable, including MUSIM and non-MUSIM gaps (type-1 and type-2). (Nokia)

It is reasonable that the priorities for MUSIM gaps and non-MUSIM type-2 gaps shall be comparable so that priority-based dropping rule could apply. However, type-1 gap is not configured with priority and some solutions to handle the collision between type-1 gaps and MUSIM gaps are still under discussion. So, type-1 gap should be precluded.

Proposal 2: The priorities among MUSIM gap and non-MUSIM type-2 gaps shall be comparable.

2.2	Collision between different MUSIM gaps

Agreements: Introduce signalling to allow UE to request to use “keep solution” collision handling mechanism for requested aperiodic and periodic MUSIM gaps and network to grant UE the use of “keep solution”. The same request applies for all MUSIM gaps altogether (i.e. one bit indication). Signalling design is up to RAN2. Agreement: NW A sends feedback to UE to let UE know NW A’s decision on “keep solution” request - Feedback signalling is up to RAN2 design.

In additional to priority-based drop solution, keep solution is introduced and it could be enabled by UE request and NW indication. One leftover issue is UE behaviour when keep solution is requested by UE but rejected by NW-A. In our view, keep solution cannot apply and UE should fall back to priority-based drop solution.

Proposal 3: When keep solution is requested by UE but rejected by NW-A, UE should fall back to use priority-based drop solution.

2.3	Collision between MUSIM and legacy gaps

Issue 2-3-1 Clarifications on collision between Type-2 MG and MUSIM gaps - Proposals - P1: when number of colliding MGs is larger than 2, collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. Note: FFS when keep solution is used simultaneously (Apple China Telecom Qualcomm Ericsson vivo oppo Huawei MTK Charter Communications) - P1a: MUSIM gaps for which “keep” solution is indicated do not collide with each other (Qualcomm) - P2: when number of colliding MGs is larger than 2, RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority. (ZTE) - P3: When at most 2 gap collide at each time instance however there are consecutive collisions, the priority rule should be applied with a chronological order. (vivo) - P4: RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on overall MUSIM gap priority handling and ‘keep solution’. (Nokia) - P5: (MTK) When number of colliding gaps is more than two (e.g., a mix of MUSIM gaps and MGs), and - a) If priority-based solution is used to handle collision between different MUSIM gaps, then: - Handle gap collisions sequentially starting from the highest priority (i.e., regardless the type of gap involved in the collision) - Then only the non-dropped gaps are compared with the remaining gaps - b) If keep solution is used to handle collisions between different MUSIM gaps, then: - First, handle gap collisions which use priority-based solution - Then apply keep solution for the remaining collided MUSIM gaps

When more than 2 MGs are collided and priority rule is used, then it is better to resolve sequentially in order of decreasing priority, starting with the gap with highest priority so that this gap can always be kept.

Proposal 4a: When number of colliding MGs is larger than 2 and priority rule is used, collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority.

When keep rule is used for MUSIM gaps, it was agreed to keep all colliding MUSIM gaps irrespective of the priority configuration. On top of that, when MUSIM gaps are collided with legacy NW-A gap, how to handle the collision is not clear. P5 suggests to use priority rule first and then keep the remaining MUSIM gaps. We don’t think it is a good way. The main motivation of keep solution is to keep MUSIM gaps for paging reception and the SSB occasions before PO for AGC. Generally, MUSIM gap #1 for paging reception will be configured with higher priority while MUSIM gap #2 for SSB occasion will have lower priority. If the priority for legacy NW-A gap #3 is higher than MUSIM gap #2 but lower than MUSIM gap #1, then UE can only receive paging without AGC. The benefits of keep solution disappear. We think the MUSIM gaps should be binded together when keep solution applies, all the MUSIM gaps should be dropped or kept if colliding with legacy NW-A gap. As a compromise, the proposal suggested by moderator in email discussion can be supported by us, that is

Proposal 4b: When number of colliding MGs is larger than 2 and keep rule is used for MUSIM gaps:

- If the highest priority of collided MUSIM gap is higher than Type-2 gap, all the MUSIM gaps should be kept and legacy gap should be dropped.

- Otherwise, all the MUSIM gaps should be dropped and legacy gap should be kept.

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or any configured gap without priority - Proposals - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (Apple xiaomi vivo oppo) - P2: MUSIM gaps are assumed to have higher priority than a Type-1 MG. (Qualcomm vivo) - P3: Collision is be handled based on the MGRP of the collided gaps (Ericsson ZTE vivo Huawei MTK) - P3-1: RAN4 to prioritize the gap with longer MGRP when: 1. Any of the collision gaps is Type-1 MG; 2. NW-A doesn’t configure a priority associated with any of the collision gaps. (Huawei Ericsson vivo MTK) - P3-2: No requirements apply if the two gaps have same MGRP. (vivo Huwei) - P3-3: If the MGRPs of the collided MUSIM gap and Type-1 MG are the same, then prioritize MUSIM gap only if it is configured with the highest priority level; otherwise prioritize Type-1 MG (MTK) - P4: Introduce priority for Type-1 MG when MUSIM gaps are configured when also having Type-1 measurement gaps allocated (vivo Nokia) FFS: For collision between MUSIM gap and Type-1 MG, collision is be handled based on the MGRP of the collided gaps

< Agreement from online session >: - Type-1 MG can be configured together with Pre-MG/NCSG - There are no requirements when type 1 MG is colliding with pre-MG or NCSG in the same FR.

Priority solution is reused to solve collisions between MUSIM gap and Type -2 MG. The collision between MUSIM gap and Type-1 MG or gap configured without priority will be further discussed in this meeting. In our view, such the configuration is incorrect, and can be avoided by NW-A implementation. For example, when receiving MUSIM gap request colliding with legacy MG for NW-A, NW-A could either reject the MUSIM gap configuration or reconfigure MG with priority for NW-A. Rel-18 MG enhancement WI also discussed Type-1 MG and agree to leave no requirements in this case. Similarly, no requirement will be defined when MUSIM gaps collide with legacy gap without priority.

Proposal 5: When a MUSIM gap collides with a legacy gap without priority, requirements shall not apply.

3	Conclusion

This contribution gave our general views on how to handle MUSIM gap collision issue and the following proposals:

Proposal 1: Do not define constraints on MUSIM request from UE side.

Proposal 2: The priorities among MUSIM gap and non-MUSIM type-2 gaps shall be comparable.

Proposal 3: When keep solution is requested by UE but rejected by NW-A, UE should fall back to use priority-based drop solution.

Proposal 4a: When number of colliding MGs is larger than 2 and priority rule is used, collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority.

Proposal 4b: When number of colliding MGs is larger than 2 and keep rule is used for MUSIM gaps:

- If the highest priority of collided MUSIM gap is higher than Type-2 gap, all the MUSIM gaps should be kept and legacy gap should be dropped.

- Otherwise, all the MUSIM gaps should be dropped and legacy gap should be kept.

Proposal 5: When a MUSIM gap collides with a legacy gap without priority, requirements shall not apply.

4	Reference

- R4-2314364, WF on R18 MUSIM, vivo