3GPP TSG-RAN WG4 Meeting # 108bis                             R4-2316201

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on ATG UE minimum output power requirements

Agenda Item:	5.13.2.2

Document for:	Approval

Introduction

In the RAN4#108 meetings, one way forward [1] were approved with the following assumptions to further discuss the minimum output power requirements.

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Frequency | 2GHz | 4GHz |
| Distance | TBD | TBD |
| PL | TBD | TBD |
| ATG BS antenna gain | TBD | TBD |
| ATG UE antenna gain | 0dB | TBD |
| Thermal noise power density | -114dBm/MHz | -114dBm/MHz |
| ATG BS Noise figure | 5dB | 5dB |
| Minimum output power | -13dBm/MHz TBD | -17dBm/MHz TBD |

In this paper, we’d like to share our views on this issue.

Discussion on Minimum output power

Referring to the contribution [2], the following assumptions were proposed to further discuss the minimum output power requirements.

- ATG BS antenna gain: Referring to Table 6.2.3.1-1 of TR 38.876, we have the following assumption for ATG BS antenna model. ATG. Base Station Antenna Characteristics. Antenna pattern; TR 38.921. Element gain (dBi) (Note 2); 7.1. Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 54º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V. Antenna polarization; Linear ±45º. Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements. Number of supported polarizations, P; 2. Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.9 of wavelength for V. Array Ohmic loss (dB) (Note 2); 2. Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); 25. Base station maximum coverage angle in the horizontal plane (degrees); 120. Base station vertical coverage range (degrees) (Note 1); 25. Mechanical uptilt (degrees); 14. Based on that, we can assume BS antenna gain = 10*log10(8*8) + 7.1 = 25.2dB - ATG BS maximum output power: (For ATG UE maximum input level evaluation) 53dBm sum of two polarizations for 4GHz 46dBm sum of two polarizations for 2GHz - ATG UE antenna gain Referring to Table 6.2.3.2-1 of TR 38.876, we have the following assumption for ATG UE antenna model for 4GHz Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 90º for V. Element gain (dBi); 5 dBi. Horizontal/vertical front-to-back ratio (dB); 30dBc. Antenna polarization; Linear ±90º. Antenna array configuration (Row × Column x Polarization) (Note 4); (8x2x2) or (16x1x2). Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.5 of wavelength for V. UE antenna orientation; Single UE panel deployed on the abdomen of the airplane facing downwards and with the longest dimension of the array aligned with the direction of the flight route. The flight route is pointed at the BS.. 4GHz ATG UE phased array antenna gain = 10*log10(16) + 5 = 17dB 2GHz ATG UE Omni-direction antenna gain = 0dB - Pathloss model: Free space Pathloss ( fc_GHz, MinimumDistance_m ) = 32.5 +20log10(fc_GHz)+20log10(MinimumDistance_m) - The assumption of minimum distance between ATG BS and ATG UE is 3km ~ 20km. - Minimum output power for ATG UE + antenna gain for ATG UE + antenna gain for BS – pathloss(MinDistance, Fc) >= RESENS of BS

Referring to the template in the WF [1], we can get the following calculations and results.

Table 1 the results for minimum output power requirements

| Parameters | case 1 | case 2 | case 3 | case 4 | case 5 | case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| center frequency GHz | 2 | 2 | 2 | 4 | 4 | 4 |
| Distance meter | 3000 | 3000 | 20000 | 3000 | 3000 | 20000 |
| Pathloss dB | 108.063 | 108.063 | 124.5412 | 114.0836 | 114.0836 | 130.5618 |
| ATG BS antenna gain | 25 | 7 | 25 | 25 | 7 | 25 |
| ATG UE antenna gain | 0 | 0 | 0 | 17 | 5 | 17 |
| Thermal noise power density dBm/MHz | -114 | -114 | -114 | -114 | -114 | -114 |
| ATG BS Noise figure dB | 5 | 5 | 5 | 5 | 5 | 5 |
| Minimum output power dBm/MHz | -25.937 | -7.93697 | -9.4588 | -36.9164 | -6.91638 | -20.4382 |

Case 1 and case 4 can be considered as the worst cases. The minimum distance between ATG BS and ATG UE is 3000m and the maximum antenna gains for BS and UE are assumed. During the discussion in last meeting, some companies think that it’s impossible to assume the maximum antenna gain when the aircraft is over the ATG BS. Thus, the element gains for ATG BS and UE are assumed for case 2 and case 5.

20000 minimum distance is assumed for case 3 and case 6, so the minimum output power at dBm/MHz is -20.43 dBm/MHz. For 5MHz channel bandwidth, the minimum output power is -20+10*log10(5) = -13dBm.

Proposal 1: to use -20dBm/MHz to derive the minimum output power, so the minimum output power is -20+10*log10(5) = -13dBm for 5MHz channel bandwidth.

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -13 | -13+10log10 (BWChannel /20) | -13+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: to use -20dBm/MHz to derive the minimum output power, so the minimum output power is -20+10*log10(5) = -13dBm for 5MHz channel bandwidth.

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -13 | -13+10log10 (BWChannel /20) | -13+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

- References

[1] R4-2314761, WF on NR_ATG_UERF_part2, Huawei, HiSilicon

[2] R4-2312962, Discussion on ATG UE Tx requirements, Huawei, HiSilicon