3GPP TSG-RAN WG4 Meeting # 108bis                             R4-2316202

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to introduce minimum output power requirements for ATG UE

Agenda Item:	5.13.2.2

Document for:	Approval

Introduction

It’s proposed to capture the following technical analysis for minimum output power requirements.

- References

[1]

[2]

- Text proposal

- <<Start of Change for TR 38.876>>

- 7.1.2.5 Minimum output power

The following assumptions can be used to derive the minimum output power requriements.

1) ATG BS antenna model can be assumed referring to Table 6.2.3.1-1. It’s highlighted that the element gain is 7.1dB and maximum antenna gain for ATG BS is 10*log10(8*8) + 7.1 = 25.2dB.

2) ATG UE antenna model can be assumed referring to Table 6.2.3.2-1 for 4GHz. It’s highlighted for 4GHz that the element gain is 5dB and maximum antenna gain for ATG BS is 10*log10(16) + 5 = 17dB. Since Omni-direction antenna is assumed for 2GHz ATG UE, the antenna gain is 0dB.

3) Pathloss model: Free space. Pathloss ( fc_GHz, MinimumDistance_m ) = 32.5 + 20log10(fc_GHz) + 20log10(MinimumDistance_m)

4) The assumption of the distance between ATG BS and ATG UE is [20km].

5) Minimum output power for ATG UE + antenna gain for ATG UE + antenna gain for BS – pathloss(MinDistance, Fc) >= RESENS of BS

The calculation and results for minimum output power are listed below.

Table 7.1.2.5-1 the calculation for minimum output power requirements

| Parameters | case 3 | case 6 |
| --- | --- | --- |
| center frequency GHz | 2 | 4 |
| Distance meter | 20000 | 20000 |
| Pathloss dB | 124.5412 | 130.5618 |
| ATG BS antenna gain | 25 | 25 |
| ATG UE antenna gain | 0 | 17 |
| Thermal noise power density dBm/MHz | -114 | -114 |
| ATG BS Noise figure dB | 5 | 5 |
| Minimum output power dBm/MHz | -9.4588 | -20.4382 |

-20dBm/MHz can be used to derive the minimum output power, so the minimum output power is -20+10*log10(5) = -13dBm for 5MHz channel bandwidth. The minimum output power requirements are specified below.

Table 7.1.2.5-2 the calculation for minimum output power requirements

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -13 | -13+10log10 (BWChannel /20) | -13+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

- <<End of Change>>