3GPP TSG-RAN WG4 Meeting #108bis	R4-2316220

Xiamen, China, October 09 – October 13, 2023

Title:	TP for UE coordinate system

Source:	OPPO

Agenda item:	5.2.1

Document for:	Approval

- Introduction

The TP of UE coordinate system including assistant coordination system was approved in the last RAN4 #108 meeting. In this paper, we provide test proposal to TR 38.871 to further refine the Annex of UE coordinate system. Based on the agreement of UE RF requirement, three candidate UE orientations can be declared by UE venders. The UE orientations, which are called as alignment options in Annex C of TR 38.810, are specified in this TP.

- Conclusions

Proposal: Adopt the TP for TR 38.871 to further refine the Annex of UE coordinate system.

- References

Text Proposal

TP for TR 38.871

Annex B:
UE coordinate system

B.1	Reference coordinate system

This annex defines the measurement coordinate system for the NR UE.  The reference coordinate system is provided in Figure B.1-1 below while Figure B.1-2 shows the DUT in the default alignment, i.e., the DUT and the reference coordinate systems are aligned with α = 0o and β = 0o and γ = 0o where α, β, and γ describe the relative angles between the two coordinate systems.

Figure B.1-1: Reference coordinate system

Figure B.1-2: DUT default alignment to coordinate system

The following aspects are necessary:

-	A basic understanding of the top and bottom of the device is needed in order to define unambiguous DUT positioning requirements for the test, e.g., in the drawings used in this annex, the three buttons are on the bottom of the device (front) and the camera is on the top of the device (back).

-	An understanding of the origin and alignment the coordinate system inside the test system, i.e. the directions in which the x, y, z axes point inside the test chamber, is needed in order to define unambiguous DUT orientation, DUT beam, signal, interference, and measurement angles.

B.2 Alternate coordinate system for 2AoA testing

Considering +AoA and -AoA are always on the meridian, the Reference coordinate system is used for Rel-18 multi-Rx testing. The Alternate coordinate system could be considered for the cases when the AoA pairs do not lie along the same meridian.

The performance of UEs with multi-Rx chain reception DL with 2 active AoA is agreed to be measured with 3D scan on the spherical surface using constant step size grids. Therefore, the test point of AoA1 on the spherical surface can be expressed as θ, ϕ in the same way it is defined for legacy RF testing with 1 AoA. In the case of the AoA2, the test points require certain angle separation with respect to AoA1. However, the direction of AoA2, which is also called as “AoA1-AoA2 DL Orientation Vector” in clause 5.3.5, may introduce ambiguity without additional instructions. The situation is illustrated in Figure B.2-1.

Figure B.2-1 Illustration of possible AoA pairs on the test point of AoA1

In order to simplify the representation of the AoA pairs per test points of AoA1, the alternate coordinate system nomenclature described in this section can be used to express the AoA pairs with one coordinate, and it could be used also for the situation whereAoA2 does not lie in the same meridian as of AoA1.

In this alternate coordinate system nomenclature for 2AoA testing. the Reference coordinates of AoA1 (θ, ϕ) could be extended to θ, ϕ,ω (θ, ϕ, ω), where ω corresponds to the AoA1-AoA2 DL Orientation Vector. In detail, ω is the angle between the AoA1-AoA2 DL Orientation Vector and the reference axis in the assistant plane.

The assistant plane is orthogonal to the AoA1 direction, and the Z’ axis correspond to the projection of the meridian (points with same fixed ϕ angle) of AoA1 into the assistant plane. I.e. in case of θ<90º, +Z’ corresponds to the cross point of +Z axis and the assistant plane (see Figure B.2-2a). In case of θ>90º, -Z’ corresponds to the cross point of -Z axis and the assistant plane (see Figure B.2-2b).

Figure B.2-2 Illustration of Assistant Plane of the alternate coordinate system

Finally, ω is the angle from the reference axis Z’ to the AoA1-AoA2 DL Orientation Vector in clockwise direction (see Figure B.2-3).

Figure B.2-3 Illustration of angle ω in the alternate coordinate system

This alternate coordinate system can also cover the theoretical cases when the AoA pairs do not lie along the same meridian, and eliminate the possible ambiguity on test point expression.

B.3 Test conditions and angle definitions

Tables B.2-1 through B.2-3 below provide the test conditions and angle definitions for three permitted device alignment for the default test condition, DUT orientation 1, and two different options for each permitted device alignment to re-position the device for DUT Orientation 2 as outlined in Figures B.2-1 and B.2-3.

Table B.2-1: Test conditions and angle definitions for Alignment Option 1

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 0º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 1 (based on re-positioning approach) | α = 180º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 3 – Option 1 (based on re-positioning approach) | α = 0º; β = 180º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in B.1-1, is maintained for each signal angle, link or interferer angle, and measurement angle. NOTE 2: The combination of rotations is captured by matrix M=Rz()•Ry()•Rx() |  |  |  |  |

Table B.2-2: Test conditions and angle definitions for Alignment Option 2

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 0º; β = -90º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 2 (based on re-positioning approach) | α = 180º; β = 90º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 3 – Option 2 (based on re-positioning approach) | α = 0º; β = 90º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in B.1-1, is maintained for each signal angle, link or interferer angle, and measurement angle. NOTE 2: The combination of rotations is captured by matrix M=Rz()•Ry()•Rx() |  |  |  |  |

Table C.2-3: Test conditions and angle definitions for Alignment Option 3

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 90º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 3 (based on re-positioning approach) | α = -90º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 3 – Option 3 (based on re-positioning approach) | α = 90º; β = 180º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in B.1-1, is maintained for each signal angle, link or interferer angle, and measurement angle. NOTE 2: The combination of rotations is captured by matrix M=Rz()•Ry()•Rx() |  |  |  |  |

For each UE requirement and test case, each of the parameters in Tables B.2-1 through B.2-3 need to be recorded, such that DUT positioning, DUT beam direction, and angles of the signal, link/interferer, and measurement are specified in terms of the fixed coordinate system.

Due to the non-commutative nature of rotations, the order of rotations is important and needs to be defined when multiple DUT orientations are tested.

The rotations around the x, y, and z axes can be defined with the following rotation matrices

and

.

with the respective angles of rotation, , ,  and

Additionally, any translation of the DUT can be defined with the translation matrix

with offsets tx, ty, tz in x, y, and z, respectively and with

The combination of rotations and translation is captured by the multiplication of rotation and translation matrices.

For instance, the matrix M

describes an initial rotation of the DUT around the x axis with angle α, a subsequent rotation around the y axis with angle β, and a final rotation around the z axis with angle γ. After those rotations, the DUT is translated by tx, ty, tz in x, y, and z, respectively.